3GPP TSG-RAN2 Meeting #108	R2-1915916

Reno, Nevada, USA, 18 - 22 November 2019

Agenda Item:	[6.7.3.1] IIoT Handling of Deprioritized transmissions

Source: 	Apple Inc

Title:  	Deprioritized PDU Retransmission Schemes

Document for:	Discussion and Decision

# Introduction

In RAN2#106 meeting, the following agreements were achieved with regards to the handling of the deprioritized transmission [1].

Agreements

1	For de-prioritized PUSCH on dynamic grant, the UE should store the de-prioritized MAC PDU in the HARQ buffer, to allow gNB to schedule re-transmission using the same HARQ process.

2	For de-prioritized PUSCH on configured grants, a) the UE could store the de-prioritized MAC PDU in the HARQ buffer, to allow gNB to schedule re-transmission. b) FFS if the UE could transmit it using the subsequent radio resources e.g. associated with the same HARQ process

3	The above agreements are at least applicable for cases when MAC has already generated the de-prioritized MAC PDU

In RAN2#107bis meeting, following discussion points were noted with regards to the handling of deprioritized transmissions [2].

Discussion Points

1	In the proposed solution UE does not indicate explicitly to the network that there is data for a deprioritized PDU

2     There is support to have “UE based autonomous retransmissions in a CG resource”

# Motivation

In IIoT, Intra UE prioritisation mechanism in the UE would result in the UE prioritizing a scheduled PDU with another high priority PDU over a CG occasion. Such a pre-emption would be known to the NW when it receives the prioritized PDU, but the NW would not be able to reliably conclude if there was any deprioritized PDU still pending in the UE side especially if the NW has configured UL skipping for the UE.

# Discussion

## NR-U CG Retransmission

In NR-U, if due to LBT conflict, PUSCH data is dropped, it may be transmitted at the next CG opportunity with the same HARQ Process ID.  For NR-IIoT, the same method used for NR-U could be considered to handle the deprioritized PUSCH PDU, thereby reducing the specification effort. Additionally, this would necessitate such NR-IIoT capable devices to support NR-U as well.

Proposal 1: For NR-IIoT devices that support NR-U, it is possible to use the NR-CG resource to schedule the deprioritized PUSCH MAC PDU transmission. The deprioritized transmission on the NR-CG resource shall use the same HARQ Process ID as the original scheduled transmission.

Certain NR-IIoT devices may not be capable of supporting NR-U. In such cases, the above proposal 1 will preclude transmission of deprioritized PDUs. Three possible scenarios could be considered.

- A Deprioritized PDU was available in the UE’s HARQ buffer and it would be scheduled in the next available CG UL resource.

- No Deprioritized PDU was available in the UE’s HARQ buffer and the NW is unaware of this situation. NW would schedule a CG UL resource (for retransmission of the deprioritized PDU), and the UE would send a padding PDU only for this CG resource. The receipt of this padding PDU would indicate to the NW that there was no deprioritized PDU available in the UE buffer in the first place, based on which NW can toggle the retransmission status of the further CG resources and set it to new transmission.

- There was no deprioritized PDU as in scenario 2 above, but there was new UL PDU which arrived after the previous CG resource, that needs to be scheduled. The NW is unaware of this situation and it would schedule a CG UL resource (for retransmission of the deprioritized PDU). There are 2 potential way to handle the current CG UL resource.

  - The UE should send a padding PDU for this CG resource. The receipt of this padding PDU would indicate to the NW that it has to toggle the retransmission status of the further CG resource and set it to new transmission. In the subsequent CG, the new UL PDU would be handled as in legacy transmission scheme.

  - The UE should use the current CG UL resource to send the new UL PDU.

As noted above, scenario 1 is handled using legacy retransmission mechanisms, but scenario 2 and 3a can be handled by sending a padding PDU only on the next CG resource to get the UE and NW in sync about the status of any deprioritized PDUs.

Proposal 2: For an NR-IIoT device that does not support NR-U and that had transmitted higher priority PDU using NR-CG in earlier transmission, UE can schedule a padding PDU when:

- there exists no deprioritized PDU awaiting retransmission or

- a new UL PDU has arrived from higher layer and has to be scheduled,

This enables the UE to explicitly indicate to the network that it has is no pending PDU awaiting a retransmission, enabling network to resume transmission of new UL PDUs. Transmission of newly arrived PDU follows the legacy transmission mechanism in the subsequent CGs.

Proposal 3: Alternatively, if new UL PDU has arrived after transmission of higher priority PDU, then UE transmit that PDU when retransmission CG resource is received from the network.

# Summary

Both the proposals presented here are in line with the discussion points mentioned in [2]

Proposal 1: For NR-IIoT devices that support NR-U, it is possible to use the NR-CG resource to schedule the Deprioritized PUSCH MAC PDU transmission. The deprioritized transmission on the NR-CG resource shall use the same HARQ Process ID as the original scheduled transmission.

Proposal 2: For an NR-IIoT device that does not support NR-U and that had transmitted higher priority PDU using NR-CG in earlier transmission, UE can schedule a padding PDU when:

- there exists no deprioritized PDU awaiting retransmission or

- a new UL PDU has arrived from higher layer and has to be scheduled,

Proposal 3: Alternatively, if new UL PDU has arrived after transmission of higher priority PDU, then UE transmit that PDU when retransmission CG resource is received from the network.

# References

- RAN2#106 chairman’s notes

- RAN2#107bis chairman’s notes