3GPP TSG-RAN WG2 Meeting #108	R2-1916269

# Reno, Nevada, 18th Nov 2019 – 22nd Nov 2019		      (revision of R2-1915005)

Agenda item:	6.13.2

Source:	Perspecta Labs, CISA ECD, AT&T, T-Mobile, Verizon, Ericsson, Qualcomm

Title:	RA Prioritization for MPS for 2-step RACH

WID/SID:	NR_2step_RACH-Core – Release 16

Document for:	Discussion and Decision

# Introduction

In RAN2#107-bis it was agreed to support random access (RA) prioritization for the 2-step RACH procedure for at least handover and beam failure recovery. An in-principle agreement was also reached to support RA prioritization for the MPS service based on its Access Identity 1 for the 4-step RACH procedure.

Therefore, it is reasonable to propose that the MPS RA prioritization mechanism also applies to the 2-step RACH mechanism.

# Discussion

In RAN2#107-bis, as an outcome of the discussion of R2-1913402, it was agreed to support RA prioritization for the 2-step contention-based RACH (CBRA) procedure for at least handover and beam failure recovery:

Agreements

- For 2 step CBRA, RA prioritisation is supported at least for handover and beam failure recovery.

- RA prioritisation for 2 step CBRA is controlled by networkRA prioritisation parameters are separately configured for 2 step CBRA and 4 step CBRA. -ra-Prioritization2Step is optionally added to BeamFailureRecoveryConfig IE and RACH-ConfigDedicated IE.

At the same time, in RAN2#107-bis, RA prioritization for the specific Access Identity 1 that corresponds to MPS service was agreed in principle for the 4-step RACH procedure in R2-1912921 and R2-1912922.

It is therefore timely to consider and propose that the same RA prioritization in the 4-step RACH is applied to the 2-step RACH procedure when triggered by the Access Identities 1 corresponding to MPS. Therefore, MPS service, when indicated independently by the corresponding field bitmap ra-PrioritizationForAI in SIB1, [1], can be RA prioritized for either the 4-step or 2-step RACH procedure.

Proposal 1. RA prioritization for MPS should be applicable for both 2-step and 4-step RACH procedures.

The prioritization parameters powerRampingStepHighPriority and scalingFactorBI for MPS are obtained from the specialized SIB1 ra-PrioritizationForAccessIdentity field. These prioritization parameters should be applicable for both 2-step and 4-step RACH per 3GPP TS 38.321.

Proposal 2. RA prioritization parameters for MPS for both 2-step and 4-step RACH procedures are obtained from the SIB1 ra-PrioritizationForAccessIdentity field as defined in TS 38.321 Subclause 5.1.1.

# Conclusion

It is proposed that RAN2 discusses and adopts the following proposals:

Proposal 1. RA prioritization for MPS should be applicable for both 2-step and 4-step RACH procedures.

Proposal 2. RA prioritization parameters for MPS for both 2-step and 4-step RACH procedures are obtained from the SIB1 ra-PrioritizationForAccessIdentity field as defined in TS 38.321 Subclause 5.1.1.

# References

[1] R2-1915006_CR1361_38331_Rel16_PRACHPrioParams_MPS_MCS, Perspecta Labs, ECD, AT&T, FirstNet, T-Mobile, Sprint, Verizon, Qualcomm, Ericsson

[2] R2-1915007_CR0675_38321_Rel16_PRACHPrioProcedure_MPS_MCS, Perspecta Labs, ECD, AT&T, FirstNet, T-Mobile, Sprint, Verizon, Qualcomm, Ericsson