3GPP TSG-RAN2 Meeting #108	R2- 19xxxxx

Reno, USA, 18th - 22th Nov 2019

Title:	LS on clarification of CLI resource configuration

Response to:

Release:	Rel-16

Work Item:	NR_CLI_RIM

Source:	RAN2

To:	RAN1

Cc:

Contact Person:

Name:	Sangwon Kim

E-mail Address:	sangwon7.kim@lge.com

Attachments:

1. Overall Description:

The existing SRS resource configuration is associated with the BWP configuration. RAN2 understanding is that the information in the BWP configuration is used to indicate the frequency position of the SRS resource. However, for CLI measurements, the SRS RSRP resource is configured without any association with the BWP configuration, and freqDomainPosition and freqDomainShift are all information to indicate the frequency position of the SRS RSR resource. Therefore RAN2 thinks additional information needs to be configured to indicate the frequency position of the SRS RSRP resource and would like to ask RAN1 whether RAN2`s understanding is right and what information is additionally required.

Question1: Is additional information needed to indicate the frequency position of the SRS RSRP resource other than freqDomainPosition and freqDomainShift? What information is additionally needed?

RAN2 also have discussed the intent of the description for nrofSymbols, which is based on the LS from RAN1 (R1-1903835), but RAN2 failed to reach a consensus on whether the meaning of the following sentences in the description is clear.

- If the SCS of configured active DL BWP(s) is larger than the reference SCS, network configures startPosition and nrofSymbols such that the configured CLI-RSSI resource not to exceed the slot boundary corresponding to the active BWP SCS. If the reference SCS is larger than SCS of active DL BWP(s), network ensures startPosition and nrofSymbols are integer multiple of reference SCS divided by active BWP SCS.

Therefore, RAN2 would like to ask RAN1 whether above sentences is clear from RAN1 perspective or needs to be modified for clarification.

Question2: Are above statements suitable to describe the nrofSymbols?

2. Actions:

To RAN1

RAN2 kindly requests RAN1 to give responses to RAN2 with regard to above questions.

3. Date of Next RAN2 Meetings:

TSG-RAN2 Meeting #109	24 - 28 February 2020	Athens, Greece

TSG-RAN2 Meeting #109bis	20 - 24 April 2020	Japan