3GPP TSG-RAN WG2 Meeting #109e	R2-2000678

Elbonia, 24 Feb – 6 Mar 2020

Agenda item:	6.10.4.2

Source:	Nokia, Nokia Shanghai Bell

Title:	BFD on dormant SCell

WID/SID:	LTE_NR_DC_CA_enh-Core - Release 16

Document for:	Discussion and Decision

# 1	Introduction

In the dormant SCell operation open items discussion it was raised whether beam failure reporting (BFR) should be supported for dormant SCell. In order to support BFR one needs to first detect the problem i.e. beam failure detection needs to be supported. As most companies think that beam failure reporting needs to be supported we consider in this paper in more details how the actual detection is working in release 15 and how to do it in for dormant SCells.

# 2	Discussion

In the open item email discussion rapporteurs summary it is proposed:

To support beam management in dormancy SCell:

The tci-StatesToAddModList in pdsch-Config IE can be configured for the dormant BWP.

We agree with this approach as it is similar to what we have already in release 15 – so there is nothing new here and basically ASN.1 already supports this. Nevertheless, problem with this approach is that it seems to imply that NW needs to always explicitly configure used RS resources. Although release 15 specify the use of explicit configuration of BFD-RS it is not beneficial to use it as the RS would need to be updated using RRC signalling frequently. In release 15 this was solved by having support for PDCCH based configuration as well i.e. from 38.331:

If no RSs are provided for the purpose of beam failure detection, the UE performs beam monitoring based on the activated TCI-State for PDCCH as described in TS 38.213 [13], clause 6. If no RSs are provided in this list for the purpose of RLF detection, the UE performs Cell-RLM based on the activated TCI-State of PDCCH as described in TS 38.213 [13], clause 5.

Thus, we think it is critical to support also similar functionality for beam failure detection for dormant SCells as otherwise signalling burden becomes unbearable:

Proposal 1: It should be possible to configure BFD-RS for dormant SCell in both implicit and explicit manner like in release 15

# 3	Conclusion

Proposal 1: It should be possible to configure BFD-RS for dormant SCell in both implicit and explicit manner like in release 15