3GPP TSG RAN WG2 Meeting# 109 electronic   	                            R2-2000834

Elbonia 24th Feb- 6th March 2020

Agenda Item:	6.7.2.3

Source: 	Sony

Title:	EHC absence of Q-Tags and NACK feedback

Document for:	Discussion

# Introduction

Figure below is proposed in the email discussion summary as the reference design of EHC:

In this contribution we discuss the need for signalling the absence /compression status of Q-Tags in EHC header and the need for NACK feedback signalling.

# Discussion

Q-Tags

RAN2 agreed that Q-Tags are static and no dynamic indication in EHC is required. However, presence/absence of Q-Tags should still be discussed even if Q-Tags are static so that the de-compressor is aware if Q-Tags were actually absent or compressed. The distinction is necessary so that the decompressor could decompress Q-tags, if compressed, and do nothing, if absent.

One approach is that a separate context is created each time when e.g. PCP or DEI field values change including the case where Q-Tags are no longer present and were present initially. Creation of separate context each time the Q-tags change is related to the size of CID. Alternatively, same flow could be used at least for when Q-Tags are not present or compressed by indicating the absence of Q-Tags and compression of Q-Tags. Any change in PCP or DEI being handled by a new context seems like a straightforward approach.

Proposal 1: RAN2 to discuss on how to indicate the absence and the compression of Q-Tags based on either the indication inside the CID field or outside the CID field with one-bit indication. Any change in Q-Tags e.g. PCP or DEI is handled by a new context.

NACK feedback

When receiving a compressed packet with an unknown CID (e.g. due to context corruption in decompressor), three solutions were proposed during the email discussion:

- Discard the packet

- Inform the compressor with a NACK

- Perform RLF

In our opinion, it is important for the compressor to know about the failure so that it can switch back to full header. Performing RLF and triggering re-establishment is an overkill for the problem and result in service interruption. Discarding the packet locally does not allow the compressor to switch back to full header.

Proposal 2: RAN2 to agree that any compressed packet with unknown CID is informed to the compressor so that the compressor could switch to full header transmission.

# Conclusion

We propose RAN2 to discuss and agree on following proposals:

Proposal 1: RAN2 to discuss on how to indicate the absence and the compression of Q-Tags based on either the indication inside the CID field or outside the CID field with one-bit indication. Any change in Q-Tags e.g. PCP or DEI is handled by a new context.

Proposal 2: RAN2 to agree that any compressed packet with unknown CID is informed to the compressor so that the compressor could switch to full header transmission.

# References

[1]	RP-191563: New WID: Private Network Support for NG-RAN, China Telecom, Nokia, Vodafone