3GPP TSG-RAN WG2 Meeting #109bis electronic	draftR2-2003807

20 – 30 April 2020

Agenda Item:	8.7

Source: 		Session Chair (Huawei)

Title: 			draft Report NB-IoT breakout session

Document for:	Approval

General

Recording of voice or video at meetings is not used in 3GPP. This applies also to this e-Meeting. At this e-Meeting, no specific actions are taken to prevent the recording of web conferences. Companies that have concerns related to recordings, if any, may express those by email in the main meeting organizational thread [AT109bis-e][000]

Please see the following Tdocs for e-meeting guidance:

R2-2002500	Agenda for RAN2#109bis-e	Chairman	agenda

R2-2003824	RAN2 109bis-e Methods and Guidance	RAN2 chairman, RAN2 vice chairmen, session chairs		discussion

Time Schedule 
Please refer to the latest schedule in the RAN2 inbox on the public 3GPP servers.

Access Tools

HTTP Upload Tool:

ETSI IT has created a facility in Inbox and Inbox/Drafts folders on the public 3GPP servers to allow delegates to upload their documents using a web browser (however Internet Explorer is not yet supported). Open your browser and navigate to your chosen folder – for example,

https://www.3gpp.org/ftp/tsg_ran/WG2_RL2/TSGR2_109bis-e/Inbox

Click the green button to log in using your EOL account. A panel will appear in the upper part of the screen and documents may be dragged and dropped onto this landing pad; this causes them to be uploaded to the folder.

Secure FTP:

Those e-delegates who prefer to use FTP-like access to our e-meeting Inbox & Draft folders but are concerned by their usernames and passwords being sent unencrypted over the internet, ETSI IT has fitted the server with FTPS (SSL) so delegates can connect from their favourite FTP client using the address: ftps.3gpp.org. Please enter your username and password when prompted.

Organizational

- Incoming LSs are noted by default. Contact companies should flag LSs that need to be replied from this meeting.

- Legacy topics will be treated by email only. Please see the list of offline email discussions below.

- Rel-16 (draft) CRs and text proposals will be handled as part of the email discussion on the corresponding CR(s) or the ASN.1 review email discussion if associated with a RIL#.

- All organizational emails and notes will be shared over the following email discussion throughout both meeting weeks:

[AT109bis-e][300][NBIOT] Organisational (Session Chair)

Status: Started

Scope: Comments to session notes. Kick-off and management of email discussions for NB-IoT session. Coordination issues. Other organisational issues and announcements.

Intended outcome: Approval of Report from NB-IoT session.

Deadline: 30-04-2020, 10:00 UTC

List and Status of Offline Email Discussions

NOTE: The official kick off date for these email discussions are Monday April 20th at 7:00 UTC. The rapporteurs can share them on the reflector earlier, however companies are not required to participate before the official kick off date. The deadlines refer to the deadline for providing company comments unless stated otherwise.

[AT109bis-e][301][NBIOT] Clarification on RLC UM SN size for NB-IoT (Huawei)

Status: Closed

Scope: Check if there is support and update based on the comments if the CR is agreeable.

Intended outcome: Report from the discussion and, if agreeable, in-principle agreed CR. The report can be provided in R2-2004036, CR in R2-2004056

Deadline: 27-04-2020, 10:00 UTC

[AT109bis-e][302][NBIOT] Optimisation on trigger for dedicated SR with HARQ-ACK (ZTE)

Status: Started

Scope: Check if there is support and update based on the comments if the CR is agreeable.

Intended outcome: Report from the discussion and, if agreeable, in-principle agreed CR. The report can be provided in R2-2004037

Deadline: 27-04-2020, 10:00 UTC

[AT109bis-e][303][NBIOT] Cell selection on the dedicated frequency after RRC connection rejection for NB-IoT (Mediatek)

Status: Closed

Scope: Check if there is support and update based on the comments if the CR is agreeable.

Intended outcome: Report from the discussion and, if agreeable, in-principle agreed CR. The report can be provided in R2-2004038

Deadline: 27-04-2020, 10:00 UTC

[AT109bis-e][304][NBIOT] 36.300 CR (Huawei)

Status: Started

Scope: Update the CR with agreements from this meeting.

Intended outcome: baseline CR for updating 36.300 in R2-2004039

Deadline: 29-04-2020, 10:00 UTC

[AT109bis-e][305][NBIOT] 36.331 CR (Huawei)

Status: Not started

Scope: Update the CR with agreements from this meeting.

Intended outcome: baseline CR for updating 36.331 in R2-2004040

Deadline: 29-04-2020, 10:00 UTC

[AT109bis-e][306][NBIOT] 36.302 CR (Huawei)

Status: Started

Scope: Update the CR with agreements from this meeting.

Intended outcome: baseline CR for updating 36.302 in R2-2004041

Deadline: 29-04-2020, 10:00 UTC

[AT109bis-e][307][NBIOT] 36.304 CR (Nokia)

Status: Started

Scope: Update the CR with agreements from this meeting, including WUS TP.

Intended outcome: baseline CR for updating 36.304 in R2-2004042

Deadline: 29-04-2020, 10:00 UTC

[AT109bis-e][308][NBIOT] 36.321 CR  (Ericsson)

Status: Started

Scope: Update the CR with agreements from this meeting.

Intended outcome: baseline CR for updating 36.321 in R2-2004043

Deadline: 29-04-2020, 10:00 UTC

[AT109bis-e][309][NBIOT] 36.306 CR  (Blackberry)

Status: Started

Scope: Update the CR with agreements from this meeting.

Intended outcome: baseline CR for updating 36.306 in R2-2004044

Deadline: 29-04-2020, 10:00 UTC

[AT109bis-e][310][NBIOT] WUS open issues (Ericsson)

Status: Closed

Scope: Remaining open issues on WUS

Intended outcome: Finalise the open issues, report in R2-2004045

Deadline: 23-04-2020, 16:00 UTC

[AT109bis-e][311][NBIOT] PUR open issues	 (Huawei)

Status: Closed

Scope: Remaining open issues on PUR

Intended outcome: Finalise the open issues, report in R2-2004046

Deadline: 22-04-2020, 16:00 UTC

Extended deadline to make further agreements: 28-04-2020 1000 UTC

[AT109bis-e][312][NBIOT] SON open issues (Session Chair)

Status: Closed

Scope: Remaining open issues on SON

Intended outcome: Finalise the open issues, report in R2-2004047

Deadline: 22-04-2020, 16:00 UTC

[AT109bis-e][313][NBIOT] UE capabilities, TDD/FDD differentiation and 5GC applicability for NB-IoT and eMTC (Huawei)

Status: Closed

Scope: Discuss the open issues on UE capabilities

Intended outcome: Finalise the issues, report in R2-2004048

Deadline: 27-04-2020, 10:00 UTC

[AT109bis-e][314][NBIOT] ASN.1 review of NB-IoT (Huawei)

Status: Started

Scope: ASN.1 WI specific issues discussion

Intended outcome: progress the ASN.1 review and conclude as much as possible, report in R2-2004049

Deadline: 27-04-2020, 10:00 UTC

[AT109bis-e][315][NBIOT]  UE specific DRX - FFSs (Huawei)

Status: Closed

Scope: Address the 2 FFS on UE specific DRX

Intended outcome: Report in R2-2004052

Deadline: 28-04-2020, 10:00 UTC

[AT109bis-e][316][NBIOT]  UE specific DRX – LSs (Huawei)

Status: Closed

Scope: Approve 2 LS on UE specific DRX. 1) to RAN4, 2) to CT1, RAN3.

Intended outcome: 2 approved LS in R2-2004050 (to:RAN4), R2-2004051 (to:CT1, RAN3)

Deadline: 22-04-2020, 10:00 UTC

[AT109bis-e][317][NBIOT] LS to SA3 on CP PUR security (Huawei)

Status: Started

Scope: Draft and approve the LS

Intended outcome: Approved LS in R2-2004055

Deadline: 28-04-2020 1000 UTC

## 4.1	NB-IoT corrections Rel-15 and earlier

Documents in this agenda item will be handled in a break out session. Common NB-IoT/eMTC parts treated jointly with 4.2.

This agenda item may not be treated during the e-meeting. No web conference is planned for this agenda item

R2-2003246	Clarification on RLC UM SN size for NB-IoT	Huawei, HiSilicon	CR	Rel-15	36.322	15.3.0	0145	-	F	NB_IOTenh2-Core

Revised in R2-2004056 ([AT109bis-e][301])

[AT109bis-e][301][NBIOT] Clarification on RLC UM SN size for NB-IoT (Huawei)

Scope: Check if there is support and update based on the comments if the CR is agreeable.

Intended outcome: Report from the discussion and, if agreeable, in-principle agreed CR. The report can be provided in R2-2004036, CR in R2-2004056

Deadline: 27-04-2020, 10:00 UTC

R2-2004036	Report of [AT109e][301][ NBIOT R14] Clarification on polling bit for RRCConnectionRelease (Huawei)

Noted ([AT109bis-e][301])

R2-2004056	Clarification on RLC UM SN size for NB-IoT	Huawei, HiSilicon	CR	Rel-15	36.322	15.3.0	0145	1	F	NB_IOTenh2-Core

Agreed in principle ([AT109bis-e][301])

R2-2003245	Optimisation on trigger for dedicated SR with HARQ-ACK	ZTE Corporation, Sanechips, MediaTek Inc.	discussion	Rel-15	LTE_eMTC4-Core

R2-2003254	Optimisation on trigger for dedicated SR with HARQ-ACK	ZTE Corporation, Sanechips, MediaTek Inc.	CR	Rel-15	36.321	15.8.0	1469	-	F	LTE_eMTC4-Core

R2-2003256	Optimisation on trigger for dedicated SR with HARQ-ACK	ZTE Corporation, Sanechips, MediaTek Inc.	CR	Rel-15	36.331	15.9.0	4254	-	F	LTE_eMTC4-Core

[AT109bis-e][302][NBIOT] Optimisation on trigger for dedicated SR with HARQ-ACK (ZTE)

Scope: Check if there is support and update based on the comments if the CR is agreeable.

Intended outcome: Report from the discussion and, if agreeable, in-principle agreed CR. The report can be provided in R2-2004037

Deadline: 27-04-2020, 10:00 UTC

R2-2004037	Report of [AT109bis-e][302][NBIOT]Trigger for dedicated SR with HARQ-ACK (ZTE)

[CB]

Proposal 1: For the case that only SR with HARQ-ACK is configured, RAN2 discuss and agree with one of the following clarification options:

Option 1: To add clarification that the pending SR is cancelled only when random access preamble is transmitted and also add clarification that the RA procedure would be cancelled if RA has been initiated and SR is then sent piggy-backed on HARQ-ACK.

Option 2: To add clarification that only when the random access preamble can be transmitted, RA procedure could be initiated and the pending SR is cancelled.

Proposal 2: It’s no need to introduce a delay timer for avoiding that the UE immediately initiates RA procedure for SR with HARQ-ACK.

R2-2003619	Discussion on dedicated frequency search after connection rejection	MediaTek Inc.	discussion	Rel-15	NB_IOTenh2-Core

R2-2003621	Cell selection on the dedicated frequency after RRC connection rejection for NB-IoT in 36.304	MediaTek Inc.	CR	Rel-15	36.304	15.5.0	0787	-	F	NB_IOTenh2-Core

R2-2003622	Cell selection on the dedicated frequency after RRC connection rejection for NB-IoT in 36.331	MediaTek Inc.	CR	Rel-15	36.331	15.9.0	4280	-	F	NB_IOTenh2-Core

[AT109bis-e][303][NBIOT] Cell selection on the dedicated frequency after RRC connection rejection for NB-IoT (Mediatek)

Scope: Check if there is support and update based on the comments if the CR is agreeable.

Intended outcome: Report from the discussion and, if agreeable, in-principle agreed CR. The report can be provided in R2-2004038

Deadline: 27-04-2020, 10:00 UTC

R2-2004038	Report of 	[AT109bis-e][303][NBIOT] Cell selection on the dedicated frequency after RRC connection rejection for NB-IoT (Mediatek)

Not pursued ([AT109bis-e][303])

## 7.2	Additional enhancements for NB-IoT

(NB_IOTenh3-Core; leading WG: RAN1; REL-16; started: Jun 18; target; June 20; WID: RP-200293; SR: RP-200440)

Time budget: 2.5 TU

Documents in this agenda item will be handled in a break out session

Some sub-items in 7.1 and 7.2 may be treated jointly.

### 7.2.1	Organisational

Including incoming LSs, draft TS, rapporteur inputs, etc

A web conference will be used for handling some of the discussions in this AI.

One CR per specification will be provided by the corresponding rapporteur. No individual company CRs are expected. Companies should provide TPs when needed.

R2-2002587	RAN2 agreements for Rel-16 additional enhancements for NB-IoT and MTC	Document Rapporteur (BlackBerry)	other	Rel-16	LTE_eMTC5-Core, NB_IOTenh3-Core

[CB]

R2-2003249	Miscellaneous corrections to TS 36.300 for Rel-16 NB-IoT	Huawei, HiSilicon	CR	Rel-16	36.300	16.1.0	1277	-	F	NB_IOTenh3-Core

[AT109bis-e][304][NBIOT] 36.300 CR (Huawei)

Scope: Update the CR with agreements from this meeting.

Intended outcome: baseline CR for updating 36.300 in R2-2004039

Deadline: 29-04-2020, 10:00 UTC

R2-2003744	Miscellaneous corrections to 36.331 for Rel-16 NB-IoT	Huawei, HiSilicon	CR	Rel-16	36.331	16.0.0	4287	-	F	NB_IOTenh3-Core	Late

[AT109bis-e][305][NBIOT] 36.331 CR (Huawei)

Scope: Update the CR with agreements from this meeting.

Intended outcome: baseline CR for updating 36.331 in R2-2004040

Deadline: 29-04-2020, 10:00 UTC

R2-2003745	Miscellaneous corrections to 36.302 for Rel-16 NB-IoT	Huawei, HiSilicon	CR	Rel-16	36.302	16.0.0	1209	-	F	NB_IOTenh3-Core	Late

[AT109bis-e][306][NBIOT] 36.302 CR (Huawei)

Scope: Update the CR with agreements from this meeting.

Intended outcome: baseline CR for updating 36.302 in R2-2004041

Deadline: 29-04-2020, 10:00 UTC

[AT109bis-e][307][NBIOT] 36.304 CR (Nokia)

Scope: Update the CR with agreements from this meeting, including WUS TP.

Intended outcome: baseline CR for updating 36.304 in R2-2004042

Deadline: 29-04-2020, 10:00 UTC

[AT109bis-e][308][NBIOT] 36.321 CR  (Ericsson)

Scope: Update the CR with agreements from this meeting.

Intended outcome: baseline CR for updating 36.321 in R2-2004043

Deadline: 29-04-2020, 10:00 UTC

[CB]

### 7.2.2	UE-group wake-up signal WUS

UE group wake Up signal for MTC and NB-IoT is treated jointly under this Agenda Item.

A web conference will be used for handling some of the discussions in this AI.

Includes [Post109e#32][NBIOT/EMTC] Finalise the 36.304 Text for WUS (Nokia)

Includes [Post109e#45][NBIOT/EMTC] WUS open issues (Ericsson)

All identified critical open issues should be provided to the rapporteur via email discussion Post109e#45 and new contributions on those topics are discouraged.

R2-2003328	E-mail-Discussion-Summary for Post109e-32 : Finalise TP for TS36.304 for WUS	Nokia, Nokia Shanghai Bell	discussion	Rel-16

Noted

R2-2003329	Draft TP for TS36.304	Nokia, Nokia Shanghai Bell	discussion	Rel-16

Can be used as a baseline for further discussion under offline#307

R2-2003431	Report - Email discussion [Post109e#45][NB-IoT/eMTC] WUS open issues	Ericsson	discussion	Rel-16	LTE_eMTC5-Core, NB_IOTenh3-Core	Late

Proposal 1	8 codepoints are used to indicate paging probability classes.

Proposal 2	The value range for paging probability classes is up to 0.9.

Proposal 3	The granularity for paging probability classes is 0.1.

Proposal 4	The value range for paging probability classes starts from 0.2.

Proposal 5	The following codepoints are used to indicate a paging probability class: {n20,n30,n40,n50,n60,n70,n80,n90}

Proposal 6	Discuss whether SA2/RAN3 should be informed with an LS if RAN2 were to decide on the value range and granularity.

Proposal 7	Discuss whether there is a need to support 4 WUS group sets considering that 1 WUS group set is assigned for UEs with no paging probability class.

Proposal 8	Discuss whether it would be beneficial for the eNB if the MME provides assistance information regarding a particular paging probability class.

Proposal 9	The mechanism proposed in SA2 for Release 15 to reduce false wake-up is used in Rel-16.

Proposal 10	Confirm the following working assumption: “Support of Release 16 WUS is independent to support of Release 15 WUS”.

Agreements: - The following codepoints are used to indicate a paging probability threshold value: {p20,p30,p40,p50,p60,p70,p80,p90}

[AT109bis-e][310][NBIOT] WUS open issues (Ericsson)

Scope: Remaining open issues on WUS

Intended outcome: Finalise the open issues, report in R2-2004045

Deadline: 22-04-2020, 16:00 UTC

R2-2004045	Report - Email discussion [AT109bis-e][310][NBIOT eMTC] WUS open issues	Ericsson

Proposal 1	The paging probability threshold values, i.e., {p20,p30,p40,p50,p60,p70,p80,p90}, are used to map the paging probabilities configured by the core network to WUS group sets.

Proposal 2	Confirm the working assumption: Maximum number probability thresholds is 3 giving 4 group.

Proposal 3	Discuss whether RAN2 should indicate to other WGs, e.g., SA2/RAN3, that it would be beneficial for the eNB if the MME provides assistance information regarding a particular paging probability class.

- QC thinks it could be raised in RAN3, we don’t need to send an LS. Huawei think it is not necessary, eNB can change the configuration, same as other paging related configs. Nokia agree with QC and HW.

- Lenovo think there is no RAN2 impact, but it can be beneficial. Ericsson think that RAN2 are the feature lead so that is why the issue was raised here.

Proposal 4	Discuss whether the mechanism proposed in SA2 for Release 15 to reduce false wake-up should be used in Rel-16.

- QC thinks SA2 already agreed this, Huawei agree and think the release is finishing so no time to make a new solution. Ericsson, Sony think there is a solution.

- Nokia think we should consider relaxing this as group can improve this. Zte think there should be a difference to Rel-15, Sony agree but think it is up to MME so not RAN2 issue.

- Huawei think there is no problem having the same solution in Rel-16 as Rel-15, and further other new solutions are not agreeable.

Proposal 5	Confirm the working assumption: “Support of Release 16 WUS is independent to support of Release 15 WUS”.

- QC thnk the capabilities could be linked, no need to have the flexibility – UE is anyway required to implement the Rel-15 functionality to support Rel-16..

- Sequans think this is useful for UE preference. HW agree with Sequans, Rel-15 is not as efficient.

- Nokia think from NW point of view this is an independent capability.

- ZTE thinks common WUS needs to be configured.

Agreements - The paging probability threshold values, i.e., {p20,p30,p40,p50,p60,p70,p80,p90}, are used to map the paging probabilities configured by the core network to WUS group sets. - Confirm the working assumption: Maximum number probability thresholds is 3 giving a total of 4 groups. - Confirm the working assumption: “Support of Release 16 WUS is independent to support of Release 15 WUS”.

R2-2002671	On supporting UE group WUS operation with mobility 	Sony	discussion	Rel-16	NB_IOTenh3-Core

R2-2003101	Consideration on WUS paging probability parameter	Lenovo, Motorola Mobility	discussion	Rel-16

R2-2003102	Group WUS for mobile UE	Lenovo, Motorola Mobility	discussion	Rel-16

R2-2003184	Clarification of WUS resource configuration	Qualcomm Incorporated	draftCR	Rel-16	36.331	16.0.0	LTE_eMTC5-Core

R2-2003485	Formula for WUS group selection	ZTE Corporation, Sanechips	discussion	Rel-16	LTE_eMTC5-Core, NB_IOTenh3-Core

R2-2003741	Signalling changes for GWUS Resource mapping for eMTC	Nokia Solutions & Networks (I)	discussion	Rel-16

### 7.2.3	Transmission in preconfigured resources

Transmission in preconfigured resources for MTC and NB-IoT is treated jointly under this Agenda Item.

A web conference will be used for handling some of the discussions in this AI.

Includes [Post109e#46][NBIOT/EMTC] PUR open issues (Huawei)

All identified critical open issues should be provided to the rapporteur via email discussions Post109e#46 and new contributions on those topics are discouraged.

R2-2003746	Report of email discussion [Post109e#46][NBIOT/EMTC] PUR open issues	Huawei	report	Rel-16	LTE_eMTC5-Core, NB_IOTenh3-Core	Late

RRC aspects:

Proposal 1-1:	For PUR TBS in eMTC, the current TBS values captured in eMTC RRC CR are supported, i.e. {b328, b408, b504, b600, b712, b808, b936, b1000, b1352, b1544, b1736, b1992, b2152, b2344, b2792, b2984}. (7/7)

Proposal 1-2:	[FFS] For PUR TBS in eMTC, TBS values larger than b2984 can be supported, FFS exact values and how many code points. (4/7)

Proposal 1-3:	For PUR TBS in NB-IoT, TBS values {b328, b408, b504, b584, b680, b808, b936, b1000, b1128, b1256, b1384, b1608, b1800, b2024, b2280, b2536} are supported. (5/7)

Proposal 1-4:	[FFS] For pur-Periodicity-r16 and requestedPeriodicity-r16, FFS whether to support hsf16384, hsf32768 and hsf65536 for both NB-IoT and eMTC (4/8).

Proposal 1-5:	[FFS] For both NB-IoT and eMTC, pur-StartTime-r16 is a 2-level start offset (5/8)

-	Level 1: startHSF: {hsf128, hsf256, hsf512, hsf1024, hsf2048, hsf4096, hsf8192, spare} (7/8)

-	Level 2: startSubframe: FFS value range (8/8)

Proposal 1-6:	[FFS] For both NB-IoT and eMTC, the granularity of requestedTimeOffset-r16 is H-SF level, FFS exact values. (4/8)

Proposal 1-7:	For both NB-IoT and eMTC, the value range of pur-TimeAlignmentTimer-r16 is INTEGER (1..8), i.e. 1~8 * PUR periodicity. (8/8)

Proposal 1-8:	PUR-RNTI is used as the name of RNTI used for PUR. (6/8)

Proposal 2-1:	All PUR parameters are stored in the eNB (7/8).

Proposal 2-2:	The eNB links CP-PUR configuration to each UE in RRC_IDLE according to PUR resource by implementation. (5/7)

Proposal 2-3:	[FFS] PUR (re-)configuration can be provided to the UE for the CP solution without AS security enabled (4/7).

MAC aspects:

Proposal 3:	Remove the Editor’s Note “FFS whether restarting the window is indended” from 36.321. (8/8)

Proposal 4:	Remove the Editor’s Note “FFS what is the impact of PUR in this section” from 36.321. (6/7)

Proposal 5: [FFS] No additional change on implicitReleaseAfter is needed in MAC specification. (4/7)

RRC-MAC interactions:

Proposal 6-0:	RAN2 to discuss whether to confirm or revert the working assumption that MAC calculates the PUR grant for each PUR occasion.

The following proposals 6-1 to 8 are conditional. If RAN2 confirms the working assumption:

Proposal 6-1:	RRC is aware of PUR grant. How RRC is aware is up to UE implementation. (5/7)

Proposal 6-2:	RRC can decide not to use the PUR grant for NAS signalling and no MAC-RRC interaction is needed. (6/7)

Proposal 6-3:	pur-NumOccasion is handed in MAC layer. (5/7)

Proposal 7:	MAC is aware of RRC state. How MAC is aware is up to UE implementation. (4/6)

Proposal 8:	MAC is aware of CP transmission using PUR. How MAC is aware is up to UE implementation. (7/7)

Proposal 9-1:	No further MAC-RRC interaction on TA validation is needed. Remove the Editor’s Note “How RRC indicates to MAC that TA is valid or instructs MAC to use PUR” from 36.321. (5/6)

Proposal 9-2:	Remove the references to PUR TA timer validation in section 5.4.7.1 from 36.321. (4/6)

Proposal 10-1:	PUR release due to RACH initiation on a new cell is captured in RRC. (7/7)

Proposal 10-2:	PUR configuration is released when the UE initiates RA procedure on a new cell for all purposes. (6/7)

Agreements: RRC: - For both NB-IoT and eMTC, the value range of pur-TimeAlignmentTimer-r16 is INTEGER (1..8), i.e. 1~8 * PUR periodicity. - All PUR parameters are stored in the eNB. RAN2 has not identified any parameters that must be stored in the MME. MAC aspects: - Remove the Editor’s Note “FFS whether restarting the window is indended” from 36.321. - Remove the Editor’s Note “FFS what is the impact of PUR in this section” from 36.321. RRC-MAC Interactions - No further MAC-RRC interaction on TA validation is needed. Remove the Editor’s Note “How RRC indicates to MAC that TA is valid or instructs MAC to use PUR” from 36.321. - Remove the references to PUR TA timer validation in section 5.4.7.1 from 36.321. - PUR release due to RACH initiation on a new cell is captured in RRC. - PUR configuration is released when the UE initiates RA procedure on a new cell for all purposes.

[AT109bis-e][311][NBIOT] PUR open issues	 (Huawei)

Scope: Remaining open issues on PUR

Intended outcome: Finalise the open issues, report in R2-2004046

Deadline: 22-04-2020, 16:00 UTC

Extended deadline to make further agreements: 28-04-2020 1000 UTC

R2-2004046	Report of offline discussion [AT109bis-e][311] on PUR open issues	Huawei

Proposal 1.	For the requested PUR TBS in eMTC, the maximum value is b2984, with spare values in requestedTBS-r16 for future extension. (4?/5)

Proposal 2.	For the requested PUR TBS in eMTC, the minimum value is b328. (4/5)

Proposal 3.	For the requested PUR TBS in eMTC, FFS how many codepoints. (3/5)

Proposal 4.	For the requested PUR TBS in NB-IoT, FFS TBS values {b328, b408, b504, b584, b680, b808, b936, b1000, b1128, b1256, b1384, b1608, b1800, b2024, b2280, b2536}. (2/5)

Proposal 5.	For pur-Periodicity-r16 and requestedPeriodicity-r16, confirm that the value range is {hsf8, hsf16, hsf32, hsf64, hsf128, hsf256, hsf512, hsf1024, hsf2048, hsf4096, hsf8192, spare5, spare4, spare3, spare2, spare1} for both NB-IoT and eMTC (5/5)

Proposal 6.	For both NB-IoT and eMTC, pur-StartTime-r16 is a 2-level start offset (3/5)

Proposal 7.	For both NB-IoT and eMTC, level-1 offset is H-SF level (5/5).

Proposal 8.	Only some of HSFN need to be included in the level-1 offset. FFS exact values. (3/5)

Proposal 9.	For both NB-IoT and eMTC, FFS details on level-2 offset.

Proposal 10.	For both NB-IoT and eMTC, requestedTimeOffset-r16 in PUR request is in H-SF level (5/5).

Proposal 11.	requestedTimeOffset-r16 in PUR request has the same value range as the level-1 offset for pur-StartTime-r16 in PUR configuration. (4/5)

Proposal 12.	PUR-RNTI is used as the name of RNTI used for PUR. (4/5).

Proposal 13.	The eNB links CP-PUR configuration to each UE in RRC_IDLE according to PUR resource by implementation. (4/5)

Proposal 14.	PUR (re-)configuration can be provided to the UE for the CP solution without AS security enabled. (4/5)

- QC, Nokia, Sequans think this needs to be checked with SA3.

- Huawei think this needs to be implemented in the spec and further action only if SA3 have an issue. QC agree.

Proposal 15.	No additional change on implicitReleaseAfter is needed in MAC specification. (3/5)

Proposal 16.	Revert the previous working assumption, PUR grant is maintained in RRC. (4/5)

- LG don’t want to revert, but can accept.

- ZTE think the proposal is acceptable as this is mainly UE implementation.

Proposal 17.	RRC configures the lower layers to use PUR grant upon initiation of transmission using PUR. (4/5)

Proposal 18.	The handling of ‘m’ counter is moved from MAC to RRC. (4/5)

Proposal 19.	Confirm that transmission using PUR cannot be used for signalling, i.e. mt-Access and mo-Signalling cannot be used for transmission using PUR. (4/5)

Agreements: - Revert the previous working assumption, PUR grant is maintained in RRC. - The handling of ‘m’ counter is moved from MAC to RRC - From RAN2 point of view PUR (re-)configuration can be provided to the UE for the CP solution without AS security enabled. - Send LS to SA3 to confirm. - PUR-RNTI is used as the name of RNTI used for PUR.

After email:

Agreements [AT109bis-e][311]: RRC: - For pur-Periodicity-r16 and requestedPeriodicity-r16, confirm that the value range is {hsf8, hsf16, hsf32, hsf64, hsf128, hsf256, hsf512, hsf1024, hsf2048, hsf4096, hsf8192, spare5, spare4, spare3, spare2, spare1} for both NB-IoT and eMTC - For both NB-IoT and eMTC, PUR request indicates requested start time/offset of PUR in H-SF level. - FFS [CB]: 2-level offset need and details for pur-StartTime-r16 - requested PUR TBS values: - For the requested PUR TBS in eMTC and NB-IoT, the minimum value is b328. - FFS [CB]: other details - FFS [CB]: It is up to eNB implementation how to link CP-PUR configuration to each UE in RRC_IDLE according to PUR resource. RRC-MAC - RRC configures the lower layers to use PUR grant upon initiation of transmission using PUR. - FFS [CB]: implicitReleaseAfter handling and other RRC-MAC interaction details Other - Confirm that transmission using PUR cannot be used for signalling, i.e. mt-Access and mo-Signalling cannot be used for transmission using PUR.

[AT109bis-e][317][NBIOT] LS to SA3 on CP PUR security (Huawei)

Status:

Scope: Draft and approve the LS

Intended outcome: Approved LS in R2-2004055

Deadline: 28-04-2020 1000 UTC

R2-2004055 	[Draft] LS on security of PUR for the CP solution, Huawei, To: SA3	NB_IOTenh3-Core, LTE_eMTC5-Core

[CB]

R2-2003257	Complete the HARQ process for PUR	ZTE Corporation, Sanechips	draftCR	Rel-16	36.321	16.0.0	NB_IOTenh3-Core, LTE_eMTC5-Core

R2-2003258	Correction on successful PUR transmission indication	ZTE Corporation, Sanechips	draftCR	Rel-16	36.321	16.0.0	LTE_eMTC5-Core, NB_IOTenh3-Core

R2-2003267	Correction on TA timer maintenance	ZTE Corporation, Sanechips	draftCR	Rel-16	36.321	16.0.0	LTE_eMTC5-Core, NB_IOTenh3-Core

R2-2003331	Security Aspects of PUR Configuration for CP	Nokia, Nokia Shanghai Bell	discussion

R2-2003355	Moving UL grant handling from MAC to RRC for PUR	Ericsson, Huawei, HiSilicon	discussion	NB_IOTenh3-Core, LTE_eMTC5-Core

R2-2003415	TA validation based on serving cell RSRP change (related to RAN4 LSes)	Sierra Wireless, S.A.	discussion	Rel-16	R2-2000443

R2-2003429	Configuration and adjustment of repetition number	Sierra Wireless, S.A.	discussion	Rel-16

R2-2003652	Remaining issues of D-PUR TA timer in MAC	ASUSTeK	discussion	Rel-16	38.321	NB_IOTenh3-Core

R2-2003653	PUR configuration maintenance during RRC state transition	ASUSTeK	discussion	Rel-16	36.331	NB_IOTenh3-Core

### 7.2.4	NB-IoT Specific

NB-IoT specific topics

This agenda item may utilize a summary document to facilitate treatment of topics during the e-meeting.

A web conference will be used for handling some of the discussions in this AI.

Includes [Post109e#15][NBIOT] UE specific DRX: DRX cycle values (Sequans)

R2-2003131	To Verify ANR Measurements	Ericsson, Nokia, Nokia Shanghai Bell, ZTE Corporation	discussion	Rel-16

R2-2003133	Logging of CE Level for RLF and ANR measurements	Ericsson	discussion	Rel-16

R2-2003139	Draft LS to RAN4 on ANR Measurements	Ericsson [To be RAN2]	LS out	Rel-16	NB_IOTenh3-Core	To:RAN4

R2-2003247	SON remaining issues	Huawei, HiSilicon	discussion	Rel-16	NB_IOTenh3-Core

R2-2003291	Remaining FFSs for SON in NB-IoT	ZTE Corporation, Sanechips	discussion	Rel-16	NB_IOTenh3-Core

R2-2003786	Summary of SON/ANR open issues	Session Chair (Huawei)

[AT109bis-e][312][NBIOT] SON open issues (Session Chair)

Scope: Remaining open issues on SON

Intended outcome: Finalise the open issues, report in R2-2004047

Deadline: 22-04-2020, 16:00 UTC

R2-2004047	Summary of SON/ANR open issues	Session Chair (Huawei)

Easy agreements:

Proposal S1-1: Confirm the Working assumption that the ANR report is discarded after 96 hours.

Proposal S1-2: ANR measurement report is discarded upon RAT change.

Proposal S1-3: Cell ID of the cell where e-establishment failed, if different to the current cell, is included in the RLF report when available.

Proposal S1-5: RLF report is discarded in the following cases:

Reporting rlf-InfoAvailable and returning to idle.

RAT change

Power off or detach.

(already agreed) after 48 hours if not fetched

Needs further discussion:

Proposal S2-2: UE stores the serving cell measurement at the time where the neighbour cell measurement is taken (in ANR-MeasResult-NB)

No consensus:

Proposal S1-4a: Include timeSpent information in ANR report to indicate the elapsed time since the generation of ANR record

Proposal S1-4b: timeSpent is defined as INTEGER (0..5760) with unit of minutes

Proposal S2-1: Send a LS to RAN4 to verify that the ANR measurements specified by RAN2 would work fine.

Agreements: - Confirm the Working assumption that the ANR report is discarded after 96 hours. - ANR measurement report is discarded upon RAT change. - Re-establishment Cell ID is included in the RLF report, only if different to the cell on which the report is sent. - RLF report is discarded in the following cases: - Reporting rlf-InfoAvailable and returning to idle. - RAT change - Power off or detach. - (already agreed) after 48 hours if not fetched - [CB]: FFS- In addition to the serving cell measurement stored when the configuration is received, UE stores the latest serving cell measurement when the neighbour cell measurement is stored (in ANR-MeasResult-NB)

R2-2003669	Report of [Post109e#15][NBIOT] UE specific DRX DRX cycle values	Sequans Communications	discussion	Rel-16	NB_IOTenh3-Core

Proposal 1: Discuss further introduction of short UE specific cycles 320ms and 640ms

R2-2003747	Introduction of UE specific DRX for NB-IoT	Huawei, HiSilicon, MediaTek, CMCC, China Unicom, Ericsson, Lenovo, Motorola Mobility	discussion	Rel-16	NB_IOTenh3-Core

=> Revised in R2-2003780

R2-2003780	Introduction of UE specific DRX for NB-IoT	Huawei, HiSilicon, MediaTek, CMCC, China Unicom, Ericsson, Lenovo, Motorola Mobility, Vodafone	discussion	Rel-16	NB_IOTenh3-Core

=> Revised in R2-2003815

R2-2003815	Introduction of UE specific DRX for NB-IoT	Huawei, HiSilicon, MediaTek, CMCC, China Unicom, Ericsson, Lenovo, Motorola Mobility, Vodafone, China Telecom 	discussion	Rel-16	NB_IOTenh3-Core

Proposal: 1: UE-specific DRX cycle values 320ms, 640ms, 1280ms, 2560ms, 5120ms and 10240ms are supported in NB-IoT for both EPS and 5GS.

Proposal: 2: Introduce an indication in SIB to enable/disable the use of UE specific DRX cycles in NB-IoT for 5GS (similar to EPS).

Proposal 3: Send a LS to CT1 and RAN3 to inform them about the UE specific DRX cycle values introduced for NB-IoT for both EPS and 5GS.

Proposal 4: Send a LS to RAN4 to inform them about the UE specific DRX cycle values introduced for NB-IoT for both EPS and 5GS and ask to update RRM requirements, if needed.

- Qualcomm is not sure how the NW configuration can address both use cases in the same cell.

- Sequans has the same concern as QC and are not sure why an indication would be needed.

- Vodafone thinks the same basestation can support multiple cells and split UEs according to the coverage. Vodafone thinks the SIB indication is different for 5GS but think it is still useful.

- Ericsson thinks worst case repetitions is not always needed for UE in good coverage even if this is configured so in practise it is possible to support both cases in the same cell.

- Huawei thinks some issues can be avoided by the correct configuration, and the SIB indication is useful for some deployments

- Huawei thinks the problems raised by Sequans and Qualcomm have been discussed several times in email discussions and the majority think these can be solved and in fact don’t exist in a typical deployment. China Telecom agree

- Sequans thinks we could capture in the minutes that the feature is not compatible with extreme coverage. Ericsson think UE can just continue until the maximum repetitions whether there is overlap with PO or not. Sequans would be fine to have clarification on UE behaviour. Nokia think some clarification would be needed but would be OK with that. QC wonders where the clarification would be specified. Ericsson thinks we can check.

Agreements: - UE-specific DRX cycle values 320ms, 640ms, 1280ms, 2560ms, 5120ms and 10240ms are supported in NB-IoT for both EPS and 5GS. - Introduce an indication in SIB to enable/disable the use of UE specific DRX cycles in NB-IoT for 5GS (similar to EPS). FFS whether it is cell or PLMN specific. - Will clarify UE behaviour in case of CSS overlap due to large repetitions needed to decode the NPDCCH for paging. FFS how. - Send a LS to CT1 and RAN3 to inform them about the UE specific DRX cycle values introduced for NB-IoT for both EPS and 5GS. - Send a LS to RAN4 to inform them about the UE specific DRX cycle values introduced for NB-IoT for both EPS and 5GS and ask to update RRM requirements, if needed.

[AT109bis-e][315][NBIOT]  UE specific DRX - FFSs (Huawei)

Status: Not started

Scope: Address the 2 FFS on UE specific DRX

Intended outcome: Report in R2-2004052

Deadline: 28-04-2020, 10:00 UTC

R2-2004052	Report of [AT109bis-e][315][NBIOT] UE specific DRX - FFSs

Proposal 1: The SIB indication to enable/disable the use of UE specified DRX for 5GS is cell specific indication. (7/7)

Proposal 2:No consensus on how to address CSS overlapping for UE specific DRX.

[CB]

R2-2003748	[Draft] Reply LS on Rel-16 NB-IoT enhancements	Huawei	LS out	Rel-16	NB_IOTenh3-Core	To:CT1, RAN3	Cc:SA2

R2-2003749	[Draft] LS on UE specific DRX in NB-IoT	Huawei	LS out	Rel-16	NB_IOTenh3-Core	To:RAN4

[AT109bis-e][316][NBIOT]  UE specific DRX – LSs (Huawei)

Status: Not started

Scope: Approve 2 LS on UE specific DRX. 1) to RAN4, 2) to CT1, RAN3.

Intended outcome: 2 approved LS in R2-2004050 (to:RAN4), R2-2004051 (to:CT1, RAN3)

Deadline: 22-04-2020, 10:00 UTC

R2-2004051	[Draft] Reply LS on Rel-16 NB-IoT enhancements	Huawei	LS out	Rel-16	NB_IOTenh3-Core	To:CT1, RAN3

LS approved in R2-2004053

R2-2004050	[Draft] LS on UE specific DRX in NB-IoT	Huawei	LS out	Rel-16	NB_IOTenh3-Core	To:RAN4

LS approved in R2-2004054

### 7.2.5	NB-IoT UE capabilities

This agenda item may utilize a summary document to facilitate treatment of topics during the e-meeting.

A web conference will be used for handling some of the discussions in this AI.

Includes [Post109e#14][NBIOT] 36.306 CR (Blackberry)

R2-2002588	Updates for Rel-16 additional enhancements NB-IoT	BlackBerry UK Limited	CR	Rel-16	36.306	16.0.0	1746	-	C	NB_IOTenh3-Core	Late

[AT109bis-e][309][NBIOT] 36.306 CR  (Blackberry)

Scope: Update the CR with agreements from this meeting.

Intended outcome: baseline CR for updating 36.306 in R2-2004044

Deadline: 29-04-2020, 10:00 UTC

R2-2003248	UE capabilities, TDD/FDD differentiation and 5GC applicability for NB-IoT and eMTC	Huawei, HiSilicon	discussion	Rel-16	NB_IOTenh3-Core, LTE_eMTC5-Core

[AT109bis-e][313][NBIOT] UE capabilities, TDD/FDD differentiation and 5GC applicability for NB-IoT and eMTC (Huawei)

Scope: Discuss the open issues on UE capabilities

Intended outcome: Finalise the issues, report in R2-2004048

Deadline: 27-04-2020, 10:00 UTC

After email:

Agreements [AT109bis-e][313]: GWUS - 1-1: For NB-IoT and eMTC, the existing capability wakeUpSignalMinGap-eDRX-r15 also applies to Rel-16 WUS. - 1-2: For NB-IoT, Rel-16 GWUS is only applicable to FDD. - 1-3: For eMTC, separate capability indications are introduced for FDD and TDD. - 1-4: For NB-IoT and eMTC, Rel-16 GWUS is applicable to both EPC and 5GC, and there is no need for capability differentiation. Multi-TB scheduling - 2-1: For NB-IoT, multiple TB scheduling in unicast and in multicast is only applicable to FDD. - 2-2: For NB-IoT and eMTC, multiple TB scheduling in unicast is applicable to both EPC and 5GC without differentiation. - 2-3: For NB-IoT and eMTC, multiple TB scheduling in multicast is only applicable to EPC SON - 3-1: For NB-IoT, support of ANR, RACH report and RLF report are applicable to both FDD and TDD and there is no need for FDD/TDD differentiation. DL channel quality reporting in MSG3 - 4-1: For NB-IoT, move the feature DL channel quality reporting in MSG3 for non-anchor carrier to section 6.17. - 4-2’: DL channel quality reporting in Msg3 for NB-IoT anchor carrier and DL channel quality reporting in Msg3 for eMTC are two separate optional features. - 4-3’: For NB-IoT, update the description of the legacy feature DL channel quality reporting in MSG3 (6.17.2) to reflect that it applies to the anchor carrier. - 4-4: For NB-IoT, DL channel quality reporting in MSG3 for non-anchor carrier is only applicable to FDD. - 4-5: For NB-IoT and eMTC, DL channel quality reporting in MSG3 is applicable to both EPC and 5GC without capability differentiation. DL channel quality reporting in connected mode - 5-1: Keep a common capability for NB-IoT and eMTC for DL channel quality reporting in connected mode and clarify in the description that reporting of the serving cell applies to E-UTRAN and reporting of the configured carrier applies to NB-IoT. - 5-2: For NB-IoT, DL channel quality reporting in MSG3 in connected mode is only applicable to FDD. For eMTC, it is applicable to both FDD and TDD. - 5-3: For NB-IoT and eMTC, DL channel quality reporting in connected mode is applicable to both EPC and 5GC without capability differentiation. NRS presence on non-anchor carrier - 6-1: For NB-IoT, Idle mode RRM measurements on non–anchor paging carrier is only applicable to FDD. - 6-2: For NB-IoT, Idle mode RRM measurement on non–anchor carrier is applicable to EPC and 5GC without capability differentiation. Inter-RAT cell selection - 7-1’: FFS [CB] - For NB-IoT and eMTC, there is no need to define a optional feature for support of assistance information for inter-RAT cell selection to/from NB-IoT. Co-existence with NR - 8-1: For NB-IoT and eMTC, UL and DL resource reservation for coexistence with NR are applicable to EPC and 5GC without capability differentiation. Connection to 5GC - 9-1: For NB-IoT, introduce a new optional feature, NB-IoT/5GC, in section 6.18. - 9-2: For NB-IoT and eMTC, remove the capabilities introduced in 6.18.1 (User Plane CIoT 5GS optimisations) and 6.18.2 (Control Plane CIoT 5GS optimisations). - 9-3: For NB-IoT and eMTC, introduce a new optional feature, MO-EDT for Control Plane CIoT 5GS Optimisation, in section 6.18 and remove the editor’s note in 6.8.4. - 9-4’: For NB-IoT, all pre-Rel15 capabilities not CIoT EPS optimisations related and other than rai-Support-r14 are applicable to 5GC without capability differentiation. FFS [CB] rai-Support-r14. - 9-5’: FFS [CB] - For NB-IoT and eMTC connected to 5GC, support of AS RAI enhancement is optional at the UE - 9-6’: For eMTC, introduce a new capability, ce-eutra-5GC, for support of connection to 5GC. - 9-6’’: For eMTC non-BL UEs, introduce new capabilities, ce-eutra-5GC-HO-ToNR-FDD-FR1, ce-eutra-5GC-HO-ToNR-TDD-FR1, ce-eutra-5GC-HO-ToNR-FDD-FR2 and ce-eutra-5GC-HO-ToNR-TDD-FR2 for support of connection to 5GC.

### 7.2.6	ASN.1 review of NB-IoT

Including documents related to Class 2/3 ASN.1 review issues that require WI-specific discussion.

A web conference will be used for handling some of the discussions in this AI.

R2-2003250	[H108][H109] TP on WUS sugnalling for per gap configuration	Huawei, HiSilicon	discussion	Rel-16	NB_IOTenh3-Core, LTE_eMTC5-Core	Late

R2-2003251	[H228][H229] TP on multipe TB schedullng in NB-IoT	Huawei, HiSilicon	discussion	Rel-16	NB_IOTenh3-Core	Late

Moved from AI 7.2.3

Related to [Z603]

R2-2003278	Capture RRC setup using PUR	ZTE Corporation, Sanechips	draftCR	Rel-16	36.331	16.0.0	LTE_eMTC5-Core, NB_IOTenh3-Core

[AT109bis-e][314][NBIOT] ASN.1 review of NB-IoT (Huawei)

Scope: ASN.1 WI specific issues discussion

Intended outcome: progress the ASN.1 review and conclude as much as possible, report in R2-2004049

Deadline: 27-04-2020, 10:00 UTC

R2-2004049	Summary of [AT109bis-e][314][NBIOT] ASN.1 review of NB-IoT (Huawei)

Potential Easy agreements [CB] 1: H084, H089, H116, H127, H130, H134, H133, H136b, H141, H144, H143, H150: Status set to ConcAgree with the additional suggestions in the comment column. 2: N014: Status set to ConcNoAct. 4: N001: Status changed to ConcAgree with parameter renamed to rrc-ACK 6a: H122/H125: Status changed to ConcAgree. No condition for inclusion of newUE-Identity-r16. 7b: H109: Status changed to ConcAgree with the changes corresponding to Alternative 2. 8: H105: Status changed to ConcAgree with the following changes for both eMTC and NB-IoT: - gwus-CommonSequence-r16 definition is changed to ENUMERATED {g0, g126} with the following field description gwus-CommonSequence Presence of the field indicates common WUS sequence is configured. Value g0 indicates common WUS sequence for the shared WUS resource is g=0. Value g126 indicates common WUS sequence for the shared WUS resource is g=126, see TS 36.211 [21]. 9: H106: Status changed to ConcAgree with the following changes (eMTC): gwus-FreqLocation Frequency location of group WUS resource 0 within paging narrowband for BL UEs and UEs in CE. Value n0 corresponds to WUS resource 0 in the 1st and 2nd PRB and value n2 represents the 3rd and 4th PRB gwus-ResourcePattern Identifies the group WUS resource mapping to time/frequency as defined in TS 36.304 [4]. If wus-Config-r15 is present in SystemInformationBlockType2, the field is set to value gwus-ResourcePatternWithLegacy; otherwise the field is set to value gwus-ResourcePatternWithoutLegacy. If the field is set to gwus-ResourcePatternWithLegacy, frequency location of group WUS resource 0 is defined by freqLocation-r15 (in WUS-Config). If the field is set to gwus-ResourcePatternWithoutLegacy, frequency location of group WUS resource 0 is defined by gwus-FreqLocation-r16.. 10: H107: Status changed to ConcAgree. 11a: H110: Status changed to ConcAgree. Only change 1) conditional presence. 12: H081/H086: Status changed to ConcAgree. 14: H094: Status changed to ConcAgree. 15: H095: Status changed to ConcAgree with anr-CarrierList-r16 being mandatory. 16: Z607: Status changed to ConcAgree. 17: H146: Status changed to ConcAgree with anr-CarrierList-r16 being mandatory in ANR-MeasConfig-NB-r16 and VarANR-MeasConfig-NB-r16. 19: H096: Status changed to ConcAgree. 20: H228/H229: Status changed to ConcAgree with the following changes: - change {interleaving, non-interleaving} to {interleaved, nonInterleaved} - move the Cond twoHARQ to the top level IEs 21: H118: Status changed to ConcAgree. 22: H148: Status changed to ConcAgree. 23a: H091: Status changed to ConcAgree. Tentative agreements [CB] 5: H098: FFS Status changed to ConcAgree with the following changes: - section 5.6.23.3: text is changed as below: 1> if UE preference is that no RRC response message is neededsent for acknowledging the reception of a transmission using PUR, include rrc-ACK set l1-ACK to TRUE;. - PURConfigurationRequest/ PURConfigurationRequest-NB : parameter is renamed to rrc-ACK-16 in the ASN.1 with the following field description: rrc-Ack Presence of this field indicates that a RRC response message for transmission using PUR is requested. For further discussion [CB] 3: Z603 – FFS pending on [AT109bis-e][311][NBIOT] PUR open issues 6b: Discuss whether newUE-Identity-r16 should be moved from RRCConnectionSetup(-NB)/ RRCRonnectionResume(-NB) to RadioResourceConfigDedicated(-NB) 7a:H108: RAN2 to discuss whether to make timeOffset-eDRX-Short optional. - if timeOffset-eDRX-Short is kept mandatory, then adopt the changes in R2-2003250 alternative 2 - if timeOffset-eDRX-Short is made optional, then adopt the equivalent changes to timeOffset-eDRX-long in R2-2003250 alternative 2 for gwus-ResourceConfig-eDRX-short 11b: H110: RAN2 to discuss whether it is possible to have no group configured for a configured probability threshold. 13: H081/H086- FFS whether and where to clarify that support for early contention resolution is mandatory for UE connected to 5GC. 18: RAN2 to discuss whether to introduce provision to introduce full carrier EARFCN value in anr-carrierList. 23b: H091: FFS whether the note should be made applicable to 5GC.