3GPP TSG-RAN WG2 Meeting #109bis-e	R2-200xyzw

Online, 20 April – 30 April 2020

Source: 	Session Chair (MediaTek)

Title:	Report of session on Rel-15 and -16 LTE and NR positioning

# Status of At-Meeting Email Discussions

This subclause is not an Agenda Item. It contains a running summary of the email discussions assigned to take place during the meeting weeks.  This section will be moved to an appendix in the final version of the report.

[AT109bis-e][600] Organisational Nathan - Positioning (MediaTek)

Status: Started

Scope: Organisational discussions and announcements, as needed throughout the meeting weeks

Intended outcome: Well-informed participants

Deadline:  Thursday 2020-04-30 1000 UTC

[AT109bis-e][601][POS] LPP ASN.1 issue gathering and easy agreements (Qualcomm)

Status: Started

Scope: Collect the issues from the contributions in agenda item 6.8.2.4, excluding R2-2003143 and R2-2003144

Intended outcome: Endorsed baseline CR starting from R2-2003350 (R2-2003981) and list of open issues (R2-2003982) for continuing ASN.1 review after this meeting

Deadline:  Comments by Monday 2020-04-27 1000 UTC; final output documents by Wednesday 2020-04-29 1000 UTC

[AT109bis-e][602][POS] LPP ASN.1 structural issues (Ericsson)

Status: Started

Scope: Initial discussion on the issues raised in R2-2003144

Intended outcome: Report of potential easy agreements and remaining open issues (R2-2003983) suitable for capture in the LPP ASN.1 issue list

Deadline:  Comments by Tuesday 2020-04-28 1000 UTC; output document Wednesday 2020-04-29 1000 UTC

[AT109bis-e][603][POS] Introduction of NavIC Keplerian set IE (Reliance Jio)

Status: Started

Scope: Review of the CR submitted in R2-2003821 to introduce the definition of NavModel-NavIC-KeplerianSet

Intended outcome: Agreed-in-principle CR

Deadline:  Wednesday 2020-04-29 1000 UTC

# 4	EUTRA corrections Rel-15 and earlier

See Appendix A for reference to Work items, work item codes and WIDs.

No documents should be submitted to 4. Please submit to 4.x

NOTE For R2 109e for R15 and earlier releases, only documents on important and urgent issues shall be submitted and treated. No text enhancements without behavioural or functional change.

## 4.4	Positioning corrections Rel-15 and earlier

Documents in this agenda item will be handled by email.  No web conference is planned for this agenda item.

# 5	WI: New Radio (NR) Access Technology

(NR_newRAT-Core; leading WG: RAN1; REL-15; started: Mar. 17; closed: Jun. 19: WID: RP-191971)

NOTE For R2 109bis-e for R15 and earlier releases, only documents on important and urgent issues shall be submitted and treated. No text enhancements without behavioural or functional change.

## 5.2	Stage 2

### 5.2.3	Positioning

Corrections to both the stage 2 and stage 3 aspects related to positioning.  Stage 2 CRs should be discussed with the specification rapporteur before submission.

Documents in this agenda item will be handled in a break out session.

R2-2002913	Clarification on UE Positioning Architecture in 38.305 for Rel-15	CATT	draftCR	Rel-15	38.305	15.5.0	B	NR_newRAT-Core

R2-2003479	Correction to periodic reporting	Huawei, HiSilicon	CR	Rel-15	37.355	15.0.0	0254	-	F	NR_newRAT-Core

R2-2003482	Correction to periodic reporting	Huawei, HiSilicon	CR	Rel-16	37.355	16.0.0	0255	-	A	NR_newRAT-Core

## 6.8	NR Positioning Support

(NR_pos-Core; leading WG: RAN1; REL-16; started: Mar 19; target; Jun 20; WID: RP-200218, SR: RP-200217). Documents in this agenda item will be handled in a break out session

Time budget: 1 TU

### 6.8.1	Organisational

Including incoming LSs, rapporteur inputs, etc.

R2-2002520	LS on outcome of email discussions on aperiodic SRS for positioning configuration from RAN1#100e (R1-2001483; contact: Ericsson)	RAN1	LS in	Rel-16	NR_pos	To:RAN2

R2-2002529	LS on gNB measurements report mapping for NR Positioning (R4-2002280; contact: Qualcomm)	RAN4	LS in	Rel-16	NR_pos-Core	To:RAN2, RAN3	Cc:RAN1

R2-2003316	Discussion on capabilities for NR positioning	Intel Corporation	discussion	Rel-16	NR_pos-Core	Late

R2-2003317	Introduction of UE positioning capabilities	Intel Corporation	draftCR	Rel-16	37.355	16.0.0	NR_pos-Core	Late

### 6.8.2	Architecture and protocol aspects

No documents should be submitted to 6.8.2.  Please submit to 6.8.2.x.

#### 6.8.2.1	Stage 2

Including impact to 36.305 and 38.305.  This agenda item may utilize a summary document to facilitate treatment of topics during the e-meeting (decision to be made based on submitted tdocs).

Including outcome of email discussion [Post109e#30][NR/Pos] Non-periodic SRS for positioning (Huawei)

Including outcome of email discussion [Post109e#31][NR/Pos] Details of spatial relation for positioning (Huawei)

Contributions on issues already resolved in email discussions [Post109e#30] and [Post109e#31] are discouraged.

Tdoc limitation: 1 tdoc

Non-periodic SRS

R2-2003068	[Post109e-30][NRPos] Non-periodic SRS for positioning (Huawei)	Huawei, HiSilicon	discussion	Rel-16	NR_pos-Core

R2-2003054	DraftLS_RAN3_non-periodicSRSPositioning	Huawei, HiSilicon	discussion	Rel-16	NR_pos-Core

Spatial relation and SSB configuration

R2-2003069	[Post109e-31][Pos] Details of spatial relation for positioning (Huawei)	Huawei, HiSilicon	discussion	Rel-16	NR_pos-Core

R2-2003055	DraftCR for SSB configuration in LPP spec	Huawei, HiSilicon	draftCR	Rel-16	37.355	16.0.0	NR_pos-Core

R2-2003056	DraftCR for SSB configuration in RRC spec	Huawei, HiSilicon	draftCR	Rel-16	38.331	16.0.0	NR_pos-Core

R2-2003057	DraftLS_RAN3_On Spatial relations for positioning	Huawei, HiSilicon	discussion	Rel-16	NR_pos-Core

Summary document

R2-2003620	Summary document for agenda item 6.8.2.1 - NR Positioning Stage 2	Nokia, Nokia Shanghai Bell	discussion	Rel-16	NR_pos-Core	Late

The following documents will not be individually treated

R2-2002914	Clarification on UE Positioning Architecture in 38.305 for Rel-16	CATT	draftCR	Rel-16	38.305	16.0.0	B	NR_pos-Core

R2-2002939	Discussion on reusing Rel-15 SRS for Multi-RTT	ZTE Corporation	discussion

R2-2003060	Text proposal to stage-2 specification	Huawei, HiSilicon	discussion	Rel-16	NR_pos-Core

R2-2003348	Various Corrections to NR Positioning	Qualcomm Incorporated	discussion

R2-2003396	Text Proposal to clarify the meaning of GNSS term	ESA, Nokia, Nokia Shanghai Bell	discussion	Rel-16	NR_pos-Core

R2-2003731	On supporting of non-periodic SRS for positioning	Samsung R&D Institute UK	discussion

#### 6.8.2.2	RRC

Including impact to 36.331 and 38.331.  This agenda item will utilize a summary document to facilitate treatment of topics during the e-meeting.

Tdoc limitation: 1 tdoc

Summary document

R2-2003769	Summary of agenda item 6.8.2.2 for RRC	Huawei	discussion	Rel-16	NR_pos-Core

The following documents will not be individually treated

R2-2002598	Broadcast of additional assistance data	NextNav, AT&T, FirstNet, Intel, Polaris Wireless	CR	Rel-16	38.331	16.0.0	1508	-	C	NR_pos, NR_pos-Core

R2-2002617	Discussion on GAP configuration and request for NR positioning	vivo	discussion	Rel-16	NR_pos-Core

R2-2003059	DraftCR on LocationMeasurementIndication	Huawei, HiSilicon	draftCR	Rel-16	38.331	16.0.0	NR_pos-Core

R2-2003136	Recommendation message from LMF to gNB for SRS configuration	Ericsson	discussion	Rel-16

R2-2003137	UL SRS UE Capability	Ericsson	discussion

R2-2003729	SSB configuration for DL-/UL-only method in RRC	Samsung R&D Institute UK	discussion

#### 6.8.2.3	LPP

This agenda item will utilize a summary document to facilitate treatment of topics during the e-meeting. Note that documents on specific ASN.1 issues should be submitted to AI 6.8.2.4.

Tdoc limitation: 1 tdoc

Summary document

R2-2003783	Summary of LPP agenda item 6.8.2.3	Qualcomm Incorporated	discussion	Rel-16	NR_pos-Core

The following documents will not be individually treated

R2-2002938	Discussion on additional path reporting	ZTE Corporation	discussion

R2-2003061	Remaining issues with LPP	Huawei, HiSilicon	discussion	Rel-16	NR_pos-Core

R2-2003130	Measurement Reporting for UE based positioning	Ericsson	discussion	Rel-16

- Revised in R2-2003811

R2-2003811	Measurement Reporting for UE based positioning	Ericsson, Deutsche Telekom	discussion	Rel-16

=> Revised in R2-2003822

R2-2003822	Measurement Reporting for UE based positioning	Ericsson, Deutsche Telekom	discussion	Rel-16	NR_pos-Core

R2-2003318	Handling on TRP-ID	Intel Corporation	discussion	Rel-16	NR_pos-Core

R2-2003730	UE Rx – Tx time difference definition in LPP	Samsung R&D Institute UK	discussion

#### 6.8.2.4	LPP ASN.1 issues

Any issues related only to the details of ASN.1 in 37.355.  CRs should not be submitted to this agenda item except by the specification rapporteur.

The ASN.1 review process for LPP will proceed from company contributions in this meeting.  Issues should be submitted under this agenda item and will be collected by the specification rapporteur.  The review process will proceed by email after this meeting with issues to be concluded at RAN2#110-e.

[AT109bis-e][601][POS] LPP ASN.1 issue gathering and easy agreements (Qualcomm)

Scope: Collect the issues from the contributions in agenda item 6.8.2.4, excluding R2-2003143 and R2-2003144

Intended outcome: Endorsed baseline CR starting from R2-2003350 (R2-2003981) and list of open issues (R2-2003982) for continuing ASN.1 review after this meeting

Deadline:  Wednesday 2020-04-29 1000 UTC

[AT109bis-e][602][POS] LPP ASN.1 structural issues (Ericsson)

Scope: Initial discussion on the issues raised in R2-2003144

Intended outcome: Report of potential easy agreements and remaining open issues (R2-2003983) suitable for capture in the LPP ASN.1 issue list

Deadline:  Comments by Tuesday 2020-04-28 1000 UTC; output document Wednesday 2020-04-29 1000 UTC

R2-2002915	Clarification on SFN0-Offset and DL-AoD report in LPP ASN.1	CATT	draftCR	Rel-16	37.355	16.0.0	B	NR_pos-Core

R2-2003066	DraftCR for NR-DL-PRS-Config	Huawei, HiSilicon	draftCR	Rel-16	37.355	16.0.0	NR_pos-Core

R2-2003067	Miscellaneous Corrections to LPP ASN.1	Huawei, HiSilicon	discussion	Rel-16	NR_pos-Core

R2-2003143	Overhead in current structure	Ericsson	discussion	Rel-16

R2-2003144	Important LPP structural aspects	Ericsson	discussion	Rel-16

R2-2003349	Various Corrections to NR Positioning	Qualcomm Incorporated	discussion

R2-2003350	LPP clean-up	Qualcomm Incorporated	discussion	Rel-16	NR_pos-Core	Late

R2-2003781	CR 37.355 V16.0.0, Corrections to the introduction of NR positioning	Ericsson	CR	Rel-16	37.355	16.0.0	0256	-	F	NR_pos-Core

#### 6.8.2.5	MAC

Including impact to 38.321.

Tdoc limitation: 1 tdoc

LCID/eLCID

R2-2003135	Change LCID to eLCID for SP Positioning SRS Activation/Deactivation MAC CE	Ericsson	CR	Rel-16	38.321	16.0.0	0720	-	F	NR_pos-Core

R2-2003062	Correction to SP SRS actication deactivation MAC CE	Huawei, HiSilicon	draftCR	Rel-16	38.321	16.0.0	NR_pos-Core

DRX

R2-2002618	Discussion on the impact of DRX on SRS for NR positioning	vivo	discussion	Rel-16	NR_pos-Core

Other

R2-2003063	Runnnig CR to MAC spec for R16 Positioning	Huawei, HiSilicon	draftCR	Rel-16	38.321	16.0.0	NR_pos-Core

=> Revised in R2-2003768

R2-2003768	Running CR to MAC spec for R16 Positioning	Huawei, HiSilicon	draftCR	Rel-16	38.321	16.0.0	NR_pos-Core

#### 6.8.2.6	Broadcast assistance data

This agenda item will utilize a summary document to facilitate treatment of topics during the e-meeting.

Tdoc limitation: 1 tdoc

Summary document

R2-2003607	Summary for Broadcast of Assistance Data	CATT	discussion	Late

The following documents will not be individually treated

R2-2002916	Summary of the agreement and left issues on Broadcast Assistance Data	CATT	discussion	Rel-16	38.331	NR_pos-Core

R2-2003058	DraftCR for on-demand SI request for positioning	Huawei, HiSilicon	draftCR	Rel-16	38.331	16.0.0	NR_pos-Core

R2-2003132	On the need of unicast tag for positioning si-BroadcastStatus	Ericsson	discussion	Rel-16

- Revised in R2-2003810

R2-2003810	On the need of unicast tag for positioning si-BroadcastStatus	Ericsson, Deutsche Telekom	discussion	Rel-16

Withdrawn/Not available

R2-2003140	On Relevance of Broadcast	Ericsson	discussion	Rel-16	Withdrawn

#### 6.8.2.7	UE-based positioning

This agenda item may utilize a summary document to facilitate treatment of topics during the e-meeting (decision to be made based on submitted tdocs).

Tdoc limitation: 1 tdoc

R2-2003064	Discussion on UE-based positioning	Huawei, HiSilicon	discussion	Rel-16	NR_pos-Core

R2-2003145	Remaining issues with NR RAT dependent UE-based positioning	Ericsson	discussion	Rel-16

### 6.8.3	Other

Tdoc limitation: 1 tdoc

R2-2003065	Discussion on UL-ECID	Huawei, HiSilicon	discussion	Rel-16	NR_pos-Core

R2-2003376	On UE RxTx Measurements	Ericsson	discussion	Rel-16

## 6.20	NR TEI16 enhancements

Small Technical Enhancements to NR. TEI should be predominantly within a single WG and fully completed within the same quarter in all affected WGs. RAN2 impact of RAN1/4-led TEI shall be limited to RRC signalling of configuration parameters and UE capabilities (no MAC impact, no RRC procedural impact, etc). Please also see RP-191602 endorsed at RAN#84. Please submit to 6.20.x.

NOTE that proponent companies are responsible to ensure that correct CRs are provided in all groups for proposals that have impact in >1 working group.

Time budget: 1 TU

Tdoc Limitation: 2 tdocs. NOTE for TEI, the tdoc limitation applies to new proposals, not to open proposals since previous meeting(s)

### 6.20.1	RAN2 led TEI16 enhancements - Control plane related

#### 6.20.1.1		Open / ongoing proposals

R2-2003142	Transfer of unicast RS observations with GNSS integer ambiguity level information	Ericsson

# 7	Rel-16 LTE Work Items

Documents in these agenda items will be handled in break out sessions

## 7.7	 Support of Indian Navigation Satellite System NavIC

(LCS_NAVIC; leading WG: RAN2; REL-16; started: Sept 19; target; March-20; WID: RP-192350)

Time budget: 0 TU Final agreement of CRs is expected

This item is 100%

R2-2003821	Update missed out definition for Information Element NavModel-NavIC-KeplerianSet	Reliance Jio	CR	Rel-16	37.355	16.0.0	0257	-	F	LCS_NAVIC-Core

[AT109bis-e][603][POS] Introduction of NavIC Keplerian set IE (Reliance Jio)

Scope: Review of the CR submitted in R2-2003821 to introduce the definition of NavModel-NavIC-KeplerianSet

Intended outcome: Agreed-in-principle CR

Deadline:  Wednesday 2020-04-29 1000 UTC