3GPP TSG-RAN WG2 Meeting #109bis-e	R2-200xyzw

Online, 20 April – 30 April 2020

Source: 	Session Chair (MediaTek)

Title:	Report of session on Rel-15 and -16 LTE and NR positioning

# Status of At-Meeting Email Discussions

This subclause is not an Agenda Item. It contains a running summary of the email discussions assigned to take place during the meeting weeks.  This section will be moved to an appendix in the final version of the report.

[AT109bis-e][600] Organisational Nathan - Positioning (MediaTek)

Status: Started

Scope: Organisational discussions and announcements, as needed throughout the meeting weeks

Intended outcome: Well-informed participants

Deadline:  Thursday 2020-04-30 1000 UTC

[AT109bis-e][601][POS] LPP ASN.1 issue gathering and easy agreements (Qualcomm)

Status: Started

Scope: Collect the issues from the contributions in agenda item 6.8.2.4, excluding R2-2003143 and R2-2003144

Intended outcome: Endorsed baseline CR starting from R2-2003350 (R2-2003981) and list of open issues (R2-2003982) for continuing ASN.1 review after this meeting

Deadline:  Comments by Monday 2020-04-27 1000 UTC; final output documents by Wednesday 2020-04-29 1000 UTC

[AT109bis-e][602][POS] LPP ASN.1 structural issues (Ericsson)

Status: Started

Scope: Initial discussion on the issues raised in R2-2003144

Intended outcome: Report of potential easy agreements and remaining open issues (R2-2003983) suitable for capture in the LPP ASN.1 issue list

Deadline:  Comments by Tuesday 2020-04-28 1000 UTC; output document Wednesday 2020-04-29 1000 UTC

[AT109bis-e][603][POS] Introduction of NavIC Keplerian set IE (Reliance Jio)

Status: Completed

Scope: Review of the CR submitted in R2-2003821 to introduce the definition of NavModel-NavIC-KeplerianSet

Intended outcome: Agreed-in-principle CR in R2-2003998

Deadline:  Wednesday 2020-04-29 1000 UTC

[AT109bis-e][604][POS] LS on support of non-periodic SRS and SSB configuration (Intel)

Status: Completed

Scope: Draft an LS to RAN1/RAN3 in reply to R2-1914310, indicating that:

- the RAN2 signalling can currently configure aperiodic SRS for positioning, but we need to know if it the specification effort is feasible to complete from RAN3 perspective.

- Activation/deactivation of SP SRS is recommended by LMF to gNB with the final activation/deactivation by gNB, and NRPPa signalling for SRS triggering is needed.

- For the assistance information in NRPPa for SSB configuration for UL-only positioning, it should include both TF configuration and SSB index in the NRPPa message.

Intended outcome: Approved LS

Deadline:  Wednesday 2020-04-22 1200 UTC

[AT109bis-e][605][POS] Checking of draft CRs on SSB configuration (Huawei)

Status: Completed

Scope: Check the draft CRs in R2-2003055 and R2-2003056.

Intended outcome: Agreed in principle CRs in R2-2003985 and R2-2003986 (the latter is revised to R2-2003987)

Deadline:  Wednesday 2020-04-22 1200 UTC

[AT109bis-e][606][POS] MAC proposals (Huawei)

Status: Started

Scope: Discuss the proposals from R2-2003135, R2-2003062, R2-2002618, and R2-2003768, and develop a CR incorporating the agreeable proposals

Intended outcome: Agreed in principle CR to 38.321, in R2-2003988

Deadline:  Wednesday 2020-04-29 1000 UTC

[AT109bis-e][607][POS] Update of 38.305 (Qualcomm)

Status: Started

Scope: Update 38.305 in light of the meeting agreements, using R2-2003348 as baseline.

Intended outcome: Agreed in principle CR in R2-2003990

Deadline:  Wednesday 2020-04-29 1000 UTC

[AT109bis-e][608][POS] GNSS terminology updates (ESA)

Status: Started

Scope: Conclude on the clarifications of GNSS terminology in 36.305 and 38.305 from R2-2003396.

Intended outcome: Agreed in principle CRs for Rel-15 and Rel-16: R2-2003991 (36.305 Rel-15), R2-2003992 (36.305 Rel-16), R2-2003993 (38.305 Rel-15), R2-2003994 (38.305 Rel-16)

Deadline:  Wednesday 2020-04-29 1000 UTC

[AT109bis-e][609][POS] Checking of CRs on proprietary interface between E-SMLC and LMF (CATT)

Status: Completed

Scope: Checking of the CRs from R2-2002914 and R2-2002913.

Intended outcome: Agreed in principle CRs for Rel-15 and Rel-16: R2-2003995 (Rel-15), R2-2003996 (Rel-16)

Deadline:  Friday 2020-04-24 1000 UTC

[AT109bis-e][610][POS] LPP proposals (Ericsson)

Status: Started

Scope: Discuss proposals 2, 4, 5, 6, 7, 8 from R2-2003783

Intended outcome: Summary of agreements in R2-2003997

Deadline:  Wednesday 2020-04-29 1000 UTC

# 4	EUTRA corrections Rel-15 and earlier

See Appendix A for reference to Work items, work item codes and WIDs.

No documents should be submitted to 4. Please submit to 4.x

NOTE For R2 109e for R15 and earlier releases, only documents on important and urgent issues shall be submitted and treated. No text enhancements without behavioural or functional change.

## 4.4	Positioning corrections Rel-15 and earlier

Documents in this agenda item will be handled by email.  No web conference is planned for this agenda item.

# 5	WI: New Radio (NR) Access Technology

(NR_newRAT-Core; leading WG: RAN1; REL-15; started: Mar. 17; closed: Jun. 19: WID: RP-191971)

NOTE For R2 109bis-e for R15 and earlier releases, only documents on important and urgent issues shall be submitted and treated. No text enhancements without behavioural or functional change.

## 5.2	Stage 2

### 5.2.3	Positioning

Corrections to both the stage 2 and stage 3 aspects related to positioning.  Stage 2 CRs should be discussed with the specification rapporteur before submission.

Documents in this agenda item will be handled in a break out session.

R2-2002913	Clarification on UE Positioning Architecture in 38.305 for Rel-15	CATT	draftCR	Rel-15	38.305	15.5.0	B	NR_newRAT-Core

- Revised in R2-2003995 (in offline discussion [AT109b-e][609])

R2-2003479	Correction to periodic reporting	Huawei, HiSilicon	CR	Rel-15	37.355	15.0.0	0254	-	F	NR_newRAT-Core

Qualcomm understand that this was intentional and values ri0-25 and ri0-5 really do map to 1 s and 2 s.

- Not pursued

R2-2003482	Correction to periodic reporting	Huawei, HiSilicon	CR	Rel-16	37.355	16.0.0	0255	-	A	NR_newRAT-Core

## 6.8	NR Positioning Support

(NR_pos-Core; leading WG: RAN1; REL-16; started: Mar 19; target; Jun 20; WID: RP-200218, SR: RP-200217). Documents in this agenda item will be handled in a break out session

Time budget: 1 TU

### 6.8.1	Organisational

Including incoming LSs, rapporteur inputs, etc.

R2-2002520	LS on outcome of email discussions on aperiodic SRS for positioning configuration from RAN1#100e (R1-2001483; contact: Ericsson)	RAN1	LS in	Rel-16	NR_pos	To:RAN2

Ericsson understand that this issue has been raised in the ASN.1 review.  Huawei confirm there is a CR.

- Noted

R2-2002529	LS on gNB measurements report mapping for NR Positioning (R4-2002280; contact: Ericsson)	RAN4	LS in	Rel-16	NR_pos-Core	To:RAN2, RAN3	Cc:RAN1

- Noted

R2-2003316	Discussion on capabilities for NR positioning	Intel Corporation	discussion	Rel-16	NR_pos-Core	Late

Intel understand that we may need to wait for RAN1 before treating this.

Come back later in the week when RAN1 have discussed.

R2-2003317	Introduction of UE positioning capabilities	Intel Corporation	draftCR	Rel-16	37.355	16.0.0	NR_pos-Core	Late

[Post109bis-e][xx][POS] UE capabilities for positioning (Intel)

Scope: Discuss documents R2-2003316 and R2-2003317, and related RAN1 decisions, and progress towards an agreeable CR introducing the capabilities into 37.355.

Intended outcome: Summary and CR to next meeting

Deadline:  Long

### 6.8.2	Architecture and protocol aspects

No documents should be submitted to 6.8.2.  Please submit to 6.8.2.x.

#### 6.8.2.1	Stage 2

Including impact to 36.305 and 38.305.  This agenda item may utilize a summary document to facilitate treatment of topics during the e-meeting (decision to be made based on submitted tdocs).

Including outcome of email discussion [Post109e#30][NR/Pos] Non-periodic SRS for positioning (Huawei)

Including outcome of email discussion [Post109e#31][NR/Pos] Details of spatial relation for positioning (Huawei)

Contributions on issues already resolved in email discussions [Post109e#30] and [Post109e#31] are discouraged.

Tdoc limitation: 1 tdoc

Non-periodic SRS

R2-2003068	[Post109e-30][NRPos] Non-periodic SRS for positioning (Huawei)	Huawei, HiSilicon	discussion	Rel-16	NR_pos-Core

Proposal1: RAN2 should discuss whether aperiodic SRS is supported for R16 positioning.

Qualcomm understand that RAN1 agreed to support this and it is already in our specs; the only thing missing is the RAN3 support.

Intel agree from RAN2 perspective the only impact is stage 2 for activation/deactivation, but they see support of aperiodic SRS for neighbour cells as difficult.

Huawei agree with Qualcomm that from RAN2 perspective there is nothing missing, but they also understand that RAN1 left it to RAN2 to evaluate if it is feasible.  We should guarantee it is feasible if we want to support it.

Ericsson think RAN1 should have confirmed feasibility, and think there would be capability signalling.  They see no reason why we couldn’t support aperiodic in the signalling.

Nokia think we should be focussing on finalising the release and this is more of a functional issue, so it should perhaps be postponed to next release if there is disagreement on the feasibility.  We also need to take RAN3 effort into account.

Qualcomm agree with Ericsson and think there is no RAN2 spec impact.  If we do not support it in Rel-16 we would need to remove it from our specs.  They agree there is NRPPa work to be done.  The complexity is in network implementation.

Intel understand that RAN1 agreed to support aperiodic SRS to the serving gNB, and had some discussion of the difficulty of supporting it for neighbouring cells, and this is why they asked RAN2/3 to confirm feasibility.  Suggest we could send an LS to RAN3 asking about their intention to support it.

Ericsson think the complexity for support at the neighbour cell is only for FR2.

Huawei think we cannot report that it is feasible from RAN2 perspective.  They see big differences between aperiodic and SP cases due to the lack of stringent requirements on the transmission timing for the aperiodic case.

Nokia have the same concern as Huawei; they think that by signalling start time we can take the timing into account, but there are questions about whether the neighbour cell can receive it.  They also wonder if RAN3 can complete their work in time.  Intel confirm there is some concern about RAN3 completion.

vivo agree there are problems with neighbour cells, but they think for TRPs under the same serving cell it can be used and at least we should support aperiodic SRS for this case.

Qualcomm think the NRPPa procedures do not distinguish between serving and neighbour cells, and restricting it to the serving cell would be more signalling work.

OPPO think we could send an LS to RAN3, but they understand that from RAN2 perspective we should not indicate that the end-to-end feature is feasible.

- Send an LS to RAN3, indicating that the RAN2 signalling can currently configure aperiodic SRS for positioning, but we need to know if it the specification effort is feasible to complete from RAN3 perspective.  Intel think this could be done in a reply to the previous RAN1 LS. (R2-1914310)

Proposal2: RAN2 should discuss how aperiodic SRS should be supported if it is agreed to be supported.

- Deferred

Proposal3: The Activation/Deactivation of the SP SRS and triggering of AP SRS is recommended by LMF to gNB; and the activation/deactivation MAC CE or triggering DCI are sent by gNB to UE.

Huawei think we can agree to the SP aspect.

Agreements:

The Activation/Deactivation of the SP SRS is recommended by LMF to gNB; and the activation/deactivation MAC CE is sent by gNB to UE (no stage 3 impact for RAN2).

Leave the design of NRPPa message for SRS triggering to RAN3 while capture the purpose and content of the message at stage 2 level in RAN2.

[AT109bis-e][604][POS] LS on support of non-periodic SRS and SSB configuration (Intel)

Scope: Draft an LS to RAN1/RAN3 in reply to R2-1914310, indicating that:

- the RAN2 signalling can currently configure aperiodic SRS for positioning, but we need to know if it the specification effort is feasible to complete from RAN3 perspective.

- Activation/deactivation of SP SRS is recommended by LMF to gNB with the final activation/deactivation by gNB, and NRPPa signalling for SRS triggering is needed.

- For the assistance information in NRPPa for SSB configuration for UL-only positioning, it should include both TF configuration and SSB index in the NRPPa message.

Intended outcome: Approved LS

Deadline:  Wednesday 2020-04-22 1200 UTC

R2-2003984	Draft Response LS on SRS and SSB configuration for NR Positioning	Intel	LSOut	Rel-16	NR_pos-Core	To: RAN3	Cc: RAN1

- Approved as R2-2003989

R2-2003054	DraftLS_RAN3_non-periodicSRSPositioning	Huawei, HiSilicon	discussion	Rel-16	NR_pos-Core

Spatial relation and SSB configuration

R2-2003069	[Post109e-31][Pos] Details of spatial relation for positioning (Huawei)	Huawei, HiSilicon	discussion	Rel-16	NR_pos-Core

P1:

Ericsson think in some cases, as when the UE is moving, the speed of the update is a concern.

Qualcomm think “decided by the gNB” is better terminology than “determined by gNB”.  Ericsson want to clarify the gNB sets the configuration and can diverge from the LMF’s recommendation.  Qualcomm agree with this and think the LMF then needs to be informed.  Nokia wonder if the LMF can take any action based on this information.  Qualcomm understand it affects which TRPs will be involved and the LMF needs to send the SRS configuration to the TRPs.

CATT think if the serving gNB does not know about the neighbouring gNBs, the LMF can help set the spatial relation so the neighbouring gNBs can collect the SRS, and this is why the LMF recommendation is needed.  ZTE think in this case it’s not right to say that the gNB “decides” since all it can do is follow the LMF’s recommendation; they would prefer to say it “chooses”.  Qualcomm and Huawei think this is a matter of gNB implementation.

P5:

OPPO wonder if this is intended for the UL-only methods.  Huawei confirm this was the intention.

P6:

Qualcomm agree we should keep the previous agreement but think this wording is unclear.  They are not sure why RRC index is required.

Agreements:

Spatial relation of SRS is recommended by the LMF and decided by the gNB.  It is up to gNB implementation whether to follow the LMF recommendation.  The gNB informs the LMF of its decision.

UE does not report RSRP of DL-PRS in RRC procedures for SRS configuration.

Keep the current SSB configuration for the DL-only positioning in the LPP message.

Keep the current configuration of SSB in RRC for UL-only positioning. This means that the RRC configuration can carry the full SSB configuration or SSB index and PCI.

For the assistance information in NRPPa for SSB configuration for UL-only positioning, it should include both TF configuration and SSB index in the NRPPa message.

- Include the RAN3-related decisions above in the LS from discussion #604.

R2-2003055	DraftCR for SSB configuration in LPP spec	Huawei, HiSilicon	draftCR	Rel-16	37.355	16.0.0	NR_pos-Core

R2-2003056	DraftCR for SSB configuration in RRC spec	Huawei, HiSilicon	draftCR	Rel-16	38.331	16.0.0	NR_pos-Core

R2-2003057	DraftLS_RAN3_On Spatial relations for positioning	Huawei, HiSilicon	discussion	Rel-16	NR_pos-Core

[AT109bis-e][605][POS] Checking of draft CRs on SSB configuration (Huawei)

Scope: Check the draft CRs in R2-2003055 and R2-2003056.

Intended outcome: Agreed in principle CRs in R2-2003985 and R2-2003986 (the latter is revised to R2-2003987)

Deadline:  Wednesday 2020-04-22 1200 UTC

R2-2003985	Correction on SSB configuration in LPP spec	Huawei, HiSilicon	draftCR	Rel-16	37.355	16.0.0	NR_pos-Core

- Endorsed in email discussion [AT109bis-e][605]

- Will be included in the LPP baseline CR (output of discussion #601)

R2-2003986	Correction on SSB configuration in RRC spec	Huawei, HiSilicon	draftCR	Rel-16	38.331	16.0.0	NR_pos-Core

- Revised in R2-2003987

R2-2003987	Correction on SSB configuration in RRC spec	Huawei, HiSilicon	draftCR	Rel-16	38.331	16.0.0	NR_pos-Core

- Endorsed in email discussion [AT109bis-e][605]

Summary document

R2-2003620	Summary document for agenda item 6.8.2.1 - NR Positioning Stage 2	Nokia, Nokia Shanghai Bell	discussion	Rel-16	NR_pos-Core	Late

P1:

CATT understand that this is a clarification to align to an SA2 spec.

Ericsson think it should be discussed in RAN3 and are not sure if the wording “proprietary” is correct.

Huawei wonder if this also applies to Rel-15.  Chair understands that this is a shadow CR of a Rel-15 proposal.

Intel think the dotted line already means there is no officially specified interface between E-SMLC and LMF.

CATT clarify that the word “proprietary” was copied from SA2.

Nokia observe that there is similar text in 36.305 for the E-SMLC and SLP, with the word “proprietary”.

Ericsson want to check if 36.305 says “proprietary” or “out of scope”.  Qualcomm confirm that “proprietary” is used and they see this CR as an alignment.

P3:

Samsung confirm that they are OK with the agreements on non-periodic SRS.

P4:

CATT think PCI is not required for the UE, but the network would benefit from it for multi-UE management.

vivo prefer to keep all the parameters and modify the TRP-ID description as in the Huawei proposal.

Intel are also fine to keep this information.

Nokia would like to understand the justification to delete it.  Qualcomm clarify that they understood it was obvious and captured elsewhere.

P5:

Intel and Huawei think this is needed.  CATT agree.

Ericsson think the formulation is not ideal as it is for the spatial relation, not reception per se.

vivo wonder if there is a connection to P9.  Huawei intend that this is TRP ID from the RAN3 perspective and P9 will not affect it.

P9:

Intel think that not all the current TRP-Id information is needed in all cases, and for transfer between LMF and gNB we need the PRS ID.  They understand that TRP ID in the RAN3 sense is not needed by the UE.

Ericsson think PRS ID is not really appropriate here and the TRP ID is a good grouping of identifiers that are regularly used together.  They are not sure if divergence between RAN2 and RAN3 terminology is a real problem here, and understand that in the UE-specific cases the PRS ID is the TRP ID.  They think TRP ID should be the identity of the TRP, not a complex structure.

Qualcomm agree that the current “TRP-Id” is not really an ID of the TRP, but a container to collect all identities that could be associated with the TRP.  They disagree that the TRP ID and PRS ID are the same, e.g. there may be TRPs without PRS.  If we break down the TRP-Id IE we would end up with separate IEs in the different cases, and the UE needs some global ID to associate the assistance data together.  They understand that the current IE matches what we have in LTE but puts it in a high-level container.  We could break out the container or rename it to something like “TRP-Identifiers”.

Huawei generally agree with Qualcomm and understand that the TRP ID in the sense of RAN3 is signalled between LMF and gNB, and the PRS ID is agreed by RAN1.  They think PRS ID is enough for the communication between the LMF and the UE, but have some sympathy for Qualcomm’s comment that a container might be needed.  If a container is needed, the name should be changed.

Intel agree that we have divergence from RAN3 and the PRS ID should be used between UE and LMF; there is no need for the RAN3 TRP ID in the LPP protocol.

Ericsson would be fine with discussing the naming and structure under LPP.

Nokia understand from the RAN1 definition that the TRP-Id container is to uniquely identify a PRS resource across different resource sets from a single TRP.  In this case “TRP-Id” would be a misnomer and something like “TRP-UniqueIdToLocatePRS-Resource” would be better.

vivo want to understand if the TRP ID definition in RAN3 matches the TRP-Id IE.  They doubt it is the same.  Chair has the same understanding.  Huawei agree.

Huawei think we can agree that PRS ID is what needs to be transferred from UE to LMF, irrespective of the container naming.  Intel and Qualcomm think this is correct.  Qualcomm understanding is that this is the only place we have confusion about the terminology and in other places we use “TRP ID” correctly (in the RAN3 sense).

P7:

CATT have a concern about the geographical coordinates.  In their understanding this would be configured by OAM to the LMF, rather than transferred by NRPPa.  They think RAN3 could discuss.

Qualcomm think the table is correct and the LMF definitely needs to know this information for position calculation.

Huawei understand that in LTE the geographical information is in SLm interface rather than OAM.  They agree that the details are RAN3 scope, but we need to capture it in stage 2.

Nokia think it could be OAM but this cannot be relied upon for all cases.

P8:

Qualcomm understand that the signalling in LTE supports this information exchange, and it was an oversight that this was not copied.

Huawei think we could take into account the RAN3 naming of this procedure as TRP information exchange.  Qualcomm clarify this is already included in R2-2003348.

P12:

Intel think this needs to be discussed separately because it affects Rel-15.  Qualcomm think we could take the Rel-16 decision now and Rel-15 can follow.

Agreements:

The interface between E-SMLC and LMF will be captured as “proprietary” in 38.305.

The information PCI/GCI/TRP-ID for UL SRS is kept in 38.305 for transfer from LMF to gNB for multi-RTT, UL-TDOA, and UL-AoA positioning.

In Table 8.10.2.4-2, Table 8.13.2.3-2 and Table 8.14.2.3-2 in TS 38.305, change “PCI, GCI, and TRP ID of the TRP for the UE to transmit UL SRS” to “PCI, GCI, and TRP ID of the TRP to receive UL SRS”.  Wording can be fine-tuned in further discussion and in light of what we decide about TRP-ID handling.  TRP ID here refers to the RAN3 definition of the term.

Stage 2 terminology for the information transferred from UE to LMF (replacing the current TRP ID for multi-RTT, DL-TDOA, and DL-AoD) will be aligned with the stage 3 conclusion on IE structure.

The following assistance data are transferred between gNB and LMF for UL-only positioning:

PCI, GCI, and TRP IDs of the TRPs served by the gNB

SSB information of the TRPs (the time/frequency occupancy of SSBs)

Geographical coordinates of the TRPs served by the gNB (include reference location for the receiving antenna of the reference TRP, relative locations for receiving antennas of other TRPs)

LMF initiated assistance data delivery procedure needs to be specified.  To be added to section 5.2.

[AT109bis-e][607][POS] Update of 38.305 (Qualcomm)

Scope: Update 38.305 in light of the meeting agreements, using R2-2003348 as baseline.

Intended outcome: Agreed in principle CR in R2-2003990

Deadline:  Wednesday 2020-04-29 1000 UTC

R2-2003990	Various Corrections to NR Positioning	Qualcomm Incorporated	draftCR	Rel-16	38.305	16.0.0	NR_newRAT-Core

[AT109bis-e][608][POS] GNSS terminology updates (ESA)

Scope: Conclude on the clarifications of GNSS terminology in 36.305 and 38.305 from R2-2003396.

Intended outcome: Agreed in principle CRs for Rel-15 and Rel-16: R2-2003991 (36.305 Rel-15), R2-2003992 (36.305 Rel-16), R2-2003993 (38.305 Rel-15), R2-2003994 (38.305 Rel-16)

Deadline:  Wednesday 2020-04-29 1000 UTC

R2-2003991	[CR to 36.305 on GNSS terminology]	ESA	CR	Rel-15	36.305	15.4.0	xxxx	-	F	NR_newRAT-Core

R2-2003992	[CR to 36.305 on GNSS terminology]	ESA	CR	Rel-16	36.305	16.0.0	xxxx	-	A	NR_newRAT-Core

R2-2003993	[CR to 38.305 on GNSS terminology]	ESA	CR	Rel-15	38.305	15.5.0	xxxx	-	F	NR_newRAT-Core

R2-2003994	[CR to 38.305 on GNSS terminology]	ESA	CR	Rel-16	38.305	16.0.0	xxxx	-	A	NR_newRAT-Core

[AT109bis-e][609][POS] Checking of CRs on proprietary interface between E-SMLC and LMF (CATT)

Scope: Checking of the CRs from R2-2002914 and R2-2002913.

Intended outcome: Agreed in principle CRs for Rel-15 and Rel-16: Agreed in principle CRs for Rel-15 and Rel-16: R2-2003995 (Rel-15), R2-2003996 (Rel-16)

Deadline:  Friday 2020-04-24 1000 UTC

R2-2003995	Clarification on UE Positioning Architecture	CATT	draftCR	Rel-15	38.305	15.5.0	F	NR_pos-Core

- Agreed in principle as R2-2004143

R2-2003996	Clarification on UE Positioning Architecture	CATT	draftCR	Rel-16	38.305	16.0.0	A	NR_pos-Core

- Agreed in principle as R2-2004144

R2-2004143	Clarification on UE Positioning Architecture	CATT	CR	Rel-15	38.305	15.5.0	0009	-	F	NR_pos-Core

- Agreed in principle in email discussion [AT109bis-e][609]

R2-2004144	Clarification on UE Positioning Architecture	CATT	CR	Rel-16	38.305	16.0.0	0010	-	A	NR_pos-Core

- Agreed in principle in email discussion [AT109bis-e][609]

The following documents will not be individually treated

R2-2002914	Clarification on UE Positioning Architecture in 38.305 for Rel-16	CATT	draftCR	Rel-16	38.305	16.0.0	B	NR_pos-Core

R2-2002939	Discussion on reusing Rel-15 SRS for Multi-RTT	ZTE Corporation	discussion

R2-2003060	Text proposal to stage-2 specification	Huawei, HiSilicon	discussion	Rel-16	NR_pos-Core

R2-2003348	Various Corrections to NR Positioning	Qualcomm Incorporated	discussion

R2-2003396	Text Proposal to clarify the meaning of GNSS term	ESA, Nokia, Nokia Shanghai Bell	discussion	Rel-16	NR_pos-Core

R2-2003731	On supporting of non-periodic SRS for positioning	Samsung R&D Institute UK	discussion

#### 6.8.2.2	RRC

Including impact to 36.331 and 38.331.  This agenda item will utilize a summary document to facilitate treatment of topics during the e-meeting.

Tdoc limitation: 1 tdoc

Summary document

R2-2003769	Summary of agenda item 6.8.2.2 for RRC	Huawei	discussion	Rel-16	NR_pos-Core

P6:

Ericsson wonder if this is in scope for the Rel-16 WI but support the concept.

NextNav understand that the broadcast objectives in the work item are broad and this just captures what we agreed in LTE, migrated to NR.  Huawei agree.

Nokia think it is a straightforward change.

Agreement:

Support posSibType4-1 and posSibType5-1 for Barometric Pressure and TBS based on MBS signals in the broadcast system information in 38.331.

The following documents will not be individually treated

R2-2002598	Broadcast of additional assistance data	NextNav, AT&T, FirstNet, Intel, Polaris Wireless	CR	Rel-16	38.331	16.0.0	1508	-	C	NR_pos, NR_pos-Core

- Agreed in principle

R2-2002617	Discussion on GAP configuration and request for NR positioning	vivo	discussion	Rel-16	NR_pos-Core

R2-2003059	DraftCR on LocationMeasurementIndication	Huawei, HiSilicon	draftCR	Rel-16	38.331	16.0.0	NR_pos-Core

R2-2003136	Recommendation message from LMF to gNB for SRS configuration	Ericsson	discussion	Rel-16

R2-2003137	UL SRS UE Capability	Ericsson	discussion

R2-2003729	SSB configuration for DL-/UL-only method in RRC	Samsung R&D Institute UK	discussion

#### 6.8.2.3	LPP

This agenda item will utilize a summary document to facilitate treatment of topics during the e-meeting. Note that documents on specific ASN.1 issues should be submitted to AI 6.8.2.4.

Tdoc limitation: 1 tdoc

Summary document

R2-2003783	Summary of LPP agenda item 6.8.2.3	Qualcomm Incorporated	discussion	Rel-16	NR_pos-Core

Rapporteur’s Proposal 2: RAN2 should discuss the interpretation of additional paths measurements (nr AdditionalPathList) in case of additional timing measurements are reported.

Rapporteur’s Proposal 3: Interesting companies should discuss any enhancements to common reporting in future Releases; e.g. as part of the positioning/latency enhancements study for Rel-17.

Rapporteur’s Proposal 4: RAN2 should discuss the use cases for a new LocationInformationType ‘locationEstimateAndMeasurementsRequired’ in IE CommonIEsProvideLocationInformation first, before introducing the feature in LPP.

Rapporteur’s Proposal 5: RAN2 should discuss whether PSCell/Scell information should be provided by a target device in CommonIEsRequestAssistanceData.

Rapporteur’s Proposal 6: RAN2 should inform RAN1 of the RAN2 discussion and concerns related to the SMTC information in the SSB assistance data, and ask RAN1 for any status update of the working assumption in RAN1.

Rapporteur’s Proposal 7: RAN2 should inform RAN1 of the RAN2 discussion and ask whether PRS-PRS QCL Type D indication is still needed (this may be a combined LS to RAN1 incl. Rapporteur’s Proposal 6).

Rapporteur’s Proposal 8: RAN2 should discuss whether

(a)	to change the name of the IE TRP-ID (e.g., to distinguish from the RAN3 TRP-ID), or

(b)	 to remove IE TRP-ID from LPP and add the relevant TRP-ID fields to the individual parent IEs.

Rapporteur’s Proposal 9: Fix the above issue (Parameter nr-UE-RxTxTimeDiff in LPP multi-RTT location IE should refer 38.215 for the definition) in the LPP Clean-up.

[AT109bis-e][610][POS] LPP proposals (Ericsson)

Scope: Discuss proposals 2, 4, 5, 6, 7, 8 from R2-2003783

Intended outcome: Summary of agreements in R2-2003997

Deadline:  Wednesday 2020-04-29 1000 UTC

R2-2003997	Email discussion report: [AT109bis-e][610][POS] LPP proposals (Ericsson)	Ericsson	discussion	Rel-16	NR_pos-Core

The following documents will not be individually treated

R2-2002938	Discussion on additional path reporting	ZTE Corporation	discussion

R2-2003061	Remaining issues with LPP	Huawei, HiSilicon	discussion	Rel-16	NR_pos-Core

R2-2003130	Measurement Reporting for UE based positioning	Ericsson	discussion	Rel-16

- Revised in R2-2003811

R2-2003811	Measurement Reporting for UE based positioning	Ericsson, Deutsche Telekom	discussion	Rel-16

=> Revised in R2-2003822

R2-2003822	Measurement Reporting for UE based positioning	Ericsson, Deutsche Telekom	discussion	Rel-16	NR_pos-Core

R2-2003318	Handling on TRP-ID	Intel Corporation	discussion	Rel-16	NR_pos-Core

R2-2003730	UE Rx – Tx time difference definition in LPP	Samsung R&D Institute UK	discussion

#### 6.8.2.4	LPP ASN.1 issues

Any issues related only to the details of ASN.1 in 37.355.  CRs should not be submitted to this agenda item except by the specification rapporteur.

The ASN.1 review process for LPP will proceed from company contributions in this meeting.  Issues should be submitted under this agenda item and will be collected by the specification rapporteur.  The review process will proceed by email after this meeting with issues to be concluded at RAN2#110-e.

[AT109bis-e][601][POS] LPP ASN.1 issue gathering and easy agreements (Qualcomm)

Scope: Collect the issues from the contributions in agenda item 6.8.2.4, excluding R2-2003143 and R2-2003144

Intended outcome: Endorsed baseline CR starting from R2-2003350 (R2-2003981) and list of open issues (R2-2003982) for continuing ASN.1 review after this meeting

Deadline:  Wednesday 2020-04-29 1000 UTC

R2-2003982	Email discussion report: [AT109bis-e][601][POS] LPP ASN.1 issue gathering and easy agreements (Qualcomm)	Qualcomm Incorporated	discussion	Rel-16	NR_pos-Core

R2-2003981	LPP Clean-Up	Qualcomm Incorporated	draftCR	Rel-16	37.355	16.0.0	NR_pos-Core

[Post109bis-e][xx][POS] LPP ASN.1 review (Qualcomm)

Scope: Gather and discuss issues and develop a running CR for ASN.1 corrections, with R2-2003981 as a baseline.

Intended outcome: Open issues list and CR to next meeting

Deadline:  Long

[AT109bis-e][602][POS] LPP ASN.1 structural issues (Ericsson)

Scope: Initial discussion on the issues raised in R2-2003144

Intended outcome: Report of potential easy agreements and remaining open issues (R2-2003983) suitable for capture in the LPP ASN.1 issue list

Deadline:  Comments by Tuesday 2020-04-28 1000 UTC; output document Wednesday 2020-04-29 1000 UTC

R2-2003983	Email discussion report: [AT109bis-e][602][POS] LPP ASN.1 structural issues (Ericsson)	Ericsson	discussion	Rel-16	NR_pos-Core

R2-2002915	Clarification on SFN0-Offset and DL-AoD report in LPP ASN.1	CATT	draftCR	Rel-16	37.355	16.0.0	B	NR_pos-Core

R2-2003066	DraftCR for NR-DL-PRS-Config	Huawei, HiSilicon	draftCR	Rel-16	37.355	16.0.0	NR_pos-Core

R2-2003067	Miscellaneous Corrections to LPP ASN.1	Huawei, HiSilicon	discussion	Rel-16	NR_pos-Core

R2-2003143	Overhead in current structure	Ericsson	discussion	Rel-16

R2-2003144	Important LPP structural aspects	Ericsson	discussion	Rel-16

R2-2003349	Various Corrections to NR Positioning	Qualcomm Incorporated	discussion

R2-2003350	LPP clean-up	Qualcomm Incorporated	discussion	Rel-16	NR_pos-Core	Late

R2-2003781	CR 37.355 V16.0.0, Corrections to the introduction of NR positioning	Ericsson	CR	Rel-16	37.355	16.0.0	0256	-	F	NR_pos-Core

#### 6.8.2.5	MAC

Including impact to 38.321.

Tdoc limitation: 1 tdoc

LCID/eLCID

R2-2003135	Change LCID to eLCID for SP Positioning SRS Activation/Deactivation MAC CE	Ericsson	CR	Rel-16	38.321	16.0.0	0720	-	F	NR_pos-Core

R2-2003062	Correction to SP SRS actication deactivation MAC CE	Huawei, HiSilicon	draftCR	Rel-16	38.321	16.0.0	NR_pos-Core

DRX

R2-2002618	Discussion on the impact of DRX on SRS for NR positioning	vivo	discussion	Rel-16	NR_pos-Core

Other

R2-2003063	Runnnig CR to MAC spec for R16 Positioning	Huawei, HiSilicon	draftCR	Rel-16	38.321	16.0.0	NR_pos-Core

=> Revised in R2-2003768

R2-2003768	Running CR to MAC spec for R16 Positioning	Huawei, HiSilicon	draftCR	Rel-16	38.321	16.0.0	NR_pos-Core

[AT109bis-e][606][POS] MAC proposals (Huawei)

Scope: Discuss the proposals from R2-2003135, R2-2003062, R2-2002618, and R2-2003768, and develop a CR incorporating the agreeable proposals

Intended outcome: Agreed in principle CR to 38.321, in R2-2003988

Deadline:  Wednesday 2020-04-29 1000 UTC

R2-2004208	Summary on MAC proposals under 6.8.2.5	Huawei, HiSilicon	discussion	Rel-16	NR_pos-Core

R2-2003988	DraftCR of MAC for R16 Positioning	Huawei, HiSilicon	CR	Rel-16	38.321	16.0.0	0735	F	NR_pos-Core

#### 6.8.2.6	Broadcast assistance data

This agenda item will utilize a summary document to facilitate treatment of topics during the e-meeting.

Tdoc limitation: 1 tdoc

Document transferred from main session

R2-2004209	Summary of [AT109bis-e][056][OdSIBconn] Ondemand SI Open issue for positioning	Ericsson	discussion	Rel-16	NR_unlic-Core, 5G_V2X_NRSL-Core, NR_IIOT-Core, LTE_NR_DC_CA_enh-Core, NR_pos-Core

Summary document

R2-2003607	Summary for Broadcast of Assistance Data	CATT	discussion	Late

The following documents will not be individually treated

R2-2002916	Summary of the agreement and left issues on Broadcast Assistance Data	CATT	discussion	Rel-16	38.331	NR_pos-Core

R2-2003058	DraftCR for on-demand SI request for positioning	Huawei, HiSilicon	draftCR	Rel-16	38.331	16.0.0	NR_pos-Core

R2-2003132	On the need of unicast tag for positioning si-BroadcastStatus	Ericsson	discussion	Rel-16

- Revised in R2-2003810

R2-2003810	On the need of unicast tag for positioning si-BroadcastStatus	Ericsson, Deutsche Telekom	discussion	Rel-16

Withdrawn/Not available

R2-2003140	On Relevance of Broadcast	Ericsson	discussion	Rel-16	Withdrawn

#### 6.8.2.7	UE-based positioning

This agenda item may utilize a summary document to facilitate treatment of topics during the e-meeting (decision to be made based on submitted tdocs).

Tdoc limitation: 1 tdoc

R2-2003064	Discussion on UE-based positioning	Huawei, HiSilicon	discussion	Rel-16	NR_pos-Core

R2-2003145	Remaining issues with NR RAT dependent UE-based positioning	Ericsson	discussion	Rel-16

### 6.8.3	Other

Tdoc limitation: 1 tdoc

R2-2003065	Discussion on UL-ECID	Huawei, HiSilicon	discussion	Rel-16	NR_pos-Core

R2-2003376	On UE RxTx Measurements	Ericsson	discussion	Rel-16

## 6.20	NR TEI16 enhancements

Small Technical Enhancements to NR. TEI should be predominantly within a single WG and fully completed within the same quarter in all affected WGs. RAN2 impact of RAN1/4-led TEI shall be limited to RRC signalling of configuration parameters and UE capabilities (no MAC impact, no RRC procedural impact, etc). Please also see RP-191602 endorsed at RAN#84. Please submit to 6.20.x.

NOTE that proponent companies are responsible to ensure that correct CRs are provided in all groups for proposals that have impact in >1 working group.

Time budget: 1 TU

Tdoc Limitation: 2 tdocs. NOTE for TEI, the tdoc limitation applies to new proposals, not to open proposals since previous meeting(s)

### 6.20.1	RAN2 led TEI16 enhancements - Control plane related

#### 6.20.1.1		Open / ongoing proposals

R2-2003142	Transfer of unicast RS observations with GNSS integer ambiguity level information	Ericsson

Qualcomm understand that this is a new proposal not connected to the previously agreed CR, and more justification is needed.  Ericsson consider that it would be useful to inform the difference between stations to the UE, and think this was previously discussed for LTE.  Qualcomm think this proposal deviates from the RTCM signalling and the benefit is not clearly shown.  Nokia think we are trying to close Rel-16 and this proposal comes quite late.  Intel have a similar view.

- Noted

Proposal 1	Define signalling to enable the UE to translate its integer ambiguity solution associated to a current reference station to an integer solution associated to a new reference station

Proposal 2	Introduce the text proposal in Appendix A into the running LPP CR

# 7	Rel-16 LTE Work Items

Documents in these agenda items will be handled in break out sessions

## 7.7	 Support of Indian Navigation Satellite System NavIC

(LCS_NAVIC; leading WG: RAN2; REL-16; started: Sept 19; target; March-20; WID: RP-192350)

Time budget: 0 TU Final agreement of CRs is expected

This item is 100%

R2-2003821	Update missed out definition for Information Element NavModel-NavIC-KeplerianSet	Reliance Jio	CR	Rel-16	37.355	16.0.0	0257	-	F	LCS_NAVIC-Core

[AT109bis-e][603][POS] Introduction of NavIC Keplerian set IE (Reliance Jio)

Scope: Review of the CR submitted in R2-2003821 to introduce the definition of NavModel-NavIC-KeplerianSet

Intended outcome: Agreed-in-principle CR in R2-2003998

Deadline:  Wednesday 2020-04-29 1000 UTC

R2-2003998	Update missed out definition for Information Element NavModel-NavIC-KeplerianSet	Reliance Jio	CR	Rel-16	37.355	16.0.0	0257	1	F	LCS_NAVIC-Core

- Agreed in principle in email discussion [AT109bis-e][603]