3GPP TSG RAN WG2 Meeting #109bis-e		R2-2003763

Electronic, 20th – 30th April 2020

Title:	[Draft] LS on RAN2 DCP Open Issues

Release:	Release 16

Work Item:	NR_UE_pow_sav-Core

Source:	InterDigital

To:	RAN WG1

Contact person:	Dylan Watts

Dylan.watts@interdigital.com

# 1	Overall description

RAN2 has been discussing remaining issues related to DCP, and would like to inform RAN1 of the following discussions/agreements:

1. Collision between DCP and RAR with C-RNTI during BFR

RAN2 has discussed UE behavior when a DCP monitoring occasion overlaps with the ra-ResponseWindow or msgB-ResponseWindow. RAN2 understanding is that according to current TS 38.213 prioritization rules, if DCP collides with RAR with C-RNTI (i.e. during BFR) and the search spaces are not quasi-collocated, DCP will be prioritized as it is type-3 CSS and thus impacting legacy RAR behavior.

RAN2 would like to ask RAN1 the following:

- To confirm RAN2 understanding that if DCP and RA search spaces are not quasi-collocated, a collision between DCP and RAR during BFR will impact legacy RAR handling and is incorrect UE behavior: i.e. DCP should not be prioritized over RAR with C-RNTI during BFR?

- If RAN1 confirms the above understanding, i.e. DCP should not be prioritized over RAR with C-RNTI during BFR, what is RAN1 preference on where to capture this behavior e.g. TS 38.213 or in TS 38.321 via a DCP monitoring exception rule similar to overlap with DRX Active time?

2. RAN2 agreements

RAN2 would also like to inform RAN1 of the following agreements:

Agreements

1	RAN2 confirms that the flags ps-TransmitPeriodicL1-RSRP and ps-TransmitPeriodicCSI are defined per cell group

2	The flags ps-TransmitPeriodicCSI and ps-TransmitPeriodicL1-RSRP are independent, and it is possible to control UE to report all types of periodic CSI apart from L1-RSRP (i.e. cri-RSRP and ssb-Index-RSRP)

3	The flag ps-TransmitPeriodicCSI  is renamed to ps-TransmitOtherPeriodicCSI

# 2	Actions

To RAN WG1:

ACTION: 	RAN2 respectfully requests RAN1 to:

- To confirm RAN2 understanding that if DCP and RA search spaces are not quasi-collocated, a collision between DCP and RAR during BFR will impact legacy RAR handling and is incorrect UE behavior: i.e. DCP should not be prioritized over RAR with C-RNTI during BFR?

- If RAN1 confirms the above understanding, i.e. DCP should not be prioritized over RAR with C-RNTI during BFR, what is RAN1 preference on where to capture this behavior e.g. TS 38.213 or in TS 38.321 via a DCP monitoring exception rule similar to overlap with DRX Active time?

- Take into consideration the agreements listed above.

# 3	Dates of next TSG RAN WG 2 meetings

RAN2#110-e	1st – 12th June 2020	Online

RAN2#111	24th – 28th August 2020	Toulouse, France