3GPP TSG-RAN WG2 Meeting #110e                                                            R2-2004750

Online, June 1 – June 12, 2020

Agenda item:       6.1.5.2

Source: 		Qualcomm Incorporated

Title: 	TP for 38331 on IP address signaling

Document for:	Other

# Introduction

This paper includes a TP with RRC IEs holding the information for IP address request, report and allocation as discussed in R2-2004749. These IEs are aligned with F1AP signalling for IP address allocation between IAB-donor-CU and IAB-donor-DU based on recent draft CR to TS 38473 in R3-202898.

Since the ongoing email discussion on open RRC issues has not yet converged on the explicit message type to be used for IP address request and report, this TP only includes information-related IEs.

# 4	TP to NR_IAB CR to 38.331

Start Change

#### –	IAB-TNL-Address-Allocation

The IE IAB-TNL-Address-Allocation is used by the IAB-donor to configure the IAB-MT with TNL addresses to be used on the BH, as defined in TS38.401[zz].

IAB-TNL-Address-Allocation information element

-- ASN1START

-- TAG-IAB-TNL-Address-Allocation-START

IAB-TNL-Address-Allocation-r16	::= SEQUENCE{

IAB-TNL-Addresses-toAddList-r16	::= SEQUENCE (SIZE(1..maxNrofTNLAddresses-r16)) OF IAB-Allocated-TNL-Address-Add-r16,

IAB-TNL-Addresses-toReplaceList-r16	::= SEQUENCE (SIZE(1..maxNrofTNLAddresses-r16)) OF IAB-Allocated-TNL-Address-Replace-r16,

IAB-TNL-Addresses-toReleaseList-r16	::= SEQUENCE (SIZE(1..maxNrofTNLAddresses-r16)) OF IAB-Allocated-TNL-Address-r16,

IAB-TNL-Allocated-Address-Add-r16	::=  SEQUENCE{

iAB-Allocated-TNL-Address	IAB-Allocated-TNL-Address-r16,

iAB-donor-DU-BAPaddress		BAPAddress-r16,

...

}

IAB-TNL-Allocated-Address-Replace-r16	::=  SEQUENCE{

old-IAB-Allocated-TNL-Address	IAB-Allocated-TNL-Address-r16,

new-IAB-Allocated-TNL-Address	IAB-Allocated-TNL-Address-r16,

iAB-donor-DU-BAPaddress		BAPAddress-r16,

...

}

-- TAG-IAB-TNL-Address-Allocation-STOP

-- ASN1STOP

IAB-TNL-Address-Allocation field descriptions

iAB-Allocated-TNL-Address Indicates the allocated TNL-address to the IAB-MT.

iAB-donor-DU-BAPaddress Indicates the BAP address of the IAB-donor-DU that anchors the TNL address contained allocated.

#### –	IAB-TNL-Address

The IE IAB-TNL-Address-Allocation is used to refer to an IP address or IP address prefix together with a usage type.

IAB-TNL-Address information element

-- ASN1START

-- TAG-IAB-TNL-Address-START

IAB-Allocated-TNL-Address-r16	::= SEQUENCE {

iABTNLAddress				IABTNLAddress-16,

iABTNLAddressUsage			IABTNLAddressUsage-16	 	OPTIONAL,

...

}

IABTNLAddress ::= CHOICE {

iPv4Address						BIT STRING (SIZE(32)),

iPv6Address						BIT STRING (SIZE(128)),

iPv6Prefix						BIT STRING (SIZE(64)),

...

}

IABTNLAddressUsage ::= ENUMERATED {

f1-c,

f1-u,

non-f1,

...

}

-- TAG-IAB-TNL-Address-STOP

-- ASN1STOP

–		IAB-TNL-Address-Request

The IE IAB-TNL-Address-Allocation is used by the IAB-MT to request from the IAB-donor TNL addresses to be used on the BH, as defined in TS38.401[zz].

IAB-TNL-Address-Request information element

-- ASN1START

-- TAG-IAB-TNL-Address-Request-START

IABAddressesRequested ::=  SEQUENCE {

iABv4AddressesRequested		IABTNLAddressesRequested,

iABv6AddressesRequested ::=  CHOICE {

iPv6Address 	IABTNLAddressesRequested,

iPv6Prefix 		ENUMERATED{ true },

...

}

...

}

IABTNLAddressesRequested	::=  SEQUENCE {

tNLAddressesRequestedF1-C			INTEGER (1..8),

tNLAddressesRequestedF1-U			INTEGER (1..8),

tNLAddressesRequestedNoNF1			INTEGER (1..8),

...

}

-- TAG-IAB-TNL-Address-Request-STOP

-- ASN1STOP

–		IAB-TNL-Address-Report

The IE IAB-TNL-Address-Allocation is used by the IAB-MT to report to the IAB-donor TNL addresses used on the BH, as defined in TS38.401[zz].

IAB-TNL-Address-Report information element

-- ASN1START

-- TAG-IAB-TNL-Address-Report-START

IAB-TNL-Address-Report-r16	::= SEQUENCE{

IAB-TNL-Addresses-toReportList-r16	::= SEQUENCE (SIZE(1..maxNrofTNLAddresses-r16)) OF IAB-Allocated-TNL-Address-r16,

}

-- TAG-IAB-TNL-Address-Report-STOP

-- ASN1STOP

End Change