3GPP TSG-RAN WG2 Meeting #110 electronic	R2-2005732

Online, June 1st - June 12th 2020

Agenda item:	8.2

Source: 	Vice Chairman (ZTE Corporation)

Title:	Report from Break-out session on SRVCC, CLI, PRN, eMIMO, RACS

Document for:	Approval

General

Recording of voice or video at meetings is not used in 3GPP. This applies also to this e-Meeting. At this e-Meeting, no specific actions are taken to prevent the recording of web conferences. Companies that have concerns related to recordings, if any, may express those by email in the main meeting organizational thread [AT110e][000]

Organizational

- With a few exceptions, only email discussions reports and summary discussion papers will be treated during the e-meeting (indicated clearly in the meeting notes)

- All organization emails and notes will be shared over the following email discussion throughout the two meeting weeks:

[AT110e][100] Organizational Sergio's session (SRVCC, CLI, PRN, eMIMO, RACS)

Scope:

    - Share plans for the meeting and list of ongoing email discussions for the sessions related to SRVCC, CLI, PRN, eMIMO, RACS

    - Share meetings notes and agreements for review and endorsement

Schedule/Plan

RACS:

The 36.331 and 38.331 CRs will be discussed and possibly agreed during the web conference call on Thursday June 4th, 12:00 - 13:30 UTC (i.e no preliminary offline email discussion)

SRVCC:

The 38.331 CR will be discussed and possibly agreed during the web conference call on Thursday June 4th, 12:00 - 13:30 UTC (i.e no preliminary offline email discussion)

CLI:

The discussion will initially happen in an offline email discussion (101) kicked off at the e-meeting start and will then continue during the web conference call(s).

Thursday June 4th, 12:00 - 13:30 UTC

- Check the status of email discussion 101 and decide on next steps

- Discuss R2-2005707 (Stage 2 CR)

eMIMO:

The discussion will initially happen in offline email discussions (102 and 103) kicked off at the e-meeting start and will then continue during the web conference call(s).

Wednesday June 3th, 12:00 - 13:30 UTC:

- Check the status of email discussion 103 and decide on next steps on MAC aspects

- Discuss and take a decision on Timer based BFR MAC CE transmission based on R2-2005123 and R2-2005493

Thursday June 4th, 12:00 - 13:30 UTC

- Check the status of email discussion 102 and decide on next steps on RRC aspects

PRN:

The discussion will initially happen in offline email discussions (104 and 105) kicked off at the e-meeting start and will then continue during the web conference call(s).

Wednesday June 3th, 12:00 - 13:30 UTC:

- Check the status of email discussion 105 and decide on next steps on 38.304 aspects

- Discuss R2-2005451 (Early Implementation of CAG Features)

- Discuss R2-2004571 (UE capabilities)

- Discuss R2-2004508 (Stage 2 CR)

Thursday June 4th, 12:00 - 13:30 UTC

- Check the status of email discussion 104 and decide on next steps on RRC aspects

List and status of offline email discussions

NOTE:  No offline email discussions will be kicked off before Monday June 1st, 07:00 UTC

[AT110e][101][CLI] RRC and UE capabilities aspects (LG)

Scope: Discuss the 38.331 and 38.306 CR on UE capabilities for CLI based on R2-2005695, R2-2005730 and R2-2004430. Also discuss the changes suggested by RAN1 in R2-2004347 and the new proposal in R2-2005309.

Initial intended outcome: summary of the offline discussion with e.g.:

    - Set of proposals with full consensus, if any (agreeable over email)

    - Set of proposals to discuss in the follow up conference call

Initial deadline (for companies' feedback):  Wednesday 2020-06-03 10:00 UTC

Initial deadline (for rapporteur's summary in R2-2005791):  Wednesday 2020-06-03 22:00 UTC

Proposed agreements in R2-2005791 indicated for email agreement and not challenged until Thursday 2020-06-04 10:00 UTC will be declared as agreed by the session chair. For the other ones, the discussion will continue online.

Status: Started

[AT110e][102][EMIMO] RRC CR (Ericsson)

Initial scope: Continue the discussion on RRC open issues, based on R2-2004465.

Initial intended outcome: summary of the offline discussion with e.g.:

- Set of proposals with full consensus, if any (agreeable over email)

    - Set of proposals to discuss in the follow up conference call

Initial deadline (for companies' feedback):  Wednesday 2020-06-03 10:00 UTC

Initial deadline (for rapporteur's summary in R2-2005792):  Wednesday 2020-06-03 22:00 UTC

Proposed agreements in R2-2005792 indicated for email agreement and not challenged until Thursday 2020-06-04 10:00 UTC will be declared as agreed by the session chair. For the other ones, the discussion will continue online.

Status: Started

[AT110e][103][EMIMO] MAC CR (Samsung)

Initial scope: Continue the discussion on MAC open issues and correction, based on the submitted contributions.

Initial intended outcome: summary of the offline discussion with e.g.:

- Set of proposals with full consensus, if any (agreeable over email)

    - Set of proposals to discuss in the follow up conference call

Initial deadline (for companies' feedback):  Tuesday 2020-06-02 10:00 UTC

Initial deadline (for rapporteur's summary in R2-2005793):  Tuesday 2020-06-02 22:00 UTC

Proposed agreements in R2-2005793 indicated for email agreement and not challenged until Wednesday 2020-06-03 10:00 UTC will be declared as agreed by the session chair. For the other ones, the discussion will continue online.

Status: Started

[AT110e][104][PRN] RRC CR (Nokia)

Initial scope: Continue the discussion on RRC open issues, based on R2-2004481, considering the new LSs from SA1 and the proposals marked "to be discussed in offline [104]". Also discuss RILs: Z112, B200 and H422.

Initial intended outcome: summary of the offline discussion with e.g.:

- Set of proposals with full consensus agreeable over email (based on the list in Section 3.1 of R2-2004481, possibly extended with new easy agreements)

    - Set of proposals to discuss in the follow up conference call

Initial deadline (for companies' feedback):  Wednesday 2020-06-03 10:00 UTC

Initial deadline (for rapporteur's summary in R2-2005794):  Wednesday 2020-06-03 22:00 UTC

Proposed agreements in R2-2005794 indicated for email agreement and not challenged until Thursday 2020-06-04 10:00 UTC will be declared as agreed by the session chair. For the other ones, the discussion will continue online.

Status: Started

[AT110e][105][PRN] 38.304 CR (Qualcomm)

Initial scope: Continue the discussion on 38.304 open issues, based on R2-2005674, and the proposals marked "to be discussed in offline [105]".

Initial intended outcome: summary of the offline discussion with e.g.:

- Set of proposals with full consensus, if any (agreeable over email)

    - Set of proposals to discuss in the follow up conference call

Initial deadline (for companies' feedback):  Tuesday 2020-06-02 10:00 UTC

Initial deadline (for rapporteur's summary in R2-2005795):  Tuesday 2020-06-02 22:00 UTC

Proposed agreements in R2-2005795 indicated for email agreement and not challenged until Wednesday 2020-06-03 10:00 UTC will be declared as agreed by the session chair. For the other ones, the discussion will continue online.

Status: Started

## 6.5	Optimisations on UE radio capability signalling

(RACS-RAN-Core; leading WG: RAN2; REL-16; started: Mar 19; target; Jun 20; WID: RP-191088, SR: RP-200163). Documents in this agenda item will be handled in a break out session.

R2 part is 100%. Only corrections.

Tdoc limitation: 1 per company

### 6.5.1	Organisational

Including incoming LSs, rapporteur inputs, etc.

Contributions in this AI are reserved for WI rapporteur inputs and do not count towards the tdoc limits.

Workplan

R2-2004711	Work plan for RACS-RAN work item	MediaTek Inc., CATT	discussion	Rel-16	RACS-RAN-Core

- Noted

Incoming LSs

R2-2004321	Reply LS on RACS and signalling of UE capabilities at handover (S2-2003483; contact: Ericsson)	SA2	LS in	Rel-16	RACS-RAN-Core	To:RAN2, RAN3

- Noted (no further RAN2 impact)

R2-2004324	LS on different coding formats (S2-2003507; contact: Samsung)	SA2	LS in	Rel-16	RACS-RAN-Core	To:RAN2, RAN3, CT4, CT3

- Noted (no RAN2 impact)

Rapporteur CRs

R2-2004710	Correction to transfer of UE capabilities at HO for RACS and correction of ASN.1 review issues [N012][N013]	MediaTek Inc., Ericsson, ZTE Corporation, Sanechips	CR	Rel-16	36.331	16.0.0	4256	2	F	RACS-RAN-Core	R2-2003906

- …

R2-2005539	Correction to transfer of UE capabilities at HO for RACS and minor ASN.1 correction (38.331)	ZTE Corporation, Ericsson, MediaTek Inc.,Sanechips,OPPO	CR	Rel-16	38.331	16.0.0	1553	2	F	RACS-RAN-Core	R2-2003905

- …

### 6.5.2	Corrections

Including contributions/TPs on RACS-specific Class 3 ASN.1 review aspects, if any. For these, no individual company CRs should be submitted: please consult with the RRC CR rapporteurs first (Nathan.Tenny@mediatek.com for 36.331 and Gao.Yuan66@zte.com.cn for 38.331).

## 6.14	Single Radio Voice Call Continuity from 5G to 3G

(SRVCC_NR_to_UMTS-Core; leading WG: RAN2; REL-16; started: Dec 18; target; Mar 20; WID: RP-190713; SR: RP-200436) Documents in this agenda item will be handled in a break out session

Tdoc Limitation: 1 tdoc

The Core part of this WI is 100% Only corrections.

### 6.14.1	Organisational

Including incoming LSs, rapporteur inputs, etc.

Contributions in this AI are reserved for WI rapporteur inputs and do not count towards the tdoc limits.

### 6.14.2	Corrections

Including contributions/TPs on SRVCC-specific Class 3 ASN.1 review aspects, if any. For these, no individual company CRs should be submitted: please consult with the RRC CR rapporteur first (tangxun@huawei.com).

R2-2005066	CR on 38.331 for SRVCC from 5G to 3G	Huawei, HiSilicon, Lenovo, Motorola Mobility, China Unicom	CR	Rel-16	38.331	16.0.0	1645	-	F	SRVCC_NR_to_UMTS-Core

- …

## 6.15	Cross Link Interference (CLI) handling and Remote Interference Management (RIM) for NR

(NR_CLI_RIM; leading WG: RAN1; REL-16; started: Dec 18; target; Jun 20; WID: RP-191997; SR: RP-200453) Documents in this agenda item will be handled in a break out session.

Tdoc Limitation: 1 tdoc

### 6.15.1	Organisational

Including incoming LSs, rapporteur inputs, etc.

Contributions in this AI are reserved for WI rapporteur inputs and do not count towards the tdoc limits.

Incoming LSs

R2-2004347	LS on subcarrier spacing for CLI-RSSI measurement (R1-2002966; contact: LGE)	RAN1	LS in	Rel-16	NR_CLI_RIM-Core	To:RAN2	Cc:RAN4

- to be discussed in offline [101]

- Noted

R2-2004365	Reply LS on CLI measurement and reporting (R4-2005297; contact: LGE)	RAN4	LS in	Rel-16	NR_CLI_RIM-Core	To:RAN1	Cc:RAN2

- Noted (no RAN2 impact)

Stage 2:

R2-2005707	CLI Corrections	Nokia (Rapporteur)	CR	Rel-16	38.300	16.1.0	0217	2	F	NR_CLI_RIM-Core	R2-2005277	Late

- …

UE capabilities

R2-2004430	UE capabilities for CLI	Qualcomm Incorporated	CR	Rel-16	38.306	16.0.0	0301	-	F	NR_CLI_RIM-Core

- to be discussed in offline [101]

R2-2005695	CR on UE capabilities for CLI	LG Electronics Inc.	CR	Rel-16	38.331	16.0.0	1689	-	F	NR_CLI_RIM-Core

- to be discussed in offline [101]

- Revised in R2-2006000

[AT110e][101][CLI] RRC and UE capabilities aspects (LG)

Scope: Discuss the 38.331 and 38.306 CR on UE capabilities for CLI based on R2-2005695, R2-2005730 and R2-2004430. Also discuss the changes suggested by RAN1 in R2-2004347 and the new proposal in R2-2005309.

Initial intended outcome: summary of the offline discussion with e.g.:

    - Set of proposals with full consensus, if any (agreeable over email)

    - Set of proposals to discuss in the follow up conference call

Initial deadline (for companies' feedback):  Wednesday 2020-06-03 10:00 UTC

Initial deadline (for rapporteur's summary in R2-2005791):  Wednesday 2020-06-03 22:00 UTC

Proposed agreements in R2-2005791 indicated for email agreement and not challenged until Thursday 2020-06-04 10:00 UTC will be declared as agreed by the session chair. For the other ones, the discussion will continue online.

R2-2005791	Offline discussion 101: CLI - RRC and UE capabilities aspects	LG (Rapporteur)	discussion	Rel-16	NR_CLI_RIM-Core

R2-2006000	CR on UE capabilities for CLI	LG Electronics Inc.	CR	Rel-16	38.331	16.0.0	1689	1	F	NR_CLI_RIM-Core

R2-2005730	UE capabilities for CLI	Ericsson	CR	Rel-16	38.331	16.0.0	1693	-	F	NR_CLI_RIM	Late

- to be discussed in offline [101]

- Noted

Withdrawn:

R2-2005277	CLI Featurre overview - Additional changes	Nokia (Rapporteur)	CR	Rel-16	38.300	16.1.0	0217	1	D	NR_CLI_RIM-Core	R2-2003365	Withdrawn

### 6.15.2	Remaining open issues

Including contributions/TPs on corrections and CLI-specific Class 3 ASN.1 review aspects, if any. For these, no individual company CRs should be submitted: please consult with the RRC CR rapporteur first (sangwon7.kim@lge.com).

R2-2005309	On the impact of DRX on CLI SRS-RSRP measurement	Huawei, HiSilicon	discussion	Rel-16

- to be discussed in offline [101]

- Noted

For the following paper, feedback from RAN3 (on exchange of information related to SRS-RSRP measurement resource configuration) is needed first

R2-2005603	Potential correction on SRS resource for Xn exchange	ZTE Corporation, Sanechips	discussion	Rel-16	NR_CLI_RIM-Core

- …

## 6.16	Enhancements on MIMO for NR

(NR_eMIMO-Core; leading WG: RAN1; REL-16; started: Jun 18; target; June 20; WID: RP-200474; SR: RP-200473). Documents in this agenda item will be handled in a break out session.

Tdoc Limitation: 2 tdocs

### 6.16.1	Organisational

Including incoming LSs, rapporteur inputs, etc.

Contributions in this AI are reserved for WI rapporteur inputs and do not count towards the tdoc limits.

R2-2004833	Miscellaneous corrections on eMIMO	Samsung, Nokia, Nokia Shanghai Bell	CR	Rel-16	38.321	16.0.0	0711	3	F	NR_eMIMO-Core	R2-2003911

- …

### 6.16.2	RRC aspects

Including output of email discussion [Post109bis-e][933][eMIMO] RRC Open issues (Ericsson).

Including contributions/TPs on eMIMO-specific Class 3 ASN.1 review aspects (such aspects should anyway be raised in the email discussion [933]). No individual company CRs should be submitted.

Also including contributions on UE capability aspects.

R2-2004465	[Post109bis-e][933][eMIMO] RRC Open Issues (Ericsson)	Ericsson	report	Rel-16	NR_eMIMO-Core

- Moved to offline email discussion [102] with the intention to go back online during the web conference call(s)

[AT110e][102][EMIMO] RRC CR (Ericsson)

Initial scope: Continue the discussion on RRC open issues, based on R2-2004465.

Initial intended outcome: summary of the offline discussion with e.g.:

- Set of proposals with full consensus, if any (agreeable over email)

    - Set of proposals to discuss in the follow up conference call

Initial deadline (for companies' feedback):  Wednesday 2020-06-03 10:00 UTC

Initial deadline (for rapporteur's summary in R2-2005792):  Wednesday 2020-06-03 22:00 UTC

Proposed agreements in R2-2005792 indicated for email agreement and not challenged until Thursday 2020-06-04 10:00 UTC will be declared as agreed by the session chair. For the other ones, the discussion will continue online.

R2-2005792	Offline discussion 102: eMIMO - RRC open issues - first round	Ericsson	report	Rel-16	NR_eMIMO-Core

### 6.16.3	Other aspects

Including contributions/TPs on MAC corrections. For these, no individual company CRs should be submitted: please consult with the MAC CR rapporteur first (seungri.jin@samsung.com).

If needed, a summary document may also be utilized to treat this agenda item.

MAC Corrections. The proposals in the following papers will be handled in offline email discussion [103]

R2-2004463	On pathloss reference RS MAC CE for SRS and PUSCH	Ericsson	discussion	Rel-16	NR_eMIMO-Core

- Noted

R2-2004464	On SRS activation/deactivation MAC CE for the list of serving cells	Ericsson	discussion	Rel-16	NR_eMIMO-Core

- Noted

R2-2004524	Issues - Beam Failure Recovery	Samsung Electronics Co., Ltd	discussion	Rel-16	NR_eMIMO-Core

- Noted

R2-2004646	Discussion on the priority of the BFR MAC CE and SR	vivo, Samsung	discussion	Rel-16	NR_eMIMO-Core

- Noted

R2-2004647	RACH cancellation after the transmission of Msg3 or MsgA	vivo	discussion	Rel-16	NR_eMIMO-Core

- Noted

R2-2004832	Remaining issues on the MAC CEs for beam enhancements	Samsung	discussion	Rel-16	NR_eMIMO-Core

- Noted

R2-2004897	Open issues for PUSCH Pathloss Reference RS Update MAC CE	OPPO	discussion	Rel-16	NR_eMIMO-Core

- Noted

R2-2004898	Open issues on Spcell BFR	OPPO	discussion	Rel-16	NR_eMIMO-Core

- Noted

R2-2005122	The Remaining issue on stopping the sr-ProhibitTimer	ZTE, Sanechips	discussion	Rel-16	NR_eMIMO-Core

- Noted

R2-2005185	Remaining issues on DL MIMO MAC CE	Qualcomm Incorporated	discussion	Rel-16	NR_eMIMO-Core

- Noted

R2-2005366	Consideration on timer based BFR MAC CE transmission	LG Electronics Inc.	discussion	NR_eMIMO-Core

- Noted

R2-2005568	Clarification on generation of BFR MAC CE	ASUSTeK	discussion	Rel-16	38.321	NR_eMIMO-Core

- Noted

[AT110e][103][EMIMO] MAC CR (Samsung)

Initial scope: Continue the discussion on MAC open issues and correction, based on the submitted contributions.

Initial intended outcome: summary of the offline discussion with e.g.:

- Set of proposals with full consensus, if any (agreeable over email)

    - Set of proposals to discuss in the follow up conference call

Initial deadline (for companies' feedback):  Tuesday 2020-06-02 10:00 UTC

Initial deadline (for rapporteur's summary in R2-2005793):  Tuesday 2020-06-02 22:00 UTC

Proposed agreements in R2-2005793 indicated for email agreement and not challenged until Wednesday 2020-06-03 10:00 UTC will be declared as agreed by the session chair. For the other ones, the discussion will continue online.

R2-2005793	Offline discussion 103: eMIMO - MAC corrections - first round	Samsung	report	Rel-16	NR_eMIMO-Core

Proposal 1: For a pending SR triggered prior to the MAC PDU assembly for beam failure recovery of an SCell sr-ProhibitTimer shall be stopped when the MAC PDU is transmitted and this PDU includes an BFR MAC CE or Truncated BFR MAC CE which contains beam failure recovery information of that Scell.

- Agreed

Proposal 2: Clarify that a MAC PDU shall contain at most one BFR MAC CE.

- Agreed

Proposal 3: Design new SP/AP SRS spatial relation indication MAC CE for multiple serving cells case. Annexure 6 is the baseline.

- Agreed

Proposal 4: Clarify that the pathloss reference RS is updated by this MAC CE in the SRI-PUSCH-powercontrol mappings provided in the same MAC CE.

- Agreed

Proposal 5: RAN2 confirm that the multiple PUCCH resources can be indicated in an Enhanced PUCCH spatial relation Activation/Deactivation MAC CE.

- Agreed

Agreements via email (from [103][EMIMO]):

- For a pending SR triggered prior to the MAC PDU assembly for beam failure recovery of an SCell sr-ProhibitTimer shall be stopped when the MAC PDU is transmitted and this PDU includes an BFR MAC CE or Truncated BFR MAC CE which contains beam failure recovery information of that Scell.

- Clarify that a MAC PDU shall contain at most one BFR MAC CE.

- Design new SP/AP SRS spatial relation indication MAC CE for multiple serving cells case. Annexure 6 is the baseline.

- Clarify that the pathloss reference RS is updated by this MAC CE in the SRI-PUSCH-powercontrol mappings provided in the same MAC CE.

- RAN2 confirm that the multiple PUCCH resources can be indicated in an Enhanced PUCCH spatial relation Activation/Deactivation MAC CE.

Timer based BFR MAC CE transmission

R2-2005123	Discussion on Beam Failure Recovery for SCell	ZTE , Sanechips, Nokia, Nokia Shanghai Bell, Huawei, HiSilicon, OPPO, Sharp, Lenovo, Apple, Asia Pacific Telecom, Futurewei	discussion	Rel-16	NR_eMIMO-Core

- …

R2-2005493	Discussion on timer based BFR MAC CE transmission	Qualcomm Incorporated, vivo, Ericsson, Samsung, MediaTek, CATT	discussion	Rel-16	NR_eMIMO-Core

- …

Withdrawn:

R2-2005257	Remaining issues on SCell BFR	Huawei, HiSilicon	discussion	Rel-16	NR_eMIMO-Core	Withdrawn

## 6.18	Private Network Support for NG-RAN

(NG_RAN_PRN-Core; leading WG: RAN3; REL-16; started: Mar 19; target; June 20; WID: RP-200122 SR; RP-200441) Documents in this agenda item will be handled in a break out session.

Tdoc Limitation: 2 tdocs

### 6.18.1	Organisational

Including incoming LSs, rapporteur inputs, etc.

Contributions in this AI are reserved for WI rapporteur inputs and do not count towards the tdoc limits.

Incoming LSs:

R2-2005739	Reply LS on CMAS/ETWS and emergency services for SNPNs (S1-202220; contact: Nokia)	SA1	LS in	Rel-16	NG_RAN_PRN	To:RAN2, SA2	Cc:CT1

- to be discussed in offline [104]

- Noted

R2-2005991	Reply LS on Manual CAG ID selection and granularity of UAC parameters for PNI-NPNs (S1-202265; contact: CMCC)	SA1	LS in	Rel-16	NG_RAN_PRN-Core	To:RAN2	Cc: CT1, SA2, RAN3

- to be discussed in offline [104]

- Noted

R2-2005993	Reply LS on manual CAG selection (S1-202277; contact: Qualcomm)	SA1	LS in	Rel-16	Vertical_LAN, NG_RAN_PRN-Core	To:RAN2, CT1	Cc: SA2, RAN3

- to be discussed in offline [104]

- Noted

Stage 2:

R2-2004508	Miscellaneous corrections to NPN	Nokia (Rapporteur)	CR	Rel-16	38.300	16.1.0	0225	-	F	NG_RAN_PRN-Core

- …

### 6.18.2	RRC aspects

Including output of email discussion [Post109bis-e][934][PRN] Remaining open issues (Nokia). Contributions related to issues addressed by this email discussions should be avoided and are discouraged for this AI.

Including contributions/TPs on PRN-specific Class 3 ASN.1 review aspects (such aspects should anyway be raised in the email discussion [934]). No individual company CRs should be submitted.

Also including contributions on UE capability aspects.

R2-2004481	Report from email discussion [Post109bis-e][934][PRN] Remaining open issues	Nokia (Rapporteur)	discussion	Rel-16	NG_RAN_PRN-Core

- Moved to offline email discussion [104] with the intention to go back online during the web conference call(s)

[AT110e][104][PRN] RRC CR (Nokia)

Initial scope: Continue the discussion on RRC open issues, based on R2-2004481, considering the new LSs from SA1 and the proposals marked "to be discussed in offline [104]". Also discuss RILs: Z112, B200 and H422.

Initial intended outcome: summary of the offline discussion with e.g.:

- Set of proposals with full consensus agreeable over email (based on the list in Section 3.1 of R2-2004481, possibly extended with new easy agreements)

    - Set of proposals to discuss in the follow up conference call

Initial deadline (for companies' feedback):  Wednesday 2020-06-03 10:00 UTC

Initial deadline (for rapporteur's summary in R2-2005794):  Wednesday 2020-06-03 22:00 UTC

Proposed agreements in R2-2005794 indicated for email agreement and not challenged until Thursday 2020-06-04 10:00 UTC will be declared as agreed by the session chair. For the other ones, the discussion will continue online.

R2-2005794	Offline discussion 104: PRN - RRC open issues - first round	Nokia (Rapporteur)	discussion	Rel-16	NG_RAN_PRN-Core

R2-2004484	Finalization of the support of Non-Public Networks	Nokia (Rapporteur)	CR	Rel-16	38.331	16.0.0	1513	1	F	NG_RAN_PRN-Core	R2-2002658	Late

Definition of selected PNI-NPN

R2-2004482	Selected NPN issue (ASN.1 Z102)	Nokia, Nokia Shanghai Bell	discussion	Rel-16	NG_RAN_PRN-Core

- to be discussed in offline [104]

- Noted

NPN indexing

R2-2004483	Discussion on NPN indexing	Nokia, Nokia Shanghai Bell	discussion	Rel-16	NG_RAN_PRN-Core

- to be discussed in offline [104]

- Noted

R2-2005592	Discussion on network indexing	Huawei, HiSilicon	discussion	Rel-16	NG_RAN_PRN-Core

- to be discussed in offline [104]

- Noted

PCI range

R2-2004521	Consideration on Cells in PCI Range for CAG	CATT	discussion	Rel-16	38.331	NG_RAN_PRN-Core

- to be discussed in offline [104]

- Noted

R2-2004572	Remaining issues for NPN	ZTE Corporation, Sanechips	discussion	Rel-16	NG_RAN_PRN-Core

- moved from 6.18.3

- to be discussed in offline [104]

- Noted

R2-2004690	Remaining aspects of NPN	Ericsson	discussion	Rel-16	NG_RAN_PRN-Core

- moved from 6.18.3

- proposals 1 and 3 to be discussed in offline [104]

- proposal 4 can be discussed in offline [105]

- Noted

R2-2005148	UE behavior for CAG PCI range	Sony	discussion	Rel-16	NG_RAN_PRN-Core

- moved from 6.18.3

- to be discussed in offline [104]

- Noted

R2-2005689	PCI List for CAG	QUALCOMM Europe Inc. - Italy	discussion

- moved from 6.16.3

- can be discussed in offline [104]

- Noted

SIB10 on demand

R2-2005593	Discussion on remaining issues on NPN	Huawei, HiSilicon	discussion	Rel-16	NG_RAN_PRN-Core

- to be discussed in offline [104]

- Noted

Early implementation

R2-2005451	Early Implementation of CAG Features	CMCC, China Telecom, CATT, ZTE, Huawei, Sony	discussion	Rel-16	NG_RAN_PRN-Core

- …

R2-2005452	CR for Early Implementation of CAG Features	CMCC, China Telecom, CATT, ZTE, Huawei, Sony	CR	Rel-16	38.331	16.0.0	1674	-	B	NG_RAN_PRN-Core

UE capability

R2-2004571	draft CR on introdcution of UE capability for non-public network	ZTE Corporation, Sanechips	draftCR	Rel-16	38.306	16.0.0	B	NG_RAN_PRN-Core

- …

Other

R2-2004743	Discussion on ANR for NPN	vivo	discussion

- can be discussed in offline [104]

- Noted

R2-2005658	Need code of hrnn-List in SIB10	LG Electronics France	discussion

- can be discussed in offline [104]

- Noted

R2-2005659	Prioritization of CAG for mobility	LG Electronics France	discussion	NG_RAN_PRN-Core

- can be discussed in offline [104]

- Noted

### 6.18.3	Other aspects

Including non-RRC issues not addressed in email discussion [934].

Including contributions/TPs on TS 38.304 corrections. For these, no individual company CRs should be submitted: please consult with the 38.304 CR rapporteur first (rprakash@qti.qualcomm.com).

If needed, a summary document may also be utilized to treat this agenda item.

R2-2005674	PRN 38304 rapporteur summary of open issues	QUALCOMM Europe Inc. - Italy	discussion	Rel-16

- Moved to offline email discussion [105] with the intention to go back online during the web conference call(s)

[AT110e][105][PRN] 38.304 CR (Qualcomm)

Initial scope: Continue the discussion on 38.304 open issues, based on R2-2005674, and the proposals marked "to be discussed in offline [105]".

Initial intended outcome: summary of the offline discussion with e.g.:

- Set of proposals with full consensus, if any (agreeable over email)

    - Set of proposals to discuss in the follow up conference call

Initial deadline (for companies' feedback):  Tuesday 2020-06-02 10:00 UTC

Initial deadline (for rapporteur's summary in R2-2005795):  Tuesday 2020-06-02 22:00 UTC

Proposed agreements in R2-2005795 indicated for email agreement and not challenged until Wednesday 2020-06-03 10:00 UTC will be declared as agreed by the session chair. For the other ones, the discussion will continue online.

R2-2005795	Offline discussion 105: PRN - 38304 open issues - first round	Qualcomm (Rapporteur)	discussion	Rel-16	NG_RAN_PRN-Core

Proposals agreeable via email:

Proposal 2.4: No change is needed for inter-RAT case with selection to CAG cells on NR.

- Agreed

Proposal 3.1: Change “registered SNPN” to “registered or selected SNPN” in the case of highest ranked cell or best not allowed.

- Agreed

Proposal 3.2: Change “cellReservedForFutureUse IE is not indicated as “true” ” to “not “true” for future use” in Section 5.3.1.

- Agreed

Proposal 3.3: Add a clarification about shared spectrum in SNPN case in Section 5.1.2.2, allowing the UE to search for multiple cells on the same frequency.

- Agreed

Agreements via email (from [105][PRN]):

- No change is needed for inter-RAT case with selection to CAG cells on NR.

- Change “registered SNPN” to “registered or selected SNPN” in the case of highest ranked cell or best not allowed.

- Change “cellReservedForFutureUse IE is not indicated as “true” ” to “not “true” for future use” in Section 5.3.1.

- Add a clarification about shared spectrum in SNPN case in Section 5.1.2.2, allowing the UE to search for multiple cells on the same frequency.

Other proposals:

Proposal 3.4: Delete SNPN from the following sentence in Section 4.1 “For the selected PLMN/SNPN, associated RAT(s) may be set, as specified in TS 23.122 [9].”

Proposal 3.5: Current 38.304 text (“PLMN is broadcast by the cell with no associated CAG-IDs…” in suitable cell definition) is sufficiently clear and no need for change.

Proposal 2.2.2: Current 38.304 text (for suitable cell definition) is sufficiently clear and no need for change to suitable cell definition considering the case of “selected CAG”.

Proposal 2.3.1: There is no need to add a definition of NPN-capable, and current text is adequate.

Proposal 2.3.2: No change needed to existing text regarding emergency calling restrictions for UEs that are not NPN-capable.

Proposal 2.2.3: During manual CAG selection, along-with the PLMN-ID and associated CAG ID, the UE AS shall report operator policy indicator in the SIB, if present, to UE NAS (i.e. indicator of whether operator allows a user to manually select a CAG-ID supported by the CAG cell but outside the UE’s allowed CAG list).

Proposal 2.1: For the case of shared spectrum and forbidden TA, UE follows the behaviour in 38.304v16.0.0, and the changes in the running CR is reverted.

R2-2005676	PRN 38304 CR (rapporteur update)	QUALCOMM Europe Inc. - Italy	CR	Rel-16	38.304	16.0.0	0156	2	F	NG_RAN_PRN	R2-2003908

- …

R2-2004522	Remaining Issues related to 38.304	CATT	discussion	Rel-16	38.304	NG_RAN_PRN-Core

- to be discussed in offline [105]

- Noted

R2-2004603	Remaining open issues in the specification of NPN	Lenovo, Motorola Mobility	discussion	Rel-16	NG_RAN_PRN-Core

- proposal 1 to be discussed in offline [104]

- other can be discussed in offline [105]

- Noted

R2-2004689	Purpose and consequences of manual CAG selection	Ericsson	discussion	Rel-16	NG_RAN_PRN-Core

- Covered by R2-2004481

- Noted

R2-2004728	Remaining issues for Manual CAG selection	Intel Corporation	discussion	Rel-16	NG_RAN_PRN-Core

- Generally covered by R2-2004481 and offline [104]. Wording can be discussed in offline [105]

- Noted

R2-2004744	Discussion on manual CAG selection	vivo	discussion

- can be discussed in offline [105]

- Noted

R2-2005364	Clarification of cell reselection for NPN-capable U	China Telecom	discussion	Rel-16

- can be discussed in offline [105]

- Noted

R2-2005365	Further consideration of UAC in PRN	China Telecom	discussion	Rel-16

- Covered by R2-2004481 and offline [104]

- Noted

R2-2005680	Emergency Calls in CAG Cells for UE supporting NPN-ANR	QUALCOMM Europe Inc. - Italy	discussion

- can be discussed in offline [105]

- Noted

## Summary

TBD