# 3GPP TSG-RAN WG2 Meeting #110	e					 R2-200xxxx

E-meeting, June 1 – 12, 2020

Agenda Item:	6.1.6 (NR_IAB-Core)

Source: 	LG Electronics

Title:         	E-mail discusson: [AT110-e][047][IAB] Particular issues III UAC

Document for: 	Discussion and Decision

# Introduction

This is to kick-off an email discussion with the following scope and intended outcome:

[AT110-e][047][IAB] Particular issues III UAC (LG)

Scope: Address open issues related to access control bypassing, i.e., address R2-2005992, R2-2005525, R2-2005653

Intended outcome: Report with functional Agreements (potentially also TPs).

Deadline: June 5, 0700 UTC

Specifically, this email discussion aims to resolve the followig issues:

- In section 2.2, whether UAC skipping should be effectively realized by AS or NAS

- In section 2.3, how to set establishmentCause and resumeCause

- In section 2.4, when UAC bypassing shouuld be applied

# Discussion

## RAN2 agreements and Liaison

At the RAN2 #109-e meeting, RAN2 made the following agreement for IAB MTs:

IAB-MTs are not under UAC control.

The agreement means that IAB-MT, being part of an IAB node which is a network node, does not perform access barring check for its access attempts to a cell. RAN2 sent an LS to SA1/CT1 to inform this decision.

RAN2 has received an reply LS [1] from SA, where the updates of TS 22.261 to reflect the RAN2 decision is informed, and the following NOTE is introduced in TS 22.261 below:

NOTE2: IAB-MT is not subject to unified access control

Given the updates of TS 22.271, RAN2 decision on UAC skipping is confirmed to be valid. Currently 38.331 already introduced the following text in red to support UAC bypassing:

### 5.3.14 Unified Access Control #### 5.3.14.1 General The purpose of this procedure is to perform access barring check for an access attempt associated with a given Access Category and one or more Access Identities upon request from upper layers according to TS 24.501 [23] or the RRC layer. This procedure does not apply to IAB-MT

## How to realize bypassing Unified Access Control

While it was confirmed that IAB-MT is not subject to UAC, there are actually two possible approaches in principle to realize “not subject to UAC”, because UAC skipping can be effectively realized by either AS or NAS.

- Approach1 (NAS based): IAB MT Access Stratum still applies UAC but NAS ensures via proper setting of Access Category (AC) and Access identify (AI) such that access by IAB MT during UAC is considered as “allowed”

- Approach2 (AS based): IAB MT Access Stratum completely skips UAC (irrespective of AC and AI provided by NAS).

UAC is executed for most of UE access attempts including connection establishment and resume, where UAC is associated with Access Category and Access Identity provided by NAS for the access. Exceptions are the access in response to NG-RAN paging and for RNA update, where a certain Access Category is selected by AS, rather than NAS. Given those, we make the following observations for idle and inactive:

- In case of access in response to NG-RAN paging (inactive),

  - AC ‘0’ is selected by UE AS, and hence the access is considered as “allowed” during UAC.

- In case of access for RNA update (inactive),

  - AC ‘2’ or ‘8’ is chosen by AS, and UE performs UAC associated with this AC and AI. Whether the access cess is allowed or not depends on UAC parameters being broadcast.

- In all other cases (idle and inactive)

  - UE AS performs UAC associated with AC and AI provided by NAS. Whether the access is allowed or not depends on UAC parameters being broadcast.

With the approach1, it is rapporteur view that RAN2 do not need to introduce any special handling to support UAC bypassing for the cases where connection is triggered by upper layers. However, we may still need to take a special treatment for RNA update (where AC ‘8’ is selected by UE AS).

With the approach2, RAN2 may need to introduce a special handing to enable IAB-MT access to completely bypass UAC. RAN2 already introduced the text in 38.331 “This procedure does not apply to IAB-MT”, but the contribution [2] claims further changes as shown below to be done if we go for this option (Note that the changes below are not exhaustive but more can be found in [2] and Annex.C of this contribution)

### 5.3.13 RRC connection resume Upon initiation of the procedure, the UE shall: 1> if the resumption of the RRC connection is triggered by response to NG-RAN paging: 2> if the resumption of the RRC connection is not for IAB-MT; 3> select '0' as the Access Category; 3> perform the unified access control procedure as specified in 5.3.14 using the selected Access Category and one or more Access Identities provided by upper layers; 4> if the access attempt is barred, the procedure ends;

Even if we take approach1, a special treatment for RNA update may need to be specified to enable effective UAC bypassing in case of RNA update by IAB MT. This implies that approach1 may not work for all cases but requires some approach2 as well. So we can add one additional approach on top of two aforementioned approaches:

- Approach3 (Mixed): To apply approach1 for all possible cases, and to apply approach2 for the rest case including RNA-update.

Question1a: Which approach do you think should be adopted to effectively enable UAC bypassing by IAB-MTs?

| Company | Answer (Approach1/2/3) | Comment |
| --- | --- | --- |
| Huawei | Prefer Approach 1, also fine with approach 3 | Rapporteur’s comments make sense. To clarify our understanding on approach 1: Upper layer provides the AC/AI/Causevalue, which are only used for AS of IAB-MT to determine the cause value. As agreed before, AS will skip the UAC check, even with the provided AC/AI. So, we may need to delete “such that access by IAB MT during UAC is considered as “allowed” in approach 1. As to the RNA updated for AC =8, IAB-MT AS just selects 8 as UE, since IAB-MT will not use this AC=8 for its access check anyway. |
| Ericsson | Approach 2 (i.e. maintain the current agreement) | In our opinion there is confusion about the “UAC does not apply to MT” and the role AI/AC plays. Some contributions have assumed that the MT will disregard AI/AC for other purposes if that is needed. However, this is a farfetched assumption. In our view, that “UAC does not apply” means that the MT does never consider the cell as barred by UAC. Some implementations may completely skip all the procedures which determine the AI/AC and that is ok as long as it does not affect other procedures that may need the AI/AC. A device that needs to determine the AI/AC for some other purpose than UAC will have to perform that determination step. So ultimately, we are really discussing which steps implementations should perform. We disagree that there is an issue to solve in this discussion rather than implementation should be done properly following correctly the specifications. |
| KDDI | Approach 2 | We agree with Ericsson. If we adopt approach 1, it impacts core networks. I mean operators have to implement s IAB dedicated UAC features to core networks. That is what we want to avoid. |
| Sony | Approach 2 | We think that it is aligned to the current agreement in RAN2 and avoids impacts to other WGs. |

Question1b: (Only if the answer to the question1a is Approach1) Do you agree that we do not need to change any further changes in RAN2 specification. If No, please provide the required changes

| Company | Answer (Yes/No) | Comment (in case answer is NO, please provide the required changes) |
| --- | --- | --- |
| Huawei | No | We need to clarify approach 1 in the spec. The current wording on that UAC does not apply to IAB-MT seems give the wrong impression upper layer will also not provide any AC/AI. “This procedure does not apply to IAB-MT. The upper layers can provide the Access Category, Access Identity, establishmentCause and resumeCause to IAB-MT RRC.” |
| Ericsson | Yes | Approach 1 has other implications in SA/CT as it also does have an impact on the setting up of e.g. the establishment cause values. This was never the intention of the agreement which RAN2 reached. |
| KDDI | Yes | We agree with Ericsson. |
| Sony | Yes |  |

In case option2 is adopted, we need to discuss whether and whether we introduce further changes, in addition to what we already have “This procedure does not apply to IAB-MT” in UAC procedure heading section. .

Question1c: (Only if the answer to the question1a is Approach2) Do you agree with the direction of changes as proposed in [2]?

| Company | Answer (Yes/No) | Comment (in case answer is NO, please provide the required changes) |
| --- | --- | --- |
| Ericsson | No | We do not think that there are any changes needed in stage 3 specs. The determination of the AI/AC is something parallel. If it is only needed for UAC, an implementation may simply skip it or may do it anyway. If AI/AC is needed for other purposes, it is obvious that it will determine them but will not consider the cell as barred (by UAC) in any case. |
| KDDI | Yes | If we complete RRC INACTIVE in Rel-16, then we are fine with the changes. |
| Sony |  | We are not keen for introducing a new behaviour for the INACTIVE state in Rel-16 but ok to go with the majority. |

Question1d: (Only if the answer to the question1a is Approach3) Companies are requested to provide view in which case the approach2 (i.e., enforced UAC bypassing by AS) should be applied?

| Company | Access case for which approach2 should apply | Comment |
| --- | --- | --- |

CT1 is discussing this issue as well, and there are some proposals to enable UAC bypassing by setting a specific AC/AI value for IAB access. CT1 final decision may be pending due to our pending decision on Q1. Hence, it seems good to send an LS to CT1 to inform our decision about this (and with SA1/2 CCed).

Question2: Do you agree to send an LS to CT1 to inform RAN2 decision on Question 1?

| Company | Answer (Yes/No) | Comment |
| --- | --- | --- |
| Huawei |  | Let’s see the need of LS after we finalize our decision. |
| Ericsson | No | RAN2 already sent an LS indicating what RAN2 agreement was. RAN2 delegates can debrief their corresponding colleagues if there is anything to clarify or communicate. At this stage, we see no reason to send another LS. |
| KDDI | No | We share the view with Ericsson. |
| Sony | No | Agree with other companies above that it is early to send the LS |

## EstablishmentCause and ResumeCause

Connection Setup

For connection establishment, a normal UE Access Stratum sets the establishment cause to the value as received from upper layers, as specified in 38.331 s5.3.3.3. The cause value is determined based on the mapping between access identity/class and causes as specified in TS 24.501.

#### 5.3.3.3 Actions related to transmission of RRCSetupRequest message The UE shall set the contents of RRCSetupRequest message as follows: 1> set the ue-Identity as follows: 2> if upper layers provide a 5G-S-TMSI: 3> set the ue-Identity to ng-5G-S-TMSI-Part1; 2> else: 3> draw a 39-bit random value in the range 0..239-1 and set the ue-Identity to this value; NOTE 1: Upper layers provide the 5G-S-TMSI if the UE is registered in the TA of the current cell. 1> set the establishmentCause in accordance with the information received from upper layers; The UE shall submit the RRCSetupRequest message to lower layers for transmission.

EstablishmentCause ::=              ENUMERATED {

emergency, highPriorityAccess, mt-Access, mo-Signalling,

mo-Data, mo-VoiceCall, mo-VideoCall, mo-SMS, mps-PriorityAccess, mcs-PriorityAccess,

spare6, spare5, spare4, spare3, spare2, spare1}

For access attempt by IAB MT, there are mainly two options how to set the establishment cause during RRC connection establishment:

- Option1: To introduce no special handling for IAB MT for establishmentCause setting. That is, IAB MT sets the establishment cause value as indicated by upper layers

- Option2: To introduce a special handling for IAB MT for establishmentCause setting. IAB MT sets the establishment cause to a specific value (e.g., highPriorityAccess)

The option1 assumes that, for connection setup, NAS always indicates the cause value to be used by AS. With this option, we do not introduce any special handling for setting establishmentCause by IAB MTs during RRC connection establishment.

The option2 enforces Access Stratum of IAB MT to set the establishment cause to a specific value. RAN2 needs to discuss which cause value shall be set for which case.

The following in the box is an excerpt from the contribution [2], where it claims that if AS of IAB-MT follows upper layer indication on AI/AC, then the IAB-MT will necessarily set the cause value to highPriorityAccess. From this rapporteur understanding, the approach in the box below [2] is to set the cause value as indicated by upper layer, i.e., to follow option1.

IAB-MT does not have other traffic except for OAM traffic, and the purpose of its access is to provide access service to the UE and other IAB-nodes, so IAB-MT can set highPriorityAccess as the cause of RRC connection establishment based on the indication from upper layer,

Question3a: Which option do you think should be adopted for IAB MT to set the establishment cause value within RRCSetupRequest?

| Company | Answer (Option1/option2) | Comment |
| --- | --- | --- |
| Huawei | Option1 | R2 agreed not to introduce new cause value in previous meeting. With option1, we don’t need to check with CT1. AS layer will use whatever upper layer provide. To set highPriorityAccess¸ we can rely on the indication provided by upper layer. |
| Ericsson | Option 1 | As said above, there seems to be some confusion about UAC/AC/AI. AC/AI are used by UAC and to determine the establishment cause. That UAC does not apply to MTs does not mean that AI/AC is ignored. That is a different thing that has never been agreed upon. So, AC/AI should be determined to set the establishment cause as in legacy. This topic was discussed in our contribution, R2-1913186. By the proper configuration of the AC/AI, the right establishment cause can be selected by the device. Thus, we do not think that there is a problem here at all. Legacy mechanisms apply i.e. option 1. |
| KDDI | Option 1 | We share the view with Ericsson. |
| Sony | Option 1 |  |

Question3b (Only if the answer to the question3a is option1) Do you agree that, we do not need to change any RAN2 specification to address cause value setting within RRCSetupRequest.

| Company | Answer (Yes/No) | Comment (in case answer is NO, please provide the required changes) |
| --- | --- | --- |
| Huawei | Agree |  |
| KDDI | Agree |  |

Question3c (Only if the answer to the question3a is option2) Please provide your view regarding how to set the cause value

| Company | Comment |
| --- | --- |

Connection Resume

UE AS needs to set resumeCause during RRC connection resume procedure. In case of resume triggered by upper layers, the resumeCause is set in accordance with the information received from upper layers, except for the access in response to RAN-paging, RNA-update, and emergency, and there are resumeCauses to be set in those exceptional cases.

ResumeCause ::=             ENUMERATED {emergency, highPriorityAccess, mt-Access, mo-Signalling,

mo-Data, mo-VoiceCall, mo-VideoCall, mo-SMS, rna-Update, mps-PriorityAccess,

mcs-PriorityAccess, spare1, spare2, spare3, spare4, spare5 }

For resume cause setting by IAB MT access, we have two options:

- Option1: To not introduce a special handling for resumeCause setting. That is, for resume triggered by upper layers, IAB MT sets the resumeCause value in accordance with the information received from upper layers, and for resume in response to RAN-paging and for RNA-update, existing respective resumeCause is set.

- Option2: To introduce a special handling in AS for resumeCause setting. That is, IAB MT sets the establishment cause to a specific value.

- Others: Please specify your preferred behaviours in the table below as part of your company comments.

Question4a: Which option do you think should be adopted for resume cause setting by IAB MT?

| Company | Answer (Option1/option2) | Comment |
| --- | --- | --- |
| Huawei | Option1 |  |
| Ericsson | Option 1 | Same comments as above. There is no reason to do anything special. Nothing is broken in the standard. |
| KDDI | Option1 |  |
| Sony | Option 1 |  |

Question4b (Only if the answer to the question4a is option1) Do you agree that, we do not need to change any RAN2 specification to address resumeCause value setting within RRCResumeRequest.

| Company | Answer (Yes/No) | Comment (in case answer is NO, please provide the required changes) |
| --- | --- | --- |
| Huawei | Agree |  |
| Ericsson | Agree | No changes at all are needed (in this or any of the previous cases). |
| KDDI | Agree |  |
| Sony | Agree |  |

Question4c (Only if the answer to the question4a is option2) Please provide your view regarding how to set the resumeCause value

| Company | Comment |
| --- | --- |

## When UAC bypassing should be applied?

The contribution [3] claims that it is not crystal when IAB-MT should or should not bypass UAC, as different from normal UE. More specifically, the contribution asks:

- Case a) What if a device capable of IAB functionalities goes outside intended IAB network coverage and happens to only identify a cell not supporting IAB operations? Is it possible for the device to access the cell as if it is a normal UEs? If this is possible (e.g. in case the IAB node has normal UE subscription as well), should the device ignore or apply UAC? What about other access control mechanism in such a case?

In addition to the case presented above (case a) and in [3], we can also consider the following case (case b) to ask if IAB node should bypass UAC at the very initial access to the network? That is, should IAB node be allowed to bypass UAC prior to actual IAB operation readiness (i.e. before receiving IAB configuration from CU).

- Casa b) Should the IAB node be able to bypass UAC even before it is ready for IAB operations? Or, Should the IAB node be able to bypass UAC only after it is ready for IAB operation?

Regarding the case b), we note that there is IAB-specific authorization procedure defined in NAS. So, from AS point of view, IAB-MT can determine that it is authorized only after receiving IAB-related RRC configuration.

Question 5a: (Related to case a) Do you think there is indeed a case where IAB node accesses a cell not broadcasting iab-Support indication as if it is a normal UE.

| Company | Answer (yes/no) | Comment |
| --- | --- | --- |
| Huawei |  | “the device to access the cell as if it is a normal UEs” this can be done by implementation. If the “IAB-MT” can act as exactly same as UE by implementation, it can be allowed. There seems no need of standard discussion, since it is just a normal UE. |
| Ericsson | No | We do not understand this section. We do not see case a) or case b) An IAB is a network node and not a UE. So we do not understand what the purpose of this section is. An IAB node does not access a cell not broadcasting iab-support IE. It will only access those nodes which do broadcast it. That cell will be considered for cell selection and it will then apply not apply UAC to access it i.e. it will consider it not barred. |
| KDDI | No | We share the view Ericsson, we also think an IAB is a network node and we do not understand the motivation for this either. |
| Sony | No |  |

If there is indeed a case where an IAB node accesses a cell not broadcasting iab-Support indication as if it is a normal UE, RAN2 should decide whether UAC and other access control mechanism should be applied to the IAB node.

- Alt1: IAB node accessing a cell not broadcasting iab-Support indication shall ignore UAC and other access control mechanisms as it does on a cell broadcasting iab-Support indication.

- Alt2: IAB node accessing a cell not broadcasting iab-Support indication shall apply UAC and other access control mechanisms as normal UEs do.

- Others: Please specify your preferred behaviours in the table below as part of your company comments.

Question 5b: If answer to question5a is yes, companies are requested to express views on the preferred alternative.

| Company | Preferred alternative | Comments |
| --- | --- | --- |
| Huawei | Alt2, but no need of agreement. | It is a UE, rather than IAB-MT from specification perspective. |

Question 6: (Related to case b) Do you agree that a device be able to bypass UAC even before it is configured to operate as IAB?

| Company | Comments |
| --- | --- |
| Huawei | Agree. There should be some test before a device claims as an IAB node. If the device passes the test, I can bypass UAC. There should be no big issue. We are also fine to go with majority. |

## Other issue?

Question 7: Companies are requested to provide view on any other issue related to UAC and cause value handling, which is essential to complete IAB WI.

| Company | Issue | Suggested changes to RAN2 specs |
| --- | --- | --- |

# Summary

FFS

# Reference

[1] R2-2005992	Reply LS on UAC applicability to IABs (S1-202274; contact: Nokia)	SA1	LS in	Rel-16	NR_IAB-Core	To:RAN2, CT1	Cc:RAN3, SA2

[2] R2-2005525	Clarification on the cause value and not supporting UAC for IAB [ToDo RIL H697]	Huawei, HiSilicon	discussion	Rel-16	NR_IAB-Core

[3] R2-2005653	Clarification of access control bypasssing	LG France	discussion	Rel-16

# Annex. A

## Definitions in TS 38.300

IAB-MT: IAB-node function that terminates the Uu interface to the parent node using the procedures and behaviours specified for UEs unless stated otherwise. IAB-MT function used in 38series of 3GPP Specifications corresponds to IAB-UE function defined in TS 23.501 [3].

IAB-node: RAN node that supports NR access links to UEs and NR backhaul links to parent nodes and child nodes. The IAB-node does not support backhauling via LTE.

# Annex. B

## Mapping between RRC causes and Access Identities/Classes

| Rule # | Access identities | Access categories | RRC establishment cause is set to |
| --- | --- | --- | --- |
| 1 | 1 | Any category | mps-PriorityAccess |
| 2 | 2 | Any category | mcs-PriorityAccess |
| 3 | 11, 15 | Any category | highPriorityAccess |
| 4 | 12,13,14, | Any category | highPriorityAccess |
| 5 | 0 | 0 (= MT_acc) | mt-Access |
|  |  | 1 (= delay tolerant) | Not applicable (NOTE 1) |
|  |  | 2 (= emergency) | emergency |
|  |  | 3 (= MO_sig) | mo-Signalling |
|  |  | 4 (= MO MMTel voice) | mo-VoiceCall |
|  |  | 5 (= MO MMTel video) | mo-VideoCall |
|  |  | 6 (= MO SMS and SMSoIP) | mo-SMS |
|  |  | 7 (= MO_data) | mo-Data |
|  |  | 9 (= MO IMS registration related signalling) | mo-Data |
| NOTE 1: A UE using access category 1 for the access barring check will determine a second access category in the range 3 to 7 that is to be used for determination of the RRC establishment cause. See subclause 4.5.2, table 4.5.2.2, NOTE 6. NOTE 2: See subclause 4.5.2, table 4.5.2.1 for use of the access identities of 0, 1, 2, and 11-15. |  |  |  |

# Annex. C Text Proposal for option 2.

### 5.3.13	RRC connection resume

#### 5.3.13.1	General

Figure 5.3.13.1-1: RRC connection resume, successful

Figure 5.3.13.1-2: RRC connection resume fallback to RRC connection establishment, successful

Figure 5.3.13.1-3: RRC connection resume followed by network release, successful

Figure 5.3.13.1-4: RRC connection resume followed by network suspend, successful

Figure 5.3.13.1-5: RRC connection resume, network reject

The purpose of this procedure is to resume a suspended RRC connection, including resuming SRB(s) and DRB(s) or perform an RNA update.

#### 5.3.13.1a	Conditions for resuming RRC Connection for NR sidelink communication

For NR sidelink communication an RRC connection is resumed only in the following cases:

1>	if configured by upper layers to transmit NR sidelink communication and related data is available for transmission:

2>	if the frequency on which the UE is configured to transmit NR sidelink communication concerns the camped frequency; and if SIB12 is provided by the cell on which the UE camps; and if the valid version of SIB12 does not include sl-TxPoolSelectedNormal for the frequency; or

2>	if the frequency on which the UE is configured to transmit NR sidelink communication is included in sl-FreqInfoList within SIB12 provided by the cell on which the UE camps; and if the valid version of SIB12 does not include sl-TxPoolSelectedNormal for the concerned frequency;

For V2X sidelink communication an RRC connection resume is initiated only when the conditions specified for V2X sidelink communication in subclause 5.3.3.1a of TS 36.331 [10] are met.

NOTE:	Upper layers initiate an RRC connection resume. The interaction with NAS is left to UE implementation.

#### 5.3.13.2	Initiation

The UE initiates the procedure when upper layers or AS (when responding to RAN paging or upon triggering RNA updates while the UE is in RRC_INACTIVE) requests the resume of a suspended RRC connection.

The UE shall ensure having valid and up to date essential system information as specified in clause 5.2.2.2 before initiating this procedure.

Upon initiation of the procedure, the UE shall:

1>	if the resumption of the RRC connection is triggered by response to NG-RAN paging:

2> if the resumption of the RRC connection is not for IAB-MT;

3> select '0' as the Access Category;

3> perform the unified access control procedure as specified in 5.3.14 using the selected Access Category and one or more Access Identities provided by upper layers;

4> if the access attempt is barred, the procedure ends;

1>	else if the resumption of the RRC connection is triggered by upper layers:

2>	if the upper layers provide an Access Category and one or more Access Identities:

3>	perform the unified access control procedure as specified in 5.3.14 using the Access Category and Access Identities provided by upper layers;

4>	if the access attempt is barred, the procedure ends;

2>	set the resumeCause in accordance with the information received from upper layers;

1>	else if the resumption of the RRC connection is triggered due to an RNA update as specified in 5.3.13.8:

2>	if an emergency service is ongoing:

NOTE:	How the RRC layer in the UE is aware of an ongoing emergency service is up to UE implementation.

3>	select '2' as the Access Category;

3>	set the resumeCause to emergency;

2>	else:

3> if the resumption of the RRC connection is not for IAB-MT;

4> select '8' as the Access Category;

2>	perform the unified access control procedure as specified in 5.3.14 using the selected Access Category and one or more Access Identities to be applied as specified in TS 24.501 [23];

3>	if the access attempt is barred:

4>	set the variable pendingRNA-Update to true;

4>	the procedure ends;

1>	if the UE is in NE-DC or NR-DC:

2>	if the UE does not support maintaining SCG configuration upon connection resumption:

3>	release the MR-DC related configurations (i.e., as specified in 5.3.5.10) from the UE Inactive AS context, if stored;

1>	if the UE does not support maintaining the MCG SCell configurations upon connection resumption:2>	release the MCG SCell(s) from the UE Inactive AS context, if stored;

1>	apply the default L1 parameter values as specified in corresponding physical layer specifications, except for the parameters for which values are provided in SIB1;

1>	apply the default SRB1 configuration as specified in 9.2.1;

1>	apply the default MAC Cell Group configuration as specified in 9.2.2;

1>	release delayBudgetReportingConfig from the UE Inactive AS context, if stored;

1>	stop timer T342, if running;

1>	release overheatingAssistanceConfig from the UE Inactive AS context, if stored;

1>	stop timer T345, if running;

1>	release idc-AssistanceConfig from the UE Inactive AS context, if stored;

1>	release drx-PreferenceConfig from the UE Inactive AS context, if stored;

1>	stop timer T346a, if running;

1>	release maxBW-PreferenceConfig from the UE Inactive AS context, if stored;

1>	stop timer T346b, if running;

1>	release maxCC-PreferenceConfig from the UE Inactive AS context, if stored;

1>	stop timer T346c, if running;

1>	release maxMIMO-LayerPreferenceConfig from the UE Inactive AS context, if stored;

1>	stop timer T346d, if running;

1>	release minSchedulingOffsetPreferenceConfig from the UE Inactive AS context, if stored;

1>	stop timer T346e, if running;

1>	release releasePreferenceConfig from the UE Inactive AS context, if stored;

1>	stop timer T346f, if running;

1>	apply the CCCH configuration as specified in 9.1.1.2;

1>	apply the timeAlignmentTimerCommon included in SIB1;

1>	start timer T319;

1>	set the variable pendingRNA-Update to false;

1>	initiate transmission of the RRCResumeRequest message or RRCResumeRequest1 in accordance with 5.3.13.3.