3GPP TSG-RAN WG2 Meeting #110-e	R2-200xxxx

# Electronic meeting, 1 – 12 June 2020

Agenda item:	6.0.1

Source:		Intel Corporation (rapporteur)

Title:		Summary of [AT110-e][066][NR16] NR ASN1 2 (Intel)

Document for:	Discussion and Decision

# Introduction

The scope of the discussion:

Scope (with some additional clarifications beyond the chair’s notes):

List extension single element list

[S654, S655], R2-2005258 [38.331][H230] Extension of a single Need M item to a list of this item,

Extending List not ToAddMod

[S655] [H005], R2-2005259 [38.331][H231] Extending the number of entries of a list not using ToAddMod list,

“Otherwise the field is absent" in Condition: Sub-issue 1: Field cannot be released

H246, R2-2005263    [38.331][H246] Usage of presence conditions for SetupRelease structures

Otherwise the field is absent" in Condition: Sub-issue 2: Missing Need node for absence:

R2-2005264    [38.331][H247] Missing need codes for absence in presence

Mechanism to release Rel-16 field

I633, I805, I803, I840, H248, I806, I804, I815, I807, I808, I820, I809, I810, I811, I812, I816, I813, I814, I818, S496, R2-2005265 [38.331][H248] Fields that cannot be released,

# Discussion

## List extension single element list

[S654, S655], R2-2005258 [38.331][H230] Extension of a single Need M item to a list of this item,

The original problem here was that a single element field in Rel-15 was extended to a list in Rel-16 and the relationship between them was not clear.   This has subsequently been updated.  The updated draft version of the spec says:

SRS-ResourceSet ::=                     SEQUENCE {

[..]

pathlossReferenceRS              PathlossReferenceRS-Config          OPTIONAL, -- Need M

[..]

...,

[[

pathlossReferenceRSToAddModList-r16     SEQUENCE (SIZE (1..maxNrofSRS-PathlossReferenceRSs-r16)) OF PathlossReferenceRS-r16                     OPTIONAL, -- Need N

pathlossReferenceRSToReleaseList-r16    SEQUENCE (SIZE (1..maxNrofSRS-PathlossReferenceRSs-r16)) OF SRS-PathlossReferenceRS-Id-r16              OPTIONAL  -- Need N

]]

}

pathlossReferenceRS A reference signal (e.g. a CSI-RS config or a SS block) to be used for SRS path loss estimation (see TS 38.213 [13], clause 7.3).

pathlossReferenceRSToAddModList Multiple candidate pathloss reference RS(s) for SRS power control, where one candidate RS can be mapped to SRS Resource Set via MAC CE (Section xxx in TS 38.321). The network can only include this field if pathlossReferenceRS is not configured in the same SRS-ResourceSet.

Since the SRS-ResourceSet itself is an IE to an addMod list, there is no real reason to reconfigure the SRS-ResourceSet from the original field pathlossReferenceRS  to the athlossReferenceRS-r16 field for the same SRS-ResourceSet element – network can simply configure another entry of SRS-ResourceSet and release the original one.  Hence the current text is sufficient and no further clarification on the relationship between the original and r16 list is needed.

Conclusion#1: No further change is necessary.

Q1: Companies are invited to indicate if they have any concerns with the above conclusion.

| Company | Any concerns with the above conclusion |
| --- | --- |
| Huawei, HiSilicon | It works but isn't ToAddModList complete overkill for a list of a single 9-bits field (1 bit for CHOICE and 6 or 8 bits)? We could just have a simple list, that compliments the existing item, like in Q2. |
| Intel | The conclusion and updates previously agreed seems OK. |
| Ericsson | No concern with conclusion |

## Extending List not ToAddMod

[S655]: [S655] [H005], R2-2005259 [38.331][H231] Extending the number of entries of a list not using ToAddMod list

There are two topics related to the following list discussed in next subsections:

BeamFailureRecoveryConfig ::=  SEQUENCE {

--

candidateBeamRSList       SEQUENCE (SIZE(1..maxA)) OF PRACH-ResourceDedicatedBFR OPTIONAL, -- Need M

...,

[[

candidateBeamRSListExt-r16 SEQUENCE (SIZE(0..maxB)) OF PRACH-ResourceDedicatedBFR OPTIONAL -- Need M

]]

}

### Is the new list is a critical extension or a non-critical extension?

In the above, an non-addMod list is extended in terms of number of entries (both lists use the same IE element).  There are two options for this looking at the functionality:

Option A: the new field (candidateBeamRSListExt-r16) is a non-critical extension to provide more entries to the list (i.e., ext is only signalled along with the original field and only when the number of entries is larger than maxA).  If so use field name to candidateBeamRSListExt-vxy.

Option B: the new field (candidateBeamRSListExt-r16) is a critical extension and a replacement of the original list (i.e., only one of these can be signalled).  If so, use field name candidateBeamRSList-r16

In both options, since the element IE is the same and as this is a non-ToAddMod list, there is no index in the IE to extend.  The UE only has one list and it doesn’t store which field was used to configure the list.

Conclusion#2: Based on the meeting discussion and agreement to use non-critical extension by default, option A should be used.

The TP for this change is also provided in  R2-2005259.

candidateBeamRSList, candidateBeamRSListExt-vxy The list of reference signals (CSI-RS and/or SSB) identifying the candidate beams for recovery and the associated RA parameters. The UE shall consider this list to include all elements of candidateBeamRSList (without suffix) and all elements of candidateBeamRSListExt-r16. The network configures these reference signals to be within the linked DL BWP (i.e., within the DL BWP with the same bwp-Id) of the UL BWP in which the BeamFailureRecoveryConfig is provided.

Q2a: Companies are invited to indicate if they have any concerns with the above conclusion of choosing option A (non-critical extension option) for this specific list.

| Company | Any concerns with using option A, the TP or the suggested field name |
| --- | --- |
| Huawei, HiSilicon | Ok |
| Intel | OK |
| Ericsson | OK |

### Can a list size 0 be used to release a non-AddMod list?

Another issue discussed in R2-2005258 is about using size 0 for the list.  A list size 0 could be used to indicate release of an non-ToAddMode list.  Another option is to use the SetupRelease structure if it is required to release the list.

Q2b: Companies are invited to comment on whether to allow, as a general rule, use of size 0 for non-ToAddMod list to release of the list.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Yes | This seems rather straightforward. |
| Intel | ? | No strong view either way. We have a SetupRelease structure that can be used and don’t see a strong need for size 0 usage as well to do the same thing. |
| Ericsson | No | In general, a cleaner approach is to use the SetupRelease structure. We need no new method. Having size 0 as a “trick” to release the list it may cause compatibility problem in the feature. |

## “Otherwise the field is absent" in Condition:

There are two issues related to this to discuss.

### Sub-issue 1: Field cannot be releasedConditional presence with SetupRelease fields

[I801, I802] R2-2004732 and R2-2005263    [38.331][H246] Usage of presence conditions for SetupRelease structures

As discussed in these documents, with conditional presence for SetupRelease fields, the original intention for the absence condition seems to be that the field is not configured.  However, it can prevent the release of the field when the condition for absence is met, which was not the original intention.  The following options are proposed to address this issue.

As discussed in the meeting, it was agreed to use:

Remove conditional presence for SetupRelease fields (use Need M) and move the intended network behaviour on when the field should be configured to field description (for example, “Network configures this field only when ....”.

A full TP for this is provided in Annex A.

Q3a: Companies are invited to comment on the text proposal in Annex A (split per WI)

| Company | Comments |
| --- | --- |
| Huawei, HiSilicon | lte-CRS-PatternList1 lte-CRS-ToMatchAround is a SetupRelease field. Is it clear enough that in "Network configures this field only if the field lte-CRS-ToMatchAround is not configured.", "is not configured" includes the case where it is included and set to release in the same message? Or should we add "or set to release"? lte-CRS-PatternList2 " Network configures this field only if the field lte-CRS-ToMatchAround is not configured and CORESETPoolIndex configured with 1." Same remark but also: this field is in ServingCellConfig while coresetPoolIndex (this is the correct name) is in ControlResourceSet, of which there can be multiple instances in the PDCCH-Config of each DL BWP. If we really need a statement, it would be "there is at least one ControlResourceSet in one DL BWP of this serving cell with coresetPoolIndex set to 1". But do we really need that? defaultDownlinkBWP-Id The initial bandwidth part is referred to by BWP-Id = 0. ID of the downlink bandwidth part to be used upon expiry of the BWP inactivity timer. This field is UE specific. When the field is absent the UE uses the initial BWP as default BWP. (see TS 38.213 [13], clause 12 and TS 38.321 [3], clause 5.15). Network configures this field only for a (non-PUCCH) SCell when the SCell is configured with a dormant BWP. firstWithinActiveTimeBWP-Id tihs -> this, witha -> with a t316 Prefers keeping a condition for the MCG as this is already done elsewhere. For the wording better to use "in the masterCellGroup" (as "for the MCG" could be understood in other ways). |
| Intel | Response to Huawei comments: Regarding “should we add "or set to release"?”: The usage “network configures this field …” has been used in 38.331 before. The configuration of the field is not the same as or directly indicate the presence of the field (RAN1 usage of “presence” for “configuration” seems to not consider the RAN2 concept of delta configuration and Need codes). This was discussed previously in Rel-15. If there is confusion about it, we can add some guidelines on the difference between “configuring a field” and “presence of a field”. lte-CRS-PatternList2 I took the proposed statement from the draft spec Conditional presence text. If it is not clear or incorrect, I am happy with go with the majority suggestion. t316 We have used MCG-Only once for a SetupRelease field dataInactivityTimer (and few times for other fields) but in that case, the condition doesn’t seem to change from presence (“for the MAC-CellGroupConfig”) to absence and hence that usage seems OK. This is not the case here as there could be change of condition from presence to absence if split SRB1 or SRB3 is changed. My suggestion to use the field description here as well but will go with the majority view. |
| Ericsson | In principle we are fine to move the intended network behaviour of the conditional presence in the field description. However, we have a small comment regarding the formulation used. In fact, if we use “Network configures this field only when…” it gives the impression that the network needs to configure the field in a mandatory way under the condition described. Since in most of the cases (at least the ones addressed in the papers) there is optionality, we would prefer to add a “may” in the proposed formulation. Therefor our proposal would be to use: “Network may configure this field only when….” On “configuring a field” vs “presence of a field”: agree we should have some guideline, since topic is often re-discussion. lte-CRS-PatternList2: Need to look more t316: We tend to agree with Intel. Proposed wording “The network configures this field in the masterCellGroup only, f the UE is configured with split SRB1 or SRB3.” |

### Sub-issue 2: Missing Need node for absence:

[I632] and R2-2005264    [38.331][H247] Missing need codes for absence in presence

RAN2 had previously agreed that Need codes should be used for the absence condition in Conditional presence if there is any risk of inter-operability issues.  This was also captured in the RRC spec.  Need codes for these are missing in some places.

A TP to correct this was provided in R2-2005264.

Q3b:  Companies are invited to comment on the TP provided in Annex B (split per WI) to add Need code for absence (extracted from R2-2005264).  Please also indicate any additional missing Need codes for absence

| Company | Comments/suggestions |
| --- | --- |
| Intel | The usage of Need Code for absence from 38.331 says: “A need code is not provided when the transition from another part of the condition to this part of the condition is not supported, when the field clearly is a one-shot or there is no difference whether UE maintains or releases the value (e.g., in case the field is mandatory present according to the other part of the condition).” Based on this criteria, Need code for absence doesnt’ seem required for: PDCP-Config -> SplitBearer2 (due to mandatory presence for presence condition) PUSCH-Config -> RepTypeB (due to Need S for presence condition) RACH-ConfigCommonTwoStepRA -> 2StepSUL, 2Step4Step, GroupBConfigured (due to mandatory presence for presence condition) |
| Ericsson | TP is ok. We also agree on the additional findings by Intel. |

## Mechanism to release Rel-16 fields

I633, I805, I803, I840, H248, I806, I804, I815, I807, I808, I820, I809, I810, I811, I812, I816, I813, I814, I818, S496, R2-2005265 [38.331][H248] Fields that cannot be released,

A text proposal to allow releasing these fields that originally used Need M, separated per WI (to allow easier WI specific checking/potential merging), is provided in Annex C.  Though use of Need R is not optimal in extension groups, it is suggested in some cases for example, where there are other fields are already using Need S or R in the extension group or the other fields are not likely to change often.

Q4: Companies are invited to provide comments on the TP in Annex C (split per WI) or provide alternative suggestions to support releasing these configurations.  Please also include any additional ones identified.

| Company | Comments/suggestions |
| --- | --- |
| Ericsson | In general, we agree with the approach taken by the Rapporteur. However, we have a small comment regarding the field discardTimerExt-r16. According to what is proposed in Q3a, if the proposal is agreed we would need to move the intended network behaviour of the Conditional presence ( -- DRB2) to the Feld description. On the bap-Address-r16, we consider SetupRelease is not needed. Parent IE bap-Config-r16 has a SetupRelease structure, and there is no reason to release the bap-Adress.r16 only. |

# Annex

## Annex A (TP related to “Otherwise the field is absent" in Condition)

### WI: eMIMO

#### –	ServingCellConfig

lte-CRS-PatternList1-r16    SetupRelease {LTE-CRS-PatternList-r16}  OPTIONAL, -- Need M

lte-CRS-PatternList2-r16    SetupRelease {LTE-CRS-PatternList-r16}  OPTIONAL  -- Need M

lte-CRS-PatternList1 A list of LTE CRS patterns around which the UE shall do rate matching for PDSCH. The LTE CRS patterns in this list shall be non-overlapping in frequency. Network configures this field only if the field lte-CRS-ToMatchAround is not configured.

lte-CRS-PatternList2 A list of LTE CRS patterns around which the UE shall do rate matching for PDSCH scheduled with a DCI detected on a CORESET with CORESETPoolIndex configured with 1. This list is configured only if CORESETPoolIndex configured with 1. The first LTE CRS pattern in this list shall be fully overlapping in frequency with the first LTE CRS pattern in lte-CRS-PatternList, The second LTE CRS pattern in this list shall be fully overlapping in frequency with the second LTE CRS pattern in lte-CRS-PatternList, and so on. Network configures this field only if the field lte-CRS-ToMatchAround is not configured and CORESETPoolIndex configured with 1.

### WI DCCA

#### –	ServingCellConfig

dormantDownlinkBWP-Id-r16          SetupRelease{BWP-Id}  OPTIONAL,   -- Need M

firstWithinActiveTimeBWP-Id-r16    SetupRelease{BWP-Id}  OPTIONAL,   -- Need M

firstOutsideActiveTimeBWP-Id-r16   SetupRelease{BWP-Id}  OPTIONAL,   -- Need M

ServingCellConfig field descriptions

defaultDownlinkBWP-Id The initial bandwidth part is referred to by BWP-Id = 0. ID of the downlink bandwidth part to be used upon expiry of the BWP inactivity timer. This field is UE specific. When the field is absent the UE uses the initial BWP as default BWP. (see TS 38.213 [13], clause 12 and TS 38.321 [3], clause 5.15). Network configures this field only for a (non-PUCCH) SCell when the UE is configured with a dormant BWP.

firstOutsideActiveTimeBWP-Id This field contains the ID of the downlink bandwidth part to be activated when receiving a DCI indication for SCell dormancy outside active time, as specified in TS 38.213 [13]. Network configures this field only when the SCell is configured with WUS and a dormant BWP.

firstWithinActiveTimeBWP-Id This field contains the ID of the downlink bandwidth part to be activated when receiving a DCI indication for SCell dormancy within active time, as specified in TS 38.213 [13]. Network configures tihs field only when the SCell is configured witha dormant BWP.

### WI MobEnh

–	RRCReconfiguration

RRCReconfiguration-v16xy-IEs ::=        SEQUENCE {

t316-r16                                SetupRelease {T316-r16 }                   OPTIONAL, -- Need M

nonCriticalExtension                    SEQUENCE {}                                OPTIONAL

}

t316 Indicates the value for timer T316 as described in clause 7.1. Value ms50 corresponds to 50 ms, value ms100 corresponds to 100 ms and so on. The network configures only this field for the NR MCG, if the UE is configured with split SRB1 or SRB3.

## Annex B: TP related to Missing Need node for absence:

The following text suggestion is From R2-2005264 (copying only relevant text – please refer to original document for more context)

#### –	PDCP-Config

| SplitBearer2 | The field is mandatory present, in case of a split bearer. Otherwise the field is absent, Need R. |
| --- | --- |

#### –	PhysicalCellGroupConfig

| twoPUCCHgroup | This field is optionally present, Need R, if secondary PUCCH group is configured. It is absent otherwise, Need R. |
| --- | --- |

#### –	PUSCH-Config

| RepTypeB | The field is optionally present, Need S, if pusch-RepTypeIndicatorForDCI-Format0-1 is set to pusch-RepTypeB. It is absent otherwise, Need R. |
| --- | --- |

#### –	RACH-ConfigCommonTwoStepRA

| 2StepSUL | The field is mandatory present in initialUplinkBWP in supplementaryUplink when both 2-step and 4-step RA type is configured; otherwise, the field is absent, Need R. |
| --- | --- |
| 2Step4Step | The field is mandatory present if both 2-step random access type and 4-step random access type are configured in the BWP, otherwise the field is not present, Need R. |
| GroupBConfigured | The field is mandatory present if msgA-PUSCH-ResourceGroupB is configured, otherwise the field is absent, Need R. |

#### –	ServingCellConfig

| CORESETPool | This field is optionally present, Need M, if the field lte-CRS-ToMatchAround is not configured and CORESETPoolIndex configured with 1. It is absent, Need R, otherwise. |
| --- | --- |
| DormantBWP | The field is optionally present, Need M, for a (non-PUCCH) SCell when the UE is configured with a dormant BWP. It is absent, Need R, otherwise. |
| LTE-CRS | This field is optionally present, Need M, if the field lte-CRS-ToMatchAround is not configured. It is absent, Need R, otherwise. |
| MultipleNonDormantBWP | The field is optionally present, Need M, when the SCell is configured witha dormant bandwidth part. It is absent, Need R, otherwise. |
| MultipleNonDormantBWP-WUS | The field is optionally present, Need M, when the SCell is configured with WUS and a dormant bandwidth part. It is absent, Need R, otherwise. |

#### –	SRS-Config

| NonNeighSSBorPRS | This field is optionally present, Need S, if ssb-IndexServing, or csi-RS-IndexServing, or srs-SpatialRelation is configured. This field is absent, Need R, if SSB or PRS from the neighbouring cell is configured |
| --- | --- |

## Annex C: TP related to Mechanism to release Rel-16 fields

### WI: URLLC

#### –	CSI-MeasConfig

CSI-MeasConfig ::=                  SEQUENCE {

[..]

...,

[[

reportTriggerSizeForDCI-Format0-2-r16       INTEGER (0..6)                                                    OPTIONAL -- Need R

]]

}

#### –	PDCP-Config

PDCP-Config ::=         SEQUENCE {

[..]

[[

discardTimerExt-r16     SetupRelease {ENUMERATED {ms0dot5, ms1, ms2, ms4, ms6, ms8, spare3, spare2, spare1}} OPTIONAL,    -- Cond DRB2

[..]

]]

}

#### –	PDSCH-Config

PDSCH-Config ::=                        SEQUENCE {

[..]

prb-BundlingTypeForDCI-Format1-2-r16    CHOICE {

staticBundling-r16                      SEQUENCE {

bundleSize-r16                          ENUMERATED { n4, wideband }                                 OPTIONAL    -- Need S

},

dynamicBundling-r16                     SEQUENCE {

bundleSizeSet1-r16                      ENUMERATED { n4, wideband, n2-wideband, n4-wideband }       OPTIONAL,   -- Need S

bundleSizeSet2-r16                      ENUMERATED { n4, wideband }                                 OPTIONAL    -- Need S

}

}                                                                                                           OPTIONAL,   -- Need R

[..]

configurableFieldForDCI-Format1-2               SEQUENCE {

harq-ProcessNumberSizeForDCI-Format1-2-r16      INTEGER (0..4)      ** URLLC                                    OPTIONAL,   -- Need R

dmrs-SequenceInitializationForDCI-Format1-2-r16 ENUMERATED {enabled}                                    OPTIONAL,   -- Need S

numberOfBitsForRV-ForDCI-Format1-2-r16          INTEGER (0..2)                                          OPTIONAL,   -- Need R

antennaPortsFieldPresenceForDCI-Format1-2-r16   ENUMERATED {enabled}                                    OPTIONAL,   -- Need S

...

},

[..]

}

#### –	PUCCH-Config

PUCCH-Config ::=                        SEQUENCE {

[..]

subslotLengthForPUCCH-r16               ENUMERATED {n2,n7}                                                    OPTIONAL, -- Need R

dl-DataToUL-ACK-ForDCI-Format1-2-r16    SetupRelease {SEQUENCE (SIZE (1..8)) OF INTEGER (0..15)}              OPTIONAL, -- Need M

numberOfBitsForPUCCH-ResourceIndicatorForDCI-Format1-2-r16  INTEGER (0..3)                                    OPTIONAL, -- Need R

[..]

]]

}

#### –	PUSCH-Config

PUSCH-Config ::=                        SEQUENCE {

[..]

pusch-RepTypeIndicator                  SEQUENCE {

pusch-RepTypeIndicatorForDCI-Format0-2-r16  ENUMERATED { pusch-RepTypeA, pusch-RepTypeB}      OPTIONAL,   -- Need R

pusch-RepTypeIndicatorForDCI-Format0-1-r16  ENUMERATED { pusch-RepTypeA, pusch-RepTypeB}      OPTIONAL    -- Need R

},

configurableFieldForDCI-Format0-2       SEQUENCE {

harq-ProcessNumberSizeForDCI-Format0-2-r16      INTEGER (0..4)                                OPTIONAL,   -- Need R

dmrs-SequenceInitializationForDCI-Format0-2-r16 ENUMERATED {enabled}                          OPTIONAL,   -- Need S

numberOfBitsForRV-ForDCI-Format0-2-r16          INTEGER (0..2)                                OPTIONAL,   -- Need R

antennaPortsFieldPresenceForDCI-Format0-2-r16   ENUMERATED {enabled}                          OPTIONAL,   -- Need S

...

},

[..]

frequencyHoppingOffsetListsForDCI-Format0-2-r16 SetupRelease {SEQUENCE (SIZE (1..4)) OF INTEGER (1.. maxNrofPhysicalResourceBlocks-1)}

OPTIONAL,   -- Need M

[..]

uci-OnPUSCH-ListForDCI-Format0-2-r16        SetupRelease {SEQUENCE (SIZE (1..2)) OF UCI-OnPUSCH-ForDCI-Format0-2-r16}  OPTIONAL,  -- Need M

uci-OnPUSCH-ListForDCI-Format0-1-r16        SetupRelease {SEQUENCE (SIZE (1..2)) OF UCI-OnPUSCH  }               OPTIONAL,   -- Need M

[..]

resourceAllocationForDCI-Format0-2-r16           ENUMERATED { resourceAllocationType0, resourceAllocationType1, dynamicSwitch} OPTIONAL, -- Need R

[..]

}

#### –	PUSCH-PowerControl

PUSCH-PowerControl-v16xy ::=        SEQUENCE {

[..]

olpc-ParameterSet                   SEQUENCE {

olpc-ParameterSetForDCI-Format0-1-r16   INTEGER (1..2)                                                  OPTIONAL, -- Need R

olpc-ParameterSetForDCI-Format0-2-r16   INTEGER (1..2)                                                  OPTIONAL  -- Need R

}     												                                                       OPTIONAL, -- Need M

...

}

#### –	PUSCH-ServingCellConfig

PUSCH-ServingCellConfig ::=             SEQUENCE {

[..]

[[

maxMIMO-LayersForDCI-Format0-2-r16      SetupRelease {INTEGER (1..4)}                                          OPTIONAL    -- Need M

]]

}

### WI: eMIMO

–	DMRS-UplinkConfig

MRS-UplinkConfig ::=               SEQUENCE {

[..]

[[

dmrs-UplinkTransformPrecoding-r16  SetupRelease {DMRS-UplinkTransformPrecoding-r16}                 OPTIONAL    -- Need M

]]

}                                                                                                       OPTIONAL,   -- Need R

...

}

DMRS-UplinkTransformPrecoding-r16  ::=  SEQUENCE {

pi2BPSK-ScramblingID0                   INTEGER(0..65535)                                               OPTIONAL,   -- Need S

pi2BPSK-ScramblingID1                   INTEGER(0..65535)                                               OPTIONAL    -- Need S

}

-- TAG-DMRS-UPLINKCONFIG-STOP

-- ASN1STOP

dmrs-UplinkTransformPrecoding This field indicates whether low PAPR DMRS is used for PUSCH with pi/2 BPSK modulation, as specified in TS38.211 [16], clause 6.4.1.1.1.2. The network configures this field only if tp-pi2BPSK is configured in PUSCH-Config.

#### –	PUCCH-PowerControl

PUCCH-PowerControl ::=              SEQUENCE {

[..]

...,

[[

pathlossReferenceRSs-r16          SetupRelease {SEQUENCE (SIZE (1..maxNrofPUCCH-PathlossReferenceRSs-r16)) OF PUCCH-PathlossReferenceRS-r16}

OPTIONAL -- Need M

]]

}

#### –	PUSCH-Config

PUSCH-Config ::=                        SEQUENCE {

[..]

pusch-PowerControl-v16xy                SetupRelease {PUSCH-PowerControl-v16xy}                                  OPTIONAL,   -- Need M

[..]

]]

}

### WI: SRVCC

#### –	QuantityConfig

The IE QuantityConfig specifies the measurement quantities and layer 3 filtering coefficients for NR and inter-RAT measurements.

QuantityConfig information element

-- ASN1START

-- TAG-QUANTITYCONFIG-START

QuantityConfig ::=                  SEQUENCE {

quantityConfigNR-List               SEQUENCE (SIZE (1..maxNrofQuantityConfig)) OF QuantityConfigNR          OPTIONAL,   -- Need M

...,

[[

quantityConfigEUTRA                 FilterConfig                                                            OPTIONAL    -- Need M

]],

[[

quantityConfigUTRA-FDD-r16          QuantityConfigUTRA-FDD-r16                                              OPTIONAL,   -- Need R

quantityConfigCLI-r16               FilterConfigCLI-r16                                                     OPTIONAL    -- Need M

]]

}

### WI: CLI_RIM

#### –	QuantityConfig

QuantityConfig ::=                  SEQUENCE {

quantityConfigNR-List               SEQUENCE (SIZE (1..maxNrofQuantityConfig)) OF QuantityConfigNR          OPTIONAL,   -- Need M

...,

[[

quantityConfigEUTRA                 FilterConfig                                                            OPTIONAL    -- Need M

]],

[[

quantityConfigUTRA-FDD-r16          QuantityConfigUTRA-FDD-r16                                              OPTIONAL,   -- Need M

quantityConfigCLI-r16               FilterConfigCLI-r16                                                     OPTIONAL    -- Need R

]]

}

### WI: IIOT

#### –	ConfiguredGrantConfig

-- ASN1START

-- TAG-CONFIGUREDGRANTCONFIG-START

ConfiguredGrantConfig ::=           SEQUENCE {

[..]

rrc-ConfiguredUplinkGrant               SEQUENCE {

[..]

[[

pusch-RepTypeIndicator-r16          ENUMERATED {pusch-RepTypeA,pusch-RepTypeB}                          OPTIONAL,   -- Need R

[..]

]]

[..]

configuredGrantConfigIndex-r16          ConfiguredGrantConfigIndex-r16                       OPTIONAL,   -- Need R

configuredGrantConfigIndexMAC-r16       ConfiguredGrantConfigIndexMAC-r16                    OPTIONAL,   -- Need R

periodicityExt-r16                      INTEGER (1..5120)                                    OPTIONAL,   -- Need R

startingFromRV0-r16                     ENUMERATED {on, off}                                 OPTIONAL,   -- Need R

phy-PriorityIndex-r16                   ENUMERATED {p0, p1}                                  OPTIONAL,    -- Need R

[..]

]]

}

#### –	SPS-Config

SPS-Config ::=                  SEQUENCE {

[..]

...,

[[

sps-ConfigIndex-r16         SPS-ConfigIndex-r16                                                             OPTIONAL,   -- Need RN

harq-ProcID-Offset-r16      INTEGER (0..15)                                                                 OPTIONAL,   -- Need RM

periodicityExt-r16          INTEGER (1..5120)                                                               OPTIONAL,   -- Need RM

harq-CodebookID-r16         INTEGER (1..2)                     										       OPTIONAL,    -- Need R

pdsch-AggregationFactor-r16 ENUMERATED {n1, n2, n4, n8 }                                                    OPTIONAL    -- Need S

]]

}

#### Additional ones with Need M for which it is not felt necessary to make changes (with reasons in comments)

#### BWP-DownlinkDedicated (no change needed)

BWP-DownlinkDedicated ::=           SEQUENCE {

[..]

...,

[[

sps-ConfigMulti-r16                SPS-ConfigMulti-r16                               OPTIONAL,   -- Need M

[..]

]]

}

#### –	BWP-UplinkDedicated (no change needed)

BWP-UplinkDedicated ::=             SEQUENCE {

[..]

configuredGrantConfigListconfigureGrantConfigMulti-r16                      OPTIONAL    -- Need M

]]

}

### WI: IAB

CellGroupConfig ::=                        SEQUENCE {

[..]

[[

bap-Address-r16                            SetupRelease {BIT STRING (SIZE (10))}                                    OPTIONAL,   -- Need M

[..]

}

### WI: Many

RRCReconfiguration-v16xy-IEs ::=        SEQUENCE {

otherConfig-v16xy                       OtherConfig-v16xy                          OPTIONAL, -- Need M

### WI: NR-U

All the NR-U related fields have already been included into the NR-U WI discussion.