3GPP TSG RAN WG2 Meeting #110-e                                     	 R2-200xxxx

# Electronic, 1st – 12th June 2020

Agenda item:		6.0.1

Source:			Lenovo

Title:		Report from email discussion [AT110-e][068][NR16] NR ASN1 4

Document for:	 	Discussion and decision

# Introduction

This contribution summarizes the discussion and result of the email discussion below that took place during RAN2#110-e:

[AT110-e][068][NR16] NR ASN1 4 (Lenovo)

Scope: ASN1 Naming A009 H001 E229 E257 E258 N033 S463, ASN1 Structure E228 E230 S656 Q022, R2-2004952 [E228][E230] On grouping similar parameters in PUSCH-Config/PDSCH-Config, Misc I654, S461, N021, R2-2004602 [I654] Adding DL AM RLC extension in NR RRC

In detail, the following issues from [1] and [2] were discussed in the email discussion:

| RIL # | Issue | Feature |
| --- | --- | --- |
| A009 | Change field name si-Periodicity | NRPos |
| H001 | Naming issue of referenceTimeInfo-r16 | General |
| E229 | Naming issue of tci -PresentInDCI-ForDCI-Format1-2-r16 | URLLC |
| E257 | Naming issue of dl-DCI-triggered-UL-ChannelAccess-CPextList-r16 | NR-U |
| E258 | Naming issue of ul-dci-triggered-UL-ChannelAccess-CPext-CAPC-List-r16 | NR-U |
| N033 | Naming issues in TDD-UL-DL-SlotConfig-IAB-MT-v16xy | IAB |
| S463 | Change names and field descriptions in AreaConfiguration-r16 | MDT |
| E228 | Grouping of configurable fields in PUSCH-Config | URLLC |
| E230 | Grouping of configurable fields in PDSCH-Config | URLLC |
| S656 | Structure of CodebookConfig-r16 | MIMO |
| Q022 | Type of RepetitionSchemeConfig-r16 | MIMO |
| I654 | Addition of DL-AM-RLC-v16xy | URLLC |
| S461 | Extension and delta signalling of LoggedMeasurementConfiguration-r16-IEs | MDT |
| N021 | ‘infinity’ value for minSchedulingOffsetPreferenceProhibitTimer-r16 | PowerSaving |

# Discussion

## A009

The description of the issue is shown below.

[RIL]: A009 [Delegate]: Apple (Zhbin Wu) [WI]: NRPos [Class]: 2 [Status]: DiscMail [TDoc]: None [Proposed Conclusion]:

[Description]: The field used to detrermine Positioning SI periodicity is called “posSI-periodicity”, not “Si-Periodicity”

[Proposed Change]: Change the wrong field name to “posSI-Periodicigty” in the two occurences of this section.

[Comments]:

The proposed change would affect subclause 5.2.2.3.2 in two occurrences of si-Periodicity, see the highlighted part in blue below.

##### 5.2.2.3.2	Acquisition of an SI message

<Text omitted>

When acquiring an SI message, the UE shall:

1>	determine the start of the SI-window for the concerned SI message as follows:

2>	if the concerned SI message is configured in the schedulingInfoList:

3>	for the concerned SI message, determine the number n which corresponds to the order of entry in the list of SI messages configured by schedulingInfoList in si-SchedulingInfo in SIB1;

3>	determine the integer value x = (n – 1) × w, where w is the si-WindowLength;

3>	the SI-window starts at the slot #a, where a = x mod N, in the radio frame for which SFN mod T = FLOOR(x/N), where T is the si-Periodicity of the concerned SI message and N is the number of slots in a radio frame as specified in TS 38.213 [13];

2>	else if the concerned SI message is configured in the posSI-SchedulingInfoList and offsetToSI-Used is not configured:

3>	create a concatented list of SI messages by appending the posSI-SchedulingInfoList in SIB1 to schedulingInfoList in si-SchedulingInfo in SIB1

3>	for the concerned SI message, determine the number n which corresponds to the order of entry in the concatenated list;

3>	determine the integer value x = (n – 1) × w, where w is the si-WindowLength;

3>	the SI-window starts at the slot #a, where a = x mod N, in the radio frame for which SFN mod T = FLOOR(x/N), where T is the si-Periodicity of the concerned SI message and N is the number of slots in a radio frame as specified in TS 38.213 [13];

2>	else if the concerned SI message is configured by the posSI-SchedulingInfoList and offsetToSI-Used is configured:

3>	determine the number m which corresponds to the number of SI messages with an associated si-Periodicity of 8 radio frames (80 ms), configured by schedulingInfoList in SystemInformationBlockType1;

3>	for the concerned SI message, determine the number n which corresponds to the order of entry in the list of SI messages configured by posSI-SchedulingInfoList in SIB1;

3>	determine the integer value x = m × w + (n – 1) × w, where w is the si-WindowLength

3>	the SI-window starts at the slot #a, where a = x mod N, in the radio frame for which SFN mod T = FLOOR(x/N), where T is the si-Periodicity of the concerned SI message and N is the number of slots in a radio frame as specified in TS 38.213 [13];

<Text omitted>

Question 1: Do companies agree to change the two occurrences of si-Periodicity to posSI-Periodicity?

| Company | Agree/Disagree | Additional comments |
| --- | --- | --- |
| Huawei, HiSilicon | Agree for the first occurrence only |  |
| Ericsson | Agree with Huawei. | There is yet another si-Periodicity (grey-marked above) that we think should be changed to posSI-Periodicigty. |
| MediaTek | Agree with Ericsson | The one marked in grey should also be changed |
| Samsung | Agree with Ericsson |  |

## H001

The description of the issue is shown below.

[RIL]: H001 [Delegate]: Yinghao/David (Huawei)  [WI]: Gen [Class]: 2 [Status]: DiscMail [TDoc]: None [Proposed Conclusion]:

[Description]: The suffix "Info" in used in a tremendous number of new names, making them longer while actually bringing no information.

[Proposed Change]: Review the new -r16 parameters and IEs and remove "Info" unless really useful.

[Comments]:

Rapp1: List of fields/IEs need to be provided.

Rapp2: Main session: “Huawei indicate that this issue is just about excessive use of the word “info”. Nokia think there is already a guideline saying this shall not be used.  Agreed (for all instances)”

The issue was raised in the context of referenceTimeInfo-r16 in DLInformationTransfer-v16xy-IEs. However, if name of referenceTimeInfo-r16 needs to be changed in accordance with the agreement made it will affect SIB9 and IE ReferenceTimeInfo as well.

DLInformationTransfer-v16xy-IEs ::= SEQUENCE {

referenceTimeInfo-r16               ReferenceTimeInfo-r16               OPTIONAL,   -- Need R

nonCriticalExtension                SEQUENCE {}                         OPTIONAL

}

SIB9 ::=                            SEQUENCE {

timeInfo                            SEQUENCE {

timeInfoUTC                         INTEGER (0..549755813887),

dayLightSavingTime                  BIT STRING (SIZE (2))                   OPTIONAL,   -- Need R

leapSeconds                         INTEGER (-127..128)                     OPTIONAL,   -- Need R

localTimeOffset                     INTEGER (-63..64)                       OPTIONAL    -- Need R

}                                                                               OPTIONAL,   -- Need R

lateNonCriticalExtension            OCTET STRING                                OPTIONAL,

...,

[[

referenceTimeInfo-r16           ReferenceTimeInfo-r16                           OPTIONAL    -- Need R

]]

}

#### –	ReferenceTimeInfo

The IE ReferenceTimeInfo contains timing information for 5G internal system clock used for, e.g., time stamping, see TS 23.501 [32], clause 5.27.1.2.

ReferenceTimeInfo-r16 ::= SEQUENCE {

time-r16                            ReferenceTime-r16,

uncertainty-r16                     INTEGER (0..32767)          OPTIONAL,   -- Need S

timeInfoType-r16                    ENUMERATED {localClock}     OPTIONAL,   -- Need S

referenceSFN-r16                    INTEGER (0..1023)           OPTIONAL    -- Cond RefTime

}

ReferenceTime-r16 ::=           SEQUENCE {

refDays-r16                         INTEGER (0..72999),

refSeconds-r16                      INTEGER (0..86399),

refMilliSeconds-r16                 INTEGER (0..999),

refTenNanoSeconds-r16               INTEGER (0..99999)

}

Question 2: How should referenceTimeInfo-r16 be renamed avoiding the use of suffix “Info”?

| Company | Agree/Disagree | Additional comments |
| --- | --- | --- |
| Huawei, HiSilicon | This is not the intention of the proposal. Suggest to postpone | The intention of the proposal is to check "Info" in all field names, there are more than 100 concerned fields. Since there is a lack of time to do it in this meeting and change of name is backward compatible, we propose to postpone this. |
| Intel |  | OK to postpone as suggested by Huawei |
| Ericsson | Disagree on particular example | On this particular IE, it is preferred to keep “info” because - The legacy Rel-15 SIB9 has the field name “timeInfo”. If “info” is removed, then it clashes with another (rel-16) IE ReferenceTime-r16 |
| MediaTek | Disagree on this example | The IE contains reference time and additional information pertinent to the signalled reference time. Therefore ‘Info’ is relevant here, to distinguish from the ReferenceTime IE. |
| Samsung |  | OK to postpone. |

## E229

The description of the issue is shown below.

[RIL]: E229 [Delegate]: Ericsson (Zhenhua) [WI]: URLLC [Class]: 2 [Status]: PropAgree2 [TDoc]: None [Proposed Conclusion]:

[Description]: The appendix of the name “ForDCI-Format1-2” is repetitive and not needed

[Proposed Change]: change to “tci-PresentInDCI-Format1-2”

[Comments]:

The proposed change would affect the field tci-PresentInDCI-ForDCI-Format1-2-r16 in IE ControlResourceSet.

-- ASN1START

-- TAG-CONTROLRESOURCESET-START

ControlResourceSet ::=              SEQUENCE {

controlResourceSetId                ControlResourceSetId,

frequencyDomainResources            BIT STRING (SIZE (45)),

duration                            INTEGER (1..maxCoReSetDuration),

cce-REG-MappingType                 CHOICE {

interleaved                         SEQUENCE {

reg-BundleSize                      ENUMERATED {n2, n3, n6},

interleaverSize                     ENUMERATED {n2, n3, n6},

shiftIndex                          INTEGER(0..maxNrofPhysicalResourceBlocks-1)       OPTIONAL -- Need S

},

nonInterleaved                      NULL

},

precoderGranularity                 ENUMERATED {sameAsREG-bundle, allContiguousRBs},

tci-StatesPDCCH-ToAddList           SEQUENCE(SIZE (1..maxNrofTCI-StatesPDCCH)) OF TCI-StateId OPTIONAL, -- Cond NotSIB1-initialBWP

tci-StatesPDCCH-ToReleaseList       SEQUENCE(SIZE (1..maxNrofTCI-StatesPDCCH)) OF TCI-StateId OPTIONAL, -- Cond NotSIB1-initialBWP

tci-PresentInDCI                        ENUMERATED {enabled}                      OPTIONAL, -- Need S

pdcch-DMRS-ScramblingID                 INTEGER (0..65535)                        OPTIONAL, -- Need S

...,

[[

rb-Offset-r16                           INTEGER (0..5)                            OPTIONAL, -- Need S

tci-PresentInDCI-ForDCI-Format1-2-r16   INTEGER (1..3)                            OPTIONAL, -- Need S

coresetPoolIndex-r16                    INTEGER (0..1)                            OPTIONAL, -- Need R

controlResourceSetId-r16                ControlResourceSetId-r16                  OPTIONAL  -- Need S

]]

}

-- TAG-CONTROLRESOURCESET-STOP

-- ASN1STOP

Question 3: Do companies agree to change field name tci-PresentInDCI-ForDCI-Format1-2-r16 to tci-PresentInDCI-Format1-2?

| Company | Agree/Disagree | Additional comments |
| --- | --- | --- |
| Huawei, HiSilicon | It depends on other places | The suffix "ForDCI-Format1-2" is used in 65 fields. Any change should be the same for all these fields. |
| Intel | No preference | Agree with Huawei comment |
| Ericsson | Disagree | There was a typo in the RIL. The intention is to remove “InDCI”, since it is repetitive. tci-PresentInDCI-ForDCI-Format1-2-r16 -> tci-PresentForDCI-Format1-2-r16 |
| MediaTek | Agree with Ericsson |  |
| Samsung | Agree with Ericsson | Agree with Huawei i.e. alignment of whole fields is preferable. |

## E257

The description of the issue is shown below.

[RIL]: E257 [Delegate]: Ericsson (Cecilia) [WI]: NR-U [Class]: 2 [Status]: DiscMeet [TDoc]: TBD [Proposed Conclusion]:

[Description]: The field description is too long and lists all parameters in the field name.

“DL-DCI triggered UL” corresponds to DCI format 1-1 and can be aligned with other names in PUCCH-Config which refer to “DCI-Format-1-X”

[Proposed Change]: change name to “channelAccessConfigListForDCI-Format1-1-r16” or preferably “channelAccessConfigListForDCI-1-1-r16”

[Comments]:

The concerned field dl-DCI-triggered-UL-ChannelAccess-CPextList-r16 in IE PUCCH-Config is shown below.

PUCCH-Config ::=                        SEQUENCE {

<Text omitted>

...,

[[

resourceToAddModList-r16                SEQUENCE (SIZE (1..maxNrofPUCCH-Resources)) OF PUCCH-Resource-r16     OPTIONAL, -- Need N

dl-DataToUL-ACK-r16                     SEQUENCE (SIZE (1..8)) OF INTEGER (-1..15)                            OPTIONAL, -- Need M

dl-DCI-triggered-UL-ChannelAccess-CPextList-r16 SEQUENCE (SIZE (1..16)) OF INTEGER (0..15)                        OPTIONAL, -- Need M

subslotLengthForPUCCH-r16               ENUMERATED {n2,n7}                       OPTIONAL, -- Need M

<Text omitted>

]]

}

Question 4: Do companies agree to change field name dl-DCI-triggered-UL-ChannelAccess-CPextList-r16? If yes, which option do companies prefer?

| Company | Agree/Disagree | Additional comments |
| --- | --- | --- |
| Huawei, HiSilicon | Agree | Agree that it is only applicable for DCI1-1, prefer the option with channelAccessConfigListForDCI-Format1-1-r16 |
| Ericsson | Agree | Any proposed option is fine. |
| MediaTek | Agree | Both options are ok. Slight preference for *ForDCI-Format1-1-r16 |
| Samsung | Agree | Agree with Huawei comment. |

## E258

The description of the issue is shown below.

[RIL]: E258 [Delegate]: Ericsson (Cecilia) [WI]: NR-U [Class]: 2 [Status]: DiscMeet2 [TDoc]: TBD [Proposed Conclusion]:

[Description]: The field description is too long and lists all parameters in the field name.

“UL-DCI triggered UL” corresponds to DCI format 0-1 and can be more generic and aligned with other names in PUSCH-Config which all refer to “DCI-Format-0-X”

[Proposed Change]: change name to “channelAccessConfigListForDCI-Format0-1-r16” or preferably “channelAccessConfigListForDCI-0-1-r16”

[Comments]:

The concerned field ul-dci-triggered-UL-ChannelAccess-CPext-CAPC-List-r16 in IE PUSCH-Config is shown below.

PUSCH-Config ::=                        SEQUENCE {

<Text omitted>

...,

[[

minimumSchedulingOffsetK2-r16           SetupRelease { MinSchedulingOffsetK2-Values-r16 }         OPTIONAL,  -- Need M

ul-dci-triggered-UL-ChannelAccess-CPext-CAPC-List-r16    SEQUENCE (SIZE (1..64)) OF INTEGER (0..63)    OPTIONAL,  -- Need M

pusch-RepTypeIndicator                  SEQUENCE {

pusch-RepTypeIndicatorForDCI-Format0-2-r16  ENUMERATED { pusch-RepTypeA, pusch-RepTypeB}      OPTIONAL,   -- Need M

pusch-RepTypeIndicatorForDCI-Format0-1-r16  ENUMERATED { pusch-RepTypeA, pusch-RepTypeB}      OPTIONAL    -- Need M

},

<Text omitted>

]]

}

Question 5: Do companies agree to change field name ul-dci-triggered-UL-ChannelAccess-CPext-CAPC-List-r16? If yes, which option do companies prefer?

| Company | Agree/Disagree | Additional comments |
| --- | --- | --- |
| Huawei, Hisilicon | Agree | Agree that the field is only applicable for DCI 0-1, Prefer the name channelAccessConfigListForDCI-Format0-1-r16 |
| Ericsson | Agree | Any proposed option is fine. |
| MediaTek | Agree | Both options are ok. Slight preference for *ForDCI-Format0-1-r16 |
| Samsung | Agree | Agree with Huawei comment. |

## N033

The description of the issue is shown below.

[RIL]: N033 [Delegate]: Nokia (Tero)  [WI]: IAB [Class]: 2 [Status]: DiscMail [Tdoc]: None [Proposed Conclusion]:

[Description]: This structure is very hard to read: first of all, there’s no need to add IAB-MT to every field since the -r16 suffixes already identify them uniquely. the differences between teh two branches of “explicit” is also very hard to see, and the field names do not help.

[Proposed Change]: suggest to use more descriptive field names:

TDD-UL-DL-SlotConfig-IAB-MT-v16xy::=    SEQUENCE {

slotIndex-r16                           TDD-UL-DL-SlotIndex,

symbols-r16                             CHOICE {

allDownlink-r16                         NULL,

allUplink-r16                           NULL,

explicit-DownlinkFirst-r16              SEQUENCE {

nrofDownlinkSymbols-r16                 INTEGER (1..maxNrofSymbols-1)    OPTIONAL, -- Need FFS

nrofUplinkSymbols-r16                   INTEGER (1..maxNrofSymbols-1)    OPTIONAL  -- Need FFS

},

explicit-UplinkFirst-r16                 SEQUENCE {

nrofUplinkSymbols-r16                   INTEGER (1..maxNrofSymbols-1)   OPTIONAL, -- Need FFS

nrofDownlinkSymbols-r16                 INTEGER (1..maxNrofSymbols-1)   OPTIONAL  -- Need FFS

}

}

}

And for field description, the following:

symbols-r16

The Symbols-r16 is used to configure an IAB-MT with the SlotConfig applicable for one serving cell. Value allDownlink indicates that all symbols in this slot are used for downlink; value allUplink indicates that all symbols in this slot are used for uplink; value explicit-DownlinkFirst indicates explicitly how many symbols in the beginning and end of this slot are allocated to downlink and uplink, respectively; value explicit-UplinkFirst indicates explicitly how many symbols in the beginning and end of this slot are allocated to uplink and downlink, respectively.

This would be far easier to read than the current one.

[Comments]: Rapp1: Looks to be exactly same structure, but shorter field names. But looks OK to me.

Referring to N033 it is proposed to change

- Field name symbols-IAB-MT-r16 to symbols-r16

- Field name explicit-r16 to explicit-DownlinkFirst-r16

- Field name explicit-IAB-MT-r16 to explicit-UplinkFirst-r16

and to update the field description of symbols-IAB-MT accordingly.

TDD-UL-DL-SlotConfig-IAB-MT-v16xy::=    SEQUENCE {

slotIndex-r16                           TDD-UL-DL-SlotIndex,

symbols-IAB-MT-r16                      CHOICE {

allDownlink-r16                         NULL,

allUplink-r16                           NULL,

explicit-r16                            SEQUENCE {

nrofDownlinkSymbols-r16                 INTEGER (1..maxNrofSymbols-1)   OPTIONAL, -- Need FFS

nrofUplinkSymbols-r16                   INTEGER (1..maxNrofSymbols-1)   OPTIONAL  -- Need FFS

},

explicit-IAB-MT-r16                     SEQUENCE {

nrofDownlinkSymbols-r16                 INTEGER (1..maxNrofSymbols-1)   OPTIONAL, -- Need FFS

nrofUplinkSymbols-r16                   INTEGER (1..maxNrofSymbols-1)   OPTIONAL  -- Need FFS

}

}

}

symbols-IAB-MT

The Symbols-IAB-MT is used to configure an IAB-MT with the SlotConfig applicable for one serving cell. Value allDownlink indicates that all symbols in this slot are used for downlink; value allUplink indicates that all symbols in this slot are used for uplink; value explicit indicates explicitly how many symbols in the beginning and end of this slot are allocated to downlink and uplink, respectively; value explicit-{IAB-MT} indicates explicitly how many symbols in the beginning and end of this slot are allocated to uplink and downlink, respectively.

Question 6: Do companies agree to change field names and field description in TDD-UL-DL-SlotConfig-IAB-MT-v16xy?

| Company | Agree/Disagree | Additional comments |
| --- | --- | --- |
| Huawei, HiSilicon | Disagree | The TDD-UL-DL-SlotConfig-IAB-MT IE is under discussion in IAB WI CR discussion [044] and is going to be deleted as the consequence of H696. Companies are welcome to disc |
| Samsung |  | We have same understanding with Huawei i.e. still under discussion in IAB WI. Regarding to change the field name, we don't have strong view. |

## S463

The description of the issue is shown below.

[RIL]: S463 [Delegate]: Samsung (Sangbum Kim)  [WI]: MDT [Class]: 2 [Status]: Tdoc [TDoc]: R2-2002826 [Proposed Conclusion]:

[Description]: In order to reflect the intention clearly, need to rename both areaConfigForServing and areaConfigForNeighbour

[Proposed Change]: Change the name and field descriptions of areaConfigForServing and areaConfigForNeighbour as to areaConfig and interFreqTargetList

[Comments]:

The proposed changes would affect IE AreaConfiguration-r16 as shown below. It should be noted that the proposed changes were already captured in the MDTSON WI CR.

-- ASN1START

-- TAG-AREACONFIGURATION-START

AreaConfiguration-r16 ::=        SEQUENCE {

areaConfig-r16         AreaConfig-r16,

interFreqTargetList-r16       InterFreqTargetList-r16              OPTIONAL

}

AreaConfig-r16 ::=     CHOICE {

cellGlobalIdList-r16             CellGlobalIdList-r16,

trackingAreaCodeList-r16         TrackingAreaCodeList-r16,

trackingAreaIdentityList-r16     TrackingAreaIdentityList-r16

}

InterFreqTargetList-r16 ::=   SEQUENCE {

dl-CarrierFreq	                   ARFCN-ValueNR,

frequencyBandList                MultiFrequencyBandListNR,

cellList                         SEQUENCE (SIZE (1..32)) OF  PhysCellId  OPTIONAL

}

CellGlobalIdList-r16 ::=         SEQUENCE (SIZE (1..32)) OF CGI-Info-Logging-r16

TrackingAreaCodeList-r16 ::=     SEQUENCE (SIZE (1..8)) OF TrackingAreaCode

TrackingAreaIdentityList-r16 ::= SEQUENCE (SIZE (1..8)) OF TrackingAreaIdentity-r16

TrackingAreaIdentity-r16 ::=     SEQUENCE {

plmn-Identity-r16                PLMN-Identity,

trackingAreaCode-r16             TrackingAreaCode

}

-- TAG-AREACONFIGURATION-STOP

-- ASN1STOP

Question 7: Do companies agree to change name and field descriptions of areaConfigForServing and areaConfigForNeighbour to areaConfig and interFreqTargetList?

| Company | Agree/Disagree | Additional comments |
| --- | --- | --- |
| Huawei, HiSilicon | Agree | At RAN2#109b-e meeting, this RIL was discussed and agreed. It has been captured in the latest MDT 38.331 CR. |
| MediaTek | Agree | Agree with Huawei’s comments. |
| Samsung | Agree | We are proponent. |

## E228

The description of the issue is shown below. E228 is related to grouping of configurable fields in PUSCH-Config.

[RIL]: E228 [Delegate]: Ericsson (Zhenhua)  [WI]: URLLC [Class]: 2 [Status]: Tdoc2 [TDoc]: R2-2004952 [Proposed Conclusion]:

[Description]:  The grouping of these configurable fields is not consistent. Some of them are grouped with DCI format, while some others are grouped with functionality, such as Priority indicator. Once the grouping is done, the name can be shortened.

[Proposed Change]:

Group the configurable fields by DCI formats and shorten the name by removing, e.g., “forDCI-Format0-2-r16”.

[Comments]: Rapp3: Ericsson is asked to provide tdoc. All these RILs concern same topic: E228 and E230.

Below the current structure of PUSCH-Config is shown (to simplify matters only the field names are shown).

PUSCH-Config ::=                        SEQUENCE {

<Text omitted>

...,

[[

minimumSchedulingOffsetK2-r16

ul-dci-triggered-UL-ChannelAccess-CPext-CAPC-List-r16

pusch-RepTypeIndicator                  SEQUENCE {

pusch-RepTypeIndicatorForDCI-Format0-2-r16

pusch-RepTypeIndicatorForDCI-Format0-1-r16

},

configurableFieldForDCI-Format0-2       SEQUENCE {

harq-ProcessNumberSizeForDCI-Format0-2-r16

dmrs-SequenceInitializationForDCI-Format0-2-r16

numberOfBitsForRV-ForDCI-Format0-2-r16

antennaPortsFieldPresenceForDCI-Format0-2-r16

...

},

resourceAllocationType1GranularityForDCI-Format0-2-r16

frequencyHoppingForDCI-Format0-2-r16    CHOICE {

pusch-RepTypeA

pusch-RepTypeB

}

frequencyHoppingOffsetListsForDCI-Format0-2-r16

uci-OnPUSCH-ForDCI-Format0-2-r16

uci-OnPUSCH-ListForDCI-Format0-2-r16

uci-OnPUSCH-ListForDCI-Format0-1-r16

pusch-TimeDomainAllocationListForDCI-Format0-2-r16

pusch-TimeDomainAllocationListForDCI-Format0-1-r16

maxRankForDCI-Format0-2-r16

codebookSubsetForDCI-Format0-2-r16

dmrs-UplinkForPUSCH-MappingTypeA-ForDCI-Format0-2-r16

dmrs-UplinkForPUSCH-MappingTypeB-ForDCI-Format0-2-r16

mcs-TableForDCI-Format0-2-r16

mcs-TableTransformPrecoderForDCI-Format0-2-r16

resourceAllocationForDCI-Format0-2-r16

priorityIndicator                           SEQUENCE {

priorityIndicatorForDCI-Format0-2-r16

priorityIndicatorForDCI-Format0-1-r16

}

invalidSymbolPatternIndicator               SEQUENCE {

invalidSymbolPatternIndicatorForDCI-Format0-1-r16

invalidSymbolPatternIndicatorForDCI-Format0-2-r16

}

frequencyHoppingForDCI-Format0-1-r16

invalidSymbolPattern-r16

pusch-PowerControl-v16xy

ul-FullPowerTransmission-r16

pusch-TimeDomainAllocationListForMultiPUSCH-r16

]]

}

In the contribution R2-2004952 [3] a proposal is given how the configurable fields in PUSCH-Config can be grouped more efficiently. In summary, the configurable fields are grouped now under the new sequences configurableFieldsForDCI-Format0-1 and configurableFieldsForDCI-Format0-2 as shown below. In this context long field names with suffix “ForDCI-Formatx-x” have been shortened.

PUSCH-Config ::=                        SEQUENCE {

<Text omitted>

...,

[[

minimumSchedulingOffsetK2-r16

ul-dci-triggered-UL-ChannelAccess-CPext-CAPC-List-r16

configurableFieldsForDCI-Format0-1   SEQUENCE {

frequencyHoppingDCI-0-1-r16

invalidSymbolPatternIndicatorDCI-0-1-r16

priorityIndicatorDCI-0-1-r16

pusch-RepTypeIndicatorDCI-0-1-r16

pusch-TimeDomainAllocationListDCI-0-1-r16

uci-OnPUSCH-ListDCI-0-1-r16

...

},

configurableFieldsForDCI-Format0-2       SEQUENCE {

antennaPortsFieldPresenceDCI-0-2-r16

dmrs-SequenceInitializationDCI-0-2-r16

dmrs-UplinkForPUSCH-MappingTypeA-DCI-0-2-r16

dmrs-UplinkForPUSCH-MappingTypeB-DCI-0-2-r16

frequencyHoppingDCI-0-2-r16    CHOICE {

forRepTypeA

forRepTypeB

}

frequencyHoppingOffsetListsDCI-0-2-r16

harq-ProcessNumberDCI-0-2-r16

invalidSymbolPatternIndicatorDCI-0-2-r16

maxRankDCI-0-2-r16

mcs-TableDCI-0-2-r16

mcs-TableTransformPrecoderDCI-0-2-r16

numberOfBitsRV-DCI-0-2-r16

priorityIndicatorDCI-0-2-r16

pusch-RepTypeIndicatorDCI-0-2-r16

pusch-TimeDomainAllocationListDCI-0-2-r16

resourceAllocationDCI-0-2-r16

resourceAllocationType1GranularityDCI-0-2-r16

uci-OnPUSCH-DCI-0-2-r16

uci-OnPUSCH-ListDCI-0-2-r16

...

},

invalidSymbolPattern-r16

pusch-PowerControl-v16xy

ul-FullPowerTransmission-r16

pusch-TimeDomainAllocationListForMultiPUSCH-r16

]]

}

Question 8: Do companies agree to re-structure the fields in PUSCH-Config? If yes, do you agree with the proposal acc. to R2-2004952?

| Company | Agree/Disagree | Additional comments |
| --- | --- | --- |
| Huawei, HiSilicon | Agree apart from the name | "configurableFieldsForDCI" means fields of the DCI that can be configured, e.g. size of HARQ process ID, RV, etc. However, things like repetition, resource allocation, etc, are just parameters for transmission, not related to configuration of fields of the DCI. So we think the name should be changed e.g. push-ParametersOnlyforDCI-Format1-2 (same for 0-2) |
| Intel | Agree | The updated structure looks tidier |
| Ericsson | Agree | Agree also with the naming suggestion from Huawei above. |
| MediaTek | Agree | Huawei’s proposal is also ok without terms ‘PUSCH’ and ‘only’ in the field name as they are redundant. |
| Samsung | Agree | We are fine to restructure but we just wonder that some fields appear in both branches with different name but with same content. I.e. they could be in common part rather than duplicated for each choice. |

## E230

The description of the issue is shown below. E230 is related to grouping of configurable fields in PDSCH-Config.

[RIL]: E230 [Delegate]: Ericsson (Zhenhua) [WI]: URLLC [Class]: 2 [Status]: Tdoc2 [TDoc]: R2-2004952 [Proposed Conclusion]:

[Description]: There is no consistent structure in introducing these new fields in Rel-16 and they are not readable. There are some other IEs introduced by the URLLC WI.

[Proposed Change]: Group the configurable fields by different DCI formats 1-2 and DCI formats 1-1. The names within the group can be shorted by removing “ForDCIFormat1-2” and the field description can be simplified.   We should change also for other IEs like SRS-Config,

[Comments]: Rapp3: Ericsson is asked to provide tdoc. All these RILs concern same topic: E228 and E230.

Below the current structure of PDSCH-Config is shown (to simplify matters only the field names are shown).

PDSCH-Config ::=                        SEQUENCE {

<Text omitted>

...,

[[

maxMIMO-Layers-r16

minimumSchedulingOffsetK0-r16

prb-BundlingTypeForDCI-Format1-2-r16    CHOICE {

staticBundling-r16                      SEQUENCE {

bundleSize-r16

},

dynamicBundling-r16                     SEQUENCE {

bundleSizeSet1-r16

bundleSizeSet2-r16

}

}

rateMatchPatternGroup1ForDCI-Format1-2-r16

rateMatchPatternGroup2ForDCI-Format1-2-r16

aperiodicZP-CSI-RS-ResourceSetsToAddModListForDCI-Format1-2-r16

aperiodicZP-CSI-RS-ResourceSetsToReleaseListForDCI-Format1-2-r16

pdsch-TimeDomainAllocationListForDCI-Format1-2-r16

configurableFieldForDCI-Format1-2               SEQUENCE {

harq-ProcessNumberSizeForDCI-Format1-2-r16

dmrs-SequenceInitializationForDCI-Format1-2-r16

numberOfBitsForRV-ForDCI-Format1-2-r16

antennaPortsFieldPresenceForDCI-Format1-2-r16

...

},

resourceAllocationType1GranularityForDCI-Format1-2-r16

vrb-ToPRB-InterleaverForDCI-Format1-2-r16

dmrs-DownlinkForPDSCH-MappingTypeAForDCI-Format1-2-r16

dmrs-DownlinkForPDSCH-MappingTypeBForDCI-Format1-2-r16

referenceOfSLIVForDCI-Format1-2-r16

mcs-TableForDCI-Format1-2-r16

resourceAllocationForDCI-Format1-2-r16

priorityIndicator                               SEQUENCE {

priorityIndicatorForDCI-Format1-2-r16

priorityIndicatorForDCI-Format1-1-r16

}

dataScramblingIdentityPDSCH2-r16

pdsch-TimeDomainAllocationList-v16xy

repetitionSchemeConfig-r16

]]

}

In the contribution R2-2004952 [3] a proposal is given how the configurable fields in PDSCH-Config can be grouped more efficiently. In summary, the configurable fields are grouped now under the new sequence configurableFieldsForDCI-Format1-2-r16 as shown below. In this context long field names with suffix “ForDCI-Formatx-x” have been shortened.

PDSCH-Config ::=                        SEQUENCE {

<Text omitted>

...,

[[

maxMIMO-Layers-r16

minimumSchedulingOffsetK0-r16

configurableFieldsForDCI-Format1-2-r16        SEQUENCE {

antennaPortsFieldPresenceDCI-1-2-r16

aperiodicZP-CSI-RS-ResourceSetsToAddModList-DCI-1-2-r16

aperiodicZP-CSI-RS-ResourceSetsToReleaseList-DCI-1-2-r16

dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2-r16

dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2-r16

dmrs-SequenceInitializationDCI-1-2-r16

harq-ProcessNumberDCI-1-2-r16

mcs-TableDCI-1-2-r16

numberOfBitsForRV-DCI-1-2-r16

pdsch-TimeDomainAllocationListDCI-1-2-r16

prb-BundlingTypeDCI-1-2-r16    CHOICE {

staticBundling-r16                     SEQUENCE {

bundleSize-r16

},

dynamicBundling-r16                     SEQUENCE {

bundleSizeSet1-r16

bundleSizeSet2-r16

}

}

priorityIndicatorDCI-1-2-r16

rateMatchPatternGroup1DCI-1-2-r16

rateMatchPatternGroup2DCI-1-2-r16

referenceOfSLIV-DCI-1-2-r16

resourceAllocationDCI-1-2-r16

resourceAllocationType1GranularityDCI-1-2-r16

vrb-ToPRB-InterleaverDCI-1-2-r16

...

},

priorityIndicatorDCI-1-1-r16

dataScramblingIdentityPDSCH2-r16

pdsch-TimeDomainAllocationList-v16xy

repetitionSchemeConfig-r16

]]

}

Offline a further alternative for grouping the fields "ForDCI-Format1-2" in PDSCH-Config was provided by Huawei. The table below list such parameters, and compares them with legacy parameters in PDSCH-Config:

|  | ForDCI-Format1-2-r16 | PDSCH-Config |
| --- | --- | --- |
| prb-BundlingType | CHOICE { staticBundling-r16 SEQUENCE { bundleSize-r16 ENUMERATED { n4, wideband } OPTIONAL -- Need S }, dynamicBundling-r16 SEQUENCE { bundleSizeSet1-r16 ENUMERATED { n4, wideband, n2-wideband, n4-wideband } OPTIONAL, -- Need S bundleSizeSet2-r16 ENUMERATED { n4, wideband } OPTIONAL -- Need S } } | CHOICE { staticBundling SEQUENCE { bundleSize ENUMERATED { n4, wideband } OPTIONAL -- Need S }, dynamicBundling SEQUENCE { bundleSizeSet1 ENUMERATED { n4, wideband, n2-wideband, n4-wideband } OPTIONAL, -- Need S bundleSizeSet2 ENUMERATED { n4, wideband } OPTIONAL -- Need S } |
| rateMatchPatternGroup1 | RateMatchPatternGroup | RateMatchPatternGroup |
| rateMatchPatternGroup2 | RateMatchPatternGroup | RateMatchPatternGroup |
| aperiodicZP-CSI-RS-ResourceSetsToAddModList | SEQUENCE (SIZE (1..maxNrofZP-CSI-RS-ResourceSets)) OF ZP-CSI-RS-ResourceSet | SEQUENCE (SIZE (1..maxNrofZP-CSI-RS-ResourceSets)) OF ZP-CSI-RS-ResourceSet |
| aperiodicZP-CSI-RS-ResourceSetsToReleaseList | SEQUENCE (SIZE (1..maxNrofZP-CSI-RS-ResourceSets)) OF ZP-CSI-RS-ResourceSetId | SEQUENCE (SIZE (1..maxNrofZP-CSI-RS-ResourceSets)) OF ZP-CSI-RS-ResourceSetId |
| pdsch-TimeDomainAllocationList | SetupRelease { PDSCH-TimeDomainResourceAllocationList } | SetupRelease { PDSCH-TimeDomainResourceAllocationList } |
| harq-ProcessNumberSize | INTEGER (0..4) | - |
| dmrs-SequenceInitialization | ENUMERATED {enabled} | - |
| numberOfBitsForRV | INTEGER (0..2) | - |
| antennaPortsFieldPresence | ENUMERATED {enabled} | - |
| resourceAllocationType1Granularity | ENUMERATED {n2,n4,n8,n16} | - |
| vrb-ToPRB-InterleaverForDCI | ENUMERATED {n2, n4} | - |
| dmrs-DownlinkForPDSCH-MappingTypeA | SetupRelease { DMRS-DownlinkConfig } | SetupRelease { DMRS-DownlinkConfig } |
| dmrs-DownlinkForPDSCH-MappingTypeB | SetupRelease { DMRS-DownlinkConfig } | SetupRelease { DMRS-DownlinkConfig } |
| referenceOfSLIV | ENUMERATED {enabled} | - |
| mcs-Table | ENUMERATED {qam256, qam64LowSE} | ENUMERATED {qam256, qam64LowSE} |
| resourceAllocation | ENUMERATED { resourceAllocationType0, resourceAllocationType1, dynamicSwitch} | ENUMERATED { resourceAllocationType0, resourceAllocationType1, dynamicSwitch} |
| priorityIndicator | ENUMERATED {enabled} | - |

In this alternative:

- Fields with green highlights are removed from PDSCH-Config, instead, they are included in another instance of PDSCH-Config, which is added in BWP-DownlinkDedicated

- Other fields in PDSCH-Config are grouped under the new sequences pdschParametersOnlyForDCI-Format1-2-r16 and pdschParametersOnlyForDCI-Format1-1-r16 as shown below.

BWP-DownlinkDedicated ::=           SEQUENCE {

pdcch-Config                        SetupRelease { PDCCH-Config }               OPTIONAL,   -- Need M

pdsch-Config                        SetupRelease { PDSCH-Config }               OPTIONAL,   -- Need M

sps-Config                          SetupRelease { SPS-Config }                 OPTIONAL,   -- Need M

radioLinkMonitoringConfig           SetupRelease { RadioLinkMonitoringConfig }  OPTIONAL,   -- Need M

...,

[[

sps-ConfigMulti-r16                  SPS-ConfigMulti-r16                         OPTIONAL,   -- Need M

beamFailureRecoverySCellConfig-r16  SetupRelease {BeamFailureRecoverySCellConfig-r16}                 OPTIONAL,    -- Cond SCellOnly

pdsch-ConfigForDCI-Format1-2        SetupRelease { PDSCH-Config }                OPTIONAL    -- Need M

]],

[[

sl-PDCCH-Config-r16                 SetupRelease { PDCCH-Config }                OPTIONAL,   -- Need M

sl-V2X-PDCCH-Config-r16             SetupRelease { PDCCH-Config }                OPTIONAL    -- Need M

]]

}

pdsch-Config, pdsch-Config-ForDCI-Fomat1-2

UE specific PDSCH configuration for one BWP. Parameters in pdsch-ConfigForDCI-Fomat1-2 apply to DCI format 1_2. pdsch-ConfigForDCI-Fomat1-2 can only be configured when pdsch-Config is also configured.

Editor's note: For a dormant BWP, if this field is configured, it is FFS if IEs other than those related to TCI state are applicable.

PDSCH-Config ::=                        SEQUENCE {

<Text omitted>

...,

[[

maxMIMO-Layers-r16

minimumSchedulingOffsetK0-r16

harq-ProcessNumberSizeForDCI-Format1-2-r16

dmrs-SequenceInitializationForDCI-Format1-2-r16

numberOfBitsForRV-ForDCI-Format1-2-r16

antennaPortsFieldPresenceForDCI-Format1-2-r16

resourceAllocationType1GranularityForDCI-Format1-2-r16

vrb-ToPRB-InterleaverForDCI-Format1-2-r16

referenceOfSLIVForDCI-Format1-2-r16

priorityIndicatorForDCI-Format1-2-r16

...

}                                  				-- Cond dciFormat1-2

pdschParametersOnlyForDCI-Format1-1-r16               SEQUENCE {

priorityIndicatorForDCI-Format1-1-r16

...

}

dataScramblingIdentityPDSCH2-r16

pdsch-TimeDomainAllocationList-v16xy

repetitionSchemeConfig-r16

]]

}

| dciFormat1-2 | The field is optionally present, Need R, in pdsch-ConfigForDCIFormat1-2. The field is absent in pdsch-Config. |
| --- | --- |

Question 9: Do companies agree to re-structure the fields in PDSCH-Config? If yes, which option do companies prefer?

| Company | Agree/Disagree | Additional comments |
| --- | --- | --- |
| Huawei, HiSilicon | Agree (see comments) | As commented before, the name "configurableFields" should not be used as it makes no sense. As explained, there are two options: option 1) keep fields unchanged from legacy as additional fields in PDSCH-Config option 2) remove these fields and use a separate instance of PDSCH-Config Option 2 is more readable. Unless there are technical issues with it, we prefer option 2. |
| Intel | Agree | Agree that grouping is quite useful. Not knowing much about the L1 configuration aspects and commenting just from the structural point of view, one possible drawback with option 2 is that it is not clear which of the common fields (outside of the group with condition dciFormat1-2) are applicable for format 1-2. Perhaps it is obvious to those familiar with L1 – if so, option 2 avoids duplication of the common fields. |
| Ericsson | Agree | Agree that option 2) is more readable. After further checking offline, there can be a lot more work to design properly based on option 2). - In Rel-15 PUSCH-Config, it is not clear if there are any fields which are applicable only for format 1-0. If there are, they should be clearly indicated. - In Rel-15 PUSCH-Config, some but not all fields are applicable for format 1-2. They should be clearly indicated. - It can be clumsy in the future to extend, if some new parameters are agreed to be included for format 1-2 while these parameters are there already for format 1-1. If RAN2 go with option 2, then one may wonder why there isn’t a similar structure for uplink. |
| MediaTek | Agree | Both options can work. Prefer option 2 as it removes a number of redundant fields and would therefore be easier to read. |

## S656

The description of the issue is shown below.

[RIL]: S656 [Delegate]: Samsung (Seungri Jin)  [WI]: MIMO [Class]: 2 [Status]: DiscMail [TDoc]: None [Proposed Conclusion]:

[Description]: No need two-level CHOICE structure in CodebookConfig-r16 IE because there are no more entries in this CHOICE structure below codebookType.

[Proposed Change]: Remove codebookType CHOICE structure and type2 SEQUENCE structure. Then change the field name of subType to codebookType-r16 to follow the RAN1 suggestion in R1-2001478.

[Comments]: ]: Nokia (Tero): We should retain the same structure as in Rel-15 for specification compatibility: Otherwise the Rel-15 and Rel-16 parameters use different structures, which may requires RAN1 changes as well. This was already discussed during RAN2#108, which is why the structure is like this.

If we follow the proposal from S656 the resulting structure of CodebookConfig-r16 will look like as below:

CodebookConfig-r16  ::=                SEQUENCE  {

codebookType                           CHOICE {

type2                                  SEQUENCE {

subTypecodebookType-r16                 CHOICE {

typeII-r16                             SEQUENCE  {

n1-n2-codebookSubsetRestriction-r16    CHOICE {

two-one                                BIT STRING (SIZE (16)),

two-two                                BIT STRING (SIZE (43)),

four-one                               BIT STRING (SIZE (32)),

three-two                              BIT STRING (SIZE (59)),

six-one                                BIT STRING (SIZE (48)),

four-two                               BIT STRING (SIZE (75)),

eight-one                              BIT STRING (SIZE (64)),

four-three                             BIT STRING (SIZE (107)),

six-two                                BIT STRING (SIZE (107)),

twelve-one                             BIT STRING (SIZE (96)),

four-four                              BIT STRING (SIZE (139)),

eight-two                              BIT STRING (SIZE (139)),

sixteen-one                            BIT STRING (SIZE (128))

},

typeII-RI-Restriction-r16              BIT STRING (SIZE(4))

},

typeII-PortSelection-r16  SEQUENCE {

portSelectionSamplingSize-r16          ENUMERATED {n1, n2, n3, n4},

typeII-PortSelectionRI-Restriction-r16 BIT STRING (SIZE (4))

}

},

numberOfPMI-SubbandsPerCQI-Subband-r16 INTEGER (1..2),

paramCombination-r16                   INTEGER (1..8)

}

}

}

Question 10: Do companies agree to change structure of IE CodebookConfig-r16?

| Company | Agree/Disagree | Additional comments |
| --- | --- | --- |
| Huawei, HiSilicon | Disagree | The current structure matches better with the field descriptions and it does not result in any useless bit. |
| Intel | Disagree | Disagree based on Tero’s comments to the RIL |
| MediaTek | Disagree | Prefer to retain Rel-15 structure. |
| Samsung |  | We are fine to not agree it. |

## Q022

The description of the issue is shown below.

[RIL]: Q022 [Delegate]: Qualcomm (Masato)  [WI]: MIMO [Class]: 2 [Status]: PropReject2 [TDoc]: None [Proposed Conclusion]:

[Description]: It was discussed whether to use 'SEQUENCE' or 'CHOICE' here in R2-2001677 and RAN2 #109e meeting. The conclusion was to ask RAN1 if schemes 2a/2b/3 and scheme 4 (slotBased) are always mutually exclusive or not. RAN1 reply LS in R2-2004251 states that the schemes 2a/2b/3 and scheme 4 should be mutually exclusive.

[Proposed Change]: Change 'SEQUENCE' to 'CHOICE'

[Comments]: Rapp3: RAN1 LS was discussed in email discussion (POST109bis-e)(903)(MIMO), and RAN2 agreed to not change asn.1, but add IE description text above (refer to restriction details as specified in RAN1 spec).

Below the latest description of IE RepetitionSchemeConfig is shown where a RAN1 reference to configuration limitations for the repetition schemes has been added.

#### –	RepetitionSchemeConfig

The IE RepetitionSchemeConfig is used to configure the UE with repetition schemes according to restrictions as specified in TS 38.214 [19] clause 5.1.

-- ASN1START

-- TAG-REPETITIONSCHEMECONFIG-START

RepetitionSchemeConfig-r16 ::= SEQUENCE {

fdm-TDM-r16                    SetupRelease { FDM-TDM-r16 }                      OPTIONAL, -- Need M

slotBased                      SetupRelease { SlotBased-r16 }                    OPTIONAL  -- Need M

}

FDM-TDM-r16 ::=                SEQUENCE {

repetitionScheme-r16           ENUMERATED {fdmSchemeA, fdmSchemeB,tdmSchemeA },

startingSymbolOffsetK-r16      INTEGER (0..7)                                    OPTIONAL  -- Need R

}

SlotBased-r16 ::=              SEQUENCE {

tciMapping-r16                 ENUMERATED {cyclicMapping, sequenticalMapping},

sequenceOffsetforRV-r16        INTEGER (1..3)

}

-- TAG-REPETITIONSCHEMECONFIG-STOP

-- ASN1STOP

Question 11: Do companies agree with rapporteur’s suggestion to add a RAN1 reference to configuration limitations for the repetition schemes in the description of IE RepetitionSchemeConfig and keep the SEQUENCE type for the IE?

| Company | Agree/Disagree | Additional comments |
| --- | --- | --- |
| Huawei, HiSilicon | Disagree | This is handled in the MIMO WI and the conclusion is that CHOICE will be used. |
| Ericsson | Agree | My understanding is different. Need to check MIMO WI status. |
| MediaTek | Disagree | We share the same understanding as Huawei |
| Samsung | Disagree | Same understanding with Huawei. |

## I654

The description of the issue is shown below.

[RIL]: I654 [Delegate]: Intel (Sudeep)  [WI]: URLLC [Class]: 2 [Status]: ConcAgree WI-CR [TDoc]: R2-2004602 [Proposed Conclusion]:

[Description]: DL-AM-RLC-v16xy is not used anywhere and is an orphan.

[Proposed Change]: Update where it is to be used.

[Comments]:

IE DL-AM-RLC-v16xy in RLC-Config was introduced by URLLC CR in order to add shorter values of [ms1, ms2, ms3, ms4] for t-StatusProhibit. In RAN2#109bis-e the issue I654 was discussed during offline discussion [071] but could not be resolved as it was not fully clear how to add IE DL-AM-RLC-v16xy in RRC. Now, in the contribution R2-2004602 [4], the following solution is proposed to resolve I654:

- Add IE DL-AM-RLC-v16xy (with Need M) as non-critical extension of IE RLC-Config in IE RLC-BearerConfig.

- Furthermore, clarify in the field description of rlc-Config (in RLC-BearerConfig field descriptions) that “The network may configure rlc-Config-v16xy only when rlc-Config (without suffix) is set to am”, see below.

RLC-BearerConfig ::=                  SEQUENCE {

logicalChannelIdentity                LogicalChannelIdentity,

servedRadioBearer                     CHOICE {

srb-Identity                          SRB-Identity,

drb-Identity                          DRB-Identity

}                                                           OPTIONAL,   -- Cond LCH-SetupOnly

reestablishRLC                            ENUMERATED {true}     OPTIONAL,   -- Need N

rlc-Config                                RLC-Config            OPTIONAL,   -- Cond LCH-Setup

mac-LogicalChannelConfig                  LogicalChannelConfig  OPTIONAL,   -- Cond LCH-Setup

...,

[[

rlc-Config-v16xy                          RLC-Config-v16xy       OPTIONAL   -- Need M

]]

}

rlc-Config

Determines the RLC mode (UM, AM) and provides corresponding parameters. RLC mode reconfiguration can only be performed by DRB release/addition or full configuration.

The network may configure rlc-Config-v16xy only when rlc-Config (without suffix) is set to am.

RLC-Config ::=                      CHOICE {

am                                  SEQUENCE {

ul-AM-RLC                           UL-AM-RLC,

dl-AM-RLC                           DL-AM-RLC

},

um-Bi-Directional                   SEQUENCE {

ul-UM-RLC                           UL-UM-RLC,

dl-UM-RLC                           DL-UM-RLC

},

um-Uni-Directional-UL               SEQUENCE {

ul-UM-RLC                           UL-UM-RLC

},

um-Uni-Directional-DL               SEQUENCE {

dl-UM-RLC                           DL-UM-RLC

},

...

}

RLC-Config-v16xy ::=			SEQUENCE {

dl-AM-RLC-v16xy					DL-AM-RLC-v16xy

}

DL-AM-RLC-v16xy ::=             SEQUENCE {

t-StatusProhibit-v16xy         T-StatusProhibit-v16xy			    OPTIONAL   -- Need R

}

T-StatusProhibit-v16xy ::=         ENUMERATED { ms1, ms2, ms3, ms4, spare4, spare3, spare2, spare1}

Question 12: Do companies agree on the solution to add IE DL-AM-RLC-v16xy (with Need M) as non-critical extension of IE RLC-Config in IE RLC-BearerConfig?

| Company | Agree/Disagree | Additional comments |
| --- | --- | --- |
| Huawei, HiSilicon | Agree |  |
| Intel | Agree (one of the proponents) |  |
| Ericsson | Agree | Probably better to add need R for dl-AM-RLC-v16xy ? |
| MediaTek | Agree |  |
| Samsung | Agree |  |

## S461

The description of the issue is shown below.

[RIL]: S461 [Delegate]: Samsung (Sangbum Kim)  [WI]: MDT [Class]: 2 [Status]: DiscMail [TDoc]: R2-2002826 [Proposed Conclusion]:

[Description]: For the future extension, add the nonCriticalExtension and laterNonCriticalExtension fields to the LoggedMeasurementConfiguration message.

In addition, RAN2 is requested to discuss whether to support delta signalling alike for RRM measurements (i.e. that in future it should e possible to merely signal an event triggered logging configuration to be added). If so, we propose to delta signalling at least for reportType (by SetupRelease with need M).

RAN2 can discuss ASN.1 details in R2-2002826.

[Proposed Change]:

LoggedMeasurementConfiguration-r16-IEs ::=       SEQUENCE {

traceReference-r16				TraceReference-r16,

traceRecordingSessionRef-r16	OCTET STRING (SIZE (2)),

tce-Id-r16						OCTET STRING (SIZE (1)),

absoluteTimeInfo-r16			AbsoluteTimeInfo-r16,

areaConfiguration-r16			AreaConfiguration-r16			OPTIONAL,	--Need R

plmn-IdentityList-r16			PLMN-IdentityList3-r16			OPTIONAL,	--Need R

bt-NameList-r16					BT-NameListConfig-r16					OPTIONAL, 	--Need R

wlan-NameList-r16				WLAN-NameListConfig-r16				OPTIONAL, 	--Need R

sensor-NameList-r16			    Sensor-NameListConfig-r16		OPTIONAL, 	--Need R,

loggingDuration-r16				LoggingDuration-r16,

reportType                                  CHOICE {

periodical                                  LoggedPeriodicalReportConfig-r16,

eventTriggered                              LoggedEventTriggerConfig-r16

}

lateNonCriticalExtension       OCTET STRING                 OPTIONAL,

nonCriticalExtension           LoggedMeasurementConfiguration-vNxy-IEs  OPTIONAL

}

[Comments]: Rapp1: Ok to add the nonCriticalExtension and laterNonCriticalExtension fields. Whether to support Delta signalling need further discussion based on R2-2002826.

The proposal for adding nonCriticalExtension and lateNonCriticalExtension fields to the LoggedMeasurementConfiguration message is straightforward and consistent with what we do when new messages are introduced in RRC. Therefore, remaining issue is whether to allow delta signalling for reportType (by SetupRelease with need M). In this context it should be noted that delta signalling for bt-NameList-r16, wlan-NameList-r16 and sensor-NameList-r16 were already captured in the MDTSON WI CR.

Question 13a: Do companies agree to add nonCriticalExtension and lateNonCriticalExtension fields to the LoggedMeasurementConfiguration message?

Question 13b: Do companies agree to allow delta signalling for reportType (by SetupRelease with need M)?

| Company | Agree/Disagree | Additional comments |
| --- | --- | --- |
| Huawei, HiSilicon | Agree | In MDT session in this meeting, RAN2 has agreed: adding nonCriticalExtension and lateNonCriticalExtension fields to the LoggedMeasurementConfiguration message And it has been captured in the latest MDT 38.331 CR. So Q13a has been resolved. For Q13b, we do not see a strong need to use delta signalling for reportType. |
| Intel | Disagree 13b | Logged measurement configuration is not likely to change frequently to motivate delta signalling. Didn’t follow properly what it meant by delta signalling of reportType here. Is it to allow reportType to be updated frequently? If so, other fields should be Need M? |
| Ericsson | Agree on 13a, disagree on 13b. |  |
| MediaTek | Agree (for 13a) Disagree (13b) | We don’t see a need to use delta signalling for reportType. |
| Samsung | Agree | We are the proponent. For 13b, we currently have mandatory field that is a choice between periodical and single event. Our main concern is that this seems not so extensible/ future proof i.e. one of these will always have to be defined. If we introduce some new options, we may end up with specifying that UE will have to ignore this. |

## N021

The description of the issue is shown below.

[RIL]: N021 [Delegate]: Nokia (Tero)  [WI]: PowSave [Class]: 2 [Status]: DiscMeet [TDoc]: None [Proposed Conclusion]:

[Description]: Inifinity value was agreed in RAN2#108 as per follows: “minimum K0/K2 value is signalled as UE assistance.  Value of infinity can be configured for the prohibit timer.”, but infinity value is not listed here.

[Proposed Change]: Add infinity to replace “spare2” as follows:

MinSchedulingOffsetPreferenceConfig-r16 ::=   SEQUENCE {

minSchedulingOffsetPreferenceProhibitTimer-r16 ENUMERATED {

s0, s0dot5, s1, s2, s3, s4, s5, s6, s7,

s8, s9, s10, s20, s30, infinity, spare1}

}

Generally, it’s not clear why infinity is not allowed for all of the prohibit timers?

[Comments]: Rapp1: Need to be discussed if infinity should be used only for this field.

RappPS: Agree that infinity needs to be added to this field only. The range of prohibit timers for the remaining timers have been discussed and agreed at R2-109e (see R2-2001912)

However, having looked at the Session minutes for NR-U, Power Savings, NTN and 2-step RACH in R2-2001664 (RAN2#109-e) the max value of 30s was agreed for all prohibit timers used for UE assistance incl. minimum scheduling offset see agreement #7 below. Therefore, it seems that previous agreement on ‘infinity’ value may not be valid anymore.

7.	The prohibit timer for UE assistance on DRX, aggregated bandwidth, number of cell, number of MIMO layers, releasePreference and minimum scheduling offset for power savings can be configured up to 30s.

Question 14: Do companies think that value ‘infinity’ should be added for minSchedulingOffsetPreferenceProhibitTimer-r16?

| Company | Agree/Disagree | Additional comments |
| --- | --- | --- |
| Huawei, HiSilicon | Disagree | Considering agreement reached in RAN2#109-e, agree that previous agreement on ‘infinity’ value may not be valid anymore. |
| Intel | Disagree | This was discussed in power saving WI and agreed not to introduce prohibit timer in general, and specifically on this timer. |
| MediaTek | Agree | The agreements from R2-109e were not intended to supersede earlier agreements, so agree that infinity can be signalled for minSchedulingOffsetPreferenceProhibitTimer only. Other prohibit timers do not need ‘infinity’, as their ranges were discussed in R2-109e. |
| Samsung | No strong view | We see no real need to have infinity. |

# Conclusion

xxx

# Reference

[1]	R2-2005318, 38331 Rel-16 Ph2 ASN.1 review file v166, Ericsson

[2]	R2-2005319, RIL list Rel-16 Ph2 ASN.1 review v166, Ericsson

[3]	R2-2004952, [E228][E230] On grouping similar parameters in PUSCH-Config/PDSCH-Config, Ericsson

[4]	R2-2004602, [I654] Adding DL AM RLC extension in NR RRC, Lenovo, Motorola Mobility, Intel Corporation