3GPP TSG-RAN WG2 Meeting #110-e	R2-2005741

Elbonia, 20 – 30 April 2020

Agenda item:	4.5.1

Source:	Nokia

Title:	Summary and discussion on [AT110e-][202][LTE15] LTE non-contiguous CA capabilities (Nokia)

Document for:	Discussion and Decision

# 1	Introduction

This document is to collect companies views on LTE non-contiguous CA capabilities, and the need to clarify their interpretation in TS36.331, based on the two set of CRs:

Scope:

    - Determine what can be agreed based on the Nokia CRs in R2-2005186, R2-2005187, R2-2005188, R2-2005189 and R2-2005190 and Huawei CRs in R2-2005481, R2-2005482, R2-2005483, R2-2005484, R2-2005485, R2-2005486 and R2-2005487

    - Determine from which release onwards a correction should be provided

# 2	Discussion

## 2.1	LTE non-contiguous CA capabilities interpretation

The documents in 1) and 2) concern pre-Rel-15 UE CA capabilities as shown below:

| Tdoc(s), Title, Company | Proposal(s) |
| --- | --- |
| 1) R2-2005186, R2-2005187, R2-2005188, R2-2005189, R2-2005190, “Clarification to UE capabilities for non-contiguous intra-band CA“ Nokia, Nokia Shanghai Bell | Discussed already in RAN2#109-e CRs from Rel-12 to clarify intra-band non-contiguous is handled as intra-band contiguous are agnostic to the order in which they are indicated in the band entries, for the CA of the same bandwidth class. |
| 2) R2-2005481, R2-2005482, R2-2005483, R2-2005484, R2-2005485, R2-2005486, R2-2005487 “Clarification on UE capability for intra-band non-continuous CA”, Huawei, Hisilicon | Discussed already in RAN2#109bis-e CRs from Rel-12 to clarify intra-band non-contiguous UE capabilities for carriers sharing the same uplink capability within intra-band non-contiguous CA are agnostic to the order in which they are indicated in the band entries. |

The set of CRs 1) was already discussed in RAN2#109-e, with the following conclusions (from RAN2#109-e email discussion [203]):

RAN2 note the following observations on differences of UE capabilities for intra-band contiguous and non-contiguous CA:

- Intra-band contiguous CA capabilities are all contained within a single band entry of a band combination, while intra-band non-contiguous CA capabilities require at least two band entries.

- For intra-band contiguous carriers, UE band combination capabilities specify that UE supports any ordering of the capabilities.

- (Based on TS36.101): The ordering of intra-band non-contiguous entries is relevant for the support of BCS.

- (Based on TS36.101): The ordering of BCS is not directly related to the MIMO capabilities.

FFS: if UE supports (2, 4) MIMO layers with CA_xA_xA, it will also support (4, 2) MIMO layers with CA_xA_xA.

The CRs in R2-2001135, R2-2001136, R2-2001137, R2-2001138 are postponed.

The CRs in R2-2001140, R2-2001141, R2-2001142 are postponed to next meeting.

The two set of CRs 1) and 2) were jointly discussed in RAN2#109bis-e, with the following conclusions (from RAN2#109bis-e email discussion [202]):

Outcome of offline discussion [202]

There are (still) several interpretations by different companies on the issue raised by Nokia in R2-2003147, R2-2003148, R2-2003149, R2-2003150, R2-2003151, and Huawei in R2-2003549, R2-2003550, R2-2003551, R2-2003552, R2-2003553, R2-2003554 all of which seem slightly different but which should be taken into account. The topic is postponed to next meeting – companies are requested to provide contributions that take all of the above aspects into account.

Based on the last meeting’s conclusion, the interpretation issue still exists but the set of documents in 1) clarified further, following a few companies converged views, that order in which UE capabilities are indicated in the band entries can be agnostic, for the CA of the same bandwidth class.

For this meeting, the remaining open point is whether and how the uplink capability should be taken into account by the UE when setting capabilities for non-contiguous intra-band CA (which is raised in documents set 2)).

Companies are requested to provide comments in the tables 1 and 2 below to get a common RAN2 understanding on the UE capabilities interpretation and a need to clarify them:

Q1: Is it necessary for the UE take into account uplink capability when signalling capabilities for non-contiguous intra-band CA?

| Company | Is it necessary for the UE take into account uplink capability when signalling capabilities for non-contiguous intra-band CA? |
| --- | --- |

Table 1. Dependency on uplink capability

Conclusion:

Proposal:

Q2: In case it is necessary to reflect the dependency on uplink capabilities, how the clarification should be formulated? (E.g. as in CRs in 2, any other alternative)

| Company | In case it is necessary to reflect the dependency on uplink capabilities, how the clarification should be formulated? (E.g. as in CRs in 2, any other alternative) |
| --- | --- |

Table 2. Wording for dependency on uplink capability

Conclusion:

Proposal:

## 2.2	Which release clarification for LTE non-contiguous CA capabilities?

Based on the last meeting’s conclusion, the issue exists but how a correction should be worded and from which release onwards should a correction be made was not decided.

Companies are requested to provide comments in the table 3 below to decide which release onwards the correction is needed:

| Company | From which release onwards should something be captured and why? |
| --- | --- |

Table 3. Starting release for the correction CRs

Conclusion:

Proposal:

# 3	Conclusion

Conclusion 1: On the necessity for the UE take into account uplink capability when signalling capabilities for non-contiguous intra-band CA

Proposal 1:

Conclusion 2: On how the clarification should be formulated? (E.g. as in CRs in 2, any other alternative)

Proposal 2:

Conclusion 3: On which release onwards clarification for LTE non-contiguous CA capabilities is needed

Proposal 3: