3GPP TSG RAN WG2 Meeting #110-e	R2-2005315

# Electronic meeting, June 01 - 12, 2020

| CR--Form--v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.355 | CR | 0261 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of Release-16 UE positioning capabilities |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel Corporation, NTT DoCoMo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos-Core |  |  |  |  |  | Date: |  |  | 2020-06-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | To capture capabilities for NR positioning into stage 3 specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To capature NR positioning UE capabilities. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Capability part for NR positioning is missing in stage 3. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3, 6.4.3.3, 6.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR... CR TBD ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 6.3	Message Body IEs

#### –	RequestCapabilities

The RequestCapabilities message body in a LPP message is used by the location server to request the target device capability information for LPP and the supported individual positioning methods.

-- ASN1START

# RequestCapabilities ::= SEQUENCE {

criticalExtensions		CHOICE {

c1						CHOICE {

requestCapabilities-r9		RequestCapabilities-r9-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture	SEQUENCE {}

}

}

# RequestCapabilities-r9-IEs ::= SEQUENCE {

commonIEsRequestCapabilities		CommonIEsRequestCapabilities		OPTIONAL,	-- Need ON

a-gnss-RequestCapabilities			A-GNSS-RequestCapabilities			OPTIONAL,	-- Need ON

otdoa-RequestCapabilities			OTDOA-RequestCapabilities			OPTIONAL,	-- Need ON

ecid-RequestCapabilities			ECID-RequestCapabilities			OPTIONAL,	-- Need ON

# epdu-RequestCapabilities			EPDU-Sequence						OPTIONAL,	-- Need ON

# ...,

# [[	sensor-RequestCapabilities-r13	Sensor-RequestCapabilities-r13		OPTIONAL,	-- Need ON

# tbs-RequestCapabilities-r13		TBS-RequestCapabilities-r13			OPTIONAL,	-- Need ON

# wlan-RequestCapabilities-r13	WLAN-RequestCapabilities-r13		OPTIONAL,	-- Need ON

# bt-RequestCapabilities-r13		BT-RequestCapabilities-r13			OPTIONAL	-- Need ON

# ]],

# [[	nr-ECID-RequestCapabilities-r16	NR-ECID-RequestCapabilities-r16		OPTIONAL,	-- Need ON

# nr-Multi-RTT-RequestCapabilities-r16

# NR-Multi-RTT-RequestCapabilities-r16

# OPTIONAL,	-- Need ON

# nr-DL-AoD-RequestCapabilities-r16

# NR-DL-AoD-RequestCapabilities-r16	OPTIONAL,	-- Need ON

# nr-DL-TDOA-RequestCapabilities-r16

# NR-DL-TDOA-RequestCapabilities-r16	OPTIONAL,	-- Need ON

# nr-UL-RequestCapabilities-r16		NR-UL-RequestCapabilities-r16	OPTIONAL	-- Need ON

# ]]

}

-- ASN1STOP

#### –	ProvideCapabilities

The ProvideCapabilities message body in a LPP message indicates the LPP capabilities of the target device to the location server.

-- ASN1START

# ProvideCapabilities ::= SEQUENCE {

criticalExtensions		CHOICE {

c1						CHOICE {

provideCapabilities-r9		ProvideCapabilities-r9-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture	SEQUENCE {}

}

}

# ProvideCapabilities-r9-IEs ::= SEQUENCE {

commonIEsProvideCapabilities		CommonIEsProvideCapabilities			OPTIONAL,

a-gnss-ProvideCapabilities			A-GNSS-ProvideCapabilities				OPTIONAL,

otdoa-ProvideCapabilities			OTDOA-ProvideCapabilities				OPTIONAL,

ecid-ProvideCapabilities			ECID-ProvideCapabilities				OPTIONAL,

# epdu-ProvideCapabilities			EPDU-Sequence							OPTIONAL,

# ...,

# [[	sensor-ProvideCapabilities-r13	Sensor-ProvideCapabilities-r13			OPTIONAL,

# tbs-ProvideCapabilities-r13		TBS-ProvideCapabilities-r13				OPTIONAL,

# wlan-ProvideCapabilities-r13	WLAN-ProvideCapabilities-r13			OPTIONAL,

# bt-ProvideCapabilities-r13		BT-ProvideCapabilities-r13				OPTIONAL

# ]],

# [[	nr-ECID-ProvideCapabilities-r16	NR-ECID-ProvideCapabilities-r16			OPTIONAL,

# nr-Multi-RTT-ProvideCapabilities-r16

# NR-Multi-RTT-ProvideCapabilities-r16	OPTIONAL,

# nr-DL-AoD-ProvideCapabilities-r16

# NR-DL-AoD-ProvideCapabilities-r16		OPTIONAL,

# nr-DL-TDOA-ProvideCapabilities-r16

# NR-DL-TDOA-ProvideCapabilities-r16		OPTIONAL,

# nr-UL-ProvideCapabilities-r16	NR-UL-ProvideCapabilities-r16			OPTIONAL

# ]]

}

-- ASN1STOP

/***start change***/

6.4.1	Common Lower-Level IEs

–	AccessTypes

The IE AccessTypes is used to indicate several cellular access types using a bit map.

-- ASN1START

# AccessTypes ::= SEQUENCE {

accessTypes		BIT STRING {	eutra		(0),

utra		(1),

gsm			(2),

nb-iot		(3),

nr-v1510	(4) } (SIZE (1..8)),

...

}

-- ASN1STOP

AccessTypes field descriptions

accessTypes This field specifies the cellular access type(s). This is represented by a bit string, with a one-value at the bit position means the particular access type is addressed; a zero-value means not addressed.

–	ARFCN-ValueEUTRA

The IEs ARFCN-ValueEUTRA and ARFCN-ValueEUTRA-v9a0 are used to indicate the ARFCN of the E-UTRA carrier frequency, as defined in TS 36.331 [12].

-- ASN1START

# ARFCN-ValueEUTRA ::= INTEGER (0..maxEARFCN)

ARFCN-ValueEUTRA-v9a0 ::=	INTEGER (maxEARFCN-Plus1..maxEARFCN2)

ARFCN-ValueEUTRA-r14 ::=	INTEGER (0..maxEARFCN2)

maxEARFCN					INTEGER ::= 65535	-- Maximum value of EUTRA carrier frequency

maxEARFCN-Plus1				INTEGER ::= 65536	-- Lowest value extended EARFCN range

maxEARFCN2					INTEGER ::= 262143	-- Highest value extended EARFCN range

-- ASN1STOP

NOTE:	For fields using the original value range, as defined by IE ARFCN-ValueEUTRA i.e. without suffix, value maxEARFCN indicates that the E-UTRA carrier frequency is indicated by means of an extension.

–	ARFCN-ValueNR

The IE ARFCN-ValueNR is used to indicate the ARFCN applicable for a downlink, uplink or bi-directional (TDD) NR global frequency raster, as defined in TS 38.101-2 [34] and TS 38.101-1 [37].

-- ASN1START

ARFCN-ValueNR-r15 ::= INTEGER (0..3279165)

-- ASN1STOP

–	ARFCN-ValueUTRA

The IE ARFCN-ValueUTRA is used to indicate the ARFCN of the UTRA carrier frequency, as defined in TS 25.331 [13].

-- ASN1START

ARFCN-ValueUTRA ::=	INTEGER (0..16383)

-- ASN1STOP

–	CarrierFreq-NB

The IE CarrierFreq-NB is used to provide the NB-IoT carrier frequency, as defined in TS 36.101 [21].

-- ASN1START

CarrierFreq-NB-r14 ::=		SEQUENCE {

carrierFreq-r14				ARFCN-ValueEUTRA-r14,

carrierFreqOffset-r14		CarrierFreqOffsetNB-r14				OPTIONAL,

...

}

-- ASN1STOP

CarrierFreq-NB field descriptions

carrierFreq This field specifies the ARFCN applicable for the NB-IoT carrier frequency as defined in TS 36.101 [21, Table 5.7.3-1].

carrierFreqOffset This field specifies the offset of the NB-IoT channel number to EARFCN as defined in TS 36.101 [21].

#### –	CarrierFreqOffsetNB

The IE CarrierFreqOffsetNB is used to provide the offset of the NB-IoT channel number to EARFCN of a NB-IoT carrier.

-- ASN1START

CarrierFreqOffsetNB-r14 ::=		ENUMERATED {

v-10, v-9, v-8,	v-7, v-6, v-5, v-4, v-3, v-2, v-1, v-0dot5,

v0, v1, v2, v3, v4, v5, v6, v7, v8, v9

}

-- ASN1STOP

CarrierFreqOffsetNB field descriptions

CarrierFreqOffsetNB This field specifies the offset of the NB-IoT channel number to EARFCN as defined in TS 36.101 [21]. Value v-10 means -10, v-9 means -9, and so on.

–	CellGlobalIdEUTRA-AndUTRA

The IE CellGlobalIdEUTRA-AndUTRA specifies the global Cell Identifier for E-UTRA or UTRA, the globally unique identity of a cell in E-UTRA or UTRA.

-- ASN1START

# CellGlobalIdEUTRA-AndUTRA ::= SEQUENCE {

plmn-Identity		SEQUENCE {

mcc		SEQUENCE (SIZE (3))	OF INTEGER (0..9),

mnc		SEQUENCE (SIZE (2..3))	OF INTEGER (0..9)

},

cellIdentity		CHOICE {

eutra	BIT STRING (SIZE (28)),

utra	BIT STRING (SIZE (32))

},

...

}

-- ASN1STOP

CellGlobalIdEUTRA-AndUTRA field descriptions

plmn-Identity This field identifies the PLMN of the cell as defined in TS 36.331 [12].

cellIdentity This field defines the identity of the cell within the context of the PLMN as defined in TS 36.331 [12] and TS 25.331 [13]. The size of the bit string allows for the 32-bit extended UTRAN cell ID; in case the cell ID is shorter, the first bits of the string are set to 0.

–	CellGlobalIdGERAN

The IE CellGlobalIdGERAN specifies the global Cell Identifier for GERAN, the globally unique identity of a cell in GERAN.

-- ASN1START

# CellGlobalIdGERAN ::= SEQUENCE {

plmn-Identity		SEQUENCE {

mcc		SEQUENCE (SIZE (3))	OF INTEGER (0..9),

mnc		SEQUENCE (SIZE (2..3))	OF INTEGER (0..9)

},

locationAreaCode		BIT STRING (SIZE (16)),

cellIdentity			BIT STRING (SIZE (16)),

...

}

-- ASN1STOP

CellGlobalIdGERAN field descriptions

plmn-Identity This field identifies the PLMN of the cell.

locationAreaCode This field is a fixed length code identifying the location area within a PLMN.

cellIdentity This field specifies the cell Identifier which is unique within the context of the GERAN location area.

–	ECGI

The IE ECGI specifies the Evolved Cell Global Identifier (ECGI), the globally unique identity of a cell in E-UTRA (TS 36.331 [12]).

NOTE:	The IE ECGI is also used for NB-IoT access.

-- ASN1START

# ECGI ::= SEQUENCE {

mcc				SEQUENCE (SIZE (3))	OF INTEGER (0..9),

mnc				SEQUENCE (SIZE (2..3))	OF INTEGER (0..9),

cellidentity	BIT STRING (SIZE (28))

}

-- ASN1STOP

–	Ellipsoid-Point

The IE Ellipsoid-Point is used to describe a geographic shape as defined in TS 23.032 [15].

-- ASN1START

# Ellipsoid-Point ::= SEQUENCE {

latitudeSign				ENUMERATED {north, south},

degreesLatitude				INTEGER (0..8388607),			-- 23 bit field

degreesLongitude			INTEGER (-8388608..8388607)		-- 24 bit field

}

-- ASN1STOP

–	Ellipsoid-PointWithUncertaintyCircle

The IE Ellipsoid-PointWithUncertaintyCircle is used to describe a geographic shape as defined in TS 23.032 [15].

-- ASN1START

# Ellipsoid-PointWithUncertaintyCircle ::= SEQUENCE {

latitudeSign				ENUMERATED {north, south},

degreesLatitude				INTEGER (0..8388607),			-- 23 bit field

degreesLongitude			INTEGER (-8388608..8388607),	-- 24 bit field

uncertainty					INTEGER (0..127)

}

-- ASN1STOP

–	EllipsoidPointWithUncertaintyEllipse

The IE EllipsoidPointWithUncertaintyEllipse is used to describe a geographic shape as defined in TS 23.032 [15].

-- ASN1START

# EllipsoidPointWithUncertaintyEllipse ::= SEQUENCE {

latitudeSign				ENUMERATED {north, south},

degreesLatitude				INTEGER (0..8388607),			-- 23 bit field

degreesLongitude			INTEGER (-8388608..8388607),	-- 24 bit field

uncertaintySemiMajor		INTEGER (0..127),

uncertaintySemiMinor		INTEGER (0..127),

orientationMajorAxis		INTEGER (0..179),

confidence					INTEGER (0..100)

}

-- ASN1STOP

–	EllipsoidPointWithAltitude

The IE EllipsoidPointWithAltitude is used to describe a geographic shape as defined in TS 23.032 [15].

-- ASN1START

# EllipsoidPointWithAltitude ::= SEQUENCE {

latitudeSign				ENUMERATED {north, south},

degreesLatitude				INTEGER (0..8388607),			-- 23 bit field

degreesLongitude			INTEGER (-8388608..8388607),	-- 24 bit field

altitudeDirection			ENUMERATED {height, depth},

altitude					INTEGER (0..32767)				-- 15 bit field

}

-- ASN1STOP

–	EllipsoidPointWithAltitudeAndUncertaintyEllipsoid

The IE EllipsoidPointWithAltitudeAndUncertaintyEllipsoid is used to describe a geographic shape as defined in TS 23.032 [15].

-- ASN1START

# EllipsoidPointWithAltitudeAndUncertaintyEllipsoid ::= SEQUENCE {

latitudeSign				ENUMERATED {north, south},

degreesLatitude				INTEGER (0..8388607),			-- 23 bit field

degreesLongitude			INTEGER (-8388608..8388607),	-- 24 bit field

altitudeDirection			ENUMERATED {height, depth},

altitude					INTEGER (0..32767),				-- 15 bit field

uncertaintySemiMajor		INTEGER (0..127),

uncertaintySemiMinor		INTEGER (0..127),

orientationMajorAxis		INTEGER (0..179),

uncertaintyAltitude			INTEGER (0..127),

confidence					INTEGER (0..100)

}

-- ASN1STOP

–	EllipsoidArc

The IE EllipsoidArc is used to describe a geographic shape as defined in TS 23.032 [15].

-- ASN1START

# EllipsoidArc ::= SEQUENCE {

latitudeSign				ENUMERATED {north, south},

degreesLatitude				INTEGER (0..8388607),			-- 23 bit field

degreesLongitude			INTEGER (-8388608..8388607),	-- 24 bit field

innerRadius					INTEGER (0..65535),				-- 16 bit field,

uncertaintyRadius			INTEGER (0..127),

offsetAngle					INTEGER (0..179),

includedAngle				INTEGER (0..179),

confidence					INTEGER (0..100)

}

-- ASN1STOP

–	EPDU-Sequence

The EPDU-Sequence contains IEs that are defined externally to LPP by other organizations.

-- ASN1START

# EPDU-Sequence ::= SEQUENCE (SIZE (1..maxEPDU)) OF EPDU

maxEPDU INTEGER ::= 16

# EPDU ::= SEQUENCE {

ePDU-Identifier			EPDU-Identifier,

ePDU-Body				EPDU-Body

}

# EPDU-Identifier ::= SEQUENCE {

ePDU-ID					EPDU-ID,

ePDU-Name				EPDU-Name		OPTIONAL,

...

}

# EPDU-ID ::= INTEGER (1..256)

# EPDU-Name ::= VisibleString (SIZE (1..32))

# EPDU-Body ::= OCTET STRING

-- ASN1STOP

EPDU-Sequence field descriptions

EPDU-ID This field provides a unique integer ID for the externally defined positioning method. Its value is assigned to the external entity that defines the EPDU. See table External PDU Identifier Definition for a list of external PDU identifiers defined in this version of the specification.

EPDU-Name This field provides an optional character encoding which can be used to provide a quasi-unique name for an external PDU – e.g., by containing the name of the defining organization and/or the name of the associated public or proprietary standard for the EPDU.

EPDU-Body The content and encoding of this field are defined externally to LPP.

# External PDU Identifier Definition

| EPDU-ID | EPDU Defining entity | Method name | Reference |
| --- | --- | --- | --- |
| 1 | OMA LOC | OMA LPP extensions (LPPe) | OMA-TS-LPPe-V1_0 [20] |

–	HighAccuracyEllipsoidPointWithUncertaintyEllipse

The IE HighAccuracyEllipsoidPointWithUncertaintyEllipse is used to describe a geographic shape as defined in TS 23.032 [15].

-- ASN1START

HighAccuracyEllipsoidPointWithUncertaintyEllipse-r15 ::= SEQUENCE {

degreesLatitude-r15				INTEGER(-2147483648..2147483647),

degreesLongitude-r15			INTEGER(-2147483648..2147483647),

uncertaintySemiMajor-r15		INTEGER (0..255),

uncertaintySemiMinor-r15		INTEGER (0..255),

orientationMajorAxis-r15		INTEGER (0..179),

confidence-r15					INTEGER (0..100)

}

-- ASN1STOP

–	HighAccuracyEllipsoidPointWithAltitudeAndUncertaintyEllipsoid

The IE HighAccuracyEllipsoidPointWithAltitudeAndUncertaintyEllipsoid is used to describe a geographic shape as defined in TS 23.032 [15].

-- ASN1START

HighAccuracyEllipsoidPointWithAltitudeAndUncertaintyEllipsoid-r15 ::= SEQUENCE {

degreesLatitude-r15				INTEGER(-2147483648..2147483647),

degreesLongitude-r15			INTEGER(-2147483648..2147483647),

altitude-r15					INTEGER(-64000..1280000),

uncertaintySemiMajor-r15		INTEGER (0..255),

uncertaintySemiMinor-r15		INTEGER (0..255),

orientationMajorAxis-r15		INTEGER (0..179),

horizontalConfidence-r15		INTEGER (0..100),

uncertaintyAltitude-r15			INTEGER (0..255),

verticalConfidence-r15			INTEGER (0..100)

}

-- ASN1STOP

–	HorizontalVelocity

The IE HorizontalVelocity is used to describe a velocity shape as defined in TS 23.032 [15].

-- ASN1START

# HorizontalVelocity ::= SEQUENCE {

bearing						INTEGER(0..359),

horizontalSpeed				INTEGER(0..2047)

}

-- ASN1STOP

–	HorizontalWithVerticalVelocity

The IE HorizontalWithVerticalVelocity is used to describe a velocity shape as defined in TS 23.032 [15].

-- ASN1START

# HorizontalWithVerticalVelocity ::= SEQUENCE {

bearing						INTEGER(0..359),

horizontalSpeed				INTEGER(0..2047),

verticalDirection			ENUMERATED{upward, downward},

verticalSpeed				INTEGER(0..255)

}

-- ASN1STOP

–	HorizontalVelocityWithUncertainty

The IE HorizontalVelocityWithUncertainty is used to describe a velocity shape as defined in TS 23.032 [15].

-- ASN1START

# HorizontalVelocityWithUncertainty ::= SEQUENCE {

bearing						INTEGER(0..359),

horizontalSpeed				INTEGER(0..2047),

uncertaintySpeed			INTEGER(0..255)

}

-- ASN1STOP

–	HorizontalWithVerticalVelocityAndUncertainty

The IE HorizontalWithVerticalVelocityAndUncertainty is used to describe a velocity shape as defined in TS 23.032 [15].

-- ASN1START

# HorizontalWithVerticalVelocityAndUncertainty ::= SEQUENCE {

bearing						INTEGER(0..359),

horizontalSpeed				INTEGER(0..2047),

verticalDirection			ENUMERATED{upward, downward},

verticalSpeed				INTEGER(0..255),

horizontalUncertaintySpeed	INTEGER(0..255),

verticalUncertaintySpeed	INTEGER(0..255)

}

-- ASN1STOP

–	LocationCoordinateTypes

The IE LocationCoordinateTypes defines a list of possible geographic shapes as defined in TS 23.032 [15].

-- ASN1START

# LocationCoordinateTypes ::= SEQUENCE {

ellipsoidPoint											BOOLEAN,

ellipsoidPointWithUncertaintyCircle						BOOLEAN,

ellipsoidPointWithUncertaintyEllipse					BOOLEAN,

polygon													BOOLEAN,

ellipsoidPointWithAltitude								BOOLEAN,

ellipsoidPointWithAltitudeAndUncertaintyEllipsoid		BOOLEAN,

ellipsoidArc											BOOLEAN,

...,

[[

highAccuracyEllipsoidPointWithUncertaintyEllipse-r15				BOOLEAN		OPTIONAL,

highAccuracyEllipsoidPointWithAltitudeAndUncertaintyEllipsoid-r15	BOOLEAN		OPTIONAL

]]

}

-- ASN1STOP

–	NCGI

The IE NCGI specifies the NR Cell Global Identifier (NCGI) which is used to identify NR cells globally (TS 38.331 [35]).

-- ASN1START

# NCGI-r15 ::= SEQUENCE {

mcc-r15					SEQUENCE (SIZE (3)) 	OF INTEGER (0..9),

mnc-r15					SEQUENCE (SIZE (2..3)) 	OF INTEGER (0..9),

nr-cellidentity-r15		BIT STRING (SIZE (36))

}

-- ASN1STOP

–	NR-PhysCellId

The IE NR-PhysCellId specifies the NR physical cell identifier (TS 38.331 [35]).

-- ASN1START

# NR-PhysCellId-r16 ::= SEQUENCE {

PhysCellId-r16				INTEGER (0..1007)}

-- ASN1STOP

–	SupportedBandNR

The IE SupportedBandNR specifies the NR band indicator (TS 38.331 [35]).

-- ASN1START

# SupportedBandNR-r16 ::= SEQUENCE {

FreqBandIndicatorNR-r16 ::=		INTEGER (1..1024)

}

-- ASN1STOP

–	PeriodicAssistanceDataControlParameters

The IE PeriodicAssistanceDataControlParameters is used in a periodic assistance data delivery procedure as described in clauses 5.2.1a and 5.2.2a.

-- ASN1START

PeriodicAssistanceDataControlParameters-r15 ::= SEQUENCE {

periodicSessionID-r15			PeriodicSessionID-r15,

...,

[[

updateCapabilities-r15		UpdateCapabilities-r15		OPTIONAL

]]

}

PeriodicSessionID-r15 ::= SEQUENCE {

periodicSessionInitiator-r15	ENUMERATED { locationServer, targetDevice, ... },

periodicSessionNumber-r15		INTEGER (0..255),

...

}

UpdateCapabilities-r15 ::= BIT STRING {primaryCellID-r15	(0)} (SIZE(1..8))

-- ASN1STOP

PeriodicAssistanceDataControlParameters field descriptions

periodicSessionID This field identifies a particular periodic assistance data delivery session and the initiator of the session.

updateCapabilities This field identifies the capabilities of the sending entity to support an update of periodic assistance data. A bit value set to one indicates a capability is supported and a bit value set to zero indicates a capability is not supported.

–	Polygon

The IE Polygon is used to describe a geographic shape as defined in TS 23.032 [15].

-- ASN1START

# Polygon ::= SEQUENCE (SIZE (3..15)) OF PolygonPoints

# PolygonPoints ::= SEQUENCE {

latitudeSign				ENUMERATED {north, south},

degreesLatitude				INTEGER (0..8388607),			-- 23 bit field

degreesLongitude			INTEGER (-8388608..8388607)		-- 24 bit field

}

-- ASN1STOP

–	PositioningModes

The IE PositioningModes is used to indicate several positioning modes using a bit map.

-- ASN1START

# PositioningModes ::= SEQUENCE {

posModes		BIT STRING {	standalone	(0),

ue-based	(1),

ue-assisted	(2)

} (SIZE (1..8)),

...

}

-- ASN1STOP

PositioningModes field descriptions

posModes This field specifies the positioning mode(s). This is represented by a bit string, with a one-value at the bit position means the particular positioning mode is addressed; a zero-value means not addressed.

–	SegmentationInfo

The IE SegmentationInfo is used by a sender to indicate that LPP message segmentation is used, as specified in clause 4.3.5.

-- ASN1START

# SegmentationInfo-r14 ::= ENUMERATED { noMoreMessages, moreMessagesOnTheWay }

-- ASN1STOP

SegmentationInfo field descriptions

SegmentationInfo noMoreMessages indicates that this is the only or last LPP message segment used to deliver the entire message body. moreMessagesOnTheWay indicates that this is one of multiple LPP message segments used to deliver the entire message body.

–	VelocityTypes

The IE VelocityTypes defines a list of possible velocity shapes as defined in TS 23.032 [15].

-- ASN1START

# VelocityTypes ::= SEQUENCE {

horizontalVelocity										BOOLEAN,

horizontalWithVerticalVelocity							BOOLEAN,

horizontalVelocityWithUncertainty						BOOLEAN,

horizontalWithVerticalVelocityAndUncertainty			BOOLEAN,

...

}

-- ASN1STOP

/***start change***/

6.4.3.3	Common NR capability Information Elements

–	NR-DL-PRS-ProcessingCapability

The IE NR-DL-PRS-ProcessingCapability defines the common downlink PRS Processing capability.

-- ASN1START

# NR-DL-PRS-ProcessingCapability-r16 ::= SEQUENCE {

prs-ProcessingCapabilityBandList-r16			SEQUENCE (SIZE (1..nrMaxBands-r16)) OF PRS-ProcessingCapabilityPerBand-r16,

maxSupportedFreqLayers-r16					INTEGER (1..4),

simulLTE-NR-PRS-r16						ENUMERATED { supported}	OPTIONAL,

...

}

PRS-ProcessingCapabilityPerBand-r16 ::= SEQUENCE {

freqBandIndicatorNR-r16 				SupportedBandNR-r16,

supportedBandwidthPRS-r16				CHOICE {

fr1										ENUMERATED {mhz5, mhz10, mhz20, mhz40, mhz50, mhz80, mhz100},

fr2										ENUMERATED {mhz50, mhz100, mhz200, mhz400}

},

dl-PRS-BufferCapability-r16		 		ENUMERATED {type1, type2},

durationOfPRS-Processing-r16			SEQUENCE {

durationOfPRS-ProcessingSysmbols-r16	ENUMERATED {nDot125, nDot25, nDot5, n1, n2, n4, n6, n8, n12, n16, n20, n25, n30, n32, n35, n40, n45, n50},

durationOfPRS-ProcessingSymbolsInEveryTms-r16	ENUMERATED {n8, n16, n20, n30, n40, n80, n160,n320, n640, n1280}

},

maxNumOfDL-PRS-ResProcessedPerSlotFR1-r16	ENUMERATED {n1, n2, n4, n8, n16, n24, n32, n48, n64},

maxNumOfDL-PRS-ResProcessedPerSlotFR2-r16	ENUMERATED {n1, n2, n4, n8, n16, n24, n32, n48, n64}

maxNumOfDL-PRS-ResProcessedPerSlot-r16	SEQUENCE {

scs15-r16								ENUMERATED {n1, n2, n4, n8, n16, n24, n32, n48, n64}		OPTIONAL,		-- Cond fr1

scs30-r16								ENUMERATED {n1, n2, n4, n8, n16, n24, n32, n48, n64}		OPTIONAL,		-- Cond fr1

scs60-r16								ENUMERATED {n1, n2, n4, n8, n16, n24, n32, n48, n64},

scs120-r16								ENUMERATED {n1, n2, n4, n8, n16, n24, n32, n48, n64}		OPTIONAL,		-- Cond fr2

}

}

nrMaxBands-r16		INTEGER ::= 1024	-- Maximum number of supported bands.

-- ASN1STOP

NR-DL-PRS-ProcessingCapability field descriptions

maxSupportedFreqLayers Indicates the maximum number of positioning frequency layers supported by UE.

supportedBandwidthPRS Indicates the maximum number of DL PRS bandwidth in MHz, which is supported and reported by UE.

dl-PRS-BufferCapability Indicates DL PRS buffering capability. Value type1 indicates sub-slot/symbol level buffering and value type2 indicates slot level buffering.

durationOfPRS-Processing Indicates the duration of DL PRS symbol in units of ms a UE can process every T ms assuming maximum DL PRS bandwidth in MHz, which is supported and reported by UE.

maxNumOfDL-PRS-ResProcessedPerSlotFR1 Indicates the maximum number of DL PRS resources that UE can process in a slot under FR1.

maxNumOfDL-PRS-ResProcessedPerSlotFR1 Indicates the maximum number of DL PRS resources that UE can process in a slot under FR2.

simulLTE-NR-PRS Indicates whether the UE supports parallel processing of LTE PRS and NR PRS.

–	NR-DL-PRS-ResourcesCapability

The IE NR-DL-PRS-ResourcesCapability defines the PRS resources capability for each positioning method. The UE can include this IE only if the UE supports NR-DL-PRS-ProcessingCapability. Otherwise, the UE does not include this IE;

-- ASN1START

# NR-DL-PRS-ResourcesCapability-r16 ::= SEQUENCE {

maxNrOfDL-PRS-ResourceSetPerTrpPerFrequencyLayer-r16	INTEGER (1..2),

maxNrOfTRP-AcrossFreqs-r16					ENUMERATED { n4, n6, n12, n16, n32, n64, n128, n256},

maxNrOfPosLayer-r16							INTEGER (1..4),

dl-PRS-ResourcesCapabilityBandList-r16		SEQUENCE (SIZE (1..nrMaxBands-r16)) OF DL-PRS-ResourcesCapabilityPerBand-r16,

dl-PRS-ResourcesBandCombinationList-r16		DL-PRS-ResourcesBandCombinationList-r16,

...

}

DL-PRS-ResourcesCapabilityPerBand-r16 ::= SEQUENCE {

freqBandIndicatorNR-r16 					SupportedBandNR-r16,

maxNrOfDL-PRS-ResourcesPerResourceSet-r16				ENUMERATED { n1, n2, n4, n8, n16, n32, n64},

maxNrOfDL-PRS-ResourcesPerPositioningFrequencylayer-r16	ENUMERATED { n6, n24, n32, n64, n96, n128, n256, n512, n1024}

}

DL-PRS-ResourcesBandCombinationList-r16 ::=			SEQUENCE (SIZE (1..maxBandComb-r16)) OF DL-PRS-ResourcesBandCombination

DL-PRS-ResourcesBandCombination ::=		SEQUENCE {

bandList-r16							SEQUENCE (SIZE (1..maxSimultaneousBands-r16)) OF SupportedBandNR-r16,

maxNrOfDL-PRS-ResAcrossAllFL-TRP-ResourceSet-FR1-r16	ENUMERATED {n6, n24, n64, n128, n192, n256, n512, n1024, n2048},

maxNrOfDL-PRS-ResAcrossAllFL-TRP-ResourceSet-FR2-r16	ENUMERATED {n24, n64, n96, n128, n192, n256, n512, n1024, n2048},

maxNrOfDL-PRS-ResAcrossAllFL-TRP-ResourceSet-FR1Mix-r16	ENUMERATED {n6, n24, n64, n128, n192, n256, n512, n1024, n2048},

maxNrOfDL-PRS-ResAcrossAllFL-TRP-ResourceSet-FR2Mix-r16	ENUMERATED {n24, n64, n96, n128, n192, n256, n512, n1024, n2048}

}

DL-PRS-ResourcesBandCombination-r16 ::=		SEQUENCE {

bandList-r16						SEQUENCE (SIZE (1..maxSimultaneousBands-r16)) OF SupportedBandNR-r16,

maxNrOfDL-PRS-ResourcesAcrossBands-r16	CHOICE{

fr1-only-r16							ENUMERATED {n6, n24, n64, n128, n192, n256, n512, n1024, n2048},

fr2-only-r16							ENUMERATED {n24, n64, n96, n128, n192, n256, n512, n1024, n2048},

fr1-fr2-mix-r16							SEQUENCE {

fr1-r16									ENUMERATED {n6, n24, n64, n96, n128, n192, n256, n512, n1024, n2048},

fr2-r16									ENUMERATED {n24, n64, n96, n128, n192, n256, n512, n1024, n2048},

}

}

# SupportedBandNR-r16 ::= SEQUENCE {

FreqBandIndicatorNR-r16 ::=		INTEGER (1..1024)

}

nrMaxBands-r16		INTEGER ::= 1024	-- Maximum number of supported bands.

maxSimultaneousBands-r16		INTEGER ::= 4	-- Maximum number of simultaneously measured bands

-- ASN1STOP

NR-DL-PRS-ResourcesCapability field descriptions

maxNrOfDL-PRS-ResourceSetPerTrpPerFrequencyLayer Indicates the maximum number of DL PRS Resource Sets per TRP per frequency layer supported by UE.

maxNrOfTRP-AcrossFreqs Indicates the maximum number of TRPs across all positioning frequency layers.

maxNrOfPosLayer Indicates the maximum number of supported positioning layer.

maxNrOfDL-PRS-ResourcesPerResourceSet Indicates the maximum number of DL PRS Resources per DL PRS Resource Set. Value 16, 32, 64 are only applicable to FR2 bands. Value 1 is not applicable for DL-AoD.

DL PRS Resources per positioning frequency layer Indicates the maximum number of DL PRS resources per TRP across all frequency layers. Value 6 is only applicable to FR1 bands.

maxNrOfDL-PRS-ResAcrossAllFL-TRP-ResourceSet-FR1 Indicates the maximum number of DL PRS Resources supported by UE across all frequency layers, TRPs and DL PRS Resource Sets for FR1-only.. This is for FR1 only bandcombination.

maxNrOfDL-PRS-ResAcrossAllFL-TRP-ResourceSet-FR1 Indicates the maximum number of DL PRS Resources supported by UE across all frequency layers, TRPs and DL PRS Resource Sets for FR2-only.. This is for FR2 only bandcombination.

maxNrOfDL-PRS-ResAcrossAllFL-TRP-ResourceSet-FR1Mix Indicates the maximum number of DL PRS Resources supported by UE across all frequency layers, TRPs and DL PRS Resource Sets for FR1 in FR1/FR2 mixed operation. This is for the bandcombination containing FR1 and FR2 bands.

maxNrOfDL-PRS-ResAcrossAllFL-TRP-ResourceSet-FR2Mix Indicates the maximum number of DL PRS Resources supported by UE across all frequency layers, TRPs and DL PRS Resource Sets for FR2 in FR1/FR2 mixed operation. This is for the bandcombination containing FR1 and FR2 bands.

–	NR-DL-AoD-MeasurementCapability

The IE NR-DL-AoD-MeasurementCapability defines the DL-AoD measurement capability. The UE can include this IE only if the UE supports NR-DL-PRS-ResourcesCapability for DL-AoD. Otherwise, the UE does not include this IE;

-- ASN1START

# NR-DL-AoD-MeasurementCapability-r16 ::= SEQUENCE {

maxDL-PRS-RSRP-MeasurementFR1-r16			INTEGER (1..8),

maxDL-PRS-RSRP-MeasurementFR2-r16			INTEGER (1..8),

dl-AoD-MeasCapabilityBandList-r16		SEQUENCE (SIZE (1..nrMaxBands-r16)) OF DL-AoD-MeasCapabilityPerBand-r16,

...

}

DL-AoD-MeasCapabilityPerBand-r16 ::= SEQUENCE {

freqBandIndicatorNR-r16 					SupportedBandNR-r16,

simul-NR-DL-AoD-DL-TDOA-r16			ENUMERATED { supported}	OPTIONAL,

simul-NR-DL-AoD-Multi-RTT-r16		ENUMERATED { supported}	OPTIONAL

}

-- ASN1STOP

NR-DL-AoD-MeasurementCapability field descriptions

maxDL-PRS-RSRP-MeasurementFR1 Indicates the maximum number of DL PRS RSRP measurements on different PRS resources from the same TRP supported by the UE on FR1.

maxDL-PRS-RSRP-MeasurementFR2 Indicates the maximum number of DL PRS RSRP measurements on different PRS resources from the same TRP supported by the UE on FR2.

simul-NR-DL-AoD-DL-TDOA Indicates whether the UE supports simultaneous processing for DL-AoD and DL-TDoA measurements. The UE can include this field only if the UE supports DL-TDOA and DL-AoD. Otherwise, the UE does not include this field;

simul-NR-DL-AoD-Multi-RTT Indicates whether the UE supports simultaneous processing for DL AoD and Multi-RTT measurements. The UE can include this field only if the UE supports Multi-RTT, srs-PosResources TS38.331 [35] and DL-AoD. Otherwise, the UE does not include this field;

–	NR-Multi-RTT-MeasurementCapability

The IE NR-Multi-RTT-MeasurementCapability defines the Multi-RTT measurement capability. The UE can include this IE only if the UE supports NR-DL-PRS-ResourcesCapability for Multi-RTT. Otherwise, the UE does not include this IE;

-- ASN1START

# NR-Multi-RTT-MeasurementCapability-r16 ::= SEQUENCE {

maxNrOfRx-TX-MeasFR1-r16						INTEGER (1..4)	OPTIONAL,

maxNrOfRx-TX-MeasFR2-r16						INTEGER (1..4)	OPTIONAL,

supportOfRSRP-MeasFR1-r16						ENUMERATED { supported}	OPTIONAL,

supportOfRSRP-MeasFR2-r16						ENUMERATED { supported}	OPTIONAL,

srs-AssocPRS-MultiLayers-r16					ENUMERATED { supported}	OPTIONAL,

...

}

-- ASN1STOP

NR-Multi-RTT-MeasurementCapability field descriptions

maxNrOfRx-TX-MeasFR1 Indicates the maximum number of UE Rx–Tx time difference measurements corresponding to a single SRS resource/resource set for positioning with each measurement corresponding to a single DL PRS resource/resource set on FR1.

maxNrOfRx-TX-MeasFR2 Indicates the maximum number of UE Rx–Tx time difference measurements corresponding to a single SRS resource/resource set for positioning with each measurement corresponding to a single DL PRS resource/resource set on FR2.

srs-AssocPRS-MultiLayers Indicates whether the UE supports measurements derived on one or more DL PRS resource/resource sets which may be in different positioning frequency layers for SRS transmitted in a single CC. PRS and SRS may be on different bands.

supportOfRSRP-MeasFR1 Indicates whether the UE supports RSRP measurement for Multi-RTT on FR1.

supportOfRSRP-MeasFR2 Indicates whether the UE supports RSRP measurement for Multi-RTT on FR2.

–	NR-DL-PRS-QCL-ProcessingCapability

The IE NR-DL-PRS-QCL-ProcessingCapability defines the common UE downlink PRS QCL Processing capability. The UE can include this IE only if the UE supports NR-DL-PRS-ProcessingCapability. Otherwise, the UE does not include this IE;

-- ASN1START

# NR-DL-PRS-QCL-ProcessingCapability-r16 ::= SEQUENCE {

dl-PRS-QCL-ProcessingCapabilityBandList-r16			SEQUENCE (SIZE (1..nrMaxBands-r16)) OF DL-PRS-QCL-ProcessingCapabilityPerBand-r16,

...

}

DL-PRS-QCL-ProcessingCapabilityPerBand-r16 ::= SEQUENCE {

freqBandIndicatorNR-r16 					SupportedBandNR-r16,

ssb-FromNeighCellAsQCL-r16					ENUMERATED { supported}	OPTIONAL,

ssb-FromNeighServingCellAsQCL-r16			ENUMERATED { supported}	OPTIONAL

prs-FromServNeighCellAsQCL-r16				ENUMERATED { supported} OPTIONAL

}

nrMaxBands-r16		INTEGER ::= 1024	-- Maximum number of supported bands.

-- ASN1STOP

NR-DL-PRS-QCL-ProcessingCapability field descriptions

ssbFromNeighCellAsQCL Indicates the support of SSB from neighbor cell as QCL source of a DL PRS. UE supporthing this feature also support reusing SSB measurement from RRM for receiving PRS Note: It refers to Type-C for FR1 and Type-C & Type-D support for FR2

sbFromNeighServingCellAsQCL Indicates the support of DL PRS from serving/neighbor cell as QCL source of a DL PRS.

PrsFromServNeighCellAsQCL Indicates the support of DL PRS from serving/neighbor cell as QCL source of a DL PRS. Note: It refers to Type-D support for FR2.

–	NR-DL-TDOA-MeasurementCapability

The IE NR-DL-TDOA-MeasurementCapability defines the DL-TDOA measurement capability. The UE can include this IE only if the UE supports NR-DL-PRS-ResourcesCapability for DL-TDOA. Otherwise, the UE does not include this IE;

-- ASN1START

# NR-DL-TDOA-MeasurementCapability-r16 ::= SEQUENCE {

dl-RSTD-MeasurementPerPairOfTRP-FR1-r16			INTEGER (1..4),

dl-RSTD-MeasurementPerPairOfTRP-FR2-r16			INTEGER (1..4),

supportOfRSRP-MeasFR1-r16						ENUMERATED { supported}	OPTIONAL,

supportOfRSRP-MeasFR2-r16						ENUMERATED { supported}	OPTIONAL,

...

}

-- ASN1STOP

NR-DL-TDOA-MeasurementCapability field descriptions

dl-RSTD-MeasurementPerPairOfTRP-FR1 Indicates number of DL RSTD measurements per pair of TRPs on FR1.

dl-RSTD-MeasurementPerPairOfTRP-FR2 Indicates number of DL RSTD measurements per pair of TRPs on FR2.

supportOfRSRP-MeasFR1 Indicates whether the UE supports RSRP measurement for DL-TDOA on FR1.

supportOfRSRP-MeasFR2 Indicates whether the UE supports RSRP measurement for DL-TDOA on FR2.

–	NR-UL-SRS-Capability

The IE NR-UL-SRS-Capability defines the UE uplink SRS capability.

-- ASN1START

# NR-UL-SRS-Capability-r16 ::= SEQUENCE {

srs-CapabilityBandList-r16			SEQUENCE (SIZE (1..nrMaxBands)) OF SRS-CapabilityPerBand-r16,

maxNumberPathLossEstimateAllCells-r16		NUMERATED {n1, n4, n8, n16}		OPTIONAL,

maxNumberSpatialRelationsAllServing-r16		NUMERATED {n0, n1, n2, n4, n8, n16}	OPTIONAL

...

}

SRS-CapabilityPerBand-r16 ::= SEQUENCE {

freqBandIndicatorNR-r16 					SupportedBandNR-r16,

olpc-SRS-Pos-r16				OLPC-SRS-Pos-r16				OPTIONAL,

spatialRelationsSRS-Pos-r16		SpatialRelationsSRS-Pos-r16		OPTIONAL,

simul-SRS-Trans-IntraBandCA-r16	INTEGER (1..2)	OPTIONAL,

simul-SRS-Trans-InterBandCA-r16	INTEGER (1..2)	OPTIONAL

}

OLPC-SRS-Pos-r16 ::=	SEQUENCE {

olpc-SRS-PosBasedOnPRS-Serving-r16		ENUMERATED {supported}	OPTIONAL,

olpc-SRS-PosBasedOnSSB-Neigh-r16 		ENUMERATED {supported}	OPTIONAL,

olpc-SRS-PosBasedOnPRS-Neigh-r16		ENUMERATED {supported}	OPTIONAL,

maxNumberPathLossEstimatePerServing-r16	NUMERATED {n1, n4, n8, n16}	OPTIONAL

}

SpatialRelationsSRS-Pos-r16 ::=		SEQUENCE {

spatialRelation-SRS-PosBasedOnSSB-Serving-r16	ENUMERATED {supported}	OPTIONAL,

spatialRelation-SRS-PosBasedOnCSI-RS-Serving-r16	ENUMERATED {supported}	OPTIONAL,

spatialRelation-SRS-PosBasedOnPRS-Serving-r16	ENUMERATED {supported}	OPTIONAL,

spatialRelation-SRS-PosBasedOnSRS-r16			ENUMERATED {supported}	OPTIONAL,

spatialRelation-SRS-PosBasedOnSSB-Neigh-r16		ENUMERATED {supported}	OPTIONAL,

spatialRelation-SRS-PosBasedOnPRS-Neigh-r16		ENUMERATED {supported}	OPTIONAL

}

nrMaxBands-r16		INTEGER ::= 1024	-- Maximum number of supported bands.

-- ASN1STOP

NR-UL-SRS-Capability field descriptions

maxNumberPathLossEstimateAllCells Indicates the maximum number of pathloss estimates that the UE can simultaneously maintain for all the SRS resource sets for positioning across all cells in addition to the up to four pathloss estimates that the UE maintains per serving cell for the PUSCH/PUCCH/SRS transmissions. The UE can include this field only if the UE supports any of olpc-SRS-PosBasedOnPRS-Serving, olpc-SRS-PosBasedOnSSB-Neigh and olpc-SRS-PosBasedOnPRS-Neigh. Otherwise, the UE does not include this field;

maxNumberSpatialRelationsAllServing indicates the maximum number of maintained spatial relations for all the SRS resource sets for positioning across all serving cells in addition to the spatial relations maintained spatial relations per serving cell for the PUSCH/PUCCH/SRS transmissions. It is only applied for FR2. The UE can include this field only if the UE supports any of spatialRelation-SRS-PosBasedOnSSB-Serving, spatialRelation-SRS-PosBasedOnCSI-RS-Serving, spatialRelation-SRS-PosBasedOnPRS-Serving, spatialRelation-SRS-PosBasedOnSSB-Neigh or spatialRelation-SRS-PosBasedOnPRS-Neigh. Otherwise, the UE does not include this field;

olpc-SRS-Pos Indicates whether the UE supports spatial relations for SRS for positioning. The capability signalling comprises the following parameters. - olpc-SRS-PosBasedOnPRS-Serving indicates whether the UE supports OLPC for SRS for positioning based on PRS from the serving cell in the same band. The UE can include this field only if the UE supports NR-DL-PRS-ProcessingCapability and srs-PosResources TS38.331 [35] Otherwise, the UE does not include this field; - olpc-SRS-PosBasedOnSSB-Neigh indicates whether the UE supports OLPC for SRS for positioning based on SSB from the neighbouring cell in the same band. The UE can include this field only if the UE supports srs-PosResources TS38.331 [35]. Otherwise, the UE does not include this field; - olpc-SRS-PosBasedOnPRS-Neigh indicates whether the UE supports OLPC for SRS for positioning based on PRS from the neighbouring cell in the same band. The UE can include this field only if the UE supports olpc-SRS-PosBasedOnPRS-Serving. Otherwise, the UE does not include this field; - maxNumberPathLossEstimatePerServing indicates the maximum number of pathloss estimates that the UE can simultaneously maintain for all the SRS resource sets for positioning per serving cell in addition to the up to four pathloss estimates that the UE maintains per serving cell for the PUSCH/PUCCH/SRS transmissions. The UE can include this field only if the UE supports any of olpc-SRS-PosBasedOnPRS-Serving, olpc-SRS-PosBasedOnSSB-Neigh and olpc-SRS-PosBasedOnPRS-Neigh. Otherwise, the UE does not include this field;

spatialRelationsSRS-Pos Indicates whether the UE supports spatial relations for SRS for positioning. It is only applicable for FR2. The capability signalling comprises the following parameters. - spatialRelation-SRS-PosBasedOnSSB-Serving indicates whether the UE supports spatial relation for SRS for positioning based on SSB from the serving cell in the same band. The UE can include this field only if the UE supports srs-PosResources TS38.331 [35]. Otherwise, the UE does not include this field; - spatialRelation-SRS-PosBasedOnCSI-RS-Serving indicates whether the UE supports spatial relation for SRS for positioning based on CSI-RS from the serving cell in the same band. The UE can include this field only if the UE supports spatialRelation-SRS-PosBasedOnSSB-Serving. Otherwise, the UE does not include this field; - spatialRelation-SRS-PosBasedOnPRS-Serving indicates whether the UE supports spatial relation for SRS for positioning based on PRS from the serving cell in the same band. The UE can include this field only if the UE supports any of DL PRS Resources for DL AoD, DL PRS Resources for DL-TDOA or DL PRS Resources for Multi-RTT, or srs-PosResources TS38.331 [35]. Otherwise, the UE does not include this field; - spatialRelation-SRS-PosBasedOnSRS indicates whether the UE supports spatial relation for SRS for positioning based on SRS in the same band. The UE can include this field only if the UE supports srs-PosResources TS38.331 [35]. Otherwise, the UE does not include this field; - spatialRelation-SRS-PosBasedOnSSB-Neigh indicates whether the UE supports spatial relation for SRS for positioning based on SSB from the neighbouring cell in the same band. The UE can include this field only if the UE supports spatialRelation-SRS-PosBasedOnSSB-Serving. Otherwise, the UE does not include this field; - spatialRelation-SRS-PosBasedOnPRS-Neigh indicates whether the UE supports spatial relation for SRS for positioning based on PRS from the neighbouring cell in the same band. The UE can include this field only if the UE supports spatialRelation-SRS-PosBasedOnPRS-Serving. Otherwise, the UE does not include this field;

simul-SRS-Trans-IntraBandCA Indicates the number of SRS resources for positioning on a symbol for intra-band CA. The UE can include this field only if the UE supports srs-PosResources TS38.331 [35]. Otherwise, the UE does not include this field;

simul-SRS-Trans-InterBandCA Indicates the number of SRS resources for positioning on a symbol for inter-band CA. The UE can include this field only if the UE supports srs-PosResources TS38.331 [35]. Otherwise, the UE does not include this field;

/***Next change***/

6.5.8	NR UL Positioning

6.5.8.1	NR UL Capability Information

–	NR-UL-ProvideCapabilities

The IE NR-UL-ProvideCapabilities is used by the target device to indicate its capability to support UL-PRS and to provide its UL-SRS capabilities to the location server.

-- ASN1START

NR-UL-ProvideCapabilities-r16 ::= SEQUENCE {

nr-UL-SRS-Capability-r16		NR-UL-SRS-Capability-r16,

...

}

-- ASN1STOP

6.5.8.2	NR UL Capability Information Request

–	NR-UL-RequestCapabilities

The IE NR-UL-RequestCapabilities is used by the location server to request the capability of the target device to support UL-PRS and to request UL-PRS capabilities from a target device.

-- ASN1START

NR-UL-RequestCapabilities-r16 ::= SEQUENCE {

...

}

-- ASN1STOP

6.5.9	NR-ECID Positioning

This clause defines the information elements for NR ECID positioning (TS 38.305 [40]).

6.5.9.1	NR-ECID Location Information

–	NR-ECID-ProvideLocationInformation

The IE NR-ECID-ProvideLocationInformation is used by the target device to provide NR ECID location measurements to the location server. It may also be used to provide NR ECID positioning specific error reason.

-- ASN1START

# NR-ECID-ProvideLocationInformation-r16 ::= SEQUENCE {

nr-ECID-SignalMeasurementInformation-r16	NR-ECID-SignalMeasurementInformation-r16 OPTIONAL,

nr-ECID-Error-r16							NR-ECID-Error-r16						OPTIONAL,

...

}

-- ASN1STOP

6.5.9.2	NR-ECID Location Information Elements

–	NR-ECID-SignalMeasurementInformation

The IE NR-ECID-SignalMeasurementInformation is used by the target device to provide NR ECID measurements to the location server.

-- ASN1START

# NR-ECID-SignalMeasurementInformation-r16 ::= SEQUENCE {

nr-PrimaryCellMeasuredResults-r16	NR-MeasuredResultsElement-r16,

nr-MeasuredResultsList-r16			NR-MeasuredResultsList-r16	OPTIONAL,

# ...

# }

# NR-MeasuredResultsList-r16 ::= SEQUENCE (SIZE(1..32)) OF MeasuredResultsElement-r16

# NR-MeasuredResultsElement-r16 ::= SEQUENCE {

systemFrameNumber				BIT STRING (SIZE (10)),

# trp-ID-r16						TRP-ID-r16			OPTIONAL,

measResultNR-r16				SEQUENCE {

cellResults-r16					SEQUENCE{

resultsSSB-Cell-r16				MeasQuantityResults-r16				OPTIONAL,

resultsCSI-RS-Cell-r16			MeasQuantityResults-r16				OPTIONAL

},

rsIndexResults-r16				SEQUENCE{

resultsSSB-Indexes-r16			ResultsPerSSB-IndexList-r16			OPTIONAL,

resultsCSI-RS-Indexes-r16		ResultsPerCSI-RS-IndexList-r16		OPTIONAL

}																		OPTIONAL

},

...

}

MeasQuantityResults-r16 ::= SEQUENCE {

nr-RSRP-r16	INTEGER (0..127)		OPTIONAL,

nr-RSRQ-r16	INTEGER (0..127)		OPTIONAL

}

ResultsPerSSB-IndexList-r16::= SEQUENCE (SIZE (1..64)) OF ResultsPerSSB-Index-r16

ResultsPerSSB-Index-r16 ::= SEQUENCE {

ssb-Index-r16					INTEGER (0..63),

ssb-Results-r16					MeasQuantityResults-r16					OPTIONAL

}

ResultsPerCSI-RS-IndexList-r16::= SEQUENCE (SIZE (1..64)) OF ResultsPerCSI-RS-Index-r16

ResultsPerCSI-RS-Index-r16 ::= SEQUENCE {

csi-RS-Index-r16				INTEGER (0..95),

csi-RS-Results-r16				MeasQuantityResults-r16					OPTIONAL

}

-- ASN1STOP

NR-ECID-SignalMeasurementInformation field descriptions

systemFrameNumber This field specifies the system frame number of the measured cell during which the measurements have been performed. The target device shall include this field if it was able to determine the SFN of the cell at the time of measurement.

resultsSSB-Cell This attribute specifies the SS reference signal received power (SS-RSRP) and quality (SS-RSRQ) measurement aggregated at cell level, as defined in TS 38.331 [35].

resultsCSI-RS-Cell This attribute specifies the CSI-RS reference signal received power (CSI-RSRP) and quality (CSI-RSRQ) measurement aggregated at cell level, as defined in TS 38.331 [35].

ssb-Results This attribute specifies the SS reference signal received power (SS-RSRP) and quality (SS-RSRQ) measurement per SSB resource, as defined in TS 38.331 [35].

csi-RS-Results This attribute specifies the CSI-RS reference signal received power (CSI-RSRP) and quality (CSI-RSRQ) per CSI-RS resource, as defined in TS 38.331 [35].

primaryCellMeasuredResults This field contains measurements for the primary cell when the target device reports measurements for both primary cell and neighbour cells. This field shall be omitted when the target device reports measurements for the primary cell only, in which case the measurements for the primary cell is reported in the measuredResultsList.

6.5.9.3	NR-ECID Location Information Request

–	NR-ECID-RequestLocationInformation

The IE NR-ECID-RequestLocationInformation is used by the location server to request NR-ECID location measurements from a target device.

-- ASN1START

# NR-ECID-RequestLocationInformation-r16 ::= SEQUENCE {

requestedMeasurements-r16		BIT STRING {	ssrsrpReq		(0),

ssrsrqReq		(1),

csirsrpReq		(2),

csirsrqReq		(3) (SIZE(1..8)),

...

}

-- ASN1STOP

NR-ECID-RequestLocationInformation field descriptions

requestedMeasurements This field specifies the NR-ECID measurements requested. This is represented by a bit string, with a one-value at the bit position means the particular measurement is requested; a zero-value means not requested.

6.5.9.4	NR-ECID Capability Information

–	NR-ECID-ProvideCapabilities

The IE NR-ECID-ProvideCapabilities is used by the target device to indicate its capability to support NR-ECID and to provide its NR-ECID positioning capabilities to the location server.

-- ASN1START

# NR-ECID-ProvideCapabilities-r16 ::= SEQUENCE {

nr-ECID-MeasSupported -r16		BIT STRING {	ssrsrpSup		(0),

ssrsrqSup		(1),

csirsrpSup		(2),

csirsrqSup		(3) (SIZE(1..8)),

periodicalReporting-r16		ENUMERATED { supported }							OPTIONAL,

triggeredReporting-r16		ENUMERATED { supported }							OPTIONAL,

...

}

-- ASN1STOP

6.5.9.5	NR-ECID Capability Information Request

–	NR-ECID-RequestCapabilities

The IE NR-ECID-RequestCapabilities is used by the location server to request the capability of the target device to support NR-ECID and to request NR-ECID positioning capabilities from a target device.

-- ASN1START

# NR-ECID-RequestCapabilities ::= SEQUENCE {

...

}

-- ASN1STOP

6.5.9.6	NR-ECID Error Elements

–	NR-ECID-Error

The IE NR-ECID-Error is used by the location server or target device to provide NR-ECID error reasons to the target device or location server, respectively.

-- ASN1START

# NR-ECID-Error-r16 ::= CHOICE {

locationServerErrorCauses-r16		NR-ECID-LocationServerErrorCauses-r16,

targetDeviceErrorCauses-r16			NR-ECID-TargetDeviceErrorCauses-r16,

...

}

-- ASN1STOP

–	NR-ECID-LocationServerErrorCauses

The IE NR-ECID-LocationServerErrorCauses is used by the location server to provide NR-ECID error reasons to the target device.

-- ASN1START

# NR-ECID-LocationServerErrorCauses-r16 ::= SEQUENCE {

Cause-r16		ENUMERATED	{	undefined,

...

},

...

}

-- ASN1STOP

–	NR-ECID-TargetDeviceErrorCauses

The IE NR-ECID-TargetDeviceErrorCauses is used by the target device to provide NR-ECID error reasons to the location server.

-- ASN1START

# NR-ECID-TargetDeviceErrorCauses-r16 ::= SEQUENCE {

Cause-r16		ENUMERATED {	undefined,

requestedMeasurementNotAvailable,

notAllrequestedMeasurementsPossible,

...

},

ss-RSRPMeasurementNotPossible				NULL		OPTIONAL,

ss-RSRQMeasurementNotPossible				NULL		OPTIONAL,

csi-RSRPMeasurementNotPossible				NULL		OPTIONAL,

csi-RSRQMeasurementNotPossible				NULL		OPTIONAL,

...

}

-- ASN1STOP

6.5.10	NR-DL-TDOA Positioning

This clause defines the information elements for NR downlink TDOA positioning (TS 38.305 [40]).

6.5.10.1	NR-DL-TDOA Assistance Data

–	NR-DL-TDOA-ProvideAssistanceData

The IE NR-DL-TDOA-ProvideAssistanceData is used by the location server to provide assistance data to enable UE-assisted and UE-based NR downlink TDOA. It may also be used to provide NR DL TDOA positioning specific error reason.

-- ASN1START

# NR-DL-TDOA-ProvideAssistanceData-r16 ::= SEQUENCE {

nr-DL-PRS-AssistanceData-r16			NR-DL-PRS-AssistanceData-r16		OPTIONAL,	-- Need ON

nr-SelectedDL-PRS-IndexList-r16	SEQUENCE (SIZE (1..nrMaxFreqLayers)) OF NR-SelectedDL-PRS-PerFreq-r16 OPTIONAL,	-- Need ON

# nr-PositionCalculationAssistanceData-r16

# NR-PositionCalculationAssistanceData-r16

# OPTIONAL, 	-- Cond UEB

nr-DL-TDOA-Error-r16				NR-DL-TDOA-Error-r16				OPTIONAL,	-- Need ON

...

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| UEB | The field is mandatory present for the UE based DL-TDOA; otherwise it is not present. |

6.5.10.2	NR-DL-TDOA Assistance Data Request

–	NR-DL-TDOA-RequestAssistanceData

The IE NR-DL-TDOA-RequestAssistanceData is used by the target device to request assistance data from a location server.

-- ASN1START

# NR-DL-TDOA-RequestAssistanceData-r16 ::= SEQUENCE {

nr-PhysCellId-r16			NR-PhysCellId-r16							OPTIONAL,

nr-AdType-r16					BIT STRING {	dl-prs (0),

posCalc (1) } (SIZE (1..8)),

...

}

-- ASN1STOP

NR-DL-TDOA-RequestAssistanceData field descriptions

nr-PhysCellId This field specifies the NR physical cell identity of the current primary cell of the target device.

nr-AdType This field indicates the requested assistance data. dl-prs means requested assistance data is nr-DL-PRS-AssistanceData, posCalc means requested assistance data is nr-PositionCalculationAssistanceData for UE based positioning.

6.5.10.3	NR-DL-TDOA Location Information

–	NR-DL-TDOA-ProvideLocationInformation

The IE NR-DL-TDOA-ProvideLocationInformation is used by the target device to provide NR-DL-TDOA location measurements to the location server. It may also be used to provide NR-DL-TDOA positioning specific error reason.

-- ASN1START

# NR-DL-TDOA-ProvideLocationInformation-r16 ::= SEQUENCE {

nr-DL-TDOA-SignalMeasurementInformation-r16

DL-TDOA-SignalMeasurementInformation-r16 			OPTIONAL,

# nr-dl-tdoa-LocationInformation-r16		NR-DL-TDOA-LocationInformation-r16			OPTIONAL, -- Cond UEB

nr-DL-TDOA-Error-r16					DL-TDOA-Error-r16							OPTIONAL,

...

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| UEB | The field is mandatory present for the UE based DL-TDOA; otherwise it is not present. |

6.5.10.4	NR-DL-TDOA Location Information Elements

–	NR-DL-TDOA-SignalMeasurementInformation

The IE NR-DL-TDOA-SignalMeasurementInformation is used by the target device to provide NR-DL TDOA measurements to the location server. The measurements are provided as a list of TRPs, where the first TRP in the list is used as reference TRP in case RSTD measurements are reported. The first TRP in the list may or may not be the reference TRP indicated in the NR-DL-PRS-AssistanceData. Furthermore, the target device selects a reference resource per TRP, and compiles the measurements per TRP based on the selected reference resource.

-- ASN1START

# NR-DL-TDOA-SignalMeasurementInformation-r16 ::= SEQUENCE {

# dl-PRS-ReferenceInfo-r16		DL-PRS-IdInfo-r16,

nr-DL-TDOA-MeasList-r16	NR-DL-TDOA-MeasList-r16,

...

}

# NR-DL-TDOA-MeasList-r16 ::= SEQUENCE (SIZE(1.. nrMaxTRPs)) OF NR-DL-TDOA-MeasElement-r16

# NR-DL-TDOA-MeasElement-r16 ::= SEQUENCE {

# trp-ID-r16						TRP-ID-r16			OPTIONAL,

nr-DL-PRS-ResourceId-r16		NR-DL-PRS-ResourceId-r16	 OPTIONAL,

nr-DL-PRS-ResourceSetId-r16		NR-DL-PRS-ResourceSetId-r16 OPTIONAL,

nr-TimeStamp-r16				NR-TimeStamp-r16,

nr-RSTD-r16						INTEGER (0..ffs),	-- FFS on the value range

nr-AdditionalPathList-r16		NR-AdditionalPathList-r16		OPTIONAL,

# nr-TimingMeasQuality-r16				NR-TimingMeasQuality-r16,

nr-PRS-RSRP-Result-r16			INTEGER (FFS)			OPTIONAL, -- FFS, value range to be decided in RAN4.

nr-DL-TDOA-AdditionalMeasurements-r16				NR-DL-TDOA-AdditionalMeasurements-r16,

...

}

NR-DL-TDOA-AdditionalMeasurements-r16 ::= SEQUENCE (SIZE (1..3)) OF NR-DL-TDOA-AdditionalMeasurementElement-r16

NR-AdditionalPathList-r16 ::= SEQUENCE (SIZE(1..2)) OF NR-AdditionalPath-r16

NR-DL-TDOA-AdditionalMeasurementElement-r16 ::= SEQUENCE {

nr-DL-PRS-ResourceId-r16        NR-DL-PRS-ResourceId-r16	 OPTIONAL,

nr-DL-PRS-ResourceSetId-r16		NR-DL-PRS-ResourceSetId-r16 OPTIONAL,

nr-TimeStamp-r16				NR-TimeStamp-r16,

nr-RSTD-ResultDiff-r16			INTEGER (0..ffs),	-- FFS on the value range to be decided in RAN4

dl-PRS-RSRP-ResultDiff-r16	INTEGER (FFS)		OPTIONAL, -- FFS on the value range	to be decided in RAN4

nr-AdditionalPathList-r16		NR-AdditionalPathList-r16		OPTIONAL,

...

}

nrMaxTRPs		INTEGER ::= 256		-- Max TRPs per UE

-- ASN1STOP

NR-DL-TDOA-SignalMeasurementInformation field descriptions

nr-PRS-RSRP-Result This field specifies the reference signal received power (RSRP) measurement, as defined in TS 38.331 [35].

nr-AdditionalPathList This field specifies one or more additional detected path timing values for the TRP or resource, relative to the path timing used for determining the nr-RSTD value. If this field was requested but is not included, it means the UE did not detect any additional path timing values.

nr-RSTD This field specifies the relative timing difference between this neighbour TRP and the PRS reference TRP, as defined in FFS. Mapping of the measured quantity is defined as in FSS.

nr-TimingMeasQuality This field specifies the target device′s best estimate of the quality of the measurement.

–	NR-DL-TDOA-LocationInformation

The IE NR-DL-TDOA-LocationInformation is included by the target device when location information derived using DL-TDOA is provided to the location server.

-- ASN1START

# NR-DL-TDOA-LocationInformation-r16 ::= SEQUENCE {

measurementReferenceTime-r16	CHOICE {

systemFrameNumber-r16			NR-TimeStamp-r16,

utc-time-r16					UTCTime,

...

}															OPTIONAL,

...

}

-- ASN1STOP

NR-DL-TDOA-LocationInformation field descriptions

measurementReferenceTime This field specifies the time for which the location estimate is valid.

6.5.10.5	NR-DL-TDOA Location Information Request

–	NR-DL-TDOA-RequestLocationInformation

The IE NR-DL-TDOA-RequestLocationInformation is used by the location server to request NR DL-TDOA location measurements from a target device.

-- ASN1START

# NR-DL-TDOA-RequestLocationInformation-r16 ::= SEQUENCE {

nr-DL-PRS-RstdMeasurementInfoRequest-r16	ENUMERATED { true }				OPTIONAL,		-- Need ON

# nr-RequestedMeasurements-r16		BIT STRING {	prsrsrpReq		(0)

# } (SIZE(1..8)),

nr-AssistanceAvailability-r16		BOOLEAN,

nr-DL-TDOA-ReportConfig-r16		NR-DL-TDOA-ReportConfig-r16		OPTIONAL,	-- Need ON

additionalPaths-r16					ENUMERATED { requested }	OPTIONAL,		-- Need ON

...

}

# NR-DL-TDOA-ReportConfig-r16 ::= SEQUENCE {

maxDL-PRS-RSRP-MeasurementsPerTRP-r16	INTEGER (1..8)	OPTIONAL,

maxDL-PRS-RSTD-MeasurementsPerTRPPair-r16	INTEGER (1..4)	OPTIONAL

timingReportingGranularityFactor-r16 	INTEGER (FFS)	OPTIONAL	-- FFS in RAN4

# }

-- ASN1STOP

NR-DL-TDOA-RequestLocationInformation field descriptions

nr-AssistanceAvailability This field indicates whether the target device may request additional PRS assistance data from the server. TRUE means allowed and FALSE means not allowed.

nr-RequestedMeasurements This field specifies the NR DL-TDOA measurements requested. This is represented by a bit string, with a one-value at the bit position means the particular measurement is requested; a zero-value means not requested.

nr-DL-PRS-RstdMeasurementInfoRequest This field indicates whether the target device is requested to report DL PRS Resource ID(s) or DL PRS Resource Set ID(s) used for determining the timing of each TRP in RSTD measurements.

maxDL-PRS-RSRP-MeasurementsPerTRP This field specifies the maximum number of DL PRS RSRP measurements on different DL PRS resources from the same TRP.

maxDL-PRS-RSTD-MeasurementsPerTRPPair This field specifies the maximum number of. DL PRS RSTD measurements per pair of TRPs. The maximum number is defined across all positioning frequency layers.

timingReportingGranularityFactor This field specifies the reporting granularity for the UE timing measurements (DL RSTD, the UE Rx-Tx time difference).

6.5.10.6	NR-DL-TDOA Capability Information

–	NR-DL-TDOA-ProvideCapabilities

The IE NR-DL-TDOA-ProvideCapabilities is used by the target device to indicate its capability to support NR DL-TDOA and to provide its NR DL-TDOA positioning capabilities to the location server.

-- ASN1START

# NR-DL-TDOA-ProvideCapabilities-r16 ::= SEQUENCE {

nr-DL-TDOA-Mode-r16					PositioningModes,

nr-DL-TDOA-PRS-Capability-r16			NR-DL-PRS-ResourcesCapability-r16,

nr-DL-TDOA-MeasurementCapability-r16	NR-DL-TDOA-MeasurementCapability-r16,

nr-DL-PRS-QCL-ProcessingCapability-r16	NR-DL-PRS-QCL-ProcessingCapability-r16,

nr-DL-PRS-ProcessingCapability-r16		NR-DL-PRS-ProcessingCapability-r16,

additionalPathsReport-r16			ENUMERATED { supported }						OPTIONAL,

periodicalReporting-r16				ENUMERATED { supported }				OPTIONAL,

...

}

-- ASN1STOP

NR-DL-TDOA-ProvideCapabilities field descriptions

nr-DL-TDOA-Mode This field specifies the DL-TDOA mode(s) supported by the target device.

6.5.10.7	NR-DL TDOA Capability Information Request

–	NR-DL-TDOA-RequestCapabilities

The IE NR-DL-TDOA-RequestCapabilities is used by the location server to request the capability of the target device to support NR DL-TDOA and to request NR DL-TDOA positioning capabilities from a target device.

-- ASN1START

# NR-DL-TDOA-RequestCapabilities ::= SEQUENCE {

...

}

-- ASN1STOP

6.5.10.8	NR-DL-TDOA Error Elements

–	NR-DL-TDOA-Error

The IE NR-DL-TDOA-Error is used by the location server or target device to provide NR DL-TDOA error reasons to the target device or location server, respectively.

-- ASN1START

# NR-DL-TDOA-Error-r16 ::= CHOICE {

locationServerErrorCauses-r16		NR-DL-TDOA-LocationServerErrorCauses-r16,

targetDeviceErrorCauses-r16			NR-DL-TDOA-TargetDeviceErrorCauses-r16,

...

}

-- ASN1STOP

–	NR-DL-TDOA-LocationServerErrorCauses

The IE NR-DL-TDOA-LocationServerErrorCauses is used by the location server to provide NR DL-TDOA error reasons to the target device.

-- ASN1START

# NR-DL-TDOA-LocationServerErrorCauses-r16 ::= SEQUENCE {

cause-r16		ENUMERATED	{	undefined,

assistanceDataNotSupportedByServer,

assistanceDataSupportedButCurrentlyNotAvailableByServer,

notProvidedAssistanceDataNotSupportedByServer, ...

},

...

}

-- ASN1STOP

–	NR-DL-TDOA-TargetDeviceErrorCauses

The IE NR-DL-TDOA-TargetDeviceErrorCauses is used by the target device to provide NR-DL-TDOA error reasons to the location server.

-- ASN1START

# DL-TDOA-TargetDeviceErrorCauses-r16 ::= SEQUENCE {

cause-r16		ENUMERATED {	undefined,

assistance-data-missing,

unableToMeasureAnyTRP,

attemptedButUnableToMeasureSomeNeighbourTRPs,

thereWereNotEnoughSignalsReceivedForUeBasedDL-TDOA,

locationCalculationAssistanceDataMissing, ...

},

nr-PRS-RSRPMeasurementNotPossible-r16				NULL		OPTIONAL,

nr-RSTDMeasurementNotPossible-r16				NULL		OPTIONAL,

...

}

-- ASN1STOP

6.5.11	NR-DL-AoD Positioning

This clause defines the information elements for NR downlink AoD positioning (TS 38.305 [40]).

6.5.11.1	NR-DL-AoD Assistance Data

–	NR-DL-AoD-ProvideAssistanceData

The IE NR-DL-AoD-ProvideAssistanceData is used by the location server to provide assistance data to enable UE-assisted Aod. It may also be used to provide NR DL AoD positioning specific error reason.

-- ASN1START

# NR-DL-AoD-ProvideAssistanceData-r16 ::= SEQUENCE {

nr-DL-PRS-AssistanceData-r16			NR-DL-PRS-AssistanceData-r16	OPTIONAL,	-- Need ON

nr-SelectedDL-PRS-IndexList-r16	SEQUENCE (SIZE (1..nrMaxFreqLayers)) OF NR-SelectedDL-PRS-PerFreq-r16 OPTIONAL,	-- Need ON

# nr-PositionCalculationAssistanceData-r16

# NR-PositionCalculationAssistanceData-r16

# OPTIONAL, 	-- Cond UEB

nr-DL-AoD-Error-r16						NR-DL-AoD-Error-r16				OPTIONAL,	-- Need ON

...

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| UEB | The field is mandatory present for the UE based DL-TDOA; otherwise it is not present. |

6.5.11.2	NR-DL-AoD Assistance Data Request

–	NR-DL-AoD-RequestAssistanceData

The IE NR-DL-AoD-RequestAssistanceData is used by the target device to request assistance data from a location server.

-- ASN1START

# NR-DL-AoD-RequestAssistanceData-r16 ::= SEQUENCE {

nr-PhysCellId-r16						NR-PhysCellId-r16						OPTIONAL,

nr-AdType-r16		BIT STRING { dl-prs (0), posCalc (1) } (SIZE (1..8)),

...

}

-- ASN1STOP

NR-DL-AoD-RequestAssistanceData field descriptions

nr-PhysCellId This field specifies the NR physical cell identity of the current primary cell of the target device.

nr-AdType This field indicates the requested assistance data. dl-prs means requested assistance data is nr-DL-PRS-AssistanceData, posCalc means requested assistance data is nr-PositionCalculationAssistanceData for UE based positioning.

6.5.11.3	NR-DL-AoD Location Information

–	NR-DL-AoD-ProvideLocationInformation

The IE NR-DL-AoD-ProvideLocationInformation is used by the target device to provide NR DL-AoD location measurements to the location server. It may also be used to provide NR DL-AoD positioning specific error reason.

-- ASN1START

# NR-DL-AoD-ProvideLocationInformation-r16 ::= SEQUENCE {

nr-DL-AoD-SignalMeasurementInformation-r16

NR-DL-AoD-SignalMeasurementInformation-r16 	OPTIONAL,

# nr-dl-aod-LocationInformation-r16		NR-DL-AoD-LocationInformation-r16			OPTIONAL, -- Cond UEB

nr-DL-AoD-Error-r16						NR-DL-AoD-Error-r16							OPTIONAL,

...

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| UEB | The field is mandatory present for the UE based DL-AoD; otherwise it is not present. |

6.5.11.4	NR-DL-AoD Location Information Elements

–	NR-DL-AoD-SignalMeasurementInformation

The IE NR-DL-AoD-SignalMeasurementInformation is used by the target device to provide NR DL AoD measurements to the location server. The measurements are provided as a list of TRPs, where the first TRP in the list is used as reference TRP.

-- ASN1START

# NR-DL-AoD-SignalMeasurementInformation-r16 ::= SEQUENCE {

nr-DL-AoD-MeasList-r16			NR-DL-AoD-MeasList-r16,

# ...

# }

# NR-DL-AoD-MeasList-r16 ::= SEQUENCE (SIZE(1..nrMaxTRPs)) OF NR-DL-AoD-MeasElement-r16

# NR-DL-AoD-MeasElement-r16 ::= SEQUENCE {

trp-ID-r16							TRP-ID-r16			OPTIONAL,

nr-DL-PRS-ResourceId-r16			NR-DL-PRS-ResourceId-r16	 OPTIONAL,

nr-DL-PRS-ResourceSetId-r16			NR-DL-PRS-ResourceSetId-r16 OPTIONAL,

nr-TimeStamp-r16					NR-TimeStamp-r16,

nr-PRS-RSRP-Result-r16				INTEGER (FFS)			OPTIONAL, -- Need RAN4 inputs on value range

nr-DL-PRS-RxBeamIndex-r16			INTEGER (1..8),

nr-TimingMeasQuality-r16				NR-TimingMeasQuality-r16,

nr-DL-Aod-AdditionalMeasurements-r16		NR-DL-AoD-AdditionalMeasurements-r16,

...

}

NR-DL-AoD-AdditionalMeasurements-r16 ::= SEQUENCE (SIZE (1..7)) OF NR-DL-AoD-AdditionalMeasurementElement-r16

NR-DL-AoD-MeasurementElement-r16 ::= SEQUENCE {

nr-DL-PRS-ResourceId-r16			NR-DL-PRS-ResourceId-r16	 OPTIONAL,

nr-DL-PRS-ResourceSetId-r16			NR-DL-PRS-ResourceSetId-r16 OPTIONAL,

nr-TimeStamp-r16					NR-TimeStamp-r16,

nr-PRS-RSRP-ResultDiff-r16			INTEGER (FFS)			OPTIONAL, -- Need RAN4 inputs on value range

nr-DL-PRS-RxBeamIndex-r16			INTEGER (1..8),

...

}

nrMaxTRPs		INTEGER ::= 256		-- Max TRPs

-- ASN1STOP

NR-DL-AoD-SignalMeasurementInformation field descriptions

nr-PRS-RSRP-Result This field specifies the reference signal received power (RSRP) measurement, as defined in TS 38.331 [35].

–	NR-DL-AoD-LocationInformation

The IE NR-DL-AoD-LocationInformation is included by the target device when location information derived using DL-AoD is provided to the location server.

-- ASN1START

# NR-DL-AoD-LocationInformation-r16 ::= SEQUENCE {

measurementReferenceTime-r16	CHOICE {

sfn-time-r16					NR-TimeStamp-r16,

utc-time-r16					UTCTime,

...

}															OPTIONAL,

...

}

-- ASN1STOP

NR-DL-AoD-LocationInformation field descriptions

measurementReferenceTime This field specifies the time for which the location estimate is valid.

6.5.11.5	NR-DL-AoD Location Information Request

–	NR-DL-AoD-RequestLocationInformation

The IE NR-DL-AoD-RequestLocationInformation is used by the location server to request NR DL-AoD location measurements from a target device.

-- ASN1START

# NR-Dl-AoD-RequestLocationInformation-r16 ::= SEQUENCE {

nr-AssistanceAvailability-r16		BOOLEAN,

nr-DL-AoD-ReportConfig-r16		NR-DL-AoD-ReportConfig-r16,

...

}

# NR-DL-AoD-ReportConfig-r16 ::= SEQUENCE {

maxDL-PRS-RSRP-MeasurementsPerTRP-r16	INTEGER (1..8)	OPTIONAL

# }

-- ASN1STOP

NR-DL-AoD-RequestLocationInformation field descriptions

nr-AssistanceAvailability This field indicates whether the target device may request additional PRS assistance data from the server. TRUE means allowed and FALSE means not allowed.

maxDL-PRS-RSRP-MeasurementsPerTRP This field specifies the maximum number of DL PRS RSRP measurements on different DL PRS resources from the same TRP.

6.5.11.6	NR-DL-AoD Capability Information

–	NR-DL-AoD-ProvideCapabilities

The IE NR-DL-AoD-ProvideCapabilities is used by the target device to indicate its capability to support NR DL-AoD and to provide its NR DL-AoD positioning capabilities to the location server.

-- ASN1START

# NR-DL-AoD-ProvideCapabilities-r16 ::= SEQUENCE {

nr-DL-TDOA-Mode-r16		PositioningModes,

# periodicalReporting-r16				ENUMERATED { supported }		OPTIONAL,

nr-DL-AoD-PRS-Capability-r16			NR-DL-PRS-ResourcesCapability-r16,

nr-DL-AoD-MeasurementCapability-r16		NR-DL-AoD-MeasurementCapability-r16,

nr-DL-PRS-QCL-ProcessingCapability-r16	NR-DL-PRS-QCL-ProcessingCapability-r16,

nr-DL-PRS-ProcessingCapability-r16		NR-DL-PRS-ProcessingCapability-r16,

...

}

-- ASN1STOP

6.5.11.7	NR-DL AoD Capability Information Request

–	NR-DL-AoD-RequestCapabilities

The IE NR-DL-AoD-RequestCapabilities is used by the location server to request the capability of the target device to support NR DL-AoD and to request NR DL-AoD positioning capabilities from a target device.

-- ASN1START

# NR-DL-AoD-RequestCapabilities ::= SEQUENCE {

...

}

-- ASN1STOP

6.5.11.8	NR-DL-AoD Error Elements

–	NR-DL-AoD-Error

The IE NR-DL-AoD-Error is used by the location server or target device to provide NR DL-AoD error reasons to the target device or location server, respectively.

-- ASN1START

# NR-DL-AoD-Error-r16 ::= CHOICE {

locationServerErrorCauses-r16		NR-DL-AoD-LocationServerErrorCauses-r16,

targetDeviceErrorCauses-r16			NR-DL-AoD-TargetDeviceErrorCauses-r16,

...

}

-- ASN1STOP

–	NR-DL-AoD-LocationServerErrorCauses

The IE NR-DL-AoD-LocationServerErrorCauses is used by the location server to provide NR DL-AoD error reasons to the target device.

-- ASN1START

# NR-DL-TDOA-LocationServerErrorCauses-r16 ::= SEQUENCE {

cause-r16		ENUMERATED	{	undefined,

assistanceDataNotSupportedByServer,

assistanceDataSupportedButCurrentlyNotAvailableByServer,

notProvidedAssistanceDataNotSupportedByServer,

...

},

...

}

-- ASN1STOP

–	NR-DL-AoD-TargetDeviceErrorCauses

The IE NR-DL-AoD-TargetDeviceErrorCauses is used by the target device to provide NR-DL-AoD error reasons to the location server.

-- ASN1START

# NR-DL-AoD-TargetDeviceErrorCauses-r16 ::= SEQUENCE {

cause-r16		ENUMERATED {	undefined,

assistance-data-missing,

unableToMeasureAnyTRP,

attemptedButUnableToMeasureSomeNeighbourTRPs,

thereWereNotEnoughSignalsReceivedForUeBasedDL-AoD,

locationCalculationAssistanceDataMissing,

...

},

nr-PRS-RSRPMeasurementNotPossible-r16				NULL		OPTIONAL,

...

}

-- ASN1STOP

6.5.12	NR-Multi-RTT Positioning

This clause defines the information elements for downlink NR-Multi-RTT positioning (TS 38.305 [40]).

6.5.12.1	NR-Multi-RTT Assistance Data

–	NR-Multi-RTT-ProvideAssistanceData

The IE NR-Multi-RTT-ProvideAssistanceData is used by the location server to provide assistance data to enable UE-assisted NR Multi-RTT. It may also be used to provide NR Multi-RTT positioning specific error reason.

-- ASN1START

# NR-Multi-RTT-ProvideAssistanceData-r16 ::= SEQUENCE {

nr-DL-PRS-AssistanceData-r16				NR-DL-PRS-AssistanceData-r16	OPTIONAL,	--Need ON

nr-SelectedDL-PRS-IndexList-r16	SEQUENCE (SIZE (1..nrMaxFreqLayers)) OF NR-SelectedDL-PRS-PerFreq-r16 OPTIONAL,	-- Need ON

nr-Multi-RTT-Error-r16						NR-Multi-RTT-Error-r16							OPTIONAL,	-- Need ON

...

}

-- ASN1STOP

6.5.12.2	NR-Multi-RTT Assistance Data Request

–	NR-Multi-RTT-RequestAssistanceData

The IE NR-Multi-RTT-RequestAssistanceData is used by the target device to request assistance data from a location server.

-- ASN1START

# NR-Multi-RTT-RequestAssistanceData-r16 ::= SEQUENCE {

nr-PhysCellId-r16						NR-PhysCellId-r16						OPTIONAL,

nr-AdType-r16							BIT STRING { dl-prs (0), ul-srs (1) } (SIZE (1..8)),

...

}

-- ASN1STOP

NR-Multi-RTT-RequestAssistanceData field descriptions

nr-PhysCellId This field specifies the NR physical cell identity of the current primary cell of the target device.

6.5.12.3	NR-Multi-RTT Location Information

–	NR-Multi-RTT-ProvideLocationInformation

The IE NR-Multi-RTT-ProvideLocationInformation is used by the target device to provide NR Multi-RTT location measurements to the location server. It may also be used to provide NR Multi-RTT positioning specific error reason.

-- ASN1START

# NR-Multi-RTT-ProvideLocationInformation-r16 ::= SEQUENCE {

nr-Multi-RTT-SignalMeasurementInformation-r16	NR-Multi-RTT-SignalMeasurementInformation-r16 OPTIONAL,

nr-Multi-RTT-Error-r16							NR-Multi-RTT-Error-r16		OPTIONAL,

...

}

-- ASN1STOP

6.5.12.4	NR-Multi-RTT Location Information Elements

–	NR-Multi-RTT-SignalMeasurementInformation

The IE NR-Multi-RTT-SignalMeasurementInformation is used by the target device to provide NR Multi-RTT measurements to the location server. The measurements are provided as a list of TRPs, where the first TRP in the list is used as reference TRP.

-- ASN1START

# NR-Multi-RTT-SignalMeasurementInformation-r16 ::= SEQUENCE {

nr-Multi-RTT-MeasList-r16	NR-Multi-RTT-MeasList-r16,

...

}

# NR-Multi-RTT-MeasList-r16 ::= SEQUENCE (SIZE(1.. nrMaxTRPs)) OF NR-Multi-RTT-MeasElement-r16

# NR-Multi-RTT-MeasElement-r16 ::= SEQUENCE {

# trp-ID-r16							TRP-ID-r16			OPTIONAL,

nr-DL-PRS-ResourceId-r16			NR-DL-PRS-ResourceId-r16	OPTIONAL,

nr-DL-PRS-ResourceSetId-r16			NR-DL-PRS-ResourceSetId-r16 OPTIONAL,

nr-UE-RxTxTimeDiff-r16				INTEGER (0..ffs)	OPTIONAL,	-- FFS on the value range to be decided in RAN4

nr-AdditionalPathList-r16			NR-AdditionalPathList-r16	OPTIONAL,

nr-TimeStamp-r16					NR-TimeStamp-r16,

nr-TimingMeasQuality-r16			NR-TimingMeasQuality-r16,

nr-PRS-RSRP-Result-r16				INTEGER (FFS)			OPTIONAL, -- FFS, value range to be decided in RAN4.

nr-Multi-RTT-AdditionalMeasurements-r16		NR-Multi-RTT-AdditionalMeasurements-r16,

...

}

NR-AdditionalPathList-r16 ::= SEQUENCE (SIZE(1..2)) OF NR-AdditionalPath-r16

NR-Multi-RTT-AdditionalMeasurements-r16 ::= SEQUENCE (SIZE (1..3)) OF NR-Multi-RTT-AdditionalMeasurementElement-r16

NR-Multi-RTT-AdditionalMeasurementElement-r16 ::= SEQUENCE {

nr-DL-PRS-ResourceId-r16			NR-DL-PRS-ResourceId-r16	OPTIONAL,

nr-DL-PRS-ResourceSetId-r16			NR-DL-PRS-ResourceSetId-r16 OPTIONAL,

nr-PRS-RSRP-ResultDiff-r16			INTEGER (FFS)			OPTIONAL, -- FFS, value range to be decided in RAN4.

nr-UE-RxTxTimeDiffAdditional-r16				INTEGER (0..ffs)	OPTIONAL,	-- FFS on the value range

nr-AdditionalPathList-r16			NR-AdditionalPathList-r16		OPTIONAL,

nr-TimeStamp-r16					NR-TimeStamp-r16,

...

}

nrMaxTRPs		INTEGER ::= 256		-- Max TRPs

-- ASN1STOP

NR-Multi-RTT-SignalMeasurementInformation field descriptions

nr-PRS-RSRP-Result This field specifies the reference signal received power (RSRP) measurement, as defined in TS 38.331 [35].

nr-UE-RxTxTimeDiff This field specifies the UE Rx–Tx time difference measurement, as defined in FFS.

nr-AdditionalPathList This field specifies one or more additional detected path timing values for the TRP or resource, relative to the path timing used for determining the nr-UE-RxTxTimeDiff value or the nr-UE-RxTxTimeDiffAdditional value. If this field was requested but is not included, it means the UE did not detect any additional path timing values.

6.5.12.5	NR-Multi-RTT Location Information Request

–	NR-Multi-RTT-RequestLocationInformation

The IE NR-Multi-RTT-RequestLocationInformation is used by the location server to request NR Multi-RTT location measurements from a target device.

-- ASN1START

# NR-Multi-RTT-RequestLocationInformation-r16 ::= SEQUENCE {

# nr-RequestedMeasurements-r16		BIT STRING {	prsrsrpReq		(0)} (SIZE(1..8)),

nr-AssistanceAvailability-r16		BOOLEAN,

nr-Multi-RTT-ReportConfig-r16		NR-Multi-RTT-ReportConfig-r16,

additionalPaths-r16					ENUMERATED { requested }	OPTIONAL,		-- Need ON

...

}

# NR-Multi-RTT-ReportConfig-r16 ::= SEQUENCE {

maxDL-PRS-RSRP-MeasurementsPerTRP-r16	INTEGER (1..8)	OPTIONAL,

maxDL-PRS-RxTxTimeDiffMeasPerTRP-r16 	INTEGER (1..4)	OPTIONAL,

timingReportingGranularityFactor-r16 	INTEGER (FFS)	OPTIONAL	-- FFS in RAN4

# }

-- ASN1STOP

NR-Multi-RTT-RequestLocationInformation field descriptions

nr-AssistanceAvailability This field indicates whether the target device may request additional PRS assistance data from the server. TRUE means allowed and FALSE means not allowed.

maxDL-PRS-RSRP-MeasurementsPerTRP This field specifies the maximum number of DL PRS RSRP measurements on different DL PRS resources from the same TRP.

maxDL-PRS-RxTxTimeDiffMeasPerTRP This field specifies the maximum number of UE-Rx-Tx time difference measurements for different DL PRS resources or DL PRS resource sets per TRP.

timingReportingGranularityFactor This field specifies the reporting granularity for the UE timing measurements (DL RSTD, the UE Rx-Tx time difference).

6.5.12.6	NR-Multi-RTT Capability Information

–	NR-Multi-RTT-ProvideCapabilities

The IE NR-Multi-RTT-ProvideCapabilities is used by the target device to indicate its capability to support NR Multi-RTT and to provide its Multi-RTT positioning capabilities to the location server.

-- ASN1START

# NR-Multi-RTT-ProvideCapabilities-r16 ::= SEQUENCE {

nr-Multi-RTT-PRS-Capability-r16			NR-DL-PRS-ResourcesCapability-r16,

nr-Multi-RTT-MeasurementCapability-r16	NR-Multi-RTT-MeasurementCapability-r16,

nr-DL-PRS-QCL-ProcessingCapability-r16	NR-DL-PRS-QCL-ProcessingCapability-r16,

nr-DL-PRS-ProcessingCapability-r16		NR-DL-PRS-ProcessingCapability-r16,

nr-UL-SRS-Capability-r16				NR-UL-SRS-Capability-r16,

additionalPathsReport-r16			ENUMERATED { supported }						OPTIONAL,

periodicalReporting-r16				ENUMERATED { supported }						OPTIONAL,

...

}

-- ASN1STOP

6.5.12.7	NR-Multi-RTT Capability Information Request

–	NR-Multi-RTT-RequestCapabilities

The IE NR-Multi-RTT-RequestCapabilities is used by the location server to request the capability of the target device to support NR Multi-RTT and to request NR Multi-RTT positioning capabilities from a target device.

-- ASN1START

# NR-Multi-RTT-RequestCapabilities ::= SEQUENCE {

...

}

-- ASN1STOP

6.5.12.8	NR-Multi-RTT Error Elements

–	NR-Multi-RTT-Error

The IE NR-Multi-RTT-Error is used by the location server or target device to provide NR Multi-RTT error reasons to the target device or location server, respectively.

-- ASN1START

# NR-Multi-RTT-Error-r16 ::= CHOICE {

locationServerErrorCauses-r16		NR-Multi-RTT-LocationServerErrorCauses-r16,

targetDeviceErrorCauses-r16			NR-Multi-RTT-TargetDeviceErrorCauses-r16,

...

}

-- ASN1STOP

–	NR-Multi-RTT-LocationServerErrorCauses

The IE NR-Multi-RTT-LocationServerErrorCauses is used by the location server to provide NR Multi-RTT error reasons to the target device.

-- ASN1START

# NR-Multi-RTT-LocationServerErrorCauses-r16 ::= SEQUENCE {

cause-r16		ENUMERATED	{	undefined,

assistanceDataNotSupportedByServer,

assistanceDataSupportedButCurrentlyNotAvailableByServer,

...

},

...

}

-- ASN1STOP

–	NR-Multi-RTT-TargetDeviceErrorCauses

The IE NR-Multi-RTT-TargetDeviceErrorCauses is used by the target device to provide NR Multi-RTT error reasons to the location server.

-- ASN1START

# NR-Multi-RTT-TargetDeviceErrorCauses-r16 ::= SEQUENCE {

cause-r16		ENUMERATED {	undefined,

dl-assistance-data-missing,

unableToMeasureAnyTRP,

attemptedButUnableToMeasureSomeNeighbourTRPs,

ul-srs-configuration-missing,

unableToTransmit-ul-prs,

...

},

nr-PRS-RSRPMeasurementNotPossible-r16				NULL		OPTIONAL,

nr-UERxTxMeasurementNotPossible-r16			NULL		OPTIONAL,

...

}

-- ASN1STOP

–	End of LPP-PDU-Definitions

-- ASN1START

# END

-- ASN1STOP