3GPP TSG-RAN WG2 Meeting #111-e	R2-2006661

Online, 17–28 August 2020

# Agenda item:	8.12.2.2

# Source:	Samsung

# Title:	Coexistence between legacy UEs and RedCap UEs

# Document for:	Discussion and Agreement

# 1	Introduction

RAN approved a new SI: Study on support of reduced capability NR devices for Rel-17 [1].

The contribution discusses how to co-exist between legacy UEs and RedCap UEs when they are in RRC_IDLE.

# 2	Discussion

Even though the SID [1] does not clearly state, we think the SI assumes a gNB that supports RedCap UEs should accommodate both legacy UEs and RedCap UEs. This can be hinted by the following statements from the SID:

- UE Bandwidth reduction Note: Rel-15 SSB bandwidth should be reused and L1 changes minimized … Note3: Coexistence with Rel-15 and Rel-16 UE should be ensured

Otherwise, this SI would create another RAT like NB-IoT. Thus,

Proposal 1:	RAN2 to confirm that a gNB that supports RedCap UEs also supports legacy UEs simultaneously.

If Proposal 1 is agreeable, the legacy signalling in MIB should be reused to support legacy UEs. From Rel-15 [2], MIB provides the bandwidth of initial BWP by controlResourceSetZero which is defined in TS 38.213 [3]:

MIB ::= SEQUENCE { systemFrameNumber BIT STRING (SIZE (6)), subCarrierSpacingCommon ENUMERATED {scs15or60, scs30or120}, ssb-SubcarrierOffset INTEGER (0..15), dmrs-TypeA-Position ENUMERATED {pos2, pos3}, pdcch-ConfigSIB1 PDCCH-ConfigSIB1, cellBarred ENUMERATED {barred, notBarred}, intraFreqReselection ENUMERATED {allowed, notAllowed}, spare BIT STRING (SIZE (1)) } PDCCH-ConfigSIB1 ::= SEQUENCE { controlResourceSetZero ControlResourceSetZero, searchSpaceZero SearchSpaceZero } ControlResourceSetZero ::= INTEGER (0..15) SearchSpaceZero ::= INTEGER (0..15)

Proposal 2:	The legacy MIB signalling is re-used to accommodate legacy UEs.

Also note that RAN1 made the following agreements last meeting:

Agreements: - For FR1, study at least 20MHz maximum UE bandwidth at least for initial access - Other bandwidths FFS - For FR2, study 50MHz and 100 MHz maximum UE bandwidth at least for initial access - Other bandwidths FFS

From their agreements, it can be observed that:

-	In FR1, all the RedCap UEs can support the bandwidth of the legacy CORESET#0, as the maximum bandwidth of CORESET#0 in FR1 is 20 MHz.

-	In FR2, RedCap UEs with 100 MHz can support the bandwidth of the legacy CORESET#0, as the maximum bandwidth of CORESET#0 in FR1 is 20 MHz. However, RedCap UEs with 50 MHz would not support the legacy CORESET#0 with 100 MHz.

For instance, suppose we consider 50 MHz bandwidth of RedCap UEs in FR2, and network is using the following table for its CORESET#0, then UE would only be able to camp on cell with the yellow-highlighted CORESETs below. So, if a RedCap UE supporting 50 MHz detects a cell with a CORESET index other than the yellow-highlighted values above, it should consider the cell as barred.:

Table 13-7: Set of resource blocks and slot symbols of CORESET for Type0-PDCCH search space set when {SS/PBCH block, PDCCH} SCS is {120, 60} kHz Index; SS/PBCH block and CORESET multiplexing pattern; Number of RBs; Number of Symbols; Offset (RBs). 0; 1; 48; 1; 0. 1; 1; 48; 1; 8. 2; 1; 48; 2; 0. 3; 1; 48; 2; 8. 4; 1; 48; 3; 0. 5; 1; 48; 3; 8. 6; 1; 96; 1; 28. 7; 1; 96; 2; 28. 8; 2; 48; 1; -41 if -42 if. 9; 2; 48; 1; 49. 10; 2; 96; 1; -41 if -42 if. 11; 2; 96; 1; 97. 12; Reserved. 13; Reserved. 14; Reserved. 15; Reserved.

Furthermore, since the field intraFreqReselection already exists in MIB, and a RedCap UE can read this information as well, the legacy field intraFreqReselection can be reused for the UE to determine whether to bar the frequency, and try to find another cell in other frequencies, if the UE cannot camp on the cell.

Proposal 3:	The legacy UE determines whether it can access the cell based on the legacy values of controlResourceSetZero.

Proposal 4:	If a RedCap UE does not support the bandwidth from the controlResourceSetZero of a cell, it considers the cell as barred.

Proposal 5:	The field intraFreqReselection is reused to determine whether a RedCap UE performs cell selection/reselection to intra-frequency cells if the cell is barred.

# 3	Conclusion

Proposal 1:	RAN2 to confirm that a gNB that supports RedCap UEs also supports legacy UEs simultaneously.

Proposal 2:	The legacy MIB signalling is re-used to accommodate legacy UEs.

Proposal 3:	The legacy UE determines whether it can access the cell based on the legacy values of controlResourceSetZero.

Proposal 4:	If a RedCap UE does not support the bandwidth from the controlResourceSetZero of a cell, it considers the cell as barred.

Proposal 5:	The field intraFreqReselection is reused to determine whether a RedCap UE performs cell selection/reselection to intra-frequency cells if the cell is barred.

# 4	References

[1]	RP-201386	Revised SID on Study on support of reduced capability NR devices	Ericsson

[2]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

[3]	3GPP TS 38.213: "NR; Physical layer procedures for control".