3GPP TSG-RAN WG2 Meeting #111 electronic	R2-2006668

# Online, August 17th - 28th, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.305 | CR | 0090 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on 36.305 in Table 4.3-1Supported versions of UE positioning methods |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2020-08-03 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - The eNB-assisted and LMU-assisted are not clearly defined and should be clarified because: “-assisted” is defined as a node that provides measurements (but which does not take the positioning calculation). But eNB-assisted and LMU-assisted is not clarified who takes the positioning calculation in 36.305, especially in Table 4.3-1: Supported versions of UE positioning methods. - “/” in the title “LMU-assisted/ E-SMLC-based” isn’t aligned with the comma in the title “UE-assisted, E-SMLC-based”. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add the definition of “UE-based” mentioned in Table 4.3-1: Supported versions of UE positioning methods. - Add the definition of “UE-assisted, E-SMLC-based” mentioned in Table 4.3-1: Supported versions of UE positioning methods. - Add the definition of “eNB-assisted, E-SMLC-based” mentioned in Table 4.3-1: Supported versions of UE positioning methods. - Add the definition of “LMU-assisted, E-SMLC-based” mentioned in Table 4.3-1: Supported versions of UE positioning methods. - “LMU-assisted/ E-SMLC-based” is updated as “LMU-assisted, E-SMLC-based” in Table 4.3-1: Supported versions of UE positioning methods. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The eNB-assisted is not clearly defined. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

4.3	Standard UE Positioning Methods

The standard positioning methods supported for E-UTRAN access are:

-	network-assisted GNSS methods;

-	downlink positioning;

-	enhanced cell ID method;

-	uplink positioning;

-	WLAN method;

-	Bluetooth method;

-	Terrestrial Beacon System method;

-	Sensor based methods:

-	Barometric Pressure Sensor;

-	Motion sensor.

Hybrid positioning using multiple methods from the list of positioning methods above is also supported.

Standalone mode (e.g. autonomous, without network assistance) using one or more methods from the list of positioning methods above is also supported.

These positioning methods may be supported in UE-based, UE-assisted, E-SMLC-based, eNB-assisted, and LMU-assisted, E-SMLC-based versions:

UE-based: The UE provides measurements and calculates its own position location;

UE-assisted, E-SMLC-based: The UE provides measurements and E-SMLC calculates position location;

eNB-assisted, E-SMLC-based: The eNB provides measurements and E-SMLC calculates position location;

LMU-assisted, E-SMLC-based: The LMU provides measurements and E-SMLC calculates position location.

Table 4.3-1 indicates which of these versions are supported in this version of the specification for the standardised positioning methods.

Table 4.3-1: Supported versions of UE positioning methods

| Method | UE-based | UE-assisted, E-SMLC-based | eNB- assisted, E-SMLC-based | LMU-assisted, E-SMLC-based | SUPL |
| --- | --- | --- | --- | --- | --- |
| A-GNSS | Yes | Yes | No | No | Yes (UE-based and UE-assisted) |
| Downlink Note1 | No | Yes | No | No | Yes (UE-assisted) |
| E-CID | No | Yes | Yes | No | Yes (UE-assisted) |
| Uplink | No | No | No | Yes | No |
| Sensor | Yes | Yes | No | No | No |
| WLAN | Yes | Yes | No | No | Yes |
| Bluetooth | No | Yes | No | No | No |
| TBS Note 2 | Yes | Yes | No | No | Yes (MBS) |
| NOTE 1: This includes TBS positioning based on PRS signals. NOTE 2: In this version of the specification only for TBS positioning based on MBS signals. |  |  |  |  |  |

END OF CHANGE