3GPP TSG-RAN WG2 #111e	R2-206979

Electronic meeting, 17 – 28 August, 2020

Agenda item:		8.12.2.1 Principles for how to define and constrain reduced capabilities

Source:		NEC

Title:		Constraint on usage of RedCap functions

Document for:		Discussion and Decision

# 1. Introduction

The RedCap SI includes the following objective [1].

Study standardization framework and principles for how to define and constrain such reduced capabilities – considering definition of a limited set of one or more device types and considering how to ensure those device types are only used for the intended use cases [RAN2, RAN1]. Study functionality that will allow devices with reduced capabilities to be explicitly identifiable to networks and network operators, and allow operators to restrict their access, if desired [RAN2, RAN1]. … Note3: Coexistence with Rel-15 and Rel-16 UE should be ensured

In this contribution, we discuss potential issues and possible solutions for identification of reduced capability UEs and constraining its usages.

# 2. Discussion

The basic issue is how to identity the reduced capability (RedCap) UEs by the network. In general, there are two ways to solve this type of issue. One way is to broadcast the specific RACH configurations differently from legacy UEs and identify the RedCap UEs upon detecting the preamble (Option 1 in Fig.1). The other way is to identify based on the UE capability or the subscription information at/from the core network (Option 2 in Fig.2).

In Option 1, the RedCap UE specific RACH configuration (e.g. PRACH preamble, time/frequency resource) is necessary and the gNB needs to broadcast them in SIB1. This is useful for the gNB to detect the RedCap UE especially in the attach case, where no UE capability is yet available in the network.

In Option 2, there is no need for specific RACH configuration. On the other hand, the gNB can identify the accessing UE as the RedCap upon receiving UE Capability Information (2a) or UE Subscription Information from 5GC (2b). This approach needs a fundamental assumption that the RedCap UEs can perform the RRC setup procedure same as legacy UEs, e.g. without any bandwidth issue. Note that examples of the UE subscription information are a device type (e.g. machine type, stationary) or a use case (e.g. sensors, video surveillance, wearables).

Observation 1. There could be two ways to identify the RedCap UEs, i.e. RACH based and UE capability or subscription based.

Fig.1: RedCap UE identification at gNB – option 1 RACH-based

Fig.2: RedCap UE identification at gNB – option 2 (2a: UE capability-based, 2b: UE subscription information-based)

For this SI (and following WI), other important objective is to ensure only a certain type of devices are accessing (i.e. selecting allowed RedCap UEs) or such devices are accessing for a certain use cases. This needs to be discussed together with the way of identification of RedCap UEs discussed above, because they have tight relationship each other.

For example, if the RACH-based identification is introduced as Fig.1, there are some mechanisms to limit an allowed RedCap UEs based on the RACH configuration. This may cause a lot of PRACH resource fragmentation or overhead due to additional PRACH resources. If the UE capability-based or the subscription-based identification is introduced, a mechanism to limit an allowed RedCap UEs could be simpler e.g. just based on capability or device type.

On the other hand, if a mechanism to allow the RedCap UEs to access for a certain use case(s) is necessary, it will require an interaction with or assistance from 5GC. This may be realized by the UE subscription-based identification (option 2b). Otherwise, at the UE side, the NAS layer needs to be aware of triggered use case.

Based on considerations above, it would be useful to discuss and reach consensus about the possible mechanism to identify RedCap UEs as well as constrain its usage only for intended devices and use cases.

Observation 2. It would be necessary to discuss how to identify the RedCap UEs and how to constrain its usage together.

Proposal 1: RAN2 to discuss possible solutions to identify the reduced capability UEs and constrain the functions only for intended UEs.

# 3. Conclusion

In this contribution we discussed potential issues and possible solutions for identification of reduced capability UEs and constraining its usages. We made following observations and proposal.

Observation 1. There could be two ways to identify the RedCap UEs, i.e. RACH based and UE capability or subscription based.

Observation 2. It would be necessary to discuss how to identify the RedCap UEs and how to constrain its usage together.

Proposal 1: RAN2 to discuss possible solutions to identify the reduced capability UEs and constrain the functions only for intended UEs.

# References

[1] RP-201386, “Revised SID on Study on support of reduced capability NR devices”, Ericsson

[2] Chairman’s Notes RAN1#101-e final