3GPP TSG-RAN WG2 Meeting #111-e	 R2-2007049

# Online, 17 Aug. – 28 Aug. 2020

Agenda Item:	8.11.2

Source:	Spreadtrum Communications

Title:	Discussion on positioning enhancements for commercial use cases

Document for:	Discussion and Decision

Introduction

This meeting is the first discussion about Rel-17 NR positioning enhancement. In the SID, one of the objective is to study enhancements and solutions of Rel-16 positioning techniques. The details are as follows:

- Study enhancements and solutions necessary to support the high accuracy (horizontal and vertical), low latency, network efficiency (scalability, RS overhead, etc.), and device efficiency (power consumption, complexity, etc.) requirements for commercial uses cases (incl. general commercial use cases and specifically (I)IoT use cases as exemplified in section 3 above (Justification)):

  - Define additional scenarios (e.g. (I)IoT) based on TR 38.901 to evaluate the performance for the use cases (e.g. (I)IoT). [RAN1]

  - Evaluate the achievable positioning accuracy and latency with the Rel-16 positioning solutions in (I)IoT scenarios and identify any performance gaps. [RAN1]

  - Identify and evaluate positioning techniques, DL/UL positioning reference signals, signalling and procedures for improved accuracy, reduced latency, network efficiency, and device efficiency.
Enhancements to Rel-16 positioning techniques, if they meet the requirements, will be prioritized, and new techniques will not be considered in this case. [RAN1, RAN2]

NOTE 1:	Sidelink is not part of this objective.

NOTE 2:	Involve RAN4 for validating assumptions for the systems evaluations where appropriate.

NOTE 3:	The commercial use cases and requirements are applicable to a limited geographic area.

In this contribution, we discuss the positioning enhancements of Rel-16 positioning techniques about scope and direction from RAN2 perspective and express our opinions.

Discussion

## 2.1 Local LMF

In the SID of Rel-17 positioning there are two exemplary use cases including general commercial use cases and IIoT use cases. The target latency requirement for this SI were set to < 100 ms; for some IIoT use cases, latency in the order of 10 ms is desired. This positioning latency requirement is very strict. Therefore reducing the positioning latency is one of the main goals of this SI.

Figure 1 illustrates the overall signalling procedure between the UE, NG-RAN and LMF for any location service [1]. Latency concerns the delay in obtaining location information for a target UE (e.g. a location estimate). From a LCS client perspective and for on demand location, the positioning (or location) latency would comprise the time between sending a request for location to the 5GC and receiving a location response from the 5GC [2]. That is the elapsed time from step 1 to step 11 in the figure 1. And there is a large of signalling interaction between AMF and LMF in the core network. To address low latency positioning requirement, introducing the local LMF (i.e. Location Management Component, LMC) is a promising direction.

In Rel-16 RAN3 has study LMC architecture [3], and it has captured three types of architecture. It is concluded that support of LMC in NG-RAN is feasible. In addition, the LMC can be expected to interact with other internal functions of the serving NG-RAN node in order to e.g. obtain position measurements for the UE and it can also interact with neighbouring NG-RAN nodes in order to e.g. obtain position measurements for the UE.

Figure 1. The overall signalling procedure

Figure 2. The overall signalling procedure with LMC

Thus, if we introduces the LMC, the overall signalling procedure will be simplified as shown in figure 2. In the step 3, when LCS service request message arrival LMC in NG-RAN, LMC can process this message and generate DL positioning message to require UE positioning capability or request location information. The LMC will sent this message to the serving NG-RAN node through the internal interface or function. And there are many LPP messages which need to interact between UE and LMC to locate an UE, including capability information, assistance data and location information. And each of the procedure of LPP may transmit many LPP messages. For example, in assistance data transmission procedure, UE sends a RequestAssistanceData message to the server. The service responds with a ProvideAssistanceData message to the target containing assistance data. The server may transmit one or more additional ProvideAssistanceData messages to the target containing further assistance data. Thus, when the LMC is introduced, the signalling overhead can be reduced and the latency also be decreased.

Proposal 1: Introduce Local LMF function in NG-RAN.

## 2.2 RRC IDLE/INACTIVE positioning

For IoT devices, power consumption is a foremost parameter since these devices are expected to operate for a long time. In this regard, IOT devices should be kept in RRC IDLE/INACTIVE state as much as possible to reduce power consumption. In LTE IDLE mode positioning is not supported. RRC_IDLE state UE has to enter connected state whenever positioning is required. And UE needs to be in RRC-CONNECTED state to receive positioning reference signals and perform positioning function. In NR, due to the fact that some devices are of low complexity and require low energy consumption, RRC-IDLE state positioning is discussed. Since PRS can be periodic, the wakeup periodicity of the UE can be matched with PRS periodicity so that when UE wakes up, it can potentially measure PRS without entering RRC-CONNECTED state.

Nowadays, in [2], the following use cases are defined.

Table 2.1 – Use cases synthesis

| Use cases |  | Potential requirements per use cases |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Environment of Use | Position Accuracy | Velocity | Avail. | Update rate or interval | TTFF | Latency | Other KPI |
| 5.2.1 | Bike sharing | 5G positioning service area - Outdoor | 2m Horizontal |  | 90 % |  | 10s | 1s |  |
|  |  | Enhanced positioning area - Outdoor | 0.2m Horizontal |  | 99 % |  | 10s | 1s |  |
| 5.2.2 | Augmented Reality | Outdoor - 5G positioning service area | 1-3m Horizontal 0.1-3m Vertical | 2 m/s 10deg. | 80 % | 1 - 10 Hz | 10s | 1s | Low Energy |
| 5.2.3 | Wearables | 5G positioning service area - -Outdoor/Indoor | 2m Horizontal 1-3m Vertical |  | 90 % | 30s - 300s | 10s |  | Power saving mode |
|  |  | 5G positioning service area - -Outdoor/Indoor | 2m Horizontal 1-3m Vertical |  | 99 % | 1s - 30s | 10s | 1s | Normal mode |
| 5.2.4 | Advertisement push | 5G positioning service area - -Outdoor/Indoor | 3m Horizontal 3m Vertical |  | 90 % |  |  | 60s |  |
| 5.2.5 | Flow management | Enhanced positioning- Outdoor/Indoor | 10m Horizontal |  | 80 % | 10s | 10s |  |  |
| 5.3.1 | Person and medical equipment location in Hospital | Enhanced positioning- Outdoor/Indoor | 3m Horizontal 2m Vertical |  | 99 % |  |  | 60s |  |
| 5.3.2 | Patient location (outside Hospital) | 5G positioning service area Outdoor/Indoor | 10m Horizontal 3m Vertical (floor) |  | 99 % |  |  |  |  |
| 5.3.3 | Trolley | Enhanced positioning- Outdoor/Indoor | 0.5m Horizontal 1-3m Vertical |  | 99 % |  |  | 20ms |  |
| 5.3.4 | Waste management | 5G positioning service area - Outdoor | 3m Horizontal |  | 99 % | 2h - 1 day |  | 60s | Very low energy (15 years) |
| 5.4.1 | Emergency call | 5G positioning service area Outdoor/Indoor | 50m Horizontal 3m Vertical |  | 95 % |  | 30s | 60s | Reliability/ Confidence |
| 5.4.2 | Accurate Positioning for First Responders | Outdoor | 1m Horizontal, 0.3 m Vertical |  | 98 % |  | 10s | 5s | MCX Confidence Event-triggered report |
|  |  | Indoor | 1m Horizontal, 2 m Vertical |  | 95 % |  | 10s | 1s |  |
| 5.4.3 | Alerting nearby emergency responders | 5G positioning service area Outdoor/Indoor | 50m Horizontal 3m Vertical (floor) |  | 99% |  | 10s |  | Privacy, scalability, cross operator |
| 5.4.4 | Emergency equipment loc. outside hospitals | 5G positioning service area Outdoor/Indoor | 10m Horizontal 3m Vertical (floor) |  | 95% |  | 10s |  | Extended sleep periods |
| 5.5.1 | Traffic Monitoring & Control | 5G positioning service area - Outdoor | 1-3m Horizontal 2.5m Vertical |  | 95 % | 10 Hz | 10s | 30ms | Antispoofing Antitampering |
| 5.5.2 | Road User Charging | 5G positioning service area - Outdoor Enhanced positioning-Tunnels | <1m (across track) 3m (along track) | 2 m/s | 99 % | 1 Hz | 10s |  | Antispoofing Antitampering |
| 5.6.1 | Asset tracking and management | 5G positioning service area - Outdoor | 10-30m Horizontal | 5 m/s | 99 % | 300s-1day |  |  | 20 mJ/fix (average), Antispoofing, Antitampering, support for "out of coverage" positioning |
|  |  | Enhanced positioning - Outdoor | 1m Horizontal |  | 99 % | 1s | 1s in enhanced positioning area |  |  |
| 5.7.1 | UAV (Data analysis) | 5G positioning service area - Outdoor | 0.1m Horizontal 0.1m Vertical | 0.5 m/s 2 deg. | 99 % |  | 10s |  | Low Energy, Antispoofing, Antitampering |
| 5.7.2 | UAV (Remote control) | 5G positioning service area - Outdoor | 0.5m Horizontal 0.3m Vertical |  | 99 % |  |  | 150ms | Antispoofing Antitampering |
|  |  | Enhanced positioning area - Outdoor | 0.5m Horizontal 0.1m Vertical |  | 99.9 % |  |  | 150ms | Antispoofing Antitampering |
| 5.8.1 | Support multiple different location service | 5G positioning service area - Outdoor | 2m Horizontal |  | 90 % |  | 10s | 1s | Management of different KPI and positioning services |
|  |  | Enhanced positioning area - Indoor | 0.1m Horizontal |  | 99 % |  | 10s | 1s |  |
| 5.8.2 | Support location method negotiation | 5G positioning service area Outdoor/Indoor |  |  |  |  |  |  | Support + negotiation of positioning methods (incl. hybrid) |
| Note: most use cases also feature potential requirements on modes of operation, intended for the UE, the Network or for the 5G system. |  |  |  |  |  |  |  |  |  |

Among all the use cases, the use cases requiring low energy or power saving mode include AR, wearables, waste management and UAV. For all these four use cases, it is clear that the devices need to report its position to the network. Take wearable devices as an example, in order to obtain minor or elderly people’s position, wearable devices, such as smart watches, can provide this services, such as tracking, activity monitoring and emergency messages.

Without supporting idle/Inactive mode positioning, UE needs to switch between connected and idle states frequently, which leads to lots of signalling overhead. If positioning can be performed without entering the connected mode, both the UE power consumption and the signalling overhead could be reduced.

Observation 1: RRC IDLE/INACTIVE positioning can improve device efficiency and signaling overhead

In order to support positioning a RRC IDLE/INACTIVE devices, there are two ways: 1) UE-based positioning method; and 2) UE-assisted positioning method with UE measurement result reporting.

For UE-based positioning, The UE does not need to report measurement result to network, instead calculates the positioning result by himself. Take DL-TDoA as an example, upon the reception of assistance data including PRS configuration, UE can start to measure the DL reference signals (e.g. PRS) from the serving and neighbouring cells to obtain the RSTD. And then UE can calculate the positioning result by itself. Moreover, in Rel-16., it has been support broadcasting assistance data. Thus, this method has weedy change to the spec, and complexity is also very low. In addition, if location request is initiated by the UE – i.e. mobile originated location request (MO-IR) [1], it is usually not needed to report location result to the network.

Observation 2：For UE-based positioning in RRC IDLE/INACTIVE state, if location request is initiated by the UE, it is not needed to report location result to the network.

If location request is initiated by AMF or location client – i.e. Mobile Terminated Location Request (MT-LR) and Network Induced Location Request (NI-LR) [1], the location results should be report to the network.

Observation 3: For UE-based positioning in RRC IDLE/INACTIVE state, if location request is initiated by AMF or location client, the location results should be report to the network.

For UE-assisted positioning method, UE needs to report its measurement result of the DL reference signals in RRC IDLE/INACTIVE state. At present, Rel-17 small data enhancements are studying to report data in RRC INACTIVE state, which can be used as the baseline for positioning data report.

And for RRC IDLE state UE, measurement result can report through MSG A/3 message of RACH procedure, which can use the early data transmission (EDT) procedure of NB-IoT as the baseline.

Observation 4：For UE-assisted positioning in RRC IDLE/INACTIVE state, the measurement results should be reported to the network.

Proposal 2: UE can report location or measurement results to the network in RRC IDLE/INACTIVE state.

Conclusions

In this contribution, we discuss the positioning enhancements of Rel-16 positioning techniques. Proposals are given as follows:

Proposal 1: Introduce Local LMF function in NG-RAN.

Observation 1: RRC IDLE/INACTIVE positioning can improve device efficiency and signaling overhead

Observation 2：For UE-based positioning in RRC IDLE/INACTIVE state, if location request is initiated by the UE, it is not needed to report location result to the network.

Observation 3: For UE-based positioning in RRC IDLE/INACTIVE state, if location request is initiated by AMF or location client, the location results should be report to the network.

Observation 4：For UE-assisted positioning in RRC IDLE/INACTIVE state, the measurement results should be reported to the network.

Proposal 2: UE can report location or measurement results to the network in RRC IDLE/INACTIVE state.

References

- TS 23.273 “5G system (5GS) Location Services (LCS)” v16.4.0, 2020-07

- TR 22.872 “Study on positioning use cases” v16.1.0, 2019-09

- TR 38.856 “Study on local NR positioning in NG-RAN” v16.0.0, 2019-12