3GPP TSG-RAN WG2 Meeting #111-e	R2-2007119

Online, August 17th - 28th, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.304 | CR | 0806 | rev | - | Current version: | 15.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on Srxlev Calculation for IRAT Cell Reselection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple, Qualcomm Incorporated, Nokia, Nokia Shanghai Bell, ZTE Corporation, Sanechips, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NewRAT-Core |  |  |  |  |  | Date: |  |  | 2020-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In the last RAN2 meeting (RAN2#110e), for NR inter-RAT cell reselection to LTE, it was common understanding that the Srxlev of the target LTE cell should be calculated according to the description in TS 36.304. But there are two parameters (q-QualMinOffsetCell and q-RxLevMinOffsetCell) broadcasted in NR SIB5 for inter-RAT cell selection and these are not taken into account in the Srxlev calculation of the E-UTRA cell in 36.304. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add q-QualMinOffsetCell and q-RxLevMinOffsetCell in Srxlev calculation in TS 36.304. Impact analysis Impacted 5G architecture options: NE-DC/NR-DC/NR SA Impacted functionality: Inter-RAT cell reselection Inter-operability - if the network is implemented according to the CR and the UE is not: the UE will not use the correct parameters for inter-RAT cell reselection from NR to LTE. - if the UE is implemented according to the CR and the network is not: there is no impact. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The two parameters (q-QualMinOffsetCell and q-RxLevMinOffsetCell) in NR SIB5 for inter-RAT cell selection will not be used by the UE for cell reselection from NR to LTE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

----------------------------- [Change Start] ---------------------------

#### 5.2.3.2	Cell Selection Criterion

For NB-IoT the cell selection criterion is defined in clause 5.2.3.2a.

The cell selection criterion S in normal coverage is fulfilled when:

Srxlev > 0 AND Squal > 0

where:

Srxlev = Qrxlevmeas – (Qrxlevmin + Qrxlevminoffset) – Pcompensation - Qoffsettemp Squal = Qqualmeas – (Qqualmin + Qqualminoffset) - Qoffsettemp

where:

| Srxlev | Cell selection RX level value (dB) |
| --- | --- |
| Squal | Cell selection quality value (dB) |
| Qoffsettemp | Offset temporarily applied to a cell as specified in TS 36.331 [3] (dB) |
| Qrxlevmeas | Measured cell RX level value (RSRP) |
| Qqualmeas | Measured cell quality value (RSRQ) |
| Qrxlevmin | Minimum required RX level in the cell (dBm) If Qrxlevminoffsetcell is signalled in NR SIB5 in TS 38.331[37] for the concerned cell, this cell specific offset is added to achieve the required minimum RX level in the concerned cell. |
| Qqualmin | Minimum required quality level in the cell (dB) If Qqualminoffsetcell is signalled is signalled in NR SIB5 in TS 38.331 [37] for the concerned cell, this cell specific offset is added to achieve the required minimum quality level in the concerned cell. |
| Qrxlevminoffset | Offset to the signalled Qrxlevmin taken into account in the Srxlev evaluation as a result of a periodic search for a higher priority PLMN while camped normally in a VPLMN TS 23.122 [5] |
| Qqualminoffset | Offset to the signalled Qqualmin taken into account in the Squal evaluation as a result of a periodic search for a higher priority PLMN while camped normally in a VPLMN TS 23.122 [5] |
| Pcompensation | If the UE supports the additionalPmax in the NS-PmaxList, if present, in SIB1, SIB3 and SIB5: max(PEMAX1 –PPowerClass, 0) – (min(PEMAX2, PPowerClass) – min(PEMAX1, PPowerClass)) (dB); else: if PPowerClass is 14 dBm: max(PEMAX1 –(PPowerClass – Poffset), 0) (dB); else: max(PEMAX1 –PPowerClass, 0) (dB) |
| PEMAX1, PEMAX2 | Maximum TX power level an UE may use when transmitting on the uplink in the cell (dBm) defined as PEMAX in TS 36.101 [33]. PEMAX1 and PEMAX2 are obtained from the p-Max and the NS-PmaxList respectively in SIB1, SIB3 and SIB5 as specified in TS 36.331 [3]. |
| PPowerClass | Maximum RF output power of the UE (dBm) according to the UE power class as defined in TS 36.101 [33] |

The signalled values Qrxlevminoffset and Qqualminoffset are only applied when a cell is evaluated for cell selection as a result of a periodic search for a higher priority PLMN while camped normally in a VPLMN TS 23.122 [5]. During this periodic search for higher priority PLMN the UE may check the S criteria of a cell using parameter values stored from a different cell of this higher priority PLMN.

If cell selection criterion S in normal coverage is not fulfilled for a cell, UE shall consider itself to be in enhanced coverage if the cell selection criterion S for enhanced coverage is fulfilled, where:

| Qrxlevmin | UE applies coverage specific value Qrxlevmin_CE (dBm) |
| --- | --- |
| Qqualmin | UE applies coverage specific value Qqualmin_CE (dB) |

If cell selection criterion S in normal coverage is not fulfilled for a cell and UE does not consider itself in enhanced coverage based on coverage specific values Qrxlevmin_CE and Qqualmin_CE, UE shall consider itself to be in enhanced coverage if UE supports CE Mode B and CE mode B is not restricted by upper layers and the cell selection criterion S for enhanced coverage is fulfilled, where:

| Qrxlevmin | UE applies coverage specific value Qrxlevmin_CE1 (dBm) |
| --- | --- |
| Qqualmin | UE applies coverage specific value Qqualmin_CE1 (dB) |

For the UE in enhanced coverage, coverage specific values Qrxlevmin_CE and Qqualmin_CE (or Qrxlevmin_CE1 and Qqualmin_CE1) are only applied for the suitability check in enhanced coverage (i.e. not used for measurement and reselection thresholds).

----------------------------- [Change End] ---------------------------