3GPP TSG-RAN WG2 Meeting #111 Electronic	R2-2007560

Elbonia, 17 – 28 August 2020

Agenda item:	8.12.2.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Cell access restrictions for REDCAP UE

WID/SID:	FS_NR_redcap - Release 17

Document for:	Discussion and Decision

# 1	Introduction

New SID on support for reduced capability (REDCAP) devices was approved in RP-193238.

In this contribution we discuss the access restriction for REDCAP devices as per the following study item objective:

- Study functionality that will allow devices with reduced capabilities to be explicitly identifiable to networks and network operators, and allow operators to restrict their access, if desired [RAN2, RAN1].

# 2	Discussion

Overload and access control functionality such as RACH back off, RRC Connection Reject, RRC Connection Release and UE based access barring mechanisms is already supported in NR specifications.

In addition unified access control framework applies to all UE states (RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED) for NR. NG-RAN broadcasts barring control information associated with Access Categories and Access Identities. The UE determines whether an access attempt is authorized based on the barring information broadcast for the selected PLMN, and the selected Access Category and Access Identity(ies) for the access attempt.

The gNB handles access attempts with establishment causes "emergency", "mps-PriorityAccess" and "mcs-PriorityAccess" (i.e. Emergency calls, MPS, MCS subscribers) with high priority and responds with RRC Reject to these access attempts only in extreme network load conditions that may threaten the gNB stability.

In general, above mentioned mechanisms are applicable for all the UEs and cannot be used as such only for REDCAP devices access control.  Given the possible high number of REDCAP devices and also their potentially reduced coverage, operators may wish to limit REDCAP devices from using certain cells/carrier. As can be seen in the SID objectives, the networks and network operators should be allowed to restrict REDCAP UE access. Therefore, we propose the following:

Proposal 1: Network can restrict REDCAP UEs access

MIB or SIB based barring could be considered for REDCAP UEs access. NR MIB has between 1-3 spare bits depending on the configuration and it seems challenging to use MIB bits for REDCAP UE barring. In addition, dedicated message e.g. RRC Reject with extended wait time could be used for restricting the access of REDCAP UE. In LTE network can authorize MTC UE to utilize coverage enhancements for the selected PLMN. Upper layers indicates to the lower layers whether the use of coverage enhancements is authorized. Network can broadcast Qoffsetauthorization which is used in cell selection criterion. REDCAP UE has potentially reduced coverage which needs to be compensated by transmission repetitions which is expensive from network resources point of view. Therefore, we propose the following:

Proposal 2: Both broadcast and dedicated access restriction mechanisms are considered for REDCAP UEs

# 3	Conclusion

In this contribution the access restriction for the REDCAP UEs was discussed, and the following is proposed:

Proposal 1: Network can restrict REDCAP UEs access

Proposal 2: Both broadcast and dedicated access restriction mechanisms are considered for REDCAP UEs