3GPP TSG RAN2 Meeting #111-e 	R2-2007572

e-Meeting, August 17th – 28th, 2020

Title:	NR NTN Reference scenarios definition for Rel-17 normative phase

Source: 	Thales

Type:	Discussion

Document for:	Decision

Agenda Item:	8.10.1

# Abstract

The objective of this paper is to down select the reference scenarios that will be considered for the normative phase on NR supports NTN. Three scenarios are proposed.

# Introduction

NR NTN WI description [1] focus the work for NR solutions to support NTN as follows:

“LEO and GEO with implicit compatibility to support HAPS (high altitude platform station) and ATG (air to ground) scenarios according to the following principles:

- FDD is assumed for core specification work for NR-NTN

  - NOTE: This does not imply that TDD cannot be used for relevant scenario, e.g. HAPS, ATG

- Earth fixed tracking area is assumed with Earth fixed and moving cells

- UEs with GNSS capabilities are assumed

- Transparent payload is assumed”

According to TR 38.821, a Transparent payload refers to payload that changes the frequency carrier of the uplink RF signal, filters and amplifies it before transmitting it on the downlink. In other word, it doesn’t embark any function or network entity defined in the 3GPP architecture.

This TDOC intends to define NTN reference scenarios for Rel.17 normative work and all related assumptions for all the RAN WGs involved in the normative work.

The objective is to support LEO, GEO as well as MEO space segments. LEO features the largest Doppler while GEO features the largest Latency.

They correspond to scenarios A, C1 and C2 in Table 4.2-1: “Reference scenarios” of 3GPP TR 38.821.

# Non-Terrestrial Network scenarios: Overview

3.1 Main assumptions

Based on the reference scenarios identified in TR 38.821, the following scenarios are selected

Six transparent payload based satellite reference scenarios are considered for the Rel-17 work item “NR_NTN_solutions” characterised in the table below:

Table 2-1 Reference satellite scenarios for Rel-17 work item “NR_NTN_solutions”

| Scenarios | C1.1 | C1.2 | C2.1 | C2.2 | A1 | A2 |
| --- | --- | --- | --- | --- | --- | --- |
| Orbit | LEO @ 600 km altitude | LEO @ 600 km altitude | LEO @ 1200 km altitude | LEO @ 1200 km altitude | GEO @ 35,786 km altitude | GEO @ 35,786 km altitude |
| Frequency band | Sub 6GHz | Above 6GHz | Sub 6GHz | Above 6GHz | Sub 6 GHz | Above 6 GHz |
| Beams generation | Earth fixed beams (Note 1) | Earth fixed beams (Note 1) | Earth moving beams | Earth moving beams | Earth fixed beams | Earth fixed beams |

NOTE 1: Each satellite has the capability to steer beams towards fixed points on earth using beam-forming techniques. This is applicable for a period of time corresponding to the visibility time of the satellite

3.2 Key parameters

The key reference scenario parameters can be found in table 4.2-2 of this document. It corresponds to the table 4.2-2 of [2] in which the scenarios referring to the regenerative payload option have been removed.

Table 4.2-2: Reference scenario parameters

| Scenarios | GEO based non-terrestrial access network (Scenario A) | LEO based non-terrestrial access network (Scenario C) |
| --- | --- | --- |
| Orbit type | notional station keeping position fixed in terms of elevation/azimuth with respect to a given earth point | circular orbiting around the earth |
| Altitude | 35,786 km | 600 km 1,200 km |
| Spectrum (service link) | <6 GHz (e.g. 2 GHz) >6 GHz (e.g. DL 20 GHz, UL 30 GHz) |  |
| Max channel bandwidth capability (service link) | 30 MHz for band < 6 GHz 400 MHz for band > 6 GHz |  |
| Payload | Scenario A: Transparent (including radio frequency function only) | Scenario C: Transparent (including radio frequency function only) |
| Inter-Satellite link | No | No |
| Earth-fixed beams | Yes | Scenario C1: Yes (steerable beams), see note 1 Scenario C2: No (the beams move with the satellite) |
| Max beam foot print size (edge to edge) regardless of the elevation angle | 3500 km (Note 5) | 1000 km |
| Min Elevation angle for both sat-gateway and user equipment | 10° for service link and 10° for feeder link | 10° for service link and 10° for feeder link |
| Max distance between satellite and user equipment at min elevation angle | 40,581 km | 1,932 km (600 km altitude) 3,132 km (1,200 km altitude) |
| Max Round Trip Delay (propagation delay only) | Scenario A: 541.46 ms (service and feeder links) | Scenario C: (transparent payload: service and feeder links) 25.77 ms (600km) 41.77 ms (1200km) |
| Max differential delay within a cell (Note 6) | 10.3 ms | 3.12 ms and 3.18 ms for respectively 600km and 1200km |
| Max Doppler shift (earth fixed user equipment) | 0.93 ppm | 24 ppm (600km) 21 ppm(1200km) |
| Max Doppler shift variation (earth fixed user equipment) | 0.000 045 ppm/s | 0.27 ppm/s (600km) 0.13 ppm/s(1200km) |
| Maximum Delay variation as seen by the UE (note 7) | Negligible | Up to +/- 48 µs/sec (600 km) Up to +/- 42 µs/sec (1200 km) |
| Service link | 3GPP defined New Radio |  |
| Feeder link | 3GPP defined Radio interface | 3GPP defined Radio interface |
| NOTE 1: Each satellite has the capability to steer beams towards fixed points on earth using beamforming techniques. This is applicable for a period of time corresponding to the visibility time of the satellite NOTE 2: Max delay variation within a beam (earth fixed user equipment) is calculated based on Min Elevation angle for both gateway and user equipment NOTE 3: Max differential delay within a beam is calculated based on Max beam foot print diameter at nadir NOTE 4: Speed of light used for delay calculation is 299792458 m/s. NOTE 5: The Maximum beam foot print size for GEO is based on current state of the art GEO High Throughput systems, assuming either spot beams at the edge of coverage (low elevation). NOTE 6: The maximum differential delay at cell level has been computed considering the one at beam level for largest beam size. It does not preclude that cell may include more than one beam when beam size are small or medium size. However the cumulated differential delay of all beams within a cell will not exceed the maximum differential delay at cell level in the table above. NOTE 7: The delay variation measures how fast the round trip delay (function of UE-satellite-NTN gateway distance) varies over time when the satellite moves towards/away from the UE. It is expressed in µs/s and is negligible for GEO scenario |  |  |

The NTN study results apply to GEO scenarios as well as all NGSO scenarios with circular orbit at altitude greater than or equal to 600 km.

3.3 User Equipment

The User equipment considered for the key reference scenario parameters can be found in table 4.3-1 of this document.

Table 4.3-1 Reference satellite scenarios: User equipment types

| User equipment characteristics | Handheld | VSAT (Note 1) |
| --- | --- | --- |
| Antenna type | Omnidirectional antenna | Directive antenna |
| Motion on the earth | 500 km/h (e.g. on board a high speed train) | Up to 1200 km/h (e.g. aircraft mounted) |
| antenna types | Omnidirectional antenna | Directional antenna (up to 60 cm equivalent aperture diameter) |
| Antenna polarisation | Linear: +/-45°X-pol | circular |
| Max transmit power | up to 200 mW (power class 3) | up to 20 W |
| Note 1 : VSAT terminal characteristics could be implemented with phased array antenna. It may be mounted on Moving platforms (e.g., aircrafts, vessels) or building |  |  |

As per [1], UEs with GNSS capabilities are assumed and hence have the capability to determine their own position.

However, both UE with and without the capability for pre-compensation of timing and frequency offset shall be supported.

The reasons are that service availability between GNSS service and NR service may differ or the processing of these signals in the UE are decoupled.

UEs with capability on timing and frequency pre-compensation using their GNSS capabilities are assumed. However the support of UEs without capability on timing and frequency pre-compensation is not precluded in the subsequent release.

# Non-Terrestrial Network scenarios: Access layer assumptions  (RAN2)

  - PCI mapping to satellite beams

Recall of some definitions:

- Satellite beam: A beam generated by an antenna on-board a satellite.

- Satellite beam foot print refers to the radio coverage of an area on Earth

- Each SS Block (SSB) is associated to a specific NR beam

- One PCI (one NR cell) can have of up to 4, 8 or even 64 SSB- depending on the frequency band

Following options should be supported in NTN:

- Only one PCI per NTN cell

- One or several satellite beams per NTN cell

- One satellite beam consists of one SSB

- Each NTN cell can have up to 4, 8 or even 64 SSB depending on the frequency band

Figure 1 Satellite beams and SSB mappings to NTN cell

In non-terrestrial network, each cell is described with a specific PCI and can be realised with one or several satellite beams, each with a SSB.

  - PLMN

The following are supported

- PLMN(s) distinct between NTN and terrestrial based NG-RAN (Roaming)

- PLMN(s) same for both NTN and terrestrial based NG-RAN (RAN sharing)

- PLMNs sharing the same NTN

The same or a distinct PLMN may be used between Non-terrestrial network and terrestrial based NG-RAN. Several PLMNs may share the same non-terrestrial network.

  - Tracking areas

The following is assumed

- Tracking Areas (TAs) are fixed on Earth

- For Earth fixed beams: a set of cells is assigned to the TA

- For Earth moving beams: cells are successively assigned to the TA

- The UE makes cell (re) selection based on similar procedures as for terrestrial NG-RAN. Based on the system information of the selected cell, the UE determines the Tracking Area it is located in..

Tracking Areas (TAs) are fixed on Earth and each entails a set of cells assigned on a permanent or temporary basis. Cell (re)selection procedures defined for NG-RAN applies.

  - Mobility procedures

The following are supported

- Intra-satellite handover:

  - Satellite beam mobility: beam level mobility at physical layer within the same cell with no explicit RRC signaling to be triggered

  - Cell handover: Inter-gNB handover procedure or Intra-gNB handover procedure (See §9.2.3 in TR 38.300)

- Inter satellite hand-over:

  - Inter-gNB handover via Xn/NG interface

  - Intra-gNB handover procedure (See §9.2.3 in TR 38.300)

Intra (inter beam or cell) and inter satellite hand-over should be based on existing mobility procedures defined for NG-RAN.

  - TN / NTN mobility

The following is assumed

- The UE is assumed to have TN and NTN access capabilities not necessarily simultaneously.

- The UE may use different antenna types for TN and NTN (e.g. directional antenna for NTN)

- TN access may be configured by the operators as preferred access (to be selected whenever available)

- TN to NTN mobility (hand-out): to be triggered at least when TN is no longer available

- NTN to TN mobility (hand-in): to be triggered when UE moves into an area with available TN coverage

For TN / NTN mobility, the UE is assumed to have TN and NTN access capabilities not necessarily simultaneously. It may use different antenna types for TN and NTN (e.g. directional antenna for NTN)

For TN / NTN mobility, TN access may be configured by the operators as preferred access (to be selected whenever available). TN to NTN mobility (hand-out) can be triggered at least when TN is no longer available. NTN to TN mobility (hand-in) can be triggered when UE moves into an area with available TN coverage.

  - Feeder link

A feeder link switchover (e.g. for Non GSO) will occur when the satellite moves out from coverage from the source NTN GW and establishes contact with the target NTN GW. The NTN-GW may be connected to the same gNB or different gNB.

For the normative work, the following are assumed

- Any satellite will always be connected to at least one NTN GW.

- A given satellite can

  - either be connected to at least one NTN GW during a feeder link switch over (hard switch).

  - or be connected to at least two NTN GW during a feeder link switch over to ensure seamless service continuity (soft switch).

- The feeder link switchover may use inter gNB mobility procedure via Xn/NG interface to exchange NTN specific information via dedicated Xn/NG messages between the respective gNB of the source and target NTN GW.

- The Doppler shift experienced on the feeder links are pre compensated by the NTN GW.

For feeder link switchover (e.g. for Non GSO), satellites may be connected to at least one NTN GW (hard switch) or at least two NTN GWs (soft switch). The Doppler shift on the feeder links are pre compensated by the NTN GW.

  - Network based positioning of UE

The following is assumed

- The NTN based positioning of UE should provide an accuracy comparable with terrestrial networks (typical Cell size).

- Location Services (LCS) framework/application protocols from Rel.16 is the basis for the NTN to locate the UE.

The NTN based positioning of UE shall should provide an accuracy comparable with terrestrial networks (typical Cell size). Location Services (LCS) framework/application protocols from Rel.16 is the basis for the NTN to locate the UE.

# UE position determination by NTN NG-RAN

This clause proposes a method of work in order to address the objective of “Identify potential issues associated to the use of the existing Location Services (LCS) application protocols to locate UE in the context of NTN and specify adaptations if any [RAN2/3]” in the “NR-NTN-Solutions” work item approved in RAN#86 and revised in RAN#88-e in RP-201256 [1]

  - Context

3GPP TS 22.071 [3] specifies that 3GPP standards shall support location service features, to allow new and innovative location-based services to be developed. It shall be possible to identify and report in a standard format (e.g. geographical co-ordinates) the current location of the user’s terminal and to make the information available to the user, the mobile entity, the network operator, the service provider, value added service providers and for PLMN internal operations.

The location is provided to identify the likely location of specific UEs. This is meant to be used for charging, location-based services, lawful interception, emergency calls, etc., as well as the positioning services.

The standard supports NG-RAN, E-UTRAN, GERAN and UTRAN to facilitate determination of the location of a mobile station.

Among the High Level Requirements, the following one can be highlighted:

- Multiple positioning methods should be supported in the different Access Networks, including (but not limited to): Modernized GPS; SBAS (Satellite Based Augmentation Systems: EGNOS, WAAS, GAGAN, MSAS); QZSS (Quasi Zenith Satellite System); GLONASS, BDS, Galileo, U-TDOA, E-OTD, IPDL-OTDOA, Network Assisted GNSS (e.g. Network Assisted GPS or Network Assisted GALILEO), Methods using cell site or sector information and Timing Advance TA or Round Trip Time RTT measurements.

The details of the location information generated by the LCS can be found in 3GPP TS 29.572 [6].

The required accuracy is related to the nature of the services, ranging from few tens of meters for asset location, routing and guidance, to several tens or hundreds of meters for emergency calls, or several kilometers or beyond for weather reports, service restriction.

The location may be:

- “Mobile Originated”: in this case the mobile station has the capability to obtain its own geographical location or have its own geographic location transferred to another LCS client, or

- “Network Supported”: in this case the provision of location services shall be possible without significantly adversely impacting the radio transmission or the signaling capabilities of the network.

In 3GPP RP-201256 [3] as approved during TSG RAN #88-E in June 2020, it is assumed that NTN UE’s would be GNSS based, i.e. with the assumed capability to determine their own geographical location thanks to the access to a local GNSS receiver attached to the UE.

Further, it is specified in 3GPP TS 33.126 [4] that the location information reported to the LEMF (Law Enforcement Monitoring Facility) shall be location information trusted by the 3GPP network (i.e. the location information is either 3GPP network derived or verified), if available. The CSP (Communication Service Provider) shall also be able to report target location information from untrusted sources (e.g. user equipment provided) in addition to or in absence of the trusted location information.

It is also important for CSP operating a satellite network to be provided with the accurate position of the UE to implement the exact corresponding tax regime.

Whilst the baseline assumption is for NTN UE to have the capability to navigate, it is essential that NTN networks have also the capabilities to locate UE to answer to some key requirements associated with the LCS functionality as specified in 3GPP TS in 3GPP TS 23.273 [5].

  - Discussion

The following approach is proposed to address the UE network supported location determination in NTN NG-RAN, with the aim at minimizing the impacts on the existing NG-RAN specifications:

- Step 1: Review of the applicability to NTN of the existing network-based location methods

Terrestrial networks and non-terrestrial networks differ by their relative speed, and thus variation of distance and delay as well as Doppler, between the target UE and the infrastructure. They also potentially differ by the actual number of available transmitters and receivers at the infrastructures to perform the measurement.

Methods such as Multiple RTTs, AOA, UL-TDOA, FDOA, etc. will be assessed.

The format of the geographic information to be provided will be identified (latitude, longitude, altitude, Cartesian coordinates, etc.), the precision will addressed against satellite constellation scenarios and associated radio frequency assumptions (number of representative configurations to be addressed according to driving parameters).

If need be modified existing network based location methods or new network based location methods will be addressed.

- Step 2: Assessment of the LCS framework [5] to [9], in particular but not excluding other TS] and its applicability to NTN will be performed, reviewing the applicability of existing protocols in parallel with the evaluation of methods as proposed in Step 1.

- Step 3: Following Step 1 & 2, down-selection of a method to be specified for locating UE by an NTN NG-RAN.

# Conclusion

Proposal 1: Six transparent payload based satellite reference scenarios are considered for the Rel-17 work item “NR_NTN_solutions” characterised in the table below:

Table 2-1 Reference satellite scenarios for Rel-17 work item “NR_NTN_solutions”

| Scenarios | C1.1 | C1.2 | C2.1 | C2.2 | A1 | A2 |
| --- | --- | --- | --- | --- | --- | --- |
| Orbit | LEO @ 600 km altitude | LEO @ 600 km altitude | LEO @ 1200 km altitude | LEO @ 1200 km altitude | GEO @ 35,786 km altitude | GEO @ 35,786 km altitude |
| Frequency band | Sub 6GHz | Above 6GHz | Sub 6GHz | Above 6GHz | Sub 6 GHz | Above 6 GHz |
| Beams generation | Earth fixed beams (Note 1) | Earth fixed beams (Note 1) | Earth moving beams | Earth moving beams | Earth fixed beams | Earth fixed beams |

NOTE 1: Each satellite has the capability to steer beams towards fixed points on earth using beam-forming techniques. This is applicable for a period of time corresponding to the visibility time of the satellite

Proposal 2: The key reference scenario parameters can be found in table 4.2-2 of this document. It corresponds to the table 4.2-2 of [2] in which the scenarios referring to the regenerative payload option have been removed.

Proposal 3: The User equipment considered for the key reference scenario parameters can be found in table 4.3-1 of this document.

Proposal 4: UEs with capability on timing and frequency pre-compensation using their GNSS capabilities are assumed. However the support of UEs without capability on timing and frequency pre-compensation is not precluded in the subsequent release.

Proposal 5: In non-terrestrial network, each cell is described with a specific PCI and can be realised with one or several satellite beams, each with a SSB.

Proposal 6: The same or a distinct PLMN may be used between Non-terrestrial network and terrestrial based NG-RAN. Several PLMNs may share the same non-terrestrial network.

Proposal 7: Tracking Areas (TAs) are fixed on Earth and each entails a set of cells assigned on a permanent or temporary basis. Cell (re)selection procedures defined for NG-RAN applies.

Proposal 8: Intra (inter beam or cell) and inter satellite hand-over should be based on existing mobility procedures defined for NG-RAN.

Proposal 9: For TN / NTN mobility, the UE is assumed to have TN and NTN access capabilities not necessarily simultaneously. It may use different antenna types for TN and NTN (e.g. directional antenna for NTN)

Proposal 10: For TN / NTN mobility, TN access may be configured by the operators as preferred access (to be selected whenever available). TN to NTN mobility (hand-out) can be triggered at least when TN is no longer available. NTN to TN mobility (hand-in) can be triggered when UE moves into an area with available TN coverage.

Proposal 11: For feeder link switchover (e.g. for Non GSO), satellites may be connected to at least one NTN GW (hard switch) or at least two NTN GWs (soft switch). The Doppler shift on the feeder links are pre compensated by the NTN GW.

Proposal 12: The NTN based positioning of UE shall should provide an accuracy comparable with terrestrial networks (typical Cell size). Location Services (LCS) framework/application protocols from Rel.16 is the basis for the NTN to locate the UE.

Proposal 13	To meet the objective of identifying potential issues associated to the use of the existing Location Services (LCS) application protocols to locate UE in the context of NTN and specify adaptations if any [RAN2/3] as part of the “NR_NTN_solutions » work item in RP-201256 [1], the following stepped approach is proposed:

- Step 1: Review of the applicability to NTN of the existing network-based location methods, adapt these methods or propose new ones if need be, and evaluate these methods.

- Step 2: Assessment the LCS framework ([5] to [9], in particular but not excluding other TS) and its applicability to NTN

- Step 3: Following Step 1 & 2, down-selection of a method to be specified for locating UE by an NTN NG-RAN.

# References

3GPP RP-201256 Work Item "Solutions for NR to support non-terrestrial networks (NTN)"

3GPP TR 38.821 “Solutions for NR to support non-terrestrial networks (NTN) (Release 16)”

3GPP TS 22.071 « Location Services (LCS) ; Service Description ; Stage 1 ; (Release 16) »

3GPP TS 33.126, ”Lawful Interception requirements, (Release 16)”

3GPP TS 23.273, “5G System (5GS) Location Services (LCS); Stage 2 (Release 16)”

3GPP TS 29.572, “Location Management Services; Stage 3 (Release 16)”

3GPP TS 38.455, “NR Positioning Protocol A (NRPPa) (Release 15)”

3GPP TS 38.305, “Functional specification of User Equipment (UE) positioning in NG-RAN (Release 16)”

3GPP TS 38.413, “NG Application Protocol (NGAP) (Release 16)”

END