3GPP TSG-RAN WG2 Meeting #111-e	R2-2007856

Electronic, 17th – 28th Aug, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 1934 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on optionality of ueCapabilityRequestFilterSidelink |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Core |  |  |  |  |  | Date: |  |  | 2020-08-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In the current specification, ueCapabilityRequestFilterSidelink is optionally present in the UECapabilityEnquirySidelink message. As agreed by R2, ue-CapabilityInformationSidelink can be optionally send together with ueCapabilityRequestFilterSidelink, with the latter one to be mandatory present in UECapabilityEnquirySidelink message. Similar to the Uu procedure, we should clarify that capability enquiry should always include ueCapabilityRequestFilterSidelink. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In 5.8.9.2, add NOTE “NOTE X: Capability enquiry without ueCapabilityRequestFilterSidelink is not supported.” Impact analysis Impacted functionality: SL UE capability Inter-operability: - If the UE is implemented according to this CR but the network is not, there is no inter-operability issue foreseen, because the CR does not impact the Uu signalling. - If the network is implemented according to this CR but the UE is not, there is no inter-operability issue foreseen, because the CR does not impact the Uu signalling. - If one UE is implemented according to this CR but the other UE is not, the transmitting and receiving UEs may have different expectation on the presence of ueCapabilityRequestFilterSidelink in UECapabilityEnquirySidelink message. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UECapabilityEnquirySidelink message without ueCapabilityRequestFilterSidelink is not the intended behaviour. The UE behaviours is not clear upon receiving UECapabilityEnquirySidelink message without ueCapabilityRequestFilterSidelink. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.8.9.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

--------------------- [Start of change] ----------------------------------

#### 5.8.9.2	Sidelink UE capability transfer

#### 5.8.9.2.1	General

This clause describes how the UE compiles and transfers its sidelink UE capability information for unicast to the initiating UE.

Figure 5.8.9.2.1-1: Sidelink UE capability transfer

#### 5.8.9.2.2	Initiation

The UE may initiate the sidelink UE capability transfer procedure upon indication from upper layer when it needs (additional) UE radio access capability information.

#### 5.8.9.2.3	Actions related to transmission of the UECapabilityEnquirySidelink by the UE

The initiating UE shall set the contents of UECapabilityEnquirySidelink message as follows:

1>	include in UE radio access capabilities for sidelink within ueCapabilityInformationSidelink;

NOTE:	It is up to initiating UE to decide whether ueCapabilityInformationSidelink should be included.

1>	set frequencyBandListFilterSidelink to include frequency bands for which the peer UE is requested to provide supported bands and band combinations;

NOTE X:	Capability enquiry without ueCapabilityRequestFilterSidelink is not supported.

1>	submit the UECapabilityEnquirySidelink message to lower layers for transmission.

#### 5.8.9.2.4	Actions related to reception of the UECapabilityEnquirySidelink by the UE

The peer UE shall set the contents of UECapabilityInformationSidelink message as follows:

1>	include in UE radio access capabilities for sidelink within ueCapabilityInformationSidelink;

1>	compile a list of "candidate band combinations" only consisting of bands included in frequencyBandListFilter, and prioritized in the order of frequencyBandListFilterSidelink (i.e. first include band combinations containing the first-listed band, then include remaining band combinations containing the second-listed band, and so on).

1>	Include into supportedBandCombinationListSidelink as many band combinations as possible from the list of "candidate band combinations", starting from the first entry;

1>	submit the UECapabilityInformationSidelink message to lower layers for transmission.

--------------------------[End of change] ------------------------------