3GPP TSG-RAN WG2 #111-e	R2-2007889

E-meeting, Aug 17 – Aug 28, 2020

Agenda Item	: 8.10.2.2 (NR_NTN_solutions-Core)

Source	: LG Electronics Inc.

Title	: Discussion on RLC/PDCP aspects for NTN

Document for	: Discussion and Decision

Introduction

In NTN WID, the following objectives are captured.

- RLC - Status reporting: Extension of the value range of t-Reassembly - Sequence Numbers: extension of the SN space only for GEO scenarios - PDCP - SDU discard: Extension of the value range of discardTimer. - Sequence Numbers: extension of the SN space for GEO scenarios.

This document shows our view on each above objective.

Discussion

Extension of the value range for timers in PDCP and RLC

Generally, the timers in PDCP and RLC, i.e., t-Reordering, discardTimer and t-Reassembly, are configured based on the QoS requirements. With this reason, a new timer value is defined only when there is a new QoS requirement. Note that, in Rel-16 IIOT WID, a new timer value for discardTimer, e.g., 0.5ms, is defined based on the QoS requirements for IIOT.

However, a new QoS requirement for NTN is not defined, and it seems that the legacy service, i.e., eMBB, may be used for the NTN services. In other words, without a new QoS requirement for NTN, there is no reason to extend the timer values. Thus, we propose that the extension of the value for timers in PDCP and RLC could be considered if a new QoS requirement for NTN is defined.

Proposal 1. The extension of the value for timers in PDCP and RLC could be considered if a new QoS requirement for NTN is defined.

Extension of the SN space for GEO scenarios

In TR 22.261, the use scenarios for NTN are defined, and the maximum throughput requirements for NTN is 360 Mbps for the downlink and 180 Mbps for the uplink as follows.

Table 7.4.2-1: Performance requirements for satellite access Scenario; Experienced data rate (DL); Experienced data rate (UL); Area traffic capacity (DL) (note 1); Area traffic capacity (UL) (note 1); Overall user density; Activity factor; UE speed; UE type. Pedestrian (note 2); [1] Mbit/s; [100] kbit/s; 1,5 Mbit/s/km2; 150 kbit/s/km2; [100]/km2; [1,5]%; Pedestrian; Handheld. Public safety; [3,5] Mbit/ss; [3,5] Mbit/s; TBD; TBD; TBD; N/A; 100 km/h; Handheld. Vehicular connectivity (note 3); 50 Mbit/s; 25 Mbit/s; TBD; TBD; TBD; 50%; Up to 250 km/h; Vehicle mounted. Airplanes connectivity (note 4); 360 Mbit/s/ plane; 180 Mbit/s/ plane; TBD; TBD; TBD; N/A; Up to 1000 km/h; Airplane mounted. Stationary; 50 Mbit/s; 25 Mbit/s; TBD; TBD; TBD; N/A; Stationary; Building mounted. Narrowband IoT connectivity; [2] kbps; [10] kbps; 8 kbps/km2; 40 kbps/km2; [400]/km2; [1]%; [Up to 100 km/h]; IoT. Note 1: Area capacity is averaged over a satellite beam. Note 2: Data rates based on Extreme long-range coverage target values in clause 6.17.2. User density based on rural area in Table 7.1-1. Note 3: Based on Table 7.1-1 Note 4: Based on an assumption of 120 users per plane 15/7.5 Mbps data rate and 20% activity factor per user Note 5: All the values in this table are targeted values and not strict requirements. Note 6: Performance requirements for all the values in this table should be analyzed independently for each scenario..

In TS 38.323 and TS 38.322, the maximum PDCP and RLC SN is 18 bits. This value could accommodate 2.9 Gbps (=1500*8*2(18-1)/0.541) data rate under the assumption that the maximum propagation delay is 541 ms. Considering that the maximum throughput requirements for NTN is 360 Mbps for the downlink, current lengths of PDCP SN and RLC SN are sufficient for NTN.

Proposal 2. Current lengths of PDCP SN and RLC SN are sufficient for NTN.

Conclusion

In this contribution, we show our view on each objective related with PDCP and RLC for NTN, and the following proposals are made.

Proposal 1. The extension of the value for timers in PDCP and RLC could be considered if a new QoS requirement for NTN is defined.

Proposal 2. Current lengths of PDCP SN and RLC SN are sufficient for NTN.

.