3GPP TSG-RAN WG2 Meeting #111 electronic				              R2-2008398

Online, August 17th - 28th, 2020                              Revision of R2-2007707

Source: 			ZTE Corporation, Sanechips

Title: 	Discussion on the compliance check failure for CPC configuration after PCell change

Agenda item:		6.7.3

Document for:		Discussion and Decision

- Introduction

At last meeting, we discussed how to handle with the stored CPC configuration in case of PCell change, and agreed “UE autonomously deletes CPC configurations upon PCell change in Rel-16” [1]. In this contribution, we discussed the possible compliance check failure for CPC configuration after PCell change.

- Discussion

According to the current RRC spec[2], the UE shall release all stored CPC configurations once the conventional PCell change procedure is executed successfully. However, for PCell change without SN involved (e.g. intra-MN PCell change without PSCell change), since the SN is not aware of the PCell change procedure, the SN has no idea whether the stored CPC configuration has been released or not on the UE side. So there may exist a mismatching for the CPC configuration between the SN side and the UE side after PCell change.

- Observation 1: According to the current RRC spec, all stored CPC configuration will be released upon PCell change. For the PCell change without SN involved, since the SN is not aware of the PCell change, the SN has no idea whether the stored CPC configuration has been released or not on the UE side. 
After PCell change without SN involved, the SN may want to modify the CPC configuration. For example, the SN may send the RRCReconfiguration message including an updated condExecutionCond but without condRRCReconfig or an updated condRRCReconfig without condExecutionCond to the UE. Since the UE has released all stored CPC configuration after PCell change (i.e. no entry within the VarConditionalReconfig), the UE may consider the received CPC configuration as a new entry within the VarConditionalReconfig. In such case, the UE may consider the received CPC configuration as erroneous since we have specified that both condExecutionCond and condRRCReconfig should be mandatory present when a new entry is being added. Thus, the UE shall declare the compliance check failure for the received message, which shall trigger the undesired SCG failure information procedure (if the message is received over SRB3) or the RRC re-establishment procedure (if the message is received over SRB1).

- Observation 2: After PCell change without SN involved, if the SN wants to modify the previous CPC configuration, a reconfiguration failure may happen since the UE may consider the received CPC configuration as a new one but the condExecutionCond and the condRRCReconfig may not be carried simultaneously, which shall cause the undesired recovery procedure (e.g. SCG failure information procedure or the RRC re-establishment procedure).

- In order to avoid triggering the unnecessary recovery procedure, we propose that the UE does not consider the message as erroneous if condRRCReconfig and condExecutionCond are not simultaneously included in the conditionalReconfiguration for CPC. And the UE ignores the received CPC configuration. The companion CR can be found in [3] and [4].

- Proposal 1: The UE does not consider the message as erroneous if condRRCReconfig and condExecutionCond are not simultaneously included in the conditionalReconfiguration for CPC. And the UE ignores the received CPC configuration.

- Conclusion and proposals

- In this contribution, we discuss the handling of compliance check failure for CPC configuration after PCell change with the following observations and proposal:

Observation 1: According to the current RRC spec, all stored CPC configuration will be released upon PCell change. For the PCell change without SN involved, since the SN is not aware of the PCell change, the SN has no idea whether the stored CPC configuration has been released or not on the UE side.

- Observation 2: After PCell change without SN involved, if the SN wants to modify the previous CPC configuration, a reconfiguration failure may happen since the UE may consider the received CPC configuration as a new one but the condExecutionCond and the condRRCReconfig may not be carried simultaneously, which shall cause the undesired recovery procedure (e.g. SCG failure information procedure or the RRC re-establishment procedure).

- Proposal 1: The UE does not consider the message as erroneous if condRRCReconfig and condExecutionCond are not simultaneously included in the conditionalReconfiguration for CPC. And the UE ignores the received CPC configuration.

- Reference

- RAN2#110e chairman notes

- TS 38.331, v 16.1.0

- R2-2008399, 38.331 CR - Compliance check failure for CPC configuration, ZTE Corporation, Sanechips

- R2-2008400, 36.331 CR - Compliance check failure for CPC configuration, ZTE Corporation, Sanechips