3GPP TSG-RAN WG2 #111-e	R2-20xxxxx

E-meeting, August 2020

Agenda Item:	8.7.4

Source:	OPPO (rapporteur)

Title:	Discovery model and procedure

Document for:	Discussion, Decision

# Introduction

The draft document intends to proceed with following offline discussion by early email:

- [AT111-e][606][Relay] Discovery model and procedure (OPPO)

Scope: Discuss proposals on the discovery model and procedures, including:

- Protocol stacks for discovery

- Potential reuse of discovery models from LTE

- Resource pool for discovery

- Visibility of discovery signalling in AS layers

- Conditions for discovery

- Authorisation related aspects

Intended outcome: Summary with potential agreeable TP

Deadline:  Wednesday 2020-08-26 1200 UTC

The technical issues of this draft paper are assumed common for both L2 and L3. They are same between U2N relay (UE to network relay) and U2U relay (UE to UE relay) also apart from section 2.5 which is only applied for U2N relay discovery. If you have different opinion from this assumption, please indicate in the detail answers to the questions.

# Discussion

# Potential reuse of discovery models from LTE

To discuss any RAN impact by discovery procedure and relevant announcement/solicitation/discovery message (called discovery message in general in the rest of the paper), we need understanding at first what kind of discovery model will be adopted in this SI.

For U2N relay (UE to network relay) contribution [8][9][11][14][15][17], propose to adopt discovery model from LTE as start point i.e. so called model A/B. Contribution [16] however proposes PC5 discovery/link establishment procedure standardized in Rel-16. The statistics in [8] also show model A or mode B discovery model is preferred in SA2 by majority companies.

Question1: Do you agree that model A/ B discovery model from LTE is reused for U2N relay?

| Company | Yes/No | Other comment |
| --- | --- | --- |
| Ericsson (Min) | No | It is sufficient to reuse NR Rel-16 link establishment procedure. The link establishment messages can be transmitted in a broadcast/groupcast manner. This will minimize RAN2 efforts. |
| Qualcomm | Yes | We think both mode A and mode B are captured in SA2 TR 23.752. Then we see no reason if RAN2 don’t follow it. |
| OPPO | Yes |  |
| Lenovo&MM | Yes | Which model (A or B) is selected for discovery purpose can be left for upper layer. It will not impact AS layer. |
| Futurewei | Yes | Models A and B kind of operations can be supported, so that relay node discovery is a separate function from PC5 link establishment. |
| Interdigital | Yes | Model A and Model B discovery for UE to NW relay is aligned with SA2 discussion. |

It is not very clear from [9][11][14][15] whether model A/B discovery model is also applicable for U2U relay. Contribution [8] and [16] propose V2X like scheme i.e. to rely on link establishment procedure while [17] explicitly propose same discovery model i.e. model A/B should be adopted for U2U relay also. As indicated in [8] the a single handshake can’t guarantee remote UE can select a right relay to reach remote UE hence model A/B is actually rather complicated scheme than R16 link establishment procedure. Since it is not very clear what is majority view in RAN2 it seems worthwhile to confirm company’s position explicitly.

Question2: which discovery model do you prefer for U2U relay?

Option1: same as U2N relay i.e. model A/B

Option2: R16 link establishment procedure i.e. exchange of Direct Communication Request/Response message

| Company | Options | Other comment |
| --- | --- | --- |
| Ericsson(Min) | Option 2 | RAN2 needs to aim for a common solution for both U2U and U2N. in this case, NR Rel-16 link establishment would be a best option from minimizing RAN2 efforts perpective. |
| Qualcomm | Option 1 | We don’t understand why different discovery models are needed for UE-to-NW relay and UE-to-UE relay. Isn’t violated note2 of SID from beginning? “NOTE 2: It is assumed that UE-to-network relay and UE-to-UE relay use the same relaying solution.” Our understanding is that Note2 means RAN2 should try to have same solution for U2U and U2W design unless some solution can’t work for U2U. However, we fail to see why modelA / modeB can’t work for U2U relay. Meanwhile, R16 link establishment procedure has PC5-S impact, which is out of scoping of RAN2. Then we are wondering why RAN2 needs to study it. |
| OPPO | Option2 | It is desirable that solution for U2U relay can align with U2N as much as possible if necessary, but it doesn’t mean they should be always the same. For discovery model we think both options are feasible but option2 is simpler from our perspective. From modelling point of view it also means no discovery phase is necessary because the handshake is also link establishment procedure. |
| Lenovo&MM | Option1 | As SID mentioned, we need to pursue the common solution for both UE-to-Network relay and UE-to-UE relay. |
| Futurewei | Option 1 | It is desirable, as noted in the SID, that same functionalities can be shared by both UE-to-Network and UE-to-UE relay. We don’t see much benefit to deviate this principle for relay node discovery. |
| Interdigital | Option 1 | From SID perspective, option 1 is preferable. There seems no need to have a different procedure for a remote UE connecting to a relay (whether UE to NW or UE to UE relay). |

As indicated in [5] eventually it should be up SA2 to make final decision which discovery model is adopted for either U2N relay or U2U relay. So regardless which option RAN2 assume for further study RAN2 should check SA2 opinion for final decision, hence a LS to SA2 for confirmation seems necessary.

Question3: Do you agree RAN2 send a LS to SA2 for confirmation of RAN2’s assumption on discovery model?

| Company | Yes/No | Other comment |
| --- | --- | --- |
| Ericsson(Min) | Yes with comment | We should emphasize in the LS that both Option 1 and Option 2 are feasible from a RAN2 point of view and that is up to SA2 to make the final decision. |
| Qualcomm | Yes but | We are still not sure why a U2U discovery option still being discussed in SA2 (i.e. option 2) needs to be discussed in RAN2. |
| OPPO | Yes | RAN2 can proceed our study based on assumption we made while waiting for SA2’s confirmation. |
| Lenovo&MM | Yes | The content of discovery message for the U2N relay and U2U relay is different if discovery model is adopted for both U2N relay and U2U relay. So, it is better to inform SA2. |
| Futurewei | Yes | It’d be worth also citing SID’s note – “NOTE 2: It is assumed that UE-to-network relay and UE-to-UE relay use the same relaying solution”. |
| Interdigital | Yes | SA2 is currently studying both options, and so they should be aware of RAN2 preference. |

# Protocol stacks for discovery

In [1] SID puts it explicitly that “Study mechanism(s) to support upper layer operations of discovery model/procedure for sidelink relaying, assuming no new physical layer channel / signal [RAN2]”. From relevant contributions [3]~[8],[11]~[13],[15] and [18] no company propose to introduce new physical channel like LTE does to transmit discovery message in NR relay operation. So it could be easily conclude that physical layer channel of sidelink communication can be reused for transmission of discovery message. Then from SLRB point of view, it is possible either discovery message is carried on SL SRB or SL DRB. Contribution [4][6][8][13] propose to take discovery message similar to PC5-S signalling and hence SL SRB is preferred while contribution [5] proposes to rely on user plane protocol stack i.e. SL DRB is preferred. One issue raised by [5] is that no size limitation is necessary like LTE does because no new physical channel is introduced. But considering existing SL SRB carrying PC5-S also support RLC UM mode for both broadcast and unicast, it seems size limitation is not a key issue to differentiate between SL DRB and SL SRB.

Question 4: which protocol stack do you prefer to transmit discovery message?

Option1: control plane protocol stack (PC5-S/PDCP/RLC/MAC/PHY) i.e. discovery message is taken as PC5-S signalling and carried on SL SRB

Option2: user plane protocol stack (data flow/SDAP/PDCP/RLC/MAC/PHY) i.e. discovery message is taken as a upper layer data flow

Option3: others (please elaborate detail protocol stack and why)

| Company | Options | Other comment |
| --- | --- | --- |
| Ericsson(Min) | Option 1 | The existing NR Rel-16 link establishment messages are already supported. It is sufficient to introduce a small extension of the message for relay discovery/link establishment. |
| Qualcomm | Slightly prefer Option 1, but with comments | Both Option 1 and Option 2 are fine with us. We slightly prefer Option1 because it requires smaller spec impacts: - Protocol stack can reuse the one specified for SL SRB0 in NR rel-16 V2X - Discovery message should not be multiplexed with other communication MAC PDU. Then, if we agree option 2, some restriction is needed to be specified on MAC PDU multiplexing However, we think it may be rushed to say it is a SL SRB. It is clearly different from existing DRB or any SRBs over PC5. And in LTE, it is carried in a special RB. So, we think it needs to further discuss whether it is SL SRB or a new RB. |
| OPPO | Option1 | Discovery message is control message in upper layer from AS layer point of view. So following existing model i.e. to carry it over SL SRB is natural choice. We don’t think message size is an issue because SL SRB can already support either RLC UM or AM mode. As for multiplexing in MAC layer we don’t see much difference between SL SRB and SL DRB since this is very first message/data to transmit or receive ahead of any other signalling or data between remote UE and relay UE i.e. there is no chance to multiplex with any other signalling or data towards same destination. |
| Lenovo&MM | Option 1 | Discovery message as PC5-S is generated by upper layer. So, it is natural to reuse the protocol stack for PC5-S. |
| Futurewei | Option 1 | It leverages Rel-16 works better. |

# Visibility of discovery signalling in AS layers

The content of the discovery message will be defined in SA2 i.e. it is transparent from AS layer, but still it could be helpful to differentiate them from other SLRB. As contribution [3][7][15] point out if AS layer want to treat discovery message differently in terms of either SL prioritization among logical channels or SL/UL prioritization then discovery message should be visible in AS layer. From RX UE point of view, if it can’t filter out discovery message properly it will receive and decode them blindly [4][7][11]. Another potential issue is that UE may need measure discovery message for relay (re)selection and hence UE need some way to find out discovery message to do proper measurement [15].

There are few solutions to differentiate discovery message in AS layer. Contribution [3] proposes one alternative is to add one new SDU type in PDCP layer. Another proposal from same [3] is to introduce new LCID in MAC layer. More contributions propose to have discovery specific source and/or destination identity [7][11]. Contribution [4] propose to add service code at MAC header for filtering purpose. Or physical layer solution is proposed in [7] and [18]. So it looks like at this stage it is difficult to converge company’s view. But it seems also either MAC layer and/or physical layer are preferred by majority companies.

Question5: Do you agree that solution is needed to differentiate discovery message in AS layer from existing SL signalling or traffic?

| Company | Yes/No | Other comment |
| --- | --- | --- |
| Ericsson(Min) | No | If using NR Rel-16 link establishment procedure, the differentiation is already in place. Therefore, no additional spec efforts are needed for this. |
| Qualcomm | Yes | We can see at least 3 benefits: - It is helpful to separate the radio resources management for discovery and other communication traffic - RX UE power saving because it can perform filtering out interested discovery message in lower layer - During Relay (re)selection, remote UE needs to perform relay RSRP measurement which only includes discovery message. |
| OPPO | Yes | From TX UE point of view: And it may impact UL/SL prioritization if different priority deems necessary. In case resource pool for discovery is mixed then LCP procedure could be also impacted From RX UE point of view: To filter out discovery message and measure only discovery message seems beneficial for RX UE. |
| Lenovo&MM | Yes | UE needs to differentiate the discovery message from other data. how to differentiate the discovery needs to consider the power consumption and signalling overhead. |
| Futurewei | Leave it to SA2 | It is up to SA2 if discovery messages should be handled differently (have higher priority) than other PC5-S messages. We don’t need to decide on AS differentiation until more details on discovery messages emerge, e.g., if discovery message would have specific source and/or destination identities. |
| Interdigital | Yes | In legacy LTE, relay selection was based on SL RSRP of the discovery message. We think this principle should be adopted, at least as a baseline, and therefore there is some need for the AS layer to be able to distinguish discovery message. |

Question6: In case answer is “yes” to Q5, which protocol layer need introduce corresponding solution?

Option1: PDCP layer

Option2: MAC layer

Option3: PHY layer

Option4: Separate resource pool for discovery message

| Company | Options | Other comment |
| --- | --- | --- |
| Qualcomm | Option 1 or Option 2. Prefer Option 2 | We show some preference on Option 2 (a new LCID) because it might help to separate the radio resources management for discovery and other communication traffic Option 3 is infeasible in this release because it has RAN1 impacts. |
| OPPO | Option2 and option3 with comment | Assuming resource pool for discovery message is mixed with other sidelink message or traffic then we need solution in both MAC and PHY layer to treat it differently. If discovery is transmit/received in separate resource pool, then resource pool itself implicitly achieve same purpose. |
| Lenovo&MM | Option4 | If separate resource pool for discovery message is agreed, the resource pool can be used to differentiate it. And, all PHY, MAC and PDCP will not impact. |
| Futurewei | Postpone | As the relevant objective in SID is “Study mechanism(s) to support upper layer operations of discovery model/procedure for sidelink relaying”, we are hesitant to discuss this level of AS details before high level agreement/understanding is reached (in SA2) on discovery model and messages. |
| Interdigital | Option 2 and 3 | Option 2 and 3 are needed for measuring RSRP of discovery transmissions. |

# Resource pool for discovery

One relevant issue is whether a separate resource pool from communication is necessary. In case resource pool is not mixed then the prioritization between discovery and communication is naturally resolved. Contribution [3] point out other benefits like discovery could have different power control scheme and allow unlicensed bands and licensed bands (i.e. “operator managed” and “non-operator managed”. Another issue is that in model A it is likely that discovery message is transmitted periodically and the period of radio resource is most likely different from sidelink communication [8]. Sharing resource pool between discovery and communication may help to improve spectrum efficiency [7] but it also brings more technical issues. Contribution [3][4][6][8][19] support to have separate resource pool. Contribution [12][17] ask RAN2 to discuss this issue. Contribution [4] point out both TX and RX resource pool should be separated.

Question7: Do you agree that a separate resource pool is necessary for discovery message from communication?

| Company | Yes/No | Other comment |
| --- | --- | --- |
| Ericsson(Min) | No | For reusing NR Rel-16 link establishment procedure perspective, a common resource pool is enough. Since link establishment and communication take place in different time. |
| Qualcomm | Yes | For the concern on spectrum efficiency, we think discovery message can be sent with a long periodicity to reduce utilization of spectrum. |
| OPPO | Yes | We share QC’s view |
| Lenovo&MM | Yes | It is beneficial for power saving. As usual, UE only needs to monitor the discovery message based on the event, such as relay reselection, rather than ‘always’. If the common resource pool for discovery and communication is configured, UE is expected to always monitor the resource pool for discovery. Therefore, if the separate resource pool for discovery message can be configured, it can save the power. In addition, the separate resource pool for discovery can be configured to differentiate the discovery message in AS layer. |
| Futurewei | Postpone | As the relevant objective in SID is “Study mechanism(s) to support upper layer operations of discovery model/procedure for sidelink relaying”, we are hesitant to discuss this level of AS details before high level agreement/understanding is reached (in SA2) on discovery model and messages. |
| Interdigital | No | Separate resource pool will result in unnecessary resource fragmentation (discovery resources cannot be reused for data transmission when there is little/no discovery traffic expected). This seems unnecessary when there are other simple methods to differentiate discovery from data. We don’t see any clear benefit of separate resource pool from power savings, since a remote UE will only monitor for discovery messages when it is looking for a relay and once one is found, will have to monitor for data as well. |

Question7a: if answer to Q7 is yes, which resource pool should be separated, TX, RX or both?

| Company | Position | Other comment |
| --- | --- | --- |
| Qualcomm | Both | It is same as LTE, i.e. separate TX and RX discovery pool |
| OPPO | Both |  |
| Lenovo&MM | Both |  |
| Futurewei | Postpone | As the relevant objective in SID is “Study mechanism(s) to support upper layer operations of discovery model/procedure for sidelink relaying”, we are hesitant to discuss this level of AS details before high level agreement/understanding is reached (in SA2) on discovery model and messages. |

# Conditions for discovery of U2N relay

Relay UE should be at least in coverage i.e. either in IDLE/INACTIVE state or CONNECTED state otherwise it can’t act as relay. From listed contributions no company differentiate between IDLE and INACTIVE state. So these two RRC states are discussed together. For relay UE in IDLE/INACTIVE state relay UE need measure Uu signalling quality i.e. RSRP to check whether it can be act as a relay UE. And it seems LTE principle is popular among contributions [8][15] i.e. two threshold approach. In order to transmit discovery message network need configure relay UE with proper parameters including communication configuration [5][8][22].

Question8: For U2N relay, do you agree that relay UE is allowed to transmit/receive discovery message when it is in coverage and relevant control parameters including Uu signal quality thresholds and communication configuration are provided by network?

| Company | Yes/No | Other comment |
| --- | --- | --- |
| Ericsson(Min) | Yes | RAN2 should aim to use LTE Rel-13 relay selection and reselection mechanism as a baseline. This can minimize RAN2 efforts. |
| Qualcomm | Yes | We prefer to follow LTE approach |
| OPPO | Yes | We can follow LTE principle |
| Lenovo&MM | Yes | Legacy mechanism for LTE relay can be reused. |
| Futurewei | Yes with the intention, some comments on the wording | The intention seems reasonable, but the wording looks more targeted at UE in RRC_IDLE/INACTIVE states. Replacing “including” with “e.g.” may be more suitable at this stage of study, especially for UE in RRC_CONNECTED state. |
| Interdigital | Yes, for Uu signal quality, but not sure about communication configuration | Agree with Futurewei, that the wording does not reflect the RRC_CONNECTED case. In LTE, a relay UE in RRC_CONNECTED uses the thresholds do decide whether to request the resources, so the final decision of whether to transmit discovery is upto the NW (not the UE). Also, the “communication configuration” seems to assume separate pool as that was the case in LTE. But for NR, it may not apply. [OPPO]: The intention of this question is to check in general whether some assistant information from network is needed for relay UE in all RRC state. I am fine to change “including” to be “e.g.” to make it more general. And both mixed or separate resource pool are covered in the context of “communication configuration” which is a general description. |

For relay UE in IDLE/INACTIVE state, more specifically LTE principle can be reused i.e. one lower threshold and one upper threshold to allow relay UE to transmit discovery message. Lower threshold is to guarantee that relay UE can have stable connection with network. And upper threshold is also needed because otherwise relay UE could be very close to network which may make it difficult for relay UE to connect with remote UE which supposes to be at cell edge or out of coverage.

Question9: For U2N relay, do you agree that LTE principle i.e. one lower threshold and one upper threshold can be reused to allow relay UE in IDLE/INACTIVE state to transmit/receive discovery message?

| Company | Yes/No | Other comment |
| --- | --- | --- |
| Ericsson(Min) | Yes | See comments in Q8 |
| Qualcomm | Yes | We think LTE approach is sufficient |
| OPPO | Yes |  |
| Lenovo&MM | Yes | Legacy mechanism can be reused. The two thresholds can be set as ‘optional’. |
| Futurewei | Yes | But other criterions don’t have to be ruled out at this stage of study. |
| Interdigital | Yes, but | Other rules/criteria should be further studied. |

For relay UE in CONNECTED state no extra condition is needed apart from proper configuration of radio resource for discovery channel in LTE. Similar principle is rather straight forward [8].

Question10: For U2N relay, do you agree that no extra condition is required apart from sidelink communication configuration for discovery message for relay UE in CONNECTED state?

| Company | Yes/No | Other comment |
| --- | --- | --- |
| Ericsson (Min) | Yes | Our understanding is that after discovery, the relay UE in CONNECTED will just use the legacy Rel-16 behaviour to establish the relay path. |
| Qualcomm | Yes (We assume “in CONNECTED state” is missing in question) | It is aligned to section 5.10.10.2 and 5.10.10.3 of current 36.331. |
| OPPO | Yes |  |
| Lenovo&MM | No | The NR V2X features e.g use of three cast types, HARQ feedback based retransmissions, MCR etc could be considered. |
| Futurewei | Not clear about the question | Extra condition for a) transmit/receive discovery message or b) for relaying operation? If it is a), why would this be different between UE in Idle/Inactive states and UE in Connectd state? If it is b), there’d be more configuration needed for the relay node (after discovery procedure concludes) [OPPO]: the intention is for a) The relay UE in IDLE/ACTIVE state need some assistant information like threshold to decide by relay UE itself whether to transmit/receive discovery message. For relay UE in CONNECTED it is totally up to network. |
| Interdigital | No | We agree that “the NW should decide – e.g. using dedicated signaling” whether an RRC_CONNECTED relay UE can transmit discovery message. And then details of the configuration can be decided later in the SI or the WI. Although we agree with legacy LTE, we are not sure stage 3 details can apply exactly. [OPPO]: As I response to your answer to Q8 communication configuration doesn’t hint separate resource pool. If you fine with this explanation, I guess your answer actually is “yes”, right? |

For remote UE, out of coverage case should be covered. Some companies also propose to confirm that remote UE in coverage should be covered for discovery [5][8][9][11][13][22]. If only out of coverage remote UE is covered, then remote UE at cell edge can’t benefit from relay scheme. In addition for both L2 and L3 U2N relay service continuity is very important. So likely switch between direct Uu connection and indirect Uu connection via relay should be supported. And such switch should happened when remote UE is still in coverage of network otherwise Uu connection is either lost or can’t be established properly.

Question11: for U2N relay, do you agree that remote UE in coverage of network should be considered for discovery?

| Company | Yes/No | Other comment |
| --- | --- | --- |
| Ericsson(Min) | Maybe | Allow the UE in coverage to trigger discovery is to make sure the UE to not being disconnected. At the same time, whenever the UE has connection to Uu (e.g., RLF is not triggered), the UE shall always only use Uu. This also mean that the gNB is in control of which path the remote UE should use (i.e., direct path or relay path). Therefore, is also the gNB to configure the remote UE to transmit or receive discovery messages. |
| Qualcomm | Yes | We prefer to follow the same way of LTE. It is allowed in LTE Prose relay |
| OPPO | Yes |  |
| Lenovo&MM | Yes | In the LTE, the remote UE located in the cell edge is also allowed to select a relay for connection. The legacy mechanism can be reused. |
| Futurewei | Yes | but its discovery procedure doesn’t have to be different from UE out of coverage. We should focus on a discovery procedure common to in-coverage and out-of-coverage scenarios. |
| Interdigital | Yes | In the email discussion [603], all companies want to consider remote UE in coverage, and so discovery is needed also in this case. |

If the answer to question 11 is yes, then following two cases can be discussed separately i.e. UE in IDLE/INACTIVE state and CONNECTED state. For UE in IDLE/INACTIVE state similar to relay UE, signalling quality of Uu interface i.e. RSRP is needed to check whether remote UE is at cell edge or not. And also similar to relay UE relevant threshold should be provided by network enable transmission/reception of discovery message [5][6][8][15][22].

Question12: for U2N relay, do you agree that remote UE in IDLE/INACTIVE state is allowed to transmit/receive discovery message when RSRP of Uu interface is lower than one configured threshold by network?

| Company | Yes/No | Other comment |
| --- | --- | --- |
| Ericsson(Min) | Yes | RAN2 should aim to use LTE Rel-13 relay selection and reselection mechanism as a baseline. This can minimize RAN2 efforts. |
| Qualcomm | Yes | We prefer to reuse same mechanism in LTE Prose relay |
| OPPO | Yes |  |
| Lenovo&MM | Yes |  |
| Futurewei | Yes |  |
| Interdigital | Yes |  |

For remote UE in CONNECTED state, the purpose for remote UE to find a relay UE is to switch from direct Uu connection to indirect Uu connection. Such kind of action should be always up to the serving gNB. Detail can be discussed further when discussing service continuity issue.

Question13: for U2N relay, do you agree when and how remote UE in CONNECTED state to transmit/receive discovery is up to serving gNB and detail is FFS?

| Company | Yes/No | Other comment |
| --- | --- | --- |
| Ericsson(Min) | Yes | We think the gNB for the case where the remote UE is in coverage should be always in control. Therefore, is also the gNB to configure the remote UE to transmit or receive discovery messages. |
| Qualcomm | No | We prefer to reuse same mechanism in LTE Prose relay. In existing 36.331, remote UE in CONNECTED state still needs to check Uu link quality: - ============================ - 5.10.11.4 Selection and reselection of sidelink relay UE A UE capable of sidelink remote UE operation that is configured by upper layers to search for a sidelink relay UE shall: 1> if out of coverage on the frequency used for sidelink communication, as defined in TS 36.304 [4], clause 11.4; or 1> if the serving frequency is used for sidelink communication and the RSRP measurement of the cell on which the UE camps (RRC_IDLE)/ the PCell (RRC_CONNECTED) is below threshHigh within remoteUE-Config : 2> search for candidate sidelink relay UEs, in accordance with TS 36.133 [16] 2> when evaluating the one or more detected sidelink relay UEs, apply layer 3 filtering as specified in 5.5.3.2 across measurements that concern the same ProSe Relay UE ID and using the filterCoefficient in SystemInformationBlockType19 (in coverage) or the preconfigured filterCoefficient as defined in 9.3(out of coverage), before using the SD-RSRP measurement results; NOTE 1: The details of the interaction with upper layers are up to UE implementation. ===================================== |
| OPPO | Yes | For both L2 or L3 solution remote UE will either keep direct Uu connection or indirect Uu connection i.e. via relay UE. In case there is direct Uu connection gNB should control when and how to transmit/receive discovery message. And it doesn’t contradict with LTE principle because such threshold for triggering relay selection can be configured by serving gNB also. |
| Lenovo&MM | Yes | The explicit signaling message or condition can be transmitted to trigger to transmit/receive discovery. |
| Futurewei | Partically Yes | We understand and are fine with the “when” part. What’d be the “how” part? We still prefer common discovery procedure for UE both in coverage an out-of-coverage. |
| Interdigital | Partially | We agree with the statement, as it is aligned with LTE, except that thresholds are also considered in the decision to request discovery resources. As for the motivation, service continuity seems more related to when the UE should switch the path, and not to when relay discovery should occur. |

For remote UE out of coverage no condition on RSRP of Uu interface is needed i.e. it can always transmit/receive discovery message based on pre-configuration [8][22].

Question14: for U2N relay, do you agree that remote UE out of coverage is always allowed to transmit/receive discovery message based on pre-configuration?

| Company | Yes/No | Other comment |
| --- | --- | --- |
| Ericsson(Min) | Yes |  |
| Qualcomm | Yes | This is aligned with current 36.331 |
| OPPO | YES |  |
| Lenovo&MM | Yes for initial relay selection. No for relay reselection | For initial relay selection, the remote UE has not connection with gNB. So, the remote UE out of coverage is always allowed to transmit/receive discovery message based on pre-configuration. For relay re-selection, the OOC remote UE has end-to-end connection with gNB. So, the OOC remote UE is not needed always to transmit/receive the discovery message. For example, if the channel quality of both the PC5 interface and Uu interface is good, it seems unnecessary to always to monitor/receive the discovery message. [OPPO]: well the question itself is checking whether remote UE is allowed to do so but not necessary have to do so. So for reselection case, I agree UE will not do it unless it is triggered to do it. But once it is triggered there is no other condition is required. |
| Futurewei | Yes | I have some sympathy with Lenovo&MM’s observation, which may be considered in later stage of the study/work item. |
| Interdigital | Yes | We prefer to keep this aligned with LTE, and we can further discuss whether enhancements are needed during the SI/WI. |

# Authorisation related aspects

In contribution [22] it is point out for both L2 and L3 relay solution authorization information for relay UE is provided by 5GC to gNB. In addition for L2 relay solution authorization information for remote UE is also needed over NG interface. It means authorization has limited RAN3 impact but no RAN2 impact which is also confirmed by contribution [11][19][21]. Contribution [20] discuss a case authentication/authorization of remote UE is done via indirect Uu connection. But again in this case it seems there is no explicit RAN impact is foreseen. Considering there is no RAN3 work will be done in study phase it seems further detail work can be left in work item phase.

Question15: Do you agree that authorization of both relay UE and remote UE has no RAN2 impact?

| Company | Yes/No | Other comment |
| --- | --- | --- |
| Ericsson(Min) | Yes |  |
| Qualcomm | Yes |  |
| OPPO | Yes |  |
| Lenovo&MM | Maybe (but bit early to conclude this) | Authorization should be discussed by SA2 and RAN3. Also, we agree with the moderator that details can be discussed in work item. |
| Futurewei | Yes |  |
| Interdigital | Yes |  |

Question16: Do you agree that limited impact on RAN3 can be left for normative work item phase?

| Company | Yes/No | Other comment |
| --- | --- | --- |
| Ericsson(Min) | Yes | In principle we agree, but it would be also good to send an LS to RAN3 so they can reply to RAN2 when their normative work is done. |
| Qualcomm | Yes |  |
| OPPO | Yes | We are fine to send a LS to RAN3 |
| Lenovo&MM | Yes |  |
| Futurewei | Yes |  |
| Interdigital | Yes |  |

# Any other issues

If you have any other issue which is not covered, please figure out details in the following table.

| Company | Issue description | comment |
| --- | --- | --- |
| Qualcomm | Prioritization between discovery TX, PC5-S/PC5-RRC TX and communication TX | Rapporteur: since QC prefer a separate TX/RX resource, then can you clarify why such prioritization is needed? |

# Conclusion

# Reference

- 3GPP RP-193253 "New SID: Study on NR sidelink relay".

- 23.752 Study on system enhancement for Proximity based Services v0.4.0

- R2-2006556	Discussion on relay discovery model / procedure   	Qualcomm Incorporated	discussion	Rel-17	FS_NR_SL_relay

- R2-2006573	Initiation of relaying operation	MediaTek Inc.	discussion	Rel-17	FS_NR_SL_relay

- R2-2006612	Discovery Model/Procedure for NR Sidelink Relay	CATT	discussion	Rel-17	FS_NR_SL_relay

- R2-2006738	Discussion on relay discovery and link management	ZTE Corporation, Sanechips	discussion	Rel-17	FS_NR_SL_relay

- R2-2006761	Discovery Procedure for SL Relaying	InterDigital	discussion	Rel-17	FS_NR_SL_relay

- R2-2006771	Discussion on SL relay discovery procedure	OPPO	discussion	Rel-17	FS_NR_SL_relay

- R2-2006862	NR Sidelink Relaying Discovery	Fraunhofer IIS, Fraunhofer HHI	discussion	Rel-17

- R2-2006868	Discovery Model and Procedure in NR Sidelink Relaying	Fujitsu	discussion	Rel-17	FS_NR_SL_relay

- R2-2006931	On Sidelink Discovery for Relaying	Intel Corporation	discussion	Rel-17	FS_NR_SL_relay

- R2-2006969	Sidelink relay discovery model and procedure	Samsung Electronics Co., Ltd	discussion	Rel-17	FS_NR_SL_relay

- R2-2007042	Discussion of Relay UE discovery	vivo	discussion	Rel-17

- R2-2007045	Discussion on discovery procedure for sidelink relay	Spreadtrum Communications	discussion

- R2-2007098	Discussion on NR Sidelink Relay Discovery	Apple, Convida Wireless	discussion	Rel-17	FS_NR_SL_relay

- R2-2007291	Discovery aspects for NR sidelink relay	Ericsson	discussion	Rel-17	FS_NR_SL_relay

- R2-2007476	Considerations on discovery procedure for sidelink relay	Lenovo, Motorola Mobility	discussion	Rel-17

- R2-2008045	Consideration of discovery model/procedure for sidelink relay	LG Electronics Inc.	discussion	Rel-17	FS_NR_SL_relay

- R2-2008049	Common aspects for L2 and L3 UE-to-Network relay	Huawei, HiSilicon	discussion	Rel-17	FS_NR_SL_relay

- R2-2006641	L2 vs L3 - Relay/Remote UE Authorization, Service Continuity	Fraunhofer HHI, Fraunhofer IIS	discussion

- R2-2006770	 Discussion on SL relay (re)selection and authorization	OPPO	discussion	Rel-17

- R2-2007040	Selection/Authorization and Security for L2 and L3 relay	vivo	discussion	Rel-17