3GPP TSG-RAN WG2 #112e	Tdoc R2-2010544

Electronic meeting, November 2nd – 12th 2020	Revision of R2-2007602

Agenda Item:	8.3.3

Source:	Ericsson

Title:	Graceful leaving for a MultiSIM device

Document for:	Discussion, Decision

# Introduction

MultiSIM devices enable use of multiple subscriptions in one device, e.g. more than one USIM and concurrent registration in more than one network. The USIMs are from same or different operator (PLMN).

A MultiSIM device needs to handle “simultaneous state” situations like:

- RRC_IDLE/INACTIVE + RRC_IDLE/INACTIVE, e.g.

  - check paging in multiple systems

  - receive system information for multiple systems

  - perform measurements on multiple systems

- RRC_CONNECTED+ RRC_INACTIVE/IDLE, e.g.

  - voice + paging/SI/measurements

RAN2 WID RP-201309 [1] defines three objectives for the support for MultiSIM devices in Rel-17. In this contribution the focus is on the objective 2, reported here:

- Specify mechanism for UE to notify Network A of its switch from Network A (for MUSIM purpose):

  - RAT Concurrency: Network A is NR. Network B can either be LTE or NR.

  - Applicable UE architecture: Single-Rx/Single-Tx, Dual-Rx/Single-Tx

The proposed mechanism is applicable to NR and LTE RAN and to 5GS and EPS Core Network. This contribution was revised to further clarify the scope of the solution described.

# Discussion

​When a Single-Rx/Single-Tx or Dual-Rx/Single-Tx MultiSIM UE, in RRC_CONNECTED or RRC_INACTIVE state in one of the PLMNs (e.g. PLMN1) needs to connect to the other PLMN (e.g. to answer a MT calls, perform RAU procedure or by user selection) the UE has to leave the PLMN currently connected.

If the MultiSIM UE interrupts abruptly the connection with the current PLMN, the network KPI might be affected negatively.

If the MultiSIM UE interrupts abruptly the connection with the current PLMN, the network KPI might be affected negatively.

Considering a MultiSIM UE in RRC_CONNECTED or RRC_INACTIVE state in PLMN1 and RRC_IDLE or RRC_INACTIVE in PLMN2, depending on how long the UE takes to perform the actions in PLMN2 (which correspond to an absence time in PLMN1), the following scenarios are possible:

- Very short UE absence: this is typically required to perform “quick” actions in PLMN2 which do not require a connection setup/resume, e.g. listen to paging, SIB acquisition and cell reselection, etc.
During this absence, the UE is kept in RRC CONNECTED in PLMN1;

- Short/long UE absence: this is relevant when the UE perform longer actions in PLMN2, which require the connection setup/resume (e.g. TAU, RNAU, mobility registration and MO/MT call). In this case the UE cannot be kept in RRC_CONNECTED in PLMN1 due to the longer activities in PLMN2. The UE notifies the current PLMN about the imminent leaving (here called “graceful leaving”) and the network moves the UE to RRC_IDLE/RRC_INACTIVE.

This contribution focuses on the “short/long UE absence” case, where the MultiSIM UE sends a message including an indication that it is leaving to other PLMN (i.e. the graceful leaving).

Either RRC or NAS signaling could be used for this, but to minimize the specification changes and have harmonized framework, it would be preferable to specify only one procedure (i.e. either by RRC or NAS signaling)

RAN2 to agree that only a single procedure should be specified for the graceful leaving, i.e. either RRC or NAS, but not both.

The usage of RRC signaling requires a higher number of changes compared to NAS and, if LTE is used in the PLMN, it would require additional changes in LTE RRC specification, which is not desirable.

For these reasons, it is recommended to specify the graceful leaving on NAS level only.

It is recommended to specify the graceful leaving indication on NAS level.

The procedure is described in the following figure (not all the NAS messages are shown):

The UE, in RRC_CONNECTED state, sends for instance NAS SERVICE REQUEST including a new indication that it is leaving to other PLMN.

Besides the leaving indication, the UE includes additional assistance information to help the network in deciding how to handle the leaving UE.

Assistance information may include the following (TBD is the complete list):

- List of PDU sessions/services which will (or will not) trigger the Paging, during the absence time.

- Absence time (short absence, long absence)

- …

The CN can use the additional assistance information to:

- move the UE to CM_IDLE and consequently gNB moves the UE to RRC_IDLE.

- modify the UE context, which may trigger the gNB to move the UE to RRC_INACTIVE.

The CN can signal the extra assistance information to the gNB (e.g. via NGAP UE Context Modification procedure). This may be beneficial when the UE is moved to RRC_INACTIVE state by gNB. The gNB can perform proper actions depending on the received assistance info: for instance, a more correct data buffer dimensioning, a better handling of RAN Paging, etc.

It would be beneficial from a RAN2 point of view if the MultiSIM UE includes the leaving information and the additional assistance information in the NAS Service Request message and that such information is signaled from CN to the gNB.

It is assumed the MultiSIM UE leaves the current PLMN after the RRCRelease message is received, and not right after having sent the NAS Service Request. That avoids possible state mismatch and misconfiguration (e.g. wrong RAN Notification Area Info for the RRC_INACTIVE state).

It is assumed that the UE leaves RRC CONNECTED (for example in order to establish an RRC connection with another network) only when receiving the RRCRelease message.

The usage of the NAS signaling to notify the current PLMN about the imminent leaving implies limited impacts to the RAN and no RAN2 specs impacts. This makes the solution applicable to both NR and LTE accesses with minor changes.

Limited RAN impacts and no RAN2 specs impact are expected if the UE uses NAS signaling to notify the current PLMN about the imminent leaving. This makes the solution applicable to both NR and LTE accesses with minor changes.

# Conclusion

In the previous sections we made the following observations:

- If the MultiSIM UE interrupts abruptly the connection with the current PLMN, the network KPI might be affected negatively.

- It is assumed the UE leaves RRC CONNECTED (for example in order to establish an RRC connection with another network) only when receiving the RRCRelease message.

- Limited RAN impacts and no RAN2 specs impact are expected if the UE uses NAS signaling to notify the current PLMN about the imminent leaving. This makes the solution applicable to both NR and LTE accesses with minor changes.

- Based on the discussion in the previous sections we propose the following:

- RAN2 to agree that only a single procedure should be specified for the graceful leaving, i.e. either RRC or NAS, but not both.

- It is recommended to specify the graceful leaving indication on NAS level.

- It would be beneficial from a RAN2 point of view if the MultiSIM UE includes the leaving information and the additional assistance information in the NAS Service Request message and that such information is signaled from CN to the gNB.

# References

RP-201309, WID: Support for Multi-SIM devices for LTE/NR.