3GPP TSG-RAN WG2 Meeting #112-e	R2-201xxxx

Online, 2 – 13 November 2020

Title:	[DRAFT] LS on per-table MCS range for mode-2

Response to:

Release:	Rel-16

Work Item:	5G_V2X_NRSL

Source:	OPPO [To be RAN2]

To:	RAN1

Cc:

Contact Person:

Name:	Qianxi Lu

Tel. Number:

E-mail Address:	qianxi.lu@oppo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

Following RAN1 request on RRC parameter definition, RAN2 defined

- Per-pool configuration on additional MCS table, for the following parameter (in R1-2003190), and

| additional-mcs-Table-SL | new | mcs-Table-SL | Indicates MCS table(s) additionally used in a resource pool. FFS in RAN1 on whether multiple MCS tables can be configured in a resource pool. 64QAM table is (pre-)configured as default. Zero, one or two additional can be additionally (pre-)configured using the 256QAM and/or low-SE MCS tables | {qam256}, {qam64LowSE}, {qam256, qam64LowSE} |
| --- | --- | --- | --- | --- |

- Per-table configuration on MCS range for each additional MCS table for mode-1 operation (in R1-2005050)

| minMcs-Mode1 | new | minMcs-Mode1 | Indicates the minimum MCS value for Mode 1 configured and dynamic grants when using the associated MCS table. If no MCS is configured, UE autonomously selects MCS from the full range of values. | 0..27 |
| --- | --- | --- | --- | --- |
| maxMcs-Mode1 | new | maxMcs-Mode1 | Indicates the maximum MCS value used for Mode 1 configured and dynamic grants when using the associated MCS table. If no MCS is configured, UE autonomously selects MCS from the full range of values. | 0..31 |

For mode-2 operation, MCS-ranges specified in RRC specification are de-coupled with MCS-tables, i.e.,

- sl-MinMCS-PSSCH and sl-MaxMCS-PSSCH included in sl-PSSCH-TxConfigList, which is the PSSCH transmission parameter table based on UE speed and synchronization source; and

- sl-MinMCS-PSSCH and sl-MaxMCS-PSSCH indicated in sl-CBR-PSSCH-TxConfigList, which is the PSSCH transmission parameter table based on CBR and priority

RAN2 would like to request feedback from RAN1 on the following question:

Q1: Similar to mode-1 operation, considering the MCS table(s) additionally used in a resource pool (i.e., MCS table for 256QAM and/or low-SE), is there a need to define per-MCS-table MCS ranges for mode-2 operation (i.e., in the PSSCH transmission parameter table based on UE speed and synchronization source, and the PSSCH transmission parameter table based on CBR and priority) or not?

2. Actions:

To RAN1 group

RAN2 respectfully requests RAN1 to feedback on Q1 as above.

3. Date of Next TSG-RAN2 Meetings:

3GPPRAN2#113	25 January – 5 February 2021	E-meeting

3GPPRAN2#113-bis 	12-21 April 2021	E-meeting