3GPP TSG RAN WG2 Meeting #112-e                                                            R2-2101007

e-Meeting, 25th January – 5th February, 2021

Agenda item:	8.1.1

Source:	Samsung

Title:	MBS Radio Bearer (MRB) Type

Document for:	Discussion and Decision

1	Introduction

This document discusses MBS radio bearer structure and its types.

2	Discussion

In RAN2#112-e, split-like bearer with a common PDCP was supported by majority of companies. The split-like bearer is configured with one PTM RLC entity shared by multiple UEs and one PTP RLC dedicated to a UE. In the split bearer of unicast, PDCP is the anchor entity and reordering function supports in-sequence delivery and duplicate detection. For MBS RB, the same reordering function can be applied.

Assuming the split-like MBS RB, we could have two switching options between PTM RLC and PTP RLC.

-  Option A: Two RLCs are always active. gNB can choose one switching option among 1) delivery via PTM RLC 2) delivery via PTP RLC 3) delivery via PTM RLC and PTP RLC (i.e., duplication) by gNB implementation. The selected switching option is not indicated to UE. Since PDCP has duplicate detection and discard function, the duplicate reception may not be a problem.

Figure 1. Option A: switching by gNB implementation

-  Option B: Active RLC (PTM or PTP) is switched by NW signalling, e.g., MAC/PDCP/RRC/etc. UE only needs to monitor G-RNTI or C-RNTI according to the data path.

Figure 2. Option B: switching by NW signaling

In uplink of unicast, data path, i.e., primary RLC, split threshold, duplication, is configured by gNB and a UE just follows the configuration. In contrast, there is no such configuration for downlink data path selection which is all up to NW implementation. MBS is mainly for downlink transmission. Considering the complexity of UE operation, Option A, similar to downlink split bearer, seems better.

Proposal 1. Split-like bearer with a common PDCP configured with one PTM RLC and one PTP RLC is considered as an MBS radio bearer structure.

Proposal 2. Assuming split-like MBS RB, dynamic switching is up to gNB. No additional switching mechanism (e.g. switching indication by MAC/PDCP/RRC) is necessary.

Proposal 3. RLC switching and MAC switching are not supported.

Figure 3. PTM-only MRB

For MBS, PTP RLC is not always necessary. If NW wants not to configure PTP RLC, an MBS bearer associated with only one PTM RLC should be possible as shown in Figure 3. To sum up, an MBS bearer can be configured with one PDCP, one PTM RLC and optionally one PTP RLC. A radio bearer without PTM RLC is the same as the unicast radio bearer (DRB/SRB). We do not need to define an MBS RB without PTM RLC.

Proposal 4. For MBS RB, RRC signalling configures one PDCP, one PTM RLC and optionally one PTP RLC, i.e. two bearer types are supported:

- Split MRB configured with one PTM RLC and one PTP RLC

- MRB configured with one PTM RLC.

We may need to consider MBS bearer re-configuration, for instance, PTP RLC is released or newly added. This reconfiguration may occur according to NW’s policy, UE’s mobility and so on. Also, bearer type change from MBS RB to unicast DRB can be considered.

Proposal 5. Reconfiguration of MRB RB from MRB to unicast DRB is supported.

Proposal 6. MRB type change is supported.

3	Conclusion

The below proposals are made:

Proposal 1. Split-like bearer with a common PDCP configured with one PTM RLC and one PTP RLC is considered as an MBS radio bearer structure.

Proposal 2. Assuming split-like MBS RB, dynamic switching is up to gNB. No additional switching mechanism (e.g. switching indication by MAC/PDCP/RRC) is necessary.

Proposal 3. RLC switching and MAC switching are not supported.

Proposal 4. For MBS RB, RRC signalling configures one PDCP, one PTM RLC and optionally one PTP RLC, i.e. two bearer types are supported:

- Split MRB configured with one PTM RLC and one PTP RLC

- Split MRB configured with one PTM RLC.

Proposal 5. Reconfiguration of MRB RB from MRB to unicast DRB is supported.

Proposal 6. MRB type change is supported.