3GPP TSG-RAN2 Meeting #113 electronic	R2-2102264

Online, Jan 25th - Feb 5th, 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.306 | CR | 0521 | rev | 1 | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on the Capability of PUCCH Transmissions for HARQ-ACK-38306 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Sanechips,Intel |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |
| Work item code: | NR_L1enh_URLLC |  |  |  | Date: |  | 2021-2-4 |
| Category: | F |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |

| Reason for change: | The component 6 of RAN1 feature 11-4/4a as below was not included in the corresponding field description. 11-4/4a component 6 Supported maximum number of actual PUCCH transmissions for HARQ-ACK within a slot. Candidate values for the component 6 of FG11-4 is: For NCP, {4, 5, 6, 7} for 2-symbol*7 sub-slot configuration; For ECP, the candidate value is {4, 5, 6} for 2-symbol*6 sub-slot configuration. |  |  |  |
| --- | --- | --- | --- | --- |
| Summary of change: | Update the field description of twoHARQ-ACK-Codebook-type1-r16/ twoHARQ-ACK-Codebook-type2-r16 to include the maximum number of actual PUCCH transmissions for HARQ-ACK within a slot. Impact analysis Impacted 5G architecture options: NR SA, NR-DC, (NG)EN-DC/NE-DC Impacted functionality: URLLC Inter-operability: - If UE implements according to the CR and the network does not, the network is unable to get the maximum number of actual PUCCH transmissions for HARQ-ACK within a slot. - If the network implements according to the CR and the UE is not, the UE is unable to report the maximum number of actual PUCCH transmissions for HARQ-ACK within a slot. |  |  |  |
| Consequences if not approved: | The network may misunderstand the capability and give the wrong configuration to the UE. |  |  |  |
| Clauses affected: | 4.2.7.7 |  |  |  |
|  | Y | N |  |  |
| Other specs | X |  | Other core specifications | TS 38331 CR2447 |
| affected: |  | X | Test specifications | TS/TR ... CR ... |
| (show related CRs) |  | X | O&M Specifications | TS/TR ... CR ... |
| Other comments: |  |  |  |  |
| This CR's revision history: |  |  |  |  |

Start change

4.2.7.7	FeatureSetUplink parameters

***************** omitted unchanged parts *****************

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| twoHARQ-ACK-Codebook-type1-r16 Indicates whether the UE supports two HARQ-ACK codebooks with up to one subslot based HARQ-ACK codebook (i.e. slot-based + slot-based, or slot-based + subslot based) simultaneously constructed for supporting HARQ-ACK codebooks with different priorities at a UE. The capability signalling comprises the following parameters: - sub-SlotConfig-NCP-r16 indicates the maximum number of actual PUCCH transmissions for HARQ-ACK within a slot for NCP with 2-symbol*7 sub-slot configuration; - sub-SlotConfig-ECP-r16 indicates the maximum number of actual PUCCH transmissions for HARQ-ACK within a slot for ECP with 2-symbol*6 sub-slot configuration; For the 7-symbol*2 sub-slot configuration of NCP or the 6-symbol*2 sub-slot configuration of ECP, the value of the maximum number of actual PUCCH transmissions for HARQ-ACK within a slot is {2}. Note: If a UE reports both multiPUCCH-r16 and twoHARQ-ACK-Codebook-type1-r16, it can support two slot-based HARQ-ACK codebooks, and one slot-based and one-sub-slot-based HARQ-ACK codebooks. If a UE reports twoHARQ-ACK-Codebook-type1-r16 but doesn’t report multiPUCCH-r16, it can only support two slot-based HARQ-ACK codebooks. | FS | No | N/A | N/A |
| twoHARQ-ACK-Codebook-type2-r16 Indicates whether the UE supports two subslot based HARQ-ACK codebooks simultaneously constructed for supporting HARQ-ACK codebooks with different priorities at a UE.The capability signalling comprises the following parameters: - sub-SlotConfig-NCP-r16 indicates the maximum number of actual PUCCH transmissions for HARQ-ACK within a slot for NCP with 2-symbol*7 sub-slot configuration; - sub-SlotConfig-ECP-r16 indicates the maximum number of actual PUCCH transmissions for HARQ-ACK within a slot for ECP with 2-symbol*6 sub-slot configuration; For the 7-symbol*2 sub-slot configuration of NCP or the 6-symbol*2 sub-slot configuration of ECP, the value of the maximum number of actual PUCCH transmissions for HARQ-ACK within a slot is {2}. | FS | No | N/A | N/A |

***************** omitted unchanged parts *****************

End of change