3GPP TSG-RAN WG2 Meeting #113-e	R2-210xxxx

# Online, January 25th – February 5th 2021

Agenda item:	6.16

Source:	Apple

Title:	Summary of [028][TEI16] Miscellaneous I (Apple)

Document for:	Discussion and Decision

1	Introduction

This document contains the summary of documents from agenda item 6.16 (“Overheating Stop Behaviour”, “Overheating Other”, “Processing time of DL Segmentation”, and “Release with Redirect”) as per below excerpt from the session chair minutes:

[AT113-e][028][TEI16] Miscellaneous I (Apple)

Scope: R2-2101434, R2-2101346, R2-2101170, R2-2101656, R2-2100872, R2-2101356, R2-2101357, R2-2101358, R2-2101359, R2-2100979, R2-2101289, R2-2101290, R2-2101291, R2-2101292, R2-2101657,

Phase 1, determine agreeable parts, Phase 2, for agreeable parts Work on CRs.

Intended outcome: Report and Agreed CRs if any is agreeable.

Deadline: Schedule A (can come back Thu Feb 4 is needed)

2	Contact Points

Respondents to the email discussion are kindly asked to fill in the following table.

| Company | Name | Email Address |
| --- | --- | --- |
| Huawei, HiSilicon | Yiru Kuang | kuangyiru@huawei.com |

2		Company comments to the contributions

  - Topic 1: Overheating Stop Behaviour

R2-2101434 is the email summary of the [Post112-e][067][NR TEI16] UE indication when it no longer experiences overheating.

Following three solutions are under the email discussion.

The e-mail discussion resulted in the following proposals:

Proposal 1	RAN2 to decide between solution 1 or 2 to address overheating for SCG in EN-DC.

Proposal 2	RAN2 to confirm that for overheating in NR-DC, the field allowedReducedConfigForOverheating should work in the same way as any other restrictions signaled within CG-ConfigInfo>configRestrictInfo.

In R2-2101346, Solution 1 with no spec change is proposed.

In R2-2101170, Solution 2 is proposed and following inter-node procedure for EN-DC is proposed to be confirmed.

Proposal 2: 	RAN2 confirms the that in EN-DC when the MN should sends the CG-ConfigInfo not containing the overheatingAssistanceForSCG to the SN that the SN understands that no change in the last signalled UE preference in regards to the detected overheating condition has occurred.

Q1: Which solution is your preference, i.e. Solution 1 or Solution 2?

| Company | Solution? | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Solution-2 (slightly) | We understand and agree the comments from Samsung(R2-2101346) that it is just one overheating feature and there is no delta reporting for LTE UAI, Solution-1 better satisfies this principle. But more changes on the Uu interface for overheating reporting seems to be needes if Solution-1 is selected, based on the current procedural text, the UE need to include overheatingAssistanceForSCG if configured. So we slightly prefer Solution 2 as only inter-node message needs clarification, but still open to hear other companies views. 3> if configured to provide overheating assistance indication for NR SCG: 4> include overheatingAssistanceForSCG in the OverheatingAssistance IE; 4> set overheatingAssistanceForSCG in accordance with clause 5.7.4.3a as specified in TS 38.331 [82]; |

Q2: Do you agree with the proposal 2 in email summary (R2-2101434)?

Proposal 2	RAN2 to confirm that for overheating in NR-DC, the field allowedReducedConfigForOverheating should work in the same way as any other restrictions signaled within CG-ConfigInfo>configRestrictInfo.

| Company | Agree or not? | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Agree |  |

Q3: Do you agree with the proposal 2 in R2-2101170 for EN-DC case?

Proposal 2: 	RAN2 confirms the that when the MN should sends the CG-ConfigInfo not containing the overheatingAssistanceForSCG to the SN that the SN understands that no change in the last signalled UE preference in regards to the detected overheating condition has occurred.

| Company | Agree or not? | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Agree (depend on Q1?) | It is related to the Q1, we understand it is aligned with Solution 2 in Q1. |

Conclusions (Overheating Stop Behaviour): TBA

  - Topic 2: Overheating Other

R2-2101656

R2-2101656	Correction on handling of overheatingAssistanceConfigForSCG when SCG is released	Huawei, HiSilicon	CR	Rel-16	36.331	16.3.0	4584	-	F	TEI16

Summary of change: Clarify that the overheatingAssistanceConfigForSCG-r16 is released when the SCG is released in (NG)EN-DC.

#### 5.3.3.4a Reception of the RRCConnectionResume by the UE The UE shall: 1>… 1> else: 2> if resuming an RRC connection from a suspended RRC connection in EPC; or 2> for NB-IoT, if resuming an RRC connection from a suspended RRC connection in 5GC and fullConfig is not present in the RRCConnectionResume message: 3>… 3> else if the UE was configured with EN-DC: 4> perform MR-DC release, as specified in TS 38.331 [82], clause 5.3.5.10; 4> release tdm-PatternConfig or tdm-PatternConfig2, if configured; 4> if overheatingAssistanceConfigForSCG is configured: 5> release overheatingAssistanceConfigForSCG and stop timer T345, if running; 3> .. 2> else if the RRCConnectionResume message includes the fullConfig (i.e., for resuming an RRC connection from RRC_INACTIVE or for resuming a suspended RRC connection in 5GC): 3> perform the radio configuration procedure as specified in 5.3.5.8; 2> else if resuming an RRC connection from RRC_INACTIVE: 3> … 3> else if the UE was configured with NGEN-DC: 4> perform MR-DC release, as specified in TS 38.331 [82], clause 5.3.5.10; 4> release tdm-PatternConfig or tdm-PatternConfig2, if configured; 4> if overheatingAssistanceConfigForSCG is configured: 5> release overheatingAssistanceConfigForSCG and stop timer T345, if running;

#### 5.3.5.3 Reception of an RRCConnectionReconfiguration not including the mobilityControlInfo by the UE #### … 1> if the received RRCConnectionReconfiguration includes endc-ReleaseAndAdd and it is set to TRUE: 2> perform MR-DC release as specified in TS 38.331 [82], clause 5.3.5.10; 2> if overheatingAssistanceConfigForSCG is configured: 3> release overheatingAssistanceConfigForSCG and stop timer T345, if running; #### …

5.3.5.4 Reception of an RRCConnectionReconfiguration including the mobilityControlInfo by the UE (handover) …. 1> if the received RRCConnectionReconfiguration includes endc-ReleaseAndAdd and it is set to TRUE: 2> perform MR-DC release as specified in TS 38.331 [82], clause 5.3.5.10; 2> if overheatingAssistanceConfigForSCG is configured: 3> release overheatingAssistanceConfigForSCG and stop timer T345, if running; ….

Q4: Do you agree with the change in R2-2101656?

| Company | Agree or not? | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Agree | Proponent. |

Conclusions (R2-2101656): TBA

R2-2100872

R2-2100872	Cleanup on Overheating UAI reporting procedure	Apple	CR	Rel-16	38.331	16.3.1	2361	-	F	TEI16

Summary of change: To make it clear that the UAI reporting procedures defined for EN-DC and NR-DC are not applicable to overheating indication.

#### 5.7.4.3 Actions related to transmission of UEAssistanceInformation message … 1> else if the UE is in (NG)EN-DC: 2> if SRB3 is configured: 3> submit the UEAssistanceInformation message via SRB3 to lower layers for transmission, except overheating assistance information indication; 2> else: 3> submit the UEAssistanceInformation message via the E-UTRA MCG embedded in E-UTRA RRC message ULInformationTransferMRDC as specified in TS 36.331 [10], except overheating assistance information indication. 1> else if the UE is in NR-DC: 2> if the UE assistance configuration that triggered this UE assistance information (except overheating assistance information) is associated with the SCG: 3> if SRB3 is configured: 4> submit the UEAssistanceInformation message via SRB3 to lower layers for transmission; 3> else: 4> submit the UEAssistanceInformation message via the NR MCG embedded in NR RRC message ULInformationTransferMRDC as specified in 5.7.2a.3; 2> else: 3> submit the UEAssistanceInformation message via SRB1 to lower layers for transmission;

Q5: Do you agree with the change in R2-2100872?

| Company | Agree or not? | Comments |
| --- | --- | --- |
| Huawei, HiSilicon |  | In our understanding, in 38.331 5.7.4.3, it describe how to set the contents of the NR UAI message and the information associated with SCG only includes the power saving UAI in DC case. The overheating assistance information associated with SCG is included in the LTE UAI message described in 36.331. Thus, we understand the intention but not sure if any clarification is really needed. |

Conclusions (R2-2100872): TBA

  - Topic 3: Processing time of DL Segmentation

R2-2102261 is the summary of Email Report of [Post112-e][063][NR TEI16] RRC processing time with segmentation.

The e-mail discussion resulted in the following proposals:

Proposal 1: 	Adopt option 2 (i.e. 16ms + (Nseg-1)*X) to define the NR RRC processing time requirement for DL RRC message with segmentation.

Proposal 2: 	Assume the X value is [2ms~16ms] in option 2, and final decision is made by RAN2.

Proposal 3: 	Send LS to RAN5 to inform the RRC processing time extension for the RRC message with segmentation.

Proposal 4: 	Adopt option 2 (i.e. 20ms + (Nseg-1)*X) to define the LTE RRC processing time requirement for DL RRC message with segmentation.

Q6: Do you agree with the proposal 1,3,4?

Proposal 1: 	Adopt option 2 (i.e. 16ms + (Nseg-1)*X) to define the NR RRC processing time requirement for DL RRC message with segmentation.

Proposal 3: 	Send LS to RAN5 to inform the RRC processing time extension for the RRC message with segmentation.

Proposal 4: 	Adopt option 2 (i.e. 20ms + (Nseg-1)*X) to define the LTE RRC processing time requirement for DL RRC message with segmentation.

| Company | Agree or not? | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Yes | This seems also the “obvious” conclusion of the email discussion [063] |

About the proposal 2 on the X value, there is not a clear consensus on a specific value. To allow the different UE implementation, in R2-2100979, it is proposed to introduce a UE capability for it.

Q7: Do you agree to introduce the UE capability for the X value as suggested in R2-2100979 ?

Proposal 1	The value of “X” for the processing delay requirement of DL RRC segmentation is signalled as a UE capability.

Proposal 2	The range of “X” (to be signalled as UE capability) is 2m, 7ms, 12ms.

| Company | Agree or not? | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Maybe | The report via UE capability seems an overkill. Nevertheless, if it is really difficult to converge on a value for X, the UE capability could be a compromise to consider. In that case for example 4 values like [4,8,12,16] could be considered. |

Q8: If UE capability for the X value is not agreed, what is your preferred value of X (i.e. 2ms, 12ms, 16ms)?

| Company | Value? | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Any value between 12 and 16 ms is fine for us | A value for X in the range [12-16] should be able to accommodate all the UEs implementations. We believe that values = or > 12 ms are not a problem on the network side. The network will wait for the UE to finish its operations and as long as this value is known to the network (either hardcoded or UE capability), there is no problem even for value X=16 ms. Companies that believe this is a problem for the network side should be able to explain why. |

Conclusions (Processing time with DL segmentation): TBA

  - Topic 4: Release with Redirect

R2-2101289	Release with Redirect in 2 steps	Ericsson	discussion	Rel-16	TEI16

R2-2101290	Release with Redirect in 2 steps	Ericsson	CR	Rel-16	38.331	16.3.1	2402	-	F	TEI16

R2-2101291	Release with Redirect in 2 steps	Ericsson	CR	Rel-16	38.306	16.3.0	0503	-	F	TEI16

R2-2101292	Release with Redirect in 2 steps	Ericsson	CR	Rel-16	38.300	16.4.0	0338	-	F	TEI16

R2-2101657	Release with redirection in 2 steps release	Huawei, HiSilicon	discussion	Rel-16	TEI16

In RAN2#112e this was discussed in R2-2009849 and [AT112-e][029][NR TEI16] Misc Corrections II (R2-2011176), and the following has been agreed:

[029] will support release with redirection in response to a ResumeRequest for both with/without anchor change cases.

[029] For anchor change scenario, the current gNB is responsible for determining the redirection.

[029] Discussion on detail mechanism and CRs is postponed to next meeting.

About the case without UE context relocation, R2-2101289 propose not to support it in R16, but R2-2101657 propose to support it and provide the detailed mechanism.

| TDoc | Proposals |
| --- | --- |
| R2-2101289 | Proposal 1: Agree on the Text Proposals to TS 38.300. Proposal 2 : Confirm that the case without UE context relocation is not supported in Rel-16. Proposal 3: Agree CRs to 38.300, 38.331 and 38.306. |
| R2-2101657 | - Proposal 1: Add new cause in RETREEVE UE CONTEXT REQUESET message to indicate anchor gNB that the new serving gNB intends to redirect this UE and the anchor gNB is responsible for determining whether to perform anchor switch. Proposal 2: In non-anchor-change scenario, the new serving gNB determines redirect configuration and sends it to the anchor gNB along with RETRIEVE UE CONTEXT REQUSET message. Proposal 3: If Proposal 1 and Proposal 2 are agreed, send LS to RAN3 to inform them. Proposal 4: Agree the Text Proposals to TS 38.300. |

Q9: Do you support case without UE context relocation?

| Company | Support? | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Support | Proponent of R2-2101657. After receiving the RRCResumeRequest message from UE, if the load of the new serving gNB is too heavy or it couldn’t support this resume cause, the new serving gNB could decide release and redirect this UE. Then, it requests UE context from the last serving gNB. After UE context is moved to the last serving gNB and Path switch is made, the new serving gNB2 will be anchor gNB. RRC release message including redirection information could be created by gNB2 and be send to UE. After receiving release message with redirect information, UE will perform cell selection and camp on a new cell, and then UE’s NAS will trigger RRC resume procedure again. In this case, anchor gNB switch has to be performed twice to finish the NAS trigger RRC resume and it is not optimal from signalling point of view. |

Conclusions (with/without UE context relocation): TBA

- Conclusions

4	Discussion contribution List

Overheating Stop Behaviour

R2-2101434	Summary of e-mail discussion on UE indication when it no longer experiences overheating	Ericsson	discussion

R2-2101346	Impacting UE to optimise inter-node transfer of SCG overheating info	Samsung Telecommunications, LG Electronics Inc.	discussion	TEI16

R2-2101170	OverheatingAssistance Restriction Release Signalling in EN-DC	Beijing Xiaomi Mobile Software	discussion	Rel-16

Overheating Other

R2-2101656	Correction on handling of overheatingAssistanceConfigForSCG when SCG is released	Huawei, HiSilicon	CR	Rel-16	36.331	16.3.0	4584	-	F	TEI16

R2-2100872	Cleanup on Overheating UAI reporting procedure	Apple	CR	Rel-16	38.331	16.3.1	2361	-	F	TEI16

Processing time of DL Segmentation

R2-2101356	Summary of Email Report of [Post112-e][063][NR TEI16] RRC processing time with segmentation	Apple	discussion	Rel-16	NR_newRAT-Core, TEI16

Revisedin=> Revised in R2-2102261

R2-2102261	Summary of Email Report of [Post112-e][063][NR TEI16] RRC processing time with segmentation	Apple	discussion	Rel-16	NR_newRAT-Core, TEI16

R2-2101357	NR RRC processing time with segmentation	Apple	CR	Rel-16	38.331	16.3.1	2405	-	F	NR_newRAT-Core, TEI16

R2-2101358	LTE RRC processing time with segmentation	Apple	CR	Rel-16	36.331	16.3.0	4572	-	F	NR_newRAT-Core, TEI16

R2-2101359	Draft LS to RAN5 on RRC processing time with segmentation	Apple	LS out	Rel-16	NR_newRAT-Core, TEI16	To:RAN5

R2-2100979	RRC processing delay for DL RRC segmentation	Ericsson	discussion	Rel-16	TEI16

Release with Redirect – Continue from last meeting

R2-2101289	Release with Redirect in 2 steps	Ericsson	discussion	Rel-16	TEI16

R2-2101290	Release with Redirect in 2 steps	Ericsson	CR	Rel-16	38.331	16.3.1	2402	-	F	TEI16

R2-2101291	Release with Redirect in 2 steps	Ericsson	CR	Rel-16	38.306	16.3.0	0503	-	F	TEI16

R2-2101292	Release with Redirect in 2 steps	Ericsson	CR	Rel-16	38.300	16.4.0	0338	-	F	TEI16

R2-2101657	Release with redirection in 2 steps release	Huawei, HiSilicon	discussion	Rel-16	TEI16