3GPP TSG-RAN WG2 Meeting #113-e 	Draft_R2-2102155

Online, 25th Jan. - 5th Feb. 2021

Agenda Item:	9.1.3

Source: 	Huawei

Title:  	Summary of [AT113-e][305][NBIOT R17] Paging carrier selection improvements

Document for:	Discussion and Decision

# Introduction

During the online session on Monday, the summary document R2-2101045 for carrier selection improvements was discussed but no agreement was made.

This document is for the following offline discussion on paging carrier selection improvements, Week 1:

- [AT113-e][305][NBIOT/eMTC R17] Paging carrier selection improvements (Huawei)

Scope:

Week 1: Discuss the details of option 1 and 2 and try to select one

Week 2: TBD online Monday 1 Feb

Intended outcome:

Week 1: Report in R2-2102155

Week 2: TBD

Deadline:

Week 1: Jan 29 1100 UTC

Week 2: TBD Feb 04 1100 UTC

# Discussion

The document focuses on the two options listed in proposal 2 in summary document R2-2101045:

Proposal 2: RAN2 to decide how to determine paging carrier based on coverage information from the following two options:

  - Option 1: The paging carrier is determined by the UE and the eNB according to the same pre-defined rule according to the RAN level information

  - Option 2: The paging carrier is configured by the eNB via dedicated signalling

The following documents submitted to RAN2#113e may be refered:

R2-2100326		Paging carriers configuration and selection			ZTE Corporation, Sanechips

R2-2100512		Paging carrier selection procedure based on CEL		Nokia, Nokia Shanghai Bell

R2-2100671		Further discussion on enhanced paging carrier selection and NPRACH carrier selection

Spreadtrum Communications

R2-2101044		Paging carrier selection improvements				Huawei, HiSilicon

R2-2101156		Support for NB-IoT carrier selection based on the coverage level

Qualcomm Incorporated

R2-2101395		NB-IoT carrier selection and configuration based on coverage level

Ericsson

R2-2101839		Carrier selection enhancement					MediaTek Inc.

## Overview for both options

During the online session, there were comments that the description on the two options are not clear enough, e.g. are they independent solution or one can be on top of another, etc. Thus we think it would be better to look at more details for both options according to companies’ contributions before starting to discuss details or make down-selection.

Option 1: The paging carrier is determined by the UE and the eNB according to the same pre-defined rule according to the RAN level information

Option 1 is summarised based on proposals in R2-2100326 (ZTE) and R2-2100512 (Nokia):

Option 1: The paging carrier is determined by the UE and the eNB according to the same pre-defined rule according to the RAN level information R2-2100326 (ZTE): Proposal 1b: The option that UE and eNB negotiate CEL information and use a same scheme to select paging carrier based on the negotiated CEL information can be further discussed and specified.. R2-2100512 (Nokia): Proposal 2: CEL information in terms of number of repetitions for NPDCCH used at the time of RRC connection Release is stored at UE and used for paging carrier selection at the time of paging carrier monitoring. FFS whether this value is explicitly signaled or deduced by UE on release of RRC connection. Proposal 3: For paging carrier selected based on CEL, the signaling procedure described in Fig 1 is considered as baseline. In figure 1, “carrier selected based on last known CEL…”.

According to the contributions, paging carrier selection in option 1 can be described as following:

- In RRC release stage, the eNB provides coverage information (e.g. certain NPDCCH repetitions for decoding NPDCCH) used for paging carrier selection to the UE explicitly or implicitly.

- In RRC release stage, the eNB also sends the coverage information to the MME/AMF. This information will be provided back to eNB in S1 (Ng) paging message.

- The mapping relationship between the coverage information and certain paging carrier(s) is broadcasted

- The UE and the eNB are aligned on the paging carrier:

  - eNB pages the UE on the paging carrier selected based on the broadcasted relationship and the coverage information received in the S1 (Ng) paging message

  - UE monitors paging on the paging carrier selected based on the broadcasted relationship and the coverage information configured by the eNB

Please note that above procedure is only about paging carrier determination for normal case, i.e. “fall back” cases when cell or coverage changes are not described.

Companies are invited to provide general comments on above procedures for option 1.

Option 1: The paging carrier is determined by the UE and the eNB according to the same pre-defined rule according to the RAN level coverage information

| Company name | Comments |
| --- | --- |
| Ericsson | The coverage of a certain paging carrier is basically determined by two static parameters: - Power - may differ by 15 dB (nrs-PowerOffsetNonAnchor-r14=dB3...dB-12) - Rmax - may differ by 33 dB (npdcch-NumRepetitionPaging-r14=r1...r2048) For example, the following two non-anchor carriers will have the same coverage: - carrier A: dB3, r8 - carrier B: dB-6, r64 This results in that a certain "negotiated CEL information" may be applicable to many of the paging carriers in a cell. A UE with a certain coverage condition will be reached on carrier A with repetition factor r2 and on carrier B with repetition factor r16 if we assume each doubling of repetitions corresponds to ~3 dB (as the power differs by 9 dB). It is not clear for option 1 how: - the UE should select the carrier based on the above two static parameters (Power/Rmax) and the "negotiated CEL information" or - carrier specific interference levels should be taken into account (this information is only known to the eNB unless the UE would be required to make quality measurements on all paging carriers in a cell and we do not think this would be good/feasible) or - a UE in poor coverage could be configured to use a "better"/"optimized" carrier from a UE power consumption perspective or - an NB-IoT cell could support having a specific carrier used only for UEs having a certain range of coverage: - better than a certain coverage condition - worse than a certain coverage condition - a coverage condition range Detailed/complex mapping rules may be possible to find and specified but the solution then needs to be able to handle a lot of the different configurations and scenarios that could apply in a NB-IoT cell and we think this is neither straight forward nor easy to be agreed among companies. Such a solution would also tend to become very complex and error prone (e.g. ambiguity of monitored carrier) risking that it would never be deployed in the NB-IoT ecosystem. Standardizing such rules would probably also limit the flexibility for certain functions. One example of this would be eNB controlled dynamic paging load balancing in a cell where capabilities of UEs of different releases need to be considered: - Rel-13 (can only use anchor) vs - Rel-14/15/16 (can only use anchor/non-anchor based on UE_ID/weights) vs - Rel-17/18/... (can be configured based on coverage condition) We also believe that other aspects like having optimized paging carriers for certain UE specific DRX cycles and/or UE differentiation information cannot easily be done using this option (compared to option 2). |
| Qualcomm | Option 1 preferred. If UE determines the paging carrier to use based on RAN level coverage information then UE can also determine when these RAN level coverage is not longer suitable to continue to use this paging carrier and then start to use the fallback carrier till RAN level coverage is suitable again. |
| ZTE | The summary on the procedure of Option 1 is generally fine. Some clarification to the above concerns on Option 1: - For concern “This results in that a certain "negotiated CEL information" may be applicable to many of the paging carriers in a cell.”, we think it’s easy for network to provide suitable paging carrier list(s) corresponding to different CEL (range). Overlap can be avoided. - For the mentioned 4 points “requirements”, we cannot understand the #4 point. But for other points, we cannot see the issue for option 1 to implement them. For example, “carrier specific interference levels should be taken into account”, if it’s the real requirement (we doubt it), as NPDCCH repetitions anyway is evaluated and assigned by NW, it’s possible to be considered into NPDCCH repetitions. Also, for “a UE in poor coverage could be configured to use a "better"/"optimized" carrier”, if here “better” carrier means a carrier configured with more repetitions, that’s just the result of Option 1, why it is not clear? - Shortly to say, we don’t think only NW can find the best suitable carrier. CEL-based paging carrier selection can achieve same or similar results. Moreover, for option 2, to prefer a sort of carrier (power boosted carrier) for some (deep coverage) UEs is easier to cause congestion on this carrier. |
| Huawei, HiSilicon | We agree with above general description if Option 1 was chosen. But we have concern on the following aspects of Option 1: - The complexity due to define the mapping rule and let the UE to select the paging carrier. Especially if we consider to combine DRX cycle based paging carrier selection, 2-level mapping rule may be needed. - The signalling overhead to broadcast the mapping relationship between paging carrier and coverage information (maybe DRX cycle also). This will also cause UE power consumption. - A new procedure needs to be defined thus there will be big specification impact. |
| Spreadtrum | We basically agree with the description in Option1. Nevertheless, the paging carrier selection mechanism based on the combination DRX with CE level also should be taken into consideration. |
| MediaTek | Whether the coverage level is informed by eNB or reported by UE is till on discussion. |
| Sequans | We are concerned of the seemingly required complexity, especially if DRX cycle based is added as a second level. Actually, that might make for a 3-level rule, as we agree ambiguity in carrier can be expected, in which case an additional rule will be needed (UE_ID-based probably, for equal spreading). Triggering fallback in case of coverage change is unrelated to initial assignment, unless some rule of moving between several threshold is applied, which sounds complex form UE POV and paging strategy. |
| Nokia | We agree with description of option 1. But this selection is applicable only if the radio condition at the time of paging reception is better than the coverage condition at the time of selection. For this scenario additional requirement needs to be identified for paging carrier selection based on CEL. |

Option 2: The paging carrier is configured by the eNB via dedicated signalling

Option 2 is summarised based on proposals in R2-2101044 (Huawei, HiSilicon) and R2-2101395 (Ericsson):

Option 2: The paging carrier is configured by the eNB via dedicated signalling R2-2101044 (Huawei, HiSilicon): Proposal 2: The eNB configures a paging carrier to the UE via dedicated signalling and the UE monitors paging on that carrier only in the last used cell.. R2-2101395 (Ericsson): Proposal 1 Dedicated RRC configuration is introduced to allow the eNB to assign a paging carrier to a UE other than that selected based on UE_ID..

According to the contributions, paging carrier selection in option 2 can be described as following:

- The eNB configures paging carrier explicitly to the UE via dedicated signalling.

- In RRC release stage, the eNB also sends the paging carrier to the MME/AMF. The paging carrier will be provided back to eNB in S1 (Ng) paging message.

- The UE and the eNB are aligned on the paging carrier:

  - eNB pages the UE on the paging carrier received in the S1 (Ng) paging message

  - UE monitors paging on the paging carrier configured by the eNB

Please note that above procedure is only about paging carrier determination for normal case, i.e. “fall back” cases when cell or coverage changes are not described.

Companies are invited to provide general comments on above procedures for option 2:

Option 2: The paging carrier is explicitly configured by the eNB via dedicated signalling

| Company name | Comments |
| --- | --- |
| Ericsson | This option can handle many different NB-IoT cell deployment scenarios for various number of paging carriers (up to 16) with potentially different carrier power levels and interference levels. It enables a flexible and future proof solution where the eNB can optimize both spectrum resource utilization and UE power consumption. The solution is robust and non-complex and does not require much standardization efforts compared to option 1. The drawbacks/problems mentioned in our comments for option 1 are not applicable for option 2. This solution will also lead to a simple UE implementation. When the eNB assigns a carrier, it can consider a number of different aspects such as: a) UE capability b) DRX cycle support c) Coverage information d) What sort of service the UE typically requires e) Carrier power boost information f) Carrier specific interference levels (that may be dynamic) g) UE differentiation information (e.g. battery operated, traffic profile, …) h) Load balancing between UEs in the cell (Rel-13 vs 14/15/16 vs ≥17) Taking all these criteria into consideration is not possible for the UE based selection in option 1. |
| ZTE | We don’t think all the above mentioned factors are real needed. For Option 2, we have the following concerns: - To prefer a sort of carrier (e.g., power boosted carrier) for some (deep coverage) UEs is easier to cause congestion on this carrier. - It is difficult for eNB to equally distribute UEs with same CEL to different paging carriers via dedicated signaling. We don’t think it’s reasonable to let eNB to record how many UEs have been assigned on a certain paging carrier. - As eNB doesn't care the previous assignment in last connection for a certain UE, and eNB is impossible to re-assign a paging carrier for an idle mode UE when updating SIB, there is a potential issue that the assigned paging carrier becomes unavailable for a UE after SIB update. - The assigned paging carrier needs to be stored in core network and sent back to eNB in next paging. But what’s the format of the assigned paging carrier in the S1/NG interface and core network storage needs further discussion. Absolute value of EARFCN looks better for avoiding unmatched issue but it has issue of signaling overhead. - The assigned paging carrier needs to be sent to UE. The format of the assigned paging carrier in Uu interface also needs further discussion. Similarly, absolute value of EARFCN looks better for avoiding unmatched issue but it has issue of signaling overhead. |
| Huawei, HiSilicon | We agree with above general description on Option 2. |
| Spreadtrum | Such option has much restriction on the flexibility regarding paging carrier selection. Moreover, we think it is not desirable to restrict a carrier selection mechanism to be used in the last cell. |
| Sequans | We have a preference towards option 2 for its simplicity on the spec, on the UE side and its generality / future flexibility. To solve the possible congestion and carrier loss issues that ZTE raises, it could be possible to define groups of carriers (size >=1) from which the UE choses based on UE_ID. So, in the end we think this could work similarly to GWUS mechanism. |
| Nokia | If paging carrier is explicitly assigned and if the decision is based on CEL, all the fallback scenarios of UE selecting another cell and UE moving to different coverage condition also needs to be addressed. We suggest to consider signalling specific carrier index within set of carriers of CEL instead of assigning paging carrier alone. UE should know the decision is based on what condition and also the selected carrier is applicable under what conditions. |

In the following sections, the following aspects will be discussed for both options based on the summary document R2-2101045. And then based on the discussion, companies are invited to indicate preference between option 1 and option 2.

- Option 1 specific issue

- Option 2 specific issue

- Common issue, including “Fall back” mechanism and CN impact

## Option 1

For Option 1, which RAN level coverage is summarised in R2-2101045 as following:

As mentioned above, if Option 1 is agreed, which RAN level information to use for the UE and the eNB to determine paging carrier needs to be decided. The following proposals related to RAN level coverage information were made: R2-2100326 (ZTE): Proposal 2b: The coverage level information can be a certain Rmax/NPDCCH repetitions for decoding NPDCCH evaluated by eNB (Hereafter referred to as Rmax-paging).. R2-2100512 (Nokia): Proposal 2: CEL information in terms of number of repetitions for NPDCCH used at the time of RRC connection Release is stored at UE and used for paging carrier selection at the time of paging carrier monitoring. FFS whether this value is explicitly signaled or deduced by UE on release of RRC connection.. R2-2101044 (Huawei, HiSilicon): No explicit proposal but in the contribution: “In general, we think there are multiple types of RAN level information that can reflect the coverage of the UE in a cell, including the 3 options discussed in offline [AT112-e][302] and also some other information, e.g.: - NRSRP (and NRSRQ). Already possible to be reported to the eNB - CQI. Already possible to be reported to the eNB - Number of NPDCCH repetitions needed for the UE to decode DCI. Can be estimated by the eNB by implementation - Number of HARQ NACK received by the eNB for a given number of NPDSCH repetitions. Can be observed by the eNB”. R2-2101156 (Qualcomm): Proposal 2: RAN2 discuss whether RAN1 and/or RAN4 should be asked suitability of physical layer metric based on NPDCCH BLER for paging carrier selection.. R2-2101395 (Ericsson): No explicit proposal but in the contribution: “2. The eNB assigns a new paging carrier different from the one based on UE_ID to the UE in connected mode or during connection establishment, whose attributes (e.g. Rmax) can be provided in a dedicated message and/or broadcasted signaling.”. R2-2101839 (MediaTek): Proposal 1: Use NRSRP or estimated NPDCCH BLER to metic the coverage level.. According to above proposals, it seems NPDCCH repetitions for decoding NPDCCH is preferred by (or acceptable to) more companies (5 companies think it is OK, 1 company suggests to ask RAN1/4). Proposal 3: If Option 1 in Proposal 2 is agreed, NPDCCH repetitions for decoding NPDCCH is used for paging carrier selection.

Question 1: For Option 1, do you agree to use NPDCCH repetitions for decoding NPDCCH for paging carrier selection? If not, which coverage information to use?

| Company name | Yes/No | Comments |
| --- | --- | --- |
| Ericsson | No | There are two aspects of this: - Carrier selection when entering IDLE mode in a cell: We do not believe that only NPDCCH repetitions for carrier selection between different carriers can be used as also other aspects needs to be taken into consideration such as carrier power levels and interference levels are needed. However, we do not want the UE to measure this on several carriers in a cell before selecting a carrier. - Carrier selection maintenance in a cell: We believe that once a carrier is used in a cell using estimated NPDCCH repetitions to determine if a “default”/“fallback” carrier should be chosen can be used. |
| Qualcomm | Maybe | This needs more discussion because how can the UE know what NPDCCH repetition level is suitable for the coverage level it is in currently? |
| ZTE | Yes | The npdcch-NumRepetitionPaging (e.g. NPDCCH repetitions for decoding NPDCCH) is already used to express CEL (e.g. in the UEPagingCoverageInformation-NB container), it can be used for CEL-based paging carrier selection. This CEL information would be provided in the RRC release stage, this is the only suitable time. Existing signaling in RAN3 specs can be re-used as much as possible. We are not clear the Ericsson’s comments. What we are talking is paging carrier selection. |
| Huawei, HiSilicon | Yes | We think estimated number of PDCCH repetitions needed to decode PDCCH for paging can be used for Option 1. |
| Spreadtrum | Yes | The NPDCCH repetitions can indirectly reflect coverage information. Therefore, it can be used for paging carrier selection. |
| MediaTek | Yes, but | We agree with Ericsson that the carrier power level should be taken into consideration. But regarding to the interference level, eNB can evaluate the interference level of each paging carrier, but it’s just what is seen from eNB side, it would be totally different from what UE actually face to. |
| Sequans | Yes, but | We think this is a good parameter, but additional parameters such as carrier power levels are needed. |
| Nokia | May be | The coverage level information sent to CN at the time of connection release indicate the number of NPDCCH repetition. The same information should be sent to UE so that when UE wake-up for paging in the same radio condition, there is match between network and UE for selection of same paging carrier |

For option 1, how to provide the coverage information to the UE is summarised in R2-2101045 as following:

How to negotiate the coverage information between the UE and the eNB also needs to be discussed for Option 1. Corresponding proposals were made in the contributions: R2-2100326 (ZTE): Proposal 3a: eNB sends Rmax-paging information to not only MME/AMF but also UE during connection release stage.. R2-2100512 (Nokia): Proposal 2: CEL information in terms of number of repetitions for NPDCCH used at the time of RRC connection Release is stored at UE and used for paging carrier selection at the time of paging carrier monitoring. FFS whether this value is explicitly signaled or deduced by UE on release of RRC connection.. Based on above two proposals, for Option 1, it is proposed that the eNB sends the coverage information to the UE during RRC connection release. Proposals 4: If Option 1 in Proposal 2 is agreed, the eNB can send the coverage information used for carrier selection to the UE during RRC connection release.

Question 2: For Option 1, do you agree that the eNB can send the coverage information used for carrier selection to the UE during RRC connection release?

- If yes, whether the value is explicitly signalled in RRC release message or derived by the UE?

- If no, why?

| Company name | Yes/No | Comments |
| --- | --- | --- |
| Ericsson | No | As indicated in previous comments we do not think it is clear what the “coverage information” would contain to get a good solution for various scenarios. We do not believe that Rmax and estimated NPDCCH repetition level would be enough to be signalled to the UE so that it autonomously selects a paging carrier that optimizes the system. |
| Qualcomm | Maybe | As for Question 1, more discussion needed to understand what information needs to be provide to the UE and whether this information should be in broadcast signalling or dedicated signalling. |
| ZTE | Yes | Since the CEL info is used for UE in idle mode and it should be consistent between eNB and UE, it is necessary that the value is explicitly signaled to UE, e.g., via RRC release message. |
| Huawei, HiSilicon | Yes | We think explicit configuration is simpler and we do not see clear benefit to let the UE derive by itself. |
| Spreadtrum | Yes | Whether the value is explicitly signalled in RRC release message or derived by the UE is mainly depends on the broadcasted content of the relationship between coverage information and certain paging carrier(s). If the coverage information stated as coverage level and only NPDCCH repetitions is signalled, the UE needs to derive the coverage level from NPDCCH repetitions. |
| MediaTek | No | It’s possible to reuse the channel quality report by UE, which is simpler. |
| Sequans | Maybe | Depends on all the parameters that will need to be used. Some may be given in RRC release, some given by SIB. Derived by UE is unlikely, unless we can be sure UE and NW will have the same conclusion in all cases |
| Nokia | Yes | Details to be discussed. |

## Option 2

For option 2, how to configure the carrier to the UE is summarised in R2-2101045 as following:

For Option 2 in Proposal 2, since there is no coverage information exchange between the UE and the eNB, it should be up to eNB implementation to configure the paging carrier to the UE considering RAN level coverage information. Proposal 5: If Option 2 in Proposal 2 is agreed, it is up to eNB implementation to take any RAN level coverage information into consideration when configuring the paging carrier.

Question 3: For Option 2, do you agree that it is up to eNB implementation to take any RAN level coverage information into consideration when configuring the paging carrier? If not, why?

| Company name | Yes/No | Comments |
| --- | --- | --- |
| Ericsson | Yes | The eNB has all necessary information to be able to optimize what carrier the UE should monitor during IDLE mode when residing in the last connected cell. |
| Qualcomm | May be | Even if the eNB decides on which paging carrier to use, how can the UE know the configuration of this paging carrier is suitable for the current coverage level? |
| ZTE | No | For this option, we have concerns as commented in section 2.1. Even what information will be used by eNB can be left to eNB implementation, it’s obvious complicated or even infeasible for eNB to distributing UEs among carriers via paging carrier assignment. |
| Huawei, HiSilicon | Yes | Considering that it is already possible for the eNB to get several information related to the coverage of the UE, we think it is up to eNB implementation regarding how to configure the paging carrier to the UE. By the way, if we go with Option 2, we think DRX cycle based paging carrier selection can also be achieved in a similar way, i.e. it is up to eNB implementation to consider the DRX cycle used by the UE when configuring paging carrier. |
| MediaTek | Yes |  |
| Sequans | Yes | We think gNB should have enough information do determine a good carrier for the UE (including DRX cycle). |
| Nokia | No | If ENB decides to assign carrier based on CEL, it should be also known to UE. This will enable UE to consider other options if the CEL on paging occasion is different. |

Question 4: For Option 2, which message should be used to configure paging carrier for the UE?

| Company name | Comments |
| --- | --- |
| Ericsson | It should be supported in at least RrcConnectionRelease but other messages could also be discussed. |
| Qualcomm | Too early to decide on the exact signalling. |
| Huawei, HiSilicon | Since the paging carrier will only be monitored in RRC_IDLE, we think RRC connection release message is enough. |
| Spreadtrum | Same viewpoint as Ericsson. |
| MediaTek | Agree with HW |
| Sequans | RRCConnectionRelease seems like a natural choice, though we are open to discussion. Some information can also be based on common signalling |
| Nokia | Further discussion needed. |

## Common aspects

### “Fall back” mechanism when cell or coverage changes

In companies’ contributions, “Fall back” mechanism when cell or coverage changes is mentioned for both options.

#### Cell changes

For the case that the UE moves to another cell, which carrier to monitor paging is summarised in R2-2101045 as following:

The following proposals were made regarding the case that the UE with coverage based paging carrier changes its cell: R2-2101044 (Huawei, HiSilicon): Proposal 4: Upon moving to another cell, the UE monitors paging as in legacy in the new cell.. R2-2101395 (Ericsson): Proposal 3 Dedicated and/or broadcast based RRC configuration is introduced for a UE to use a paging carrier other than the dedicated paging carrier in case a new cell is selected.. R2-2101839 (MediaTek): Proposal 2:It is allowed to use the same selected paging carrier in another cell when possible.. According to above proposals, two companies think that the coverage based paging carrier (selected in Option 1, configured in Option 2) cannot be used in the new cell. One company thinks this should be allowed if possible. Thus, it is proposed: Proposal 6: Upon moving to another cell, the UE does not monitor paging on the carrier selected/configured in the previous cell. FFS which carrier to use: - The carrier selected as in legacy - Another preconfigured carrier

For both options, if the UE moves to a new cell, we need to discuss which carrier to monitor in the new cell. The possible solutions for this issue may be different between option 1 and option 2.

Question 5: For option 1, upon moving to another cell, which paging carrier should be monitored by the UE?

  - Paging carrier selected in the new cell based on the previously configured coverage information

  - The carrier selected as in legacy

  - Other

| Company name | Comments |
| --- | --- |
| Ericsson | We think there can be multiple options and the carrier to use could be configured by the eNB in for example SIB22-NB by indicating one of the following (exact details/options to be discussed more): anchor carrier, legacy/UE-ID based or same as previous cell (if available and coverage conditions allow). Hence it should be left to NW to decide depending upon the deployment scenario. |
| Qualcomm | The legacy paging carrier selection scheme can be the default and study other approaches and then make final decision. |
| ZTE | The carrier selected as in legacy. When UE moves to another cell, it’s naturally for the UE to deactivate the CEL-based paging carrier selection. |
| Huawei, HiSilicon | We think the carrier selected as in legacy is the simplest solution. We do not think using the paging carrier selected in the new cell based on the previously configured coverage information is a good way as it is very likely that the coverage in the new cell will be changed. Thus the same coverage information is not valid in the new cell. |
| Spreadtrum | Upon moving to another cell, the UE needs to select paging carrier based on the previous determined coverage and the broadcasted mapping relationship between coverage information and paging carrier(s) in the new cell. |
| MediaTek | To allow the UE to benefit from paging carrier selection even when moving to another cell, UE can be allowed to select the same paging carrier when: - The coverage level has not been changed when moving to another cell - The paging carrier configuration in SIBs are same as the pervious cell |
| Sequans | We prefer legacy carrier selection as it simplest. Even if the selection is based on some rules and/or broadcast information we would expect possible discrepancies between UE and NW as coverage information may be very different in a new cell. |
| Nokia | If the new cell is configured with CEL based paging carrier configuration, UE need to select carrier in new cell based on CEL. |

Question 6: For option 2, upon moving to another cell, which paging carrier should be monitored by the UE?

  - The carrier selected as in legacy

  - Another preconfigured carrier

  - Other

| Company name | Comments |
| --- | --- |
| Ericsson | Same answer as for Question 5 above |
| Qualcomm | Same reply as to Q5. |
| ZTE | Not so sure, maybe also the carrier selected as in legacy is the safe choice. Any other ways, e.g., assigning other dedicated carrier seems infeasible as NW cannot predict which target cell UE would go. If only anchor carrier can be used, the benefit of multi-carrier paging scheme would be lost after UE moving. Anchor carrier in those target cells are easy to congestion. |
| Huawei, HiSilicon | We think the carrier selected as in legacy is the simplest solution. We do not think using another preconfigured carrier in the new cell is feasible and beneficial. The eNB cannot predict which new cell the UE will camp on and pre-configure paging carrier used in other cell will need eNB negotiation. |
| Spreadtrum | The UE needs to select paging carrier as in legacy. |
| MediaTek | The carrier selected as in legacy. |
| Sequans | Legacy carrier selection, see Q5. |
| Nokia | See our response to Q5 |

#### Coverage changes

For the case that the UE is still in the same cell but the coverage changes, which carrier to monitor paging is summarised in R2-2101045 as following:

The following proposals were made regarding the case that the UE with coverage based paging carrier changes its coverage: R2-2100326 (ZTE): Proposal 3f: RAN2 discuss the following two schemes to deal with the case that UE detects the change of situation and determines the previous Rmax-paging information is no longer suitable: - Scheme 1: UE sends another request to eNB to indicate the change and eNB can re-estimate the Rmax-paging and send updated Rmax-paging to UE and core network. Or UE can also request to completely deactivate the CE level based paging carrier selection. - Scheme 2: The eNB can assign an additional Rmax-paging-fallback to UE along with provision of the evaluated Rmax-paging. When UE detects the radio situation change, e.g., it’s different from Rmax-paging, the UE can use this assigned Rmax-paging-fallback to select paging carrier. And eNB can use Rmax-paging-fallback after the first time paging failure, e.g., to send paging on both the carrier determined by the Rmax-paging and the carrier determined by Rmax-paging-fallback.. R2-2100512 (Nokia): In Figure 1, “UE select paging carrier closest to the estimated repetition as per current RSRP” Proposal 4: RAN2 to discuss further enhancements to minimize the impact of mismatch of selected paging carrier at UE and ENB.. R2-2101044 (Huawei, HiSilicon): Proposal 6: In case“coverage change” happens, the UE monitors paging on the carrier selected based on legacy mechanism (before Rel-17). Proposal 7: The eNB pages the UE only on the configured paging carrier at the first attempt and then on both the configured and legacy paging carrier.. R2-2101156 (Qualcomm): Proposal 5: With paging carrier selection based on coverage level, avoid mechanism that require UE to report coverage level when coverage level changes.. R2-2101395 (Ericsson): Proposal 2 Dedicated and/or broadcast based RRC configuration is introduced for a UE to use a paging carrier other than the dedicated paging carrier in case its coverage level/condition deteriorates within the cell.. R2-2101839 (MediaTek): Proposal 3: No autonomous indication from UE to network when radio condition deteriorates. Proposal 4: The UE can switch back to the default paging carrier when radio condition deteriorates. Different solutions were described in above proposals, but we think there are two aspects in common: - When coverage changes, the mechanism for the UE to indicate the change to the NW can/should be avoided. It is possible to avoid coverage change report in all of the solutions (including Scheme in R2-2100326) - When coverage changes, a “default” carrier can be used for the UE to monitor paging. The “default” carrier is determined differently in companies’ contributions: - Calculated/derived by pre-defined rule (R2-2100326, R2-2100512) - Determined based on legacy mechanism (R2-2101044) - Preconfigured carrier (R2-2101395) The following are proposed when coverage changes based on above. Proposal 7: Avoid mechanism that requires UE to report the update of coverage when coverage changes. Proposal 8: In case“coverage change” happens, the UE monitors paging on a “default” paging carrier. FFS how to determine the “default” carrier: - Calculated/derived by pre-defined rule - Determined based on legacy mechanism - Preconfigured carrier

Question 7: For both options, do you agree that when coverage changes, mechanism that requires UE to report the update of coverage when coverage changes should be avoided?

| Company name | Yes/No, why? | Comments |
| --- | --- | --- |
| Ericsson | Yes | - UE should not provide report for this thing. |
| Qualcomm | May be | In principle it is not a good idea for UE to report change in its coverage level as this increases power consumption and diminishes one of the possible benefit of this feature. |
| ZTE | Yes | Considering that the CEL-based paging carrier selection would be mainly used for geo-stationary or low-mobility UEs, we assume coverage will not change frequently. Thus UE reports its coverage change might not increase signaling overhead and UE power consumption much. But we can understand the concerns from UE vendors as such report might trigger a RA procedure for UE in idle. So we are fine to not pursue such report. |
| Huawei, HiSilicon | Yes | We think the report of coverage update will add UE complexity and power consumption thus should be avoided. |
| Spreadtrum | Yes | Reporting the update of coverage when coverage changes will cause much signalling overhead and power consumption, which is not desirable in NB_IoT. |
| MediaTek | Yes | It cost too much signaling overhead and power consumption. |
| Sequans | Yes | It is a long-standing understanding that UE should not be required to report CEL change in Idle. The additional power consumption is very unlikely to be worth it, and the UE can always go back to Connected if it thinks it would be. |
| Nokia | Yes | Reporting of coverage change is not needed in this case. |

Question 8: For both options, in case“coverage change” happens, do you agree that the UE monitors paging on a “default” paging carrier. If no, why? If yes, which option do you prefer for the “default” carrier?

  - Calculated/derived by pre-defined rule

  - Determined based on legacy mechanism

  - Preconfigured carrier

  - Other

| Company name | Yes/No | Comments |
| --- | --- | --- |
| Ericsson | Legacy mechanism or preconfigured carrier; pls see comments | We think there can be multiple options and the carrier to use could be configured from the eNB by either dedicated signalling or in for example SIB22-NB by indicating one of the following (exact details/options to be discussed more): anchor carrier, legacy/UE-ID based, no “default”/”fallback” or any other specific carrier (). Hence it should be left to NW to decide depending upon the deployment scenario. |
| Qualcomm | May be | As with cell change, we think the default can be to select paging carrier based on legacy scheme and study other schemes before making final decision. |
| ZTE | No | Here we assume the explicit issue is UE in same cell but just CEL changes. For option 2, the more possible way for dealing with “coverage change” may be to fall-back to a “default” paging carrier. But for Option 1, we have the following ways (maybe still rely on CEL information other than monitor an explicitly indicated carrier): - As mentioned in [R2-2100326], an additional NPDCCH repetitions-fallback might be assigned from NW to UE along with provision of the evaluated NPDCCH repetitions (the value mentioned in Q1). When UE detects CEL change, the UE can use this assigned NPDCCH repetitions-fallback to select paging carrier. And eNB can use NPDCCH repetitions-fallback after the first time paging failure, e.g., to send paging on both carriers determined by the NPDCCH repetitions and NPDCCH repetitions-fallback. - A simpler way may be, UE can still monitor the carrier selected based on the NPDCCH repetitions assigned previously, but with a larger NPDCCH repetitions for NPDCCH reception. E.g., the paging carrier does not need to change and the successful paging can be guaranteed by more NPDCCH repetitions (eNB should also use the larger NPDCCH repetitions for transmission). |
| Huawei, HiSilicon | Yes | We think a “default” carrier is needed in this case. We prefer to use legacy carrier as the “default” carrier as it is the simplest solution. We cannot see clear benefit of other solutions. |
| Spreadtrum | Yes | When “coverage change” happens, it is better to determine paging carrier based on the legacy mechanism, which is easy for both UE and eNB to align the used paging carrier. |
| MediaTek | Yes |  |
| Sequans | Yes | We would prefer to use legacy carrier as it is the simplest solution and we don’t see much benefit to other solutions. However, we are more open (compared to cell change scenario) to consider another preconfigured carrier if benefits can be shown. |
| Nokia | May be | If there is CEL based carriers are configured, UE need to select based on CEL. Default carrier will not work unless if there is one carrier (anchor carrier) which always support maximum repetition. |

For both options, regarding how to determine whether “coverage change” has happened, the following is summarised in R2-2101045:

In all options in Proposal 8, it is obvious that a criterion is needed for the UE to determine whether “coverage change” has happened. This was proposed in the following contributions: R2-2100326 (ZTE): Proposal 3f: RAN2 discuss the following two schemes to deal with the case that UE detects the change of situation and determines the previous Rmax-paging information is no longer suitable: - Scheme 1: UE sends another request to eNB to indicate the change and eNB can re-estimate the Rmax-paging and send updated Rmax-paging to UE and core network. Or UE can also request to completely deactivate the CE level based paging carrier selection. - Scheme 2: The eNB can assign an additional Rmax-paging-fallback to UE along with provision of the evaluated Rmax-paging. When UE detects the radio situation change, e.g., it’s different from Rmax-paging, the UE can use this assigned Rmax-paging-fallback to select paging carrier. And eNB can use Rmax-paging-fallback after the first time paging failure, e.g., to send paging on both the carrier determined by the Rmax-paging and the carrier determined by Rmax-paging-fallback.. R2-2100512 (Nokia): In Figure 1, “If CEL lesser than last known … Else,…”. R2-2101044 (Huawei, HiSilicon): Proposal 5: A criterion is defined for the UE to determine whether “coverage change” has happened. Details are FFS.. R2-2101395 (Ericsson): No explicit proposal but in the contribution: “Another item to standardize is how the UE should detect the intra-cell coverage condition change and thus if the assigned paging carrier is no longer good enough and needs to be changed (for example according to one of the above three alternatives). How this should be solved needs to be evaluated further but it could for example be based on a change in NRSRP or an estimated NPDCCH BLER using paging Rmax going above a certain percentage threshold, e.g. X%. The latter could be based on same/similar criteria as already used for the Msg3 CQI reporting. ”. According to comments received in the offline email, the following two options are possible for the UE to determine whether “coverage change” has happened: - Option 1: A criterion is specified in the specification. Details are FFS. - Option 2: Leave it to UE implementation. Proposal 9: RAN2 to decide how does the UE determine whether “coverage change” has happened from the following options: - Option 1: A criterion is specified in the specification. Details are FFS. - Option 2: Leave it to UE implementation.

Question 9: For both options, regarding how the UE determines whether “coverage change” has happened, do you think a criterion needs to be specified in the specification? Or is it OK to leave it to UE implementation?

| Company name | Yes/No, why? | Comments |
| --- | --- | --- |
| Ericsson | Yes |  |
| Qualcomm | May be | This aspect is important to study before deciding whether it should be left to implementation or be a spec defined criteria. |
| ZTE | - | Not sure now. Maybe at least such criterion doesn’t need to be configured from NW to UE. In our previous thought, it can be left to UE implementation. |
| Huawei, HiSilicon | Yes | We think a criterion needs to be specified. We assume that the “default carrier” will have more repetition number and the eNB will page the UE first on the selected/configured carrier and then on the “default carrier”. If the criterion is not specified, it is difficult for the eNB to configure properly and the UE may lose the first paging with higher probability. |
| Spreadtrum |  | It can be left to UE implementation. |
| MediaTek | Yes |  |
| Sequans | Maybe | Especially if the fallback carrier is the legacy carrier we think this can be left to UE implementation, as in either case the NW cannot know when the UE “falls back”. If another preconfigured carrier is agreed then some rule is probably needed so it can be well configured. |
| Nokia | May be | Some threshold for coverage change may be useful for UE to decide on the fallback mechanism. |

### CN impact

For both options, the CN impacts are summarised in R2-2101045 as following:

For coverage based paging carrier selection, the following proposals were made regarding CN impact on the signalling: R2-2100326 (ZTE): Proposal 3a: eNB sends Rmax-paging information to not only MME/AMF but also UE during connection release stage.. R2-2100512 (Nokia): No explicit proposal but see the CEL-Infor in S1 Release/Paging message in Figure 1.. R2-2101044 (Huawei, HiSilicon): Proposal 3: The configured paging carrier is added to the UEPagingCoverageInformation-NB container, transmitted transparently from eNB to MME(AMF) and provided it back to eNB in S1 (Ng) paging message.. R2-2101395 (Ericsson): No explicit proposal but in the contribution: “3. When the UE is released to idle mode the eNB via paging container informs the MME about the assigned paging carrier and the UE monitors paging using the assigned carrier when it camps on the same cell where it was released and experiences similar coverage conditions.”. All above four companies agree that some information related to coverage needs to be exchanged between the eNB and MME(AMF) but different understanding and have the same view on the signalling procedure, but companies have different view on the content to be exchanged because of different preferences between Options 1 and 2 in Proposal 2. Thus, we suggest the following proposal: Proposal 10: The information related to coverage based paging carrier selection is added to the UEPagingCoverageInformation-NB container, transmitted transparently from eNB to MME(AMF) and provided back to eNB in S1 (Ng) paging message. The details of the information depends on the outcome of Proposal 2: - For Option 1, the coverage information used for carrier selection - For Option 2, the configured carrier

Question 10: For both options, do you agree with above proposals 10?

| Company name | Yes/No, why? | Comments |
| --- | --- | --- |
| Ericsson | Yes |  |
| Qualcomm | May be | There is likely to be impact on eNB-CN signalling to restrict the eNB to use a specific paging carrier. Again, the exact details will depend on the eventual solution. |
| ZTE | Yes | Proposal 10 can be baseline. For option 1, existing signaling and IE in RAN3 spec can be re-used as much as possible. Moreover, in current spec, Cell Identifier and Coverage Enhancement Level information can already be provided in the S1AP/NgAP procedure, this CEL info can be used for this purpose. To differentiate from the legacy paging carrier behavior, an additional simple indication might be considered to indicate that CEL-based paging carrier is used. |
| Huawei, HiSilicon | Yes |  |
| Spreadtrum | Yes |  |
| MediaTek | Yes |  |
| Sequans | Yes | This can be agreed as WA or baseline if companies have concerns. At least for option 2, it could be a group of carriers. |
| Nokia | Yes |  |

## Option 1 v.s. Option 2

Based on above discussion, please companies provide comments on pros. and cons. for each option and indicate your preference.

- Option 1: The paging carrier is determined by the UE and the eNB according to the same pre-defined rule according to the RAN level coverage information

- Option 2: The paging carrier is explicitly configured by the eNB via dedicated signalling

Question 11: Pros. and cons. for each option and companies’ preference

| Company name | Preference | Comments, Pros. and Cons. |
| --- | --- | --- |
| Ericsson | Option 2 | - Option 2 compared to Option 1: - is more flexible as it can handle many different deployment scenarios including different number of carriers having different power/interference levels, - is less complex, - requires less standardization efforts, - can better optimize both spectrum resources and UE power consumption, - can be combined with other service specific aspects such as UE specific DRX and UE differentiation information, - can achieve better paging carrier load balance - Option 1 has uncertainties such as: - details on what “negotiated CEL information” is (as we do not think Rmax and/or estimated NPDCCH repetition level would be enough) how selection rules should be standardized given that a NB-IoT cell may contain up to 16 carriers used for paging having a set of very different characteristics (Rmax, Power level, Interference level) where the system would benefit of using certain carriers for certain services and/or coverage situation for a set of UEs |
| Qualcomm | - | For both solutions need to look at the solutions to decide on pros/cons. Making judgement on hypothetical solution is not helpful. Companies should present the design for their preferred solution with the design answering as many of the questions above. |
| ZTE | Option 1 | Pros. - It is a simple solution and tries to align with the legacy paging carrier selection scheme (maybe we can say the difference is just to use a sub set paging carriers which corresponding to UE’s CEL). The effect of distributing UE among carriers still can be kept. - eNB provide CEL based paging carrier list, in which different npdcch-NumRepetitionPaging, nB, defaultDrxCycle can be configured for different carrier. Moreover, these carriers can be selected by UEs with corresponding CEL. This can achieve carrier differentiation to fulfil UE’s different service requirements and make trade-off between performance and resources efficiency. - To have less impacts on RAN3/SA2 specs. Cons. Without reporting CEL change from UE to network, we need some “fallback” scheme to guarantee the successful paging. But it seems this is needed for any of options. The Cons. for option 2 have been mentioned in Section 2.1 and we cannot see obvious benefits. |
| Huawei, HiSilicon | Option 2 | For Option 1, we do not see any additional benefit compared to Option 2. But the spec impact and complexity is very big at least because: - A coverage information needs to be chosen/defined and we need to specify a rule for the UE to select paging carrier, which is complicated from the UE point of view - The mapping relationship between coverage information and the paging carrier will cause lot of signalling overhead and UE power consumption (especially 2-level mapping may be needed if we consider to combine DRX based carrier selection). - A new procedure needs to be defined. Option 2 is much simpler than Option 1. In addition, for Option 2: - All the information related to coverage can be used by the eNB by implementation when configuring the carrier to the UE. - It is much easier to support DRX based carrier selection. Thus we prefer Option 2. |
| Spreadtrum | Option 1 | Taking into consideration of the pros/cons of the two options, it can be find that option 1 is more flexible to select a paging carrier than option 2. Moreover, for option 2, the new paging carrier selection mechanism cannot be used when the UE moves out of the last cell. |
| MediaTek | Option 1 | In option 1, It’s possible for UE to benefit from paging carrier selection when moving to another cell. It’s not possible for option2. |
| Sequans | Option 2 | Considerations are already presented in 2.1-2.3, and we agree in general with HW and Ericsson |
| Nokia | - | Further discussion needed on the details of solution considering all the possible scenarios of coverage and cell change from RRC connection release to paging occasion |

# Conclusion

This paper focused on coverage based paging carrier selection improvements. Corresponding proposals are listed as follows:

TBD

# References

# Contact delegates

| Delegate | Company name | Email |
| --- | --- | --- |
| Baokun Shan | Huawei, HiSilicon | baokun.shan@huawei.com |
| Ritesh Shreevastav | Ericsson | Ritesh.shreevastav@ericsson.com |
| Mungal Dhanda | Qualcomm | mdhanda@qti.qualcomm.com |
| Ting Lu | ZTE | lu.ting@zte.com.cn |
| Xu Liu | Spreadtrum | xu.liu1@unisoc.com |
| Aaron Cai | MediaTek | Aaron.cai@mediatek.com |