3GPP TSG-RAN WG2 Meeting #113 bis electronic	R2-2102794
Online, April 12 – April 20, 2021

Source:	OPPO

Title:	Paging with Service Indication

Agenda Item:	8.3.4

Document for:	Discussion and Decision

- Introduction

In RAN#91e meeting, a revised WID scope of Rel-17 Multi-SIM WI was agreed [1]:

- Specify, if necessary, enhancement(s) to address the collision due to reception of paging when the UE is in IDLE/INACTIVE mode in both the networks associated with respective SIMs [RAN2]

  - RAT Concurrency: Network A can be NR or LTE. Network B can either be LTE or NR.

  - Applicable UE architecture: Single-Rx/Single-Tx.

- Specify mechanism for UE to notify Network A of its switch from Network A (for MUSIM purpose) [RAN2]:

  - RAT Concurrency: Network A is NR. Network B can either be LTE or NR.

  - Applicable UE architecture: Single-Rx/Single-Tx, Dual-Rx/Single-Tx

- Unless SA2 find an alternative solution or decides otherwise, specify mechanism for an incoming page to indicate to the UE whether the service is voLTE/VoNR[ RAN2].

  - RAT Concurrency: Network A is either LTE or NR. Network B is either LTE or NR.

  - Applicable UE architecture: Single-Rx/Dual-Rx/Single-Tx

UE SIMs may belong to same or different operators.

USIM can be a physical SIM or eSIM.

Coordination with relevant WGs, such as SA2, should be considered where relevant.

For objective 1, specification change should focus on NR side and the change on LTE side is only for IDLE mode (i.e. related to EPC enhancement in SA2)

And the following agreements were made in RAN2#112e meeting for objective 3 [2]:

Provide SA2 with information on paging cause costs based on the email discussion + contributions. Indicate that this may change if assumptions change.

From RAN2 perspective, we haven't decided on paging cause feasibility yet.

RAN2 will evaluate short/long time switching in this WI

In this contribution, we will focus on the highlighted yellow parts, i.e. paging cause.

- Discussion

In RAN2#112e meeting, a reply LS was sent to SA2 and for paging cause, the following response was agreed in the LS [3]:

Paging Cause

Q1) Please confirm the feasibility and overhead of sending a Paging Cause in [Uu] Paging message for EPS and for 5GS. [RAN2, RAN3]

Q2) Please indicate whether adding the paging cause (e.g.  3-4bits) per UE in the paging message would reduce the number of paging records that could be included in a single paging message, and if so by what magnitude. (For NR and E-UTRA) [RAN2]

Q3) Please indicate how the paging cause is expected to be supported in RAN nodes (e.g. per PLMN, per TA, per RAN node, per cell) (For NR and E-UTRA) [RAN2, RAN3]

RAN2 haven’t decided on paging cause feasibility, including how it should be supported in RAN nodes at this meeting.

For Q2, the overhead of paging cause is expected additional 1 + log2(number of paging cause) bits per UE at most in NR and LTE, if a parallel list (a solution adopted in R16 LTE paging message) is applied for introducing paging causes.

This means that, assuming 3 bits paging cause, additional 17 bytes (9 bytes) are expected at most when keeping the maximum number of paging records in NR (LTE) paging message, which is roughly ~8%  for NR and ~6% for LTE increase in size at most. Whether 3/4 bits paging cause will impact the real deployment about paging volume and coverage is still under RAN2’s discussion.

- The above overhead analysis may be changed based on SA3 feedback on security requirements for paging cause.

SA2 group already agreed that only ‘voice’ is introduced for Paging cause, but RAN2 has not decided the feasibility to introduce paging cause in RAN side. According to overhead evaluation in the reply LS, the extra paging overhead introduced by paging cause is still acceptable from RAN2 perspective.

Observation1: The extra paging overhead introduced by paging cause is still acceptable in RAN2.

On the top of above, it’s feasible to introduce paging cause.

Proposal 1: From RAN2 perspective, it’s feasible to introduce paging cause into paging message.

Only ‘voice’ is defined by SA2 for paging cause, but from RAN2 signalling perspective, it’s still unclear whether a paging cause ‘other’ is needed to explicitly differentiate ‘voice’ from ‘non-voice’. RAN2 can consider this further later.

Proposal 2: Only ‘voice’ is defined for paging cause.

FFS: whether a paging cause ‘other’ is needed to explicitly differentiate ‘non-voice’ from ‘voice’.

The next open issue is for coding on paging cause, an incoming SA2 LS on paging cause was agreed during SA2#143e meeting [4]:

SA2 has concluded that the UE shall be able to discriminate the case where it is being paged for non-voice service from the case where it is being paged (for any service).

A RAN node either:

-	does not support the Paging Cause feature (i.e. the RAN node pages (for any service)), or

-	supports and uses the Paging Cause feature (i.e. the RAN node pages for voice service, or for non-voice service), or

-	supports but does not use the Paging Cause feature when in case of RAN sharing it is configured to operate without this feature for some of the connected CNs (i.e. the RAN node pages (for any service)).

SA2 would like to ask RAN2 to take into account the above solution, and provide feedback if necessary.

Based on the LS content, SA2 would like RAN2 to clarify how to differentiate the three cases from RAN2 perspective. But in our view, this issue is not a paging cause specific issue. Every time we introduce new features in the next release, this coding issue may happen. We may solve the concern or may not, this depends on how these cases will impact UE behaviour.

Observation2: The issue listed by SA2 LS is not a paging specific issue.

Before RAN2 reply the LS, RAN2 should give some analysis, the following ASN.1 extension can be used as baseline for further discussion:

-- ASN1START

-- TAG-PAGING-START

Paging ::=                          SEQUENCE {

pagingRecordList                    PagingRecordList                                                        OPTIONAL, -- Need N

lateNonCriticalExtension            OCTET STRING                                                            OPTIONAL,

nonCriticalExtension                Paging-v17xy-IEs                                                              OPTIONAL

}

PagingRecordList ::=                SEQUENCE (SIZE(1..maxNrofPageRec)) OF PagingRecord

PagingRecord ::=                    SEQUENCE {

ue-Identity                         PagingUE-Identity,

accessType                          ENUMERATED {non3GPP}    OPTIONAL,   -- Need N

...

}

PagingUE-Identity ::=               CHOICE {

ng-5G-S-TMSI                        NG-5G-S-TMSI,

fullI-RNTI                          I-RNTI-Value,

...

}

Paging-v17xy-IEs ::=			     SEQUENCE {

pagingRecordList-v17xy			     PagingRecordList-v17xy				OPTIONAL,	-- Need N

nonCriticalExtension			     SEQUENCE {}						OPTIONAL

}

PagingRecordList-v17xy ::=			 SEQUENCE (SIZE(1..maxNrofPageRec)) OF PagingRecord-v17xy

PagingRecord-v17xy ::=				  SEQUENCE {

pagingCause-r17	ENUMERATED {voice}			OPTIONAL		-- Need N

}

-- TAG-PAGING-STOP

-- ASN1STOP

Let’s assume we have the following type of UEs in the system:

Type1: legacy UE(R15/R16 UEs);

Type2: R17 UEs without MUSIM capability;

Type3: R17 UEs with MUSIM capability;

Scenario1: The RAN node is a legacy RAN node, i.e. RAN node cannot support Paging Cause feature.

Scenario2: The RAN node support Paging Cause feature, but not all UEs send MUSIM mode indication to CN to activate Paging Cause delivery.

For Scenario1, all three type of UEs above will not receive paging cause as the pagingRecordList-v17xy is absent, the baseline ASN.1 extension can be workable.

For Scenario2, for type1 UE, UE will follow the legacy behaviour, i.e. the pagingRecordList-v17xy is invisible to legacy UE. For type2 UE, even if the type2 UE can read the R17 IE pagingRecordList-v17xy, the desirable UE behaviour is to follow the legacy behaviour, i.e. type2 UE will ignore R17 IE pagingRecordList-v17xy.

As for Type3 UE, there are two sub-cases:

Sub-case1: Type3 UE has not sent MUSIM mode indication to CN to activate Paging Cause delivery. In this case for Scenario2, the desirable UE behaviour is to follow the legacy behaviour, i.e. type2 UE will ignore R17 IE pagingRecordList-v17xy even received.

Sub-case2: Type3 UE has already sent MUSIM mode indication to CN to activate Paging Cause delivery. In this case for Scenario2, Type3 UE will process R17 IE pagingRecordList-v17xy. More addition, when paging cause is for ‘voice’, pagingCause-r17 will be set; otherwise (for non-voice case), pagingCause-r17 will be absent, so the baseline ASN.1 can also differentiate ‘voice’ and ‘non-voice’.

Based on the analysis above, the baseline ASN.1 can cover all the concerns listed in SA2 LS. In other words, SA2 concern listed in LS can be solved by RAN2 ASN .1 coding, no extra spec impact is identified.

Observation3: All the concerns/requirements listed in SA2 LS for paging cause can be solved by RAN2 ASN.1 coding.

Proposal 3: RAN2 to agree the above ASN.1 extension for paging cause as baseline and the details can be discussed further.

RAN2 should reply LS to SA2 for paging cause, the draft version is given in clause 5.

Proposal 4: Send reply LS to SA2 that all the concerns/requirements listed in SA2 LS for paging cause can be solved by RAN2 ASN.1 coding, no extra spec impact is identified.

The next issue is to discuss something for security requirements, paging message is carried by PCCH, which has no integrity and ciphering. There may have security concern if paging cause is exposed in an unprotected message. Coordination with SA3 is needed, but from RAN2 perspective, we can give some suggestion to avoid the security issue. For instance, a dynamic mapping rule can be configured from AMF to UE NAS, only UE NAS can understand the paging cause in the paging message. But this method should be checked by SA2/SA3.

Proposal 5: Postpone the security issue for paging cause.

- Conclusion

In conclusion, we propose the followings:

Observation1: The extra paging overhead introduced by paging cause is still acceptable in RAN2.

Proposal 1: From RAN2 perspective, it’s feasible to introduce paging cause into paging message.

Proposal 2: Only ‘voice’ is defined for paging cause.

FFS: whether a paging cause ‘other’ is needed to explicitly differentiate ‘non-voice’ from ‘voice’.

Observation2: The issue listed by SA2 LS is not a paging specific issue.

Observation3: All the concerns/requirements listed in SA2 LS for paging cause can be solved by RAN2 ASN.1 coding.

Proposal 3: RAN2 to agree the above ASN.1 extension for paging cause as baseline and the details can be discussed further.

Proposal 4: Send reply LS to SA2 that all the concerns/requirements listed in SA2 LS for paging cause can be solved by RAN2 ASN.1 coding, no extra spec impact is identified.

Proposal 5: Postpone the security issue for paging cause.

- References

[1] 3GPP RP-210316 Revised WID: Core part: Support for Multi-SIM devices for LTE/NR

[2] 3GPP RAN2#112e Chairman Notes

[3] R2-2010738 Draft reply LS on System support for Multi-USIM devices (S2-2006037; contact: Intel)

[4] S2-2100308 LS on System support for Multi-USIM devices

- Draft Reply LS to SA2

3GPP TSG-RAN WG2 Meeting #113 bis electronic	R2-2xxxxxx
Online, April 12 – April 20, 2021

Title:	Draft reply LS on System support for Multi-USIM devices

Response to:	R2-21XXXX

Release:	Release 17

Work Item:	LTE_NR_MUSIM-Core

Source:	RAN2

To:	SA2

Cc:	RAN3/SA3

Contact Person:

#### Name:	Jiangsheng Fan

E-mail Address:	fanjiangsheng(at)oppo(dot)com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN2 has discussed the paging cause issue listed in SA2 LS and would like to give the following feedback:

All the concerns/requirements listed in SA2 LS for paging cause can be solved by RAN2 ASN.1 coding, no extra spec impact is identified.

2. Actions:

To SA2 group.

ACTION: 	RAN2 respectfully asks SA2 to take the above info into account and gives the feedback if any.

3. Date of Next TSG-RAN WG2 Meetings:

3GPP RAN2#114-e       from 2021-05-19	to 2021-05-27		Electronic Meeting

3GPP RAN2#115-e       from 2021-08-23             to 2021-08-27                TBD		TBD