3GPP TSG-RAN WG2 Meeting #113-bis-e                             	R2-2102937

Online, April 12 – April 20 2021

Agenda item:	8.1.2.4

Source:	Samsung

Title:	On MBS DRX, Data-Inactivity & BWP aspects

Document for:	Discussion & Decision

# Introduction

In this contribution, we discuss certain issues and aspects related to DRX, Data-Inactivity & BWP operation for MBS services [1].

# Discussion

## DRX for MBS

Adhering to SCPTM principle, each MBS session can have a service specific DRX for reception with PTM mode and is associated with a specific G-RNTI. Whereas, PTP mode is closely related to legacy unicast and UE specific unicast C-DRX is suitable. There are two important aspects that need to be considered.

- Service specific DRX for PTM is also applicable for delivery mode 2. There may be a large number of broadcast services (SC-PTM considered max 1023 services). It would be efficient from power saving as well as signaling perspective to group multiple services which share same DRX pattern (unlike SCPTM which carries separate and independent signaling for DRX configuration for each service)

- Though PTM and PTP have different DRX i.e. service specific and UE specific DRX respectively, they may be linked in some cases for HARQ retransmissions. That is, PTM provides initial transmission and PTP carries HARQ retransmissions (addressed by C-RNTI or UE specific RNTI). This implies that PTP should be in active time to receive HARQ retransmissions. More specifically, drx-HARQ-RTT-TimerDL and drx-RetransmissionTimerDL should be operated for PTP when MBS initial transmission is received for PTM and HARQ retransmission is configured to be received through PTP.

Proposal 1: Each MBS session reception via PTM mode is associated with a service specific DRX and UE receives MBS traffic in the active time of the associated DRX

Proposal 2: MBS session reception via PTP mode is associated with UE specific unicast DRX.

Proposal 3: Multiple MBS services can share a common DRX configuration.

Proposal 4: For delivery mode 2, multiple MBS services that share a common DRX configuration, are grouped and signalled together in MCCH.

Proposal 5: When PTM HARQ retransmission are configured to be received on PTP, PTP should be in active time to receive HARQ retransmissions. That is, drx-HARQ-RTT-TimerDL and drx-RetransmissionTimerDL are operated for PTP when MBS initial transmission is received for PTM and HARQ retransmission is configured to be received through PTP.

## Data Inactivity for MBS

In RAN2#112e meeting, following agreement was made “When there is no data ongoing for the multicast session, the UE can stay in RRC_CONNECTED. Other cases FFS”.

For legacy connected UE, data inactivity monitoring can be applied and UE transits to Idle state once data-InactivityTimer is expired. Therefore, we see two issues with the mentioned MBS agreement

- It conflicts with the legacy mechanism of data inactivity monitoring for unicast

- There is no clear limit as to how long UE can stay in RRC_CONNECTED when there is no data ongoing for the multicast session

To address these two issues, it seems imperative that data Inactivity monitoring is applied together for unicast and MBS to decide state transition for UE. As in legacy, UE transits to Idle mode when data-InactivityTimer is expired. Further,

- It seems obvious that network can suitably configure data-InactivityTimer to consider both unicast and MBS.

- It can also be noted that in LTE the timer was not operated for eMBMS traffic because LTE eMBMS traffic can be received transparently from any RRC state, unlike NR multicast.

Proposal 6: Data Inactivity monitoring is applied together for unicast and MBS to decide state transition for UE. UE transits to Idle mode when data-InactivityTimer is expired.

## BWP Aspect in Connected Mode

UE’s interested MBS service may be ongoing on non-active BWP.

Option 1: Active BWP shall be switched for MBS service

Option 2: UE is provided MBS service in PTP mode on active BWP

Option 3: MBS-active BWP and Unicast-active BWP can be different or not receivable together. A mechanism to resolve this issue is necessary, e.g., reporting this mismatch, autonomous BWP switch for MBS, TDM-like switch, etc.

Option 4: MBS capable UE has two RF chains per cell. UE can receive both MBS BWP and active unicast BWP.

Considering the complexity from RAN2 perspective, option 1 and option 2 look simple. But there is some RAN1 impact also.

Proposal 7: RAN2 to further discuss the alternatives to receive UE’s interested MBS service ongoing on non-active BWP.

## BWP Switching

Consider that MBS service is being received on MBS BWP or frequency resource associated with unicast BWP. UE currently receiving its interested MBS services should not switch to other unicast BWP by e.g. Network’s BWP switching indication or bwp-InactivityTimer expiry in order to support MBS service reception.

- Network’s BWP switching: Network shall not indicate BWP switching to UE receiving MBS service. For multicast MBS service received by UE in connected mode, gNB is aware of it. For broadcast services in Idle/Inactive or Connected mode, MBS service would be provided in initial BWP and moreover, it is provided in best effort manner and therefore, any guaranteed mechanism is not needed.

- bwp-InactivityTimer expiry: UE is not needed to switch to default BWP or initial BWP when UE is receiving it’s interested MBS service.

Proposal 8: UE is not allowed to switch BWP at bwp-InactivityTimer expiry, if it is receiving interested MBS service.

# Conclusion

We request RAN2 to discuss and possibly agree to the proposals made as:

Proposal 1: Each MBS session reception via PTM mode is associated with a service specific DRX and UE receives MBS traffic in the active time of the associated DRX

Proposal 2: MBS session reception via PTP mode is associated with UE specific unicast DRX.

Proposal 3: Multiple MBS services can share a common DRX configuration.

Proposal 4: For delivery mode 2, multiple MBS services that share a common DRX configuration, are grouped and signalled together in MCCH.

Proposal 5: When PTM HARQ retransmission are configured to be received on PTP, PTP should be in active time to receive HARQ retransmissions. That is, drx-HARQ-RTT-TimerDL and drx-RetransmissionTimerDL are operated for PTP when MBS initial transmission is received for PTM and HARQ retransmission is configured to be received through PTP.

Proposal 6: Data Inactivity monitoring is applied together for unicast and MBS to decide state transition for UE. UE transits to Idle mode when data-InactivityTimer is expired.

Proposal 7: RAN2 to further discuss the alternatives to receive UE’s interested MBS service ongoing on non-active BWP.

Proposal 8: UE is not allowed to switch BWP at bwp-InactivityTimer expiry, if it is receiving interested MBS service.

# References

[1] RP-201038 WID revision: NR Multicast and Broadcast Services, Huawei, HiSilicon