3GPP TSG-RAN WG2 Meeting #113bis electronic	R2-2103104

Online, April 12 – April 20, 2021

Source:	CATT

Title:	Considerations on Procedures without Anchor Relocation

Agenda Item:	8.6.4

Document for:	Discussion and Decision

Introduction

RAN3 had an initial discussion on small data in RAN3#111-e. In this contribution, we would like to discuss possible solutions for context fetch/data forwarding without anchor relocation and procedure of transition into RRC_CONNECTED during SDT procedure without anchor relocation.

Discussion

- Context fetch/data forwarding without anchor relocation

In RAN2#112e meeting, we made the following assumption in LS [1]:

The RLC configuration used for the SDT DRB data will be based on a UE stored configuration. RAN2 assumption is that the RLC PDU will be processed in the receiving gNB.

In corresponding reply LS [2], RAN3 inform RAN2 about initial progress as follows:

- RAN3 discussed the assumption in the LS that RLC handling is processed in the receiving gNB and would like to ask if this is confirmed as a firm agreement in RAN2. Several companies in RAN3 proposed to analyse the topic further.

In the section, we analyze possible solutions for context fetch/data forwarding without anchor relocation from RAN2 point of view.

There are two possible solutions for data forwarding without anchor relocation:

Option 1: RLC PDU is processed in the current serving gNB. PDCP PDU for small data is forwarded between the current serving gNB and anchor gNB.

Option 2: RLC PDU is processed in anchor gNB. MAC PDU for small data is forwarded between the current serving gNB and anchor gNB.

With option 1, RLC PDU will be processed in the current serving gNB. That means the current serving gNB needs to retrieve UE context which at least contains RLC configuration used for the received SDT DRB data. But if full UE context is provided to the current serving gNB, there may be security issues as AS security info exists in both serving gNB and anchor gNB at the same time. With option 1, tunnels for forwarding UL/DL PDCP PDU need to be setup during/after UE context retrieval procedure. The same issues exists for CU-DU scenarios.

Figure 1. SDT without anchor relocation, RLC PDU is processed in the current serving gNB (Option 1)

With option 2, the first UL small data can be sent together with UE context retrieval request. Compared with option 1, this can reduce the delay for the first UL SDT packet. But in CU-DU scenario, the anchor gNB CU has to setup UE context with an arbitrary DU and RLC PDU is processed in the anchor gNB DU. This may introduce additional delay for subsequent UL/DL SDT as shown in figure 2.

Regarding option 2, the RLC layer processing of the SDT data which includes segmentation is performed at anchor gNB DL, while the MAC layer processing the SDT data which includes grant assignment is performed at Serving gNB DU. However, these two nodes are not in the same physical location. And the delay between these two nodes can’t be ignored. Therefore, there is a problem that how RLC layer in one RAN node can be aware of the transmission opportunity in MAC layer in another RAN node. Hence, the grant assignment for SDT is low efficiency which is awkward.

Figure 2. SDT without anchor relocation, RLC PDU is processed in anchor gNB (Option 2)

In our understanding, UE context retrieval procedure, tunnel establishment, and special procedures for CU/DU scenarios are in the scope of RAN3. We can leave these corresponding comparisons to RAN3. But based on the above analyses, from RAN2 point of view, we can observe that:

Observation 1: If subsequent UL small data transmissions and/or subsequent DL small data transmissions require segementation and re-segementation, there is a problem that how RLC layer in one RAN node can be aware of the transmission opportunity in MAC layer in another RAN node if RLC PDU is processed in anchor gNB.

- Procedure of transition into RRC_CONNECTED

With anchor relocation, legacy RRC resume procedure can be reused if the gNB decides to transit UE into RRC_CONNECTED during subsequent SDT.

Without anchor relocation, the current serving gNB cannot obtain the whole UE context. Small data is processed in anchor gNB with updated security key. During subsequent SDT procedure, the network may decide to transit the UE into RRC_CONNECTED. The following issues need to be discussed:

- Which node can decide to transit the UE into RRC_CONNECTED?

Option 1: The current serving gNB decides to transit the UE into RRC_CONNECTED.

Option 2: The anchor gNB decides to transit the UE into RRC_CONNECTED.

As the current serving gNB can be aware of the size of UL data to be transmitted and performance of data transmission via radio interface, it is reasonable for the current serving gNB to decide to transit the UE into RRC_CONNECTED.

On the other hand, the anchor gNB can be aware of DL data arrival. Hence, it is reasonable for the anchor gNB to trigger the transition to RRC_CONNECTED.

As the issue may impact signalling exchange via Xn interface, RAN3 can discuss and decide it further.

Observation 2: From RAN2 point of view, both serving gNB and anchor gNB can trigger the UE transition to RRC_CONNECTED in the case of without anchor relocation. Due to Xn signaling involvement, RAN3 should make the final decision.

- Which node decides the content of RRCResume message?

Option 1: The current serving gNB decides the content of RRCResume message.

Option 2: The anchor gNB decides the content of RRCResume message.

As the content of RRCResume message is related to configurations of radio interface, it is more reasonable for the current serving gNB to decide the content of RRCResume message. Option 1 is preferred.

Observation 3: The content of RRCResume message should be decided by the serving gNB in the case of without anchor relocation as serving gNB decides radio configurations.

- Which node performs ciphering and integrity protection for RRCResume message?

Option 1: The current serving gNB.

Option 2: The anchor gNB.

When the network decides to transit the UE into RRC_CONNECTED, the UE is not aware of this. The UE uses updated security key (marked as KgNB1), which is updated upon initiation of RRCResumeRequest message, to receive DL data and DL signalling (including RRCResume message). Hence, ciphering and integrity protection for RRCResume message need to be performed based on KgNB1. As KgNB1 is used by the anchor gNB for UL/DL data during SDT without anchor relocation, the anchor gNB should perform ciphering and integrity protection for RRCResume message. Option 2 should be adopted.

Observation 4: The anchor gNB should perform ciphering and integrity protection for RRCResume message in the case of without anchor relocation as PDCP is processed in anchor gNB.

- Which node is RRCResumeComplete message sent to?

Option 1: The current serving gNB.

Option 2: The anchor gNB.

Upon receiving RRCResumeRequest message, the UE enters RRC_CONNECTED mode and sends RRCResumeComplete message for response. When the network decides to transit the UE into RRC_CONNECTED mode, there is no benefit to keep the UE context and handle UL/DL data in anchor gNB. Thus, anchor relocation will be performed. And it is straightforward for the UE to send RRCResumeComplete message to the current serving gNB, which is similar as handover procedure. Option 1 is preferred.

Observation 5: The UE sends RRCResumeComplete message to the current serving gNB in the case of without anchor relocation, which is similar as handover procedure, as anchor relocation is performed when transition to RRC_CONNECTED.

- Security issue

As analyzed above, updated security key (KgNB1) will be applied to ciphering and integrity protection for RRCResume message.

And similar as handover procedure, we propose that the UE sends RRCResumeComplete message to the current serving gNB. Then subsequent UL/DL data is handled in the current serving gNB. Different security keys should be used in different network nodes. But according to current TS, security key cannot be updated via RRCResume message. Thus, we need to ask SA3 how to ensure security in this case.

The following is the general picture of transition into RRC_CONNECTED during subsequent SDT without anchor relocation.

Figure 3 Transition into RRC_CONNECTED during subsequent SDT without anchor relocation

Observation 6: Without anchor relocation, the issue that how to ensure security separation between two different gNBs after transaction to RRC_CONNECTED needs to be resolved.

- Summary

If we agree that anchor relocation is always performed if SDT cannot be completed within one shot UL/DL SDT, above observations don’t need to be considered. Hence, we propose:

Proposal 1: RAN2 discuss whether anchor relocation is always performed if SDT cannot be completed within one shot UL/DL SDT.

If we agree to support subsequent SDT without anchor relocation, considering the RLC segementation/resegementation issue in observation 1, we propose:

Proposal 2: If subsequent SDT without anchor relocation is supported, RAN2 confirm RLC handling is processed in the receiving gNB and send response LS to RAN3.

Furthermore, we need to discuss how to support the transaction into RRC_CONNECTED during subsequent SDT in the case of without anchor relocation.

Proposal 3: If subsequent SDT without anchor relocation is supported, we propose for the case of without anchor relocation:

- Ask RAN3 to decide which node can decide to transit the UE into RRC_CONNECTED.

- The current serving gNB decides the content of RRCResume message.

- The anchor gNB performs ciphering and integrity protection for RRCResume message.

- The UE sends RRCResumeComplete message to the current serving gNB.

- Ask SA3 how to ensure security key separation if UL/DL data and RRCResume message are treated by anchor gNB with updated security key then RRCResumeComplete message and subsequent UL/DL are treated by the current serving gNB.

Conclusion

In this contribution, we discuss possible solutions for context fetch/data forwarding without anchor relocation and procedure of transition into RRC_CONNECTED during SDT procedure without anchor relocation.

For context fetch/data forwarding without anchor relocation, we observe:

Observation 1: If subsequent UL small data transmissions and/or subsequent DL small data transmissions require segementation and re-segementation, there is a problem that how RLC layer in one RAN node can be aware of the transmission opportunity in MAC layer in another RAN node if RLC PDU is processed in anchor gNB.

Without anchor relocation, the network may decide to transit the UE into RRC_CONNECTED. We observe:

Observation 2: From RAN2 point of view, both serving gNB and anchor gNB can trigger the UE transition to RRC_CONNECTED in the case of without anchor relocation. Due to Xn signaling involvement, RAN3 should make the final decision.

Observation 3: The content of RRCResume message should be decided by the serving gNB in the case of without anchor relocation as serving gNB decides radio configurations.

Observation 4: The anchor gNB should perform ciphering and integrity protection for RRCResume message in the case of without anchor relocation as PDCP is processed in anchor gNB.

Observation 5: The UE sends RRCResumeComplete message to the current serving gNB in the case of without anchor relocation, which is similar as handover procedure, as anchor relocation is performed when transition to RRC_CONNECTED.

Observation 6: Without anchor relocation, the issue that how to ensure security separation between two different gNBs after transaction to RRC_CONNECTED needs to be resolved.

If we agree that anchor relocation is always performed if SDT cannot be completed within one shot UL/DL SDT, above observations don’t need to be considered. Hence, we propose:

Proposal 1: RAN2 discuss whether anchor relocation is always performed if SDT cannot be completed within one shot UL/DL SDT.

Proposal 2: If subsequent SDT without anchor relocation is supported, RAN2 confirm RLC handling is processed in the receiving gNB and send response LS to RAN3.

Proposal 3: If subsequent SDT without anchor relocation is supported, we propose for the case of without anchor relocation:

- Ask RAN3 to decide which node can decide to transit the UE into RRC_CONNECTED.

- The current serving gNB decides the content of RRCResume message.

- The anchor gNB performs ciphering and integrity protection for RRCResume message.

- The UE sends RRCResumeComplete message to the current serving gNB.

- Ask SA3 how to ensure security key separation if UL/DL data and RRCResume message are treated by anchor gNB with updated security key then RRCResumeComplete message and subsequent UL/DL are treated by the current serving gNB.

Reference

- R2-2010839, LS to RAN3 on small data transmission, Ericsson, RAN2#112e

- R3-211280, LS to RAN3 on small data transmission (R2-2010839), Ericsson, RAN3#111e