3GPP TSG-RAN WG2 Meeting #113bis e                  	R2-2103377

E-Meeting, 12th – 20th April, 2021

Source:	vivo

Title:	QoE measurement configuration and reporting

Agenda Item:	8.14.2.1

Document for:	Discussion and Decision

- Introduction

At RAN#91 meeting, a new WI on QoE management and optimizations for diverse services has been approved, with the following objective [1]:

The detailed objectives of the work item are as follows: - Specify the support for QoE measurement collection in NR standalone mode. [RAN2, RAN3] - Specify configuration, activation, and deactivation procedures for both signalling-based and management-based QoE measurement collection and reporting, taking LTE QoE solutions as baseline, as defined in TR 38.890. - Specify configuration and reporting for multiple simultaneous QoE measurements at a UE. - Specify QoE measurement handling at RAN overload, including pause and resume of QoE measurement reporting. - Specify QoE measurement handling in RRC_INACTIVE, i.e. keeping the QoE measurement configuration without measuring and reusing the same configuration upon transition from RRC_INACTIVE to RRC_CONNECTED. NOTE: RRC segmentation may be needed for transmission of QoE reports, and any potential solutions need detailed technical specification of the procedures (if time allows in RAN2). - Specify the support for QoE measurement collection and reporting continuity in intra-system intra-RAT mobility scenario for signaling based QoE. Mobility support for management based QoE measurements is pending input from SA5. [RAN3, RAN2] - To support RAN visible QoE, evaluate and specify an initial relevant set of RAN-visible QoE parameters, then specify configuration and reporting. [RAN3, RAN2] - Specify the support for per-slice QoE measurement. [RAN3, RAN2] - Specify the necessary mechanism to support alignment of radio-related measurement (i.e. MDT) and QoE measurement. [RAN3, RAN2] Note: If needed, co-operate with other working groups, e.g. SA4/SA5/CT1.

RAN3 has begun discussion on QoE management and optimizations and sent Ls to RAN2 [2] to inform RAN2 about RAN3 progress.

This contribution further discusses some QoE management and optimizations issues.

- Discussion

UE may be configured with a single service application or multiple service applications, at the same time. In case of multiple services applications configured to UE, the different service applications may have different QoE requirements, for example a UE configured with VR and OTT service applications. For such kinds of multiple service applications configured to UE, if a unique QoE is configured for both VR application and OTT application, network may not have a clear idea of UE service quality experience. This would not allow per service application QoE optimization.

- Single QoE configuration for multiple service applications is not helpful for QoE monitoring and optimization.

If for each UE service application, UE is configured with QoE reporting, UE would be able to report service-specific QoE. And based on per service QoE reporting, the network would be able to apply per service QoE optimization. Therefore,

- Network can configure UE with multiple QoE reporting and UE should be able to report QoE measurement per service application

Collection and reporting of radio-related QoE measurements can be done using new signaling procedure, but as QoE measurement collection and MDT measurements collection have some similarity, current Rel-17 MDT measurement collection can be used for radio-related QoE measurements collection.

Therefore,

- Methods for collection and reporting of radio related QoE measurements should be done by existing collection and reporting of radio related Rel-17 MDT measurements methods, as far as applicable.

As discussed during SI phase, QoE measurement can be configured as Signalling-based or Management-based. And the QoE activation or de-activation can also be either Signalling-based or Management-based.

Therefore,

- Both Signalling-based or Management-based QoE measurement activate or de-activate are supported in the WI.

For QoE measurement, if both Signalling-based or Management-based configuration are supported, QoE measurement may first be configured as Management-based and then QoE measurement may be configured as Signalling-based. In such the Signalling-based configuration may override the Management-based configuration. But, Management-based configuration should not override Signalling-based configuration.

Therefore,

- Management based QoE configuration should not override signalling based QoE configuration.

- Conclusion

This paper further discussed QoE management configuration and handling. The paper concludes with:

Observation 1:	Single QoE configuration for multiple service application is not helpful for QoE monitoring and optimization.

Proposal 1：	Network can configure UE with multiple QoE reporting and UE should be able to report QoE measurement per service application

Proposal 2：	Methods for collection and reporting of radio related QoE measurements should be done by existing collection and reporting of radio related Rel-17 MDT measurements methods, as far as applicable.

Proposal 3：	Both Signalling-based or Management-based QoE measurement activate or de-activate are supported in the WI.

Proposal 4：	Management based QoE configuration should not override signalling based QoE configuration.

- References

- RP-210913, New WID on NR QoE management and optimizations for diverse services