3GPP TSG-RAN WG2 Meeting #113 bis electronic	R2-2103411

Online, April 12 – April 20, 2021                             Revision of R2-2101131

Agenda item:		9.2.3

Source:		Lenovo, Motorola Mobility

Title:		Potential issues in IoT NTN with discontinuous coverage

Document for:		Discussion

- Introduction

During the email discussion for IoT NTN CP in RAN2#113e, 11 companies agreed that RAN2 should study the effect of discontinuous coverage of IoT-NTN over cell re-selection. In this contribution we provide further views on potential issues in cell selection/reselection and connection re-establishment due to discontinuous coverage in IoT NTN.

- Discussion

In the RAN1 summary for IoT NTN scenarios, Cube satellite has the size and power limitations typically associated with microsatellites and low-density constellations:

- Restricted link budget consistent with extreme coverage assumption, due to relatively much smaller maximum transmission power, smaller antenna gains and number of beams.

- Discontinuous service link coverage due to very sparse satellite constellation where UE devices can remain long periods of time without being able to detect a satellite cell.

As the link budget is mainly in RAN1 scope, we focus on the second bullet i.e. discontinuous service link coverage and give further analysis from RAN2 perspective.

Although for now it is not clear whether Cube satellite can support moving beam/cell or fixed beam/cell (or both), as shown in Figure 1 Cube satellite can be considered as a special case of LEO scenario in which the discontinuous service link coverage may occur in space and/or time domain due to limited capability or deployment. And similarly normal LEO satellites or HAPs may cause coverage interruption as well e.g. due to movement or low-density constellation.

Figure 1. Discontinuous coverage of Cube satellites: a) earth moving cell & b) earth fixed cell

Observation 1: Service link coverage interruption (i.e. discontinuous coverage) may occur for LEO satellites (including Cube satellites) and HAPs due to capability, movement or low-density constellation.

For UE other than NB-IoT, serving cell measurement is always performed and neighboring cell measurement is mandatory if serving cell quality is lower than threshold. Therefore in discontinuous coverage scenario when serving cell becomes unavailable (e.g. due to Cube satellite movement) and there is no available neighboring cell, no suitable or acceptable cell will be found so UE will keep on measuring/scanning before and after entering “Any Cell Selection” state according to current specification. The same issue exists for NB-IoT UE except that cell quality threshold only includes RSRP and inter-frequency/RAT measurement is not considered.

If following legacy cell selection/reselection procedures, for discontinuous coverage scenario when serving cell becomes unavailable (e.g. due to Cube satellite movement), no suitable cell (or even no acceptable cell) will be found so the UE will keep on scanning all RF channels, which consumes power but is useless until coverage restores.

Observation 2: UE in IDLE/INACTIVE always measures serving cell RSRP/RSRQ and mandatorily triggers neighboring cell measurement when the measured RSRP/RSRQ is lower than threshold, which consumes power but is useless during the no coverage duration in discontinuous coverage scenario.

The relaxed monitoring is introduced to further reduce power consumption for NB-IoT with the intention to avoid triggering measurement when the serving cell quality decline is not significant (SrxlevRef – Srxlev < SSearchDeltaP, which can be 6,9,12,15 dB) and the situation last for a period of time (TSearchDeltaP = max {5min, eDRX cycle (if configured)}). To use relaxed monitoring it is also required that less than 24 hours have passed since measurements for cell reselection were last performed and the UE has performed intra-frequency or inter-frequency measurements for at least TSearchDeltaP after selecting or reselecting a new cell.

It is obvious that relaxed monitoring is a trade-off between cell quality decline and measurement triggering, but not for discontinuous coverage. To leverage relaxed monitoring for discontinuous coverage, the cell quality tolerance (SSearchDeltaP) has to be large enough (e.g. maximum RSRP possible) which will forbid further measurement. Meanwhile TSearchDeltaP is better to be shorter (now at least 5min) for power saving. Besides, the maximum duration of relaxed monitoring is 24 hours.

Observation 3: Relaxed monitoring mechanism may not work for discontinuous coverage due to limited value range of cell quality tolerance, and cannot avoid useless neighboring cell measurement in TSearchDeltaP (at least 5min) and every 24 hours.

Proposal 1: RAN2 to study enhancement to cell selection/reselection considering service link coverage interruption in discontinuous coverage scenario.

We further consider a CONNECTED UE that may experience coverage interruption in discontinuous coverage scenario. The UE performs RLM and will detect RLF when approaching the edge of discontinuous network coverage. The UE will first try to find a suitable cell to re-establish RRC connection, after which it will finally transit to IDLE as T301 will expire due to coverage interruption. In this case the RLM, RLF detection and RRC re-establishment attempt are unnecessary. From the perspective of power saving or signalling overhead, it is expected that network can indicate UE to omit these procedures. Considering that the configurations for CONNECTED will be released or discarded when UE enters IDLE, the UE has to initiate random access and establish new RRC connection when coverage restores (e.g. a new cell belonging to the same or different satellite is available again after coverage interruption).

Observation 4: When UE encounters coverage interruption in CONNECTED mode, the RLM, RLF detection and RRC re-establishment attempt are unnecessary, and it has to to initiate random access and establish new RRC connection when coverage restores.

Proposal 2: RAN2 to study enhancement to RLM, RLF and RRC re-establishment considering service link coverage interruption.

- Conclusion

In this contribution we further discuss potential issues in cell selection/reselection and connect re-establishment in discontinuous coverage scenario for IoT NTN. The following observation are given:

Observation 1: Service link coverage interruption (i.e. discontinuous coverage) may occur for LEO satellites (including Cube satellites) and HAPs due to capability, movement or low-density constellation.

Observation 2: UE in IDLE/INACTIVE always measures serving cell RSRP/RSRQ and mandatorily triggers neighboring cell measurement when the measured RSRP/RSRQ is lower than threshold, which consumes power but is useless during the no coverage duration in discontinuous coverage scenario.

Observation 3: Relaxed monitoring mechanism may not work for discontinuous coverage due to limited value range of cell quality tolerance, and cannot avoid useless neighboring cell measurement in TSearchDeltaP (at least 5min) and every 24 hours.

Observation 4: When UE encounters coverage interruption in CONNECTED mode, the RLM, RLF detection and RRC re-establishment attempt are unnecessary, and it has to to initiate random access and establish new RRC connection when coverage restores.

And we propose:

Proposal 1: RAN2 to study enhancement to cell selection/reselection considering service link coverage interruption in discontinuous coverage scenario.

Proposal 2: RAN2 to study enhancement to RLM, RLF and RRC re-establishment considering service link coverage interruption.