3GPP TSG-RAN2 Meeting #113bis-e	R2-2103491

Online, April 12 – 20, 2021

Agenda Item:	7.2.3

Source:			Huawei, HiSilicon

Title:				RSRQ measurements when RSS is used

Document for:	Discussion and decision

# Introduction

In the LS in [1] RAN4 have asked RAN2 to decide whether or not RSRQ based on RSS measurements should be supported. In this document we provide some considerations from RAN2 perspective.

# Discussion

In RAN4#98-e it was discussed whether to introduce RSS based RSRQ measurements, as this requirement is currently missing from RAN4 (and RAN1) specifications even though it is required by RAN2 specifications. RAN4 could not conclude so they have asked RAN2 to discuss and decide between the 2 options.

- Option 1: Remove RSRQ from the cell selection and cell re-selection criterion when a cell is measured using RSS.

- Option 2: Define RSRQ for RSS measurements.

In NB-IoT, RSRQ is not supported for the case of measuring NRS on the non-anchor carrier. RSRQ is only used for the cell suitability, so it only needs one “if” statement for the case of NRS on non-anchor carrier, and no big change to the overall idle mode mobility behaviour because it is not used in cell reselection.

For eMTC it should be noted that RSRQ is used for cell reselection evaluation, for determining whether to perform neighbour cell measurements, for determining normal/enhanced coverage. So the impact to the (RAN2) specification is a lot greater for eMTC if we do not support RSS based RSRQ measurements and it would be significantly changing the overall idle mode mobility behaviour if we remove it. From a RAN2 point of view clearly there is more impact if RSS based RSRQ is not supported, than if it is supported. We also need to consider the difference in cell reselection behaviour between UEs not supporting RSS measurements and UEs supporting RSS measurements. In addition we need to consider the case that some cells enable RSS measurements and others may not – in this case there would be different criteria depending on the camped cell (i.e. RSRQ would be applied in a cell not configuring RSS based measurements, and RSRQ would not be applied in a cell configuring it, introducing a potential ping-pong situation).

On the other hand, it may be argued that it is less painful from a specifications point of view to remove from RAN2 specification for this particular case, rather than to add the missing requirements in RAN1 and RAN4. It may also be argued that the accuracy of RSRQ may not be sufficient when performing measurement using RSS, although this is a matter that would need to be addressed in RAN1 and RAN4.

Given the above, it is our opinion that at least from a RAN2 perspective it is preferable not to remove RSRQ from the cell selection and reselection criteria both in terms of specification impact and in terms of introducing inconsistent requirements, both between UEs supporting/not supporting RSS measurements potential, and between cells enabling/not enabling RSS measurements.

Observation: From a RAN2 perspective option 2 is preferable.

In order to facilitate progress, we propose to discuss the pros and cons of each option and provide draft LSs, draft CRs, and skeleton offline discussion in the appendices.

Proposal: RAN2 to discuss the pros and cons of options 1 and 2 to decide how to proceed.

# Conclusion

In this paper we have provided some consideration on the LS from RAN4 regarding RSRQ measurement when RSS is used.

Observation: From a RAN2 perspective option 2 is preferable.

Proposal: RAN2 to discuss the pros and cons of options 1 and 2 to decide how to proceed.

# Reference

- R2-2102653, “LS related to RSS based RSRQ for LTE-MTC”’, RAN4

# Appendix A: Offline discussion skeleton

3GPP TSG-RAN2 Meeting #113bis-e	R2-21xxxxx

Online, April 12 – 20, 2021

Agenda Item:	9.1.4

Source:			Huawei, HiSilicon

Title:				Offline 4xx - RSRQ measurements when RSS is used

Document for:	Discussion and decision

- Introduction

This document is the summary of the offline email discussion “[AT113bis-e][xxx][ eMTC R16] RSRQ measurements when RSS is used (Huawei)”, as indicated below:

- [AT113bis-e][4xx][eMTC R16] RSRQ measurements when RSS is used (Huawei)

Scope:

TBD

Intended outcome:

TBD

Deadline:

TBD

- Discussion

Companies are requested to provide comments in the table below (one row for each new comment to better keep track of the discussion – please don’t edit the previous comments).

| Company | Option 1 or Option 2? | Detailed comments |
| --- | --- | --- |

- Conclusion

TBD

# Appendix B.1: Draft Reply LS for Option 1

3GPP TSG-RAN2 Meeting #113bis-e	R2-21xxxxx

Online, April 12 – 20, 2021

Title:	[draft] Reply LS related to RSS based RSRQ for LTE-MTC

Response to:	R4-2103728

Release:	Release 16

Work Item:	LTE_eMTC5-Core

Source:	[Huawei – to be] RAN2

To:	RAN4,

Cc:	RAN1

Contact Person:

#### Name:	Brian Martin

#### E-mail Address:	brian DOT alexander DOT martin AT huawei DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments: [Agreed CR]

1. Overall Description:

RAN2 thanks RAN4 for the LS in R4-2103728.

RAN2 have discussed the options listed in the LS and have agreed the attached CR for option 1 (i.e. RSRQ has been removed from the cell selection and cell re-selection criterion when a cell is measured using RSS).

2. Actions:

To RAN4:

RAN2 kindly asks RAN4 to take the above into account.

3. Date of Next TSG-RAN2 Meetings:

TSG-RAN2 Meeting #114-e		  	19 – 27 May, 2021

TSG-RAN2 Meeting #115		23 – 27 Aug, 2021

# Appendix B.2: Draft CR for Option 1

3GPP TSG-RAN2 Meeting #113bis-e	R2-21xxxxx

Online, April 12 – 20, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.304 | CR | xyz | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Removal of RSS based RSRQ from cell selection and reselection criterion. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, [company A], [company B] |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_eMTC5-Core |  |  |  |  |  | Date: |  |  | 2021-04-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | There are no requirements defined in RAN1 or RAN4 for RSS based RSRQ measurements therefore it is not clear how the UE will obtain the values to determine cell selection and reselection criterion. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove RSS based RSRQ from the cell selection and reselection criterion. Impact analysis Impacted functionality: eMTC cell selection and reselection when RSS measurements are used. Inter-operability: No interoperability issue, UE idle mode behaviour, |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not clear how the UE will obtain the RSRQ values to determine cell selection and reselection criterion when RSS measurements are used. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.3.2, 5.2.4.2, 5.2.4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

#### 5.2.3.2	Cell Selection Criterion

For NB-IoT the cell selection criterion is defined in clause 5.2.3.2a.

If the measurements are performed using RSS as specified in [10], the cell selection criterion S in normal coverage is fulfilled when:

Srxlev > 0

Else, the cell selection criterion S in normal coverage is fulfilled when:

Srxlev > 0 AND Squal > 0

where:

Srxlev = Qrxlevmeas – (Qrxlevmin + Qrxlevminoffset) – Pcompensation - Qoffsettemp Squal = Qqualmeas – (Qqualmin + Qqualminoffset) - Qoffsettemp

where:

| Srxlev | Cell selection RX level value (dB) |
| --- | --- |
| Squal | Cell selection quality value (dB) |
| Qoffsettemp | Offset temporarily applied to a cell as specified in TS 36.331 [3] (dB) |
| Qrxlevmeas | Measured cell RX level value (RSRP) |
| Qqualmeas | Measured cell quality value (RSRQ) |
| Qrxlevmin | Minimum required RX level in the cell (dBm). Qrxlevmin is obtained from q-RxLevMin in SIB1, SIB3, SIB5, or NR SIB5. When the UE who is camped on a NR cell is evaluating an E-UTRA cell, if Qrxlevminoffsetcell is signalled in NR SIB5 in TS 38.331 [37] for the E-UTRA cell, this cell specific offset is added to q-RxLevMin to achieve the required minimum RX level in the E-UTRA cell. |
| Qqualmin | Minimum required quality level in the cell (dB) When the UE who is camped on a NR cell is evaluating an E-UTRA cell, if Qqualminoffsetcell is signalled in NR SIB5 in TS 38.331 [37] for the E-UTRA cell, this cell specific offset is added to achieve the required minimum quality level in the E-UTRA cell. |
| Qrxlevminoffset | Offset to the signalled Qrxlevmin taken into account in the Srxlev evaluation as a result of a periodic search for a higher priority PLMN while camped normally in a VPLMN TS 23.122 [5] |
| Qqualminoffset | Offset to the signalled Qqualmin taken into account in the Squal evaluation as a result of a periodic search for a higher priority PLMN while camped normally in a VPLMN TS 23.122 [5] |
| Pcompensation | If the UE supports the additionalPmax in the NS-PmaxList, if present, in SIB1, SIB3 and SIB5: max(PEMAX1 –PPowerClass, 0) – (min(PEMAX2, PPowerClass) – min(PEMAX1, PPowerClass)) (dB); else: if PPowerClass is 14 dBm: max(PEMAX1 –(PPowerClass – Poffset), 0) (dB); else: max(PEMAX1 –PPowerClass, 0) (dB) |
| PEMAX1, PEMAX2 | Maximum TX power level an UE may use when transmitting on the uplink in the cell (dBm) defined as PEMAX in TS 36.101 [33]. PEMAX1 and PEMAX2 are obtained from the p-Max and the NS-PmaxList respectively in SIB1, SIB3 and SIB5 as specified in TS 36.331 [3]. |
| PPowerClass | Maximum RF output power of the UE (dBm) according to the UE power class as defined in TS 36.101 [33] |

The signalled values Qrxlevminoffset and Qqualminoffset are only applied when a cell is evaluated for cell selection as a result of a periodic search for a higher priority PLMN while camped normally in a VPLMN TS 23.122 [5]. During this periodic search for higher priority PLMN the UE may check the S criteria of a cell using parameter values stored from a different cell of this higher priority PLMN.

If cell selection criterion S in normal coverage is not fulfilled for a cell, UE shall consider itself to be in enhanced coverage if the cell selection criterion S for enhanced coverage is fulfilled, where:

| Qrxlevmin | UE applies coverage specific value Qrxlevmin_CE (dBm) |
| --- | --- |
| Qqualmin | UE applies coverage specific value Qqualmin_CE (dB) |

If cell selection criteria S in normal coverage is fulfilled for a cell, UE may consider itself to be in enhanced coverage if SystemInformationBlockType1 cannot be acquired but UE is able to acquire MasterInformationBlock, SystemInformationBlockType1-BR and SystemInformationBlockType2.

If cell selection criterion S in normal coverage is not fulfilled for a cell and UE does not consider itself in enhanced coverage based on coverage specific values Qrxlevmin_CE and, when the measurements are not performed using RSS as specified in [10], Qqualmin_CE, UE shall consider itself to be in enhanced coverage if UE supports CE Mode B and CE mode B is not restricted by upper layers and the cell selection criterion S for enhanced coverage is fulfilled, where:

| Qrxlevmin | UE applies coverage specific value Qrxlevmin_CE1 (dBm) |
| --- | --- |
| Qqualmin | UE applies coverage specific value Qqualmin_CE1 (dB) |

For the UE in enhanced coverage, coverage specific values Qrxlevmin_CE and Qqualmin_CE (or Qrxlevmin_CE1 and Qqualmin_CE1) are only applied for the suitability check in enhanced coverage (i.e. not used for measurement and reselection thresholds).

NEXT CHANGE

#### 5.2.4.2	Measurement rules for cell re-selection

For NB-IoT measurement rules for cell re-selection is defined in clause 5.2.4.2.a.

When evaluating Srxlev and Squal of non-serving cells for reselection purposes, the UE shall use parameters provided by the serving cell.

Following rules are used by the UE to limit needed measurements:

-	If the serving cell fulfils Srxlev > SIntraSearchP and, when the measurements are not performed using RSS as specified in [10], Squal > SIntraSearchQ, the UE may choose not to perform intra-frequency measurements.

-	Otherwise, the UE shall perform intra-frequency measurements.

-	The UE shall apply the following rules for E-UTRAN inter-frequencies and inter-RAT frequencies which are indicated in system information and for which the UE has priority provided as defined in 5.2.4.1:

-	For an E-UTRAN inter-frequency or inter-RAT frequency with a reselection priority higher than the reselection priority of the current E-UTRA frequency the UE shall perform measurements of higher priority E-UTRAN inter-frequency or inter-RAT frequencies according to TS 36.133 [10].

-	For an E-UTRAN inter-frequency with an equal or lower reselection priority than the reselection priority of the current E-UTRA frequency and for inter-RAT frequency with lower reselection priority than the reselection priority of the current E-UTRAN frequency:

-	If the serving cell fulfils Srxlev > SnonIntraSearchP and, when the measurements are not performed using RSS as specified in [10], Squal > SnonIntraSearchQ, the UE may choose not to perform measurements of E-UTRAN inter-frequencies or inter-RAT frequency cells of equal or lower priority unless the UE is triggered to measure an E-UTRAN inter-frequency which is configured with redistributionInterFreqInfo.

-	Otherwise, the UE shall perform measurements of E-UTRAN inter-frequencies or inter-RAT frequency cells of equal or lower priority according to TS 36.133 [10].

-	If the UE supports relaxed monitoring and s-SearchDeltaP is present in SystemInformationBlockType3, the UE may further limit the needed measurements, as specified in clause 5.2.4.12.

NEXT CHANGE

#### 5.2.4.5	E-UTRAN Inter-frequency and inter-RAT Cell Reselection criteria

For NB-IoT inter-frequency cell reselection shall be based on ranking as defined in clause 5.2.4.6.

If threshServingLowQ is provided in SystemInformationBlockType3 and more than 1 second has elapsed since the UE camped on the current serving cell, and if the measurements are not performed using RSS as specified in [10], cell reselection to a cell on a higher priority E-UTRAN frequency or inter-RAT frequency than the serving frequency shall be performed if:

-	A cell of a higher priority EUTRAN, NR or UTRAN FDD RAT/ frequency fulfils Squal > ThreshX, HighQ during a time interval TreselectionRAT; or

-	A cell of a higher priority UTRAN TDD, GERAN or CDMA2000 RAT/ frequency fulfils Srxlev > ThreshX, HighP during a time interval TreselectionRAT.

Otherwise, cell reselection to a cell on a higher priority E-UTRAN frequency or inter-RAT frequency than the serving frequency shall be performed if:

-	A cell of a higher priority RAT/ frequency fulfils Srxlev > ThreshX, HighP during a time interval TreselectionRAT; and

-	More than 1 second has elapsed since the UE camped on the current serving cell.

Cell reselection to a cell on an equal priority E-UTRAN frequency shall be based on ranking for Intra-frequency cell reselection as defined in clause 5.2.4.6.

If threshServingLowQ is provided in SystemInformationBlockType3 and more than 1 second has elapsed since the UE camped on the current serving cell, and if the measurements are not performed using RSS as specified in [10], cell reselection to a cell on a lower priority E-UTRAN frequency or inter-RAT frequency than the serving frequency shall be performed if:

-	The serving cell fulfils Squal < ThreshServing, LowQ and a cell of a lower priority EUTRAN, NR or UTRAN FDD RAT/ frequency fulfils Squal > ThreshX, LowQ during a time interval TreselectionRAT; or

-	The serving cell fulfils Squal < ThreshServing, LowQ and a cell of a lower priority UTRAN TDD, GERAN or CDMA2000 RAT/ frequency fulfils Srxlev > ThreshX, LowP during a time interval TreselectionRAT.

Otherwise, cell reselection to a cell on a lower priority E-UTRAN frequency or inter-RAT frequency than the serving frequency shall be performed if:

-	The serving cell fulfils Srxlev < ThreshServing, LowP and a cell of a lower priority RAT/ frequency fulfils Srxlev > ThreshX, LowP during a time interval TreselectionRAT; and

-	More than 1 second has elapsed since the UE camped on the current serving cell.

Cell reselection to a higher priority RAT/ frequency shall take precedence over a lower priority RAT/ frequency, if multiple cells of different priorities fulfil the cell reselection criteria.

The UE shall not perform cell reselection to NR or UTRAN FDD cells for which the cell selection criterion S is not fulfilled.

For cdma2000 RATs, Srxlev is equal to -FLOOR(-2 x 10 x log10 Ec/Io) in units of 0.5 dB, as defined in [18], with Ec/Io referring to the value measured from the evaluated cell.

For cdma2000 RATs, ThreshX, HighP and ThreshX, LowP are equal to -1 times the values signalled for the corresponding parameters in the system information.

In all the above criteria the value of TreselectionRAT is scaled when the UE is in the medium or high mobility state as defined in clause 5.2.4.3.1. If more than one cell meets the above criteria, the UE shall reselect a cell as follows:

-	If the highest-priority frequency is an E-UTRAN frequency, a cell ranked as the best cell among the cells on the highest priority frequency(ies) meeting the criteria according to clause 5.2.4.6;

-	If the highest-priority frequency is from another RAT, a cell ranked as the best cell among the cells on the highest priority frequency(ies) meeting the criteria of that RAT.

Cell reselection to another RAT, for which Squal based cell reselection parameters are broadcast in system information, shall be performed based on the Squal criteria if the UE supports Squal (RSRQ) based cell reselection to E-UTRAN from all the other RATs provided by system information which UE supports. Otherwise, cell reselection to another RAT shall be performed based on Srxlev criteria.

Cell reselection to NR, for which a cell reselection parameter, q-RxLevMinSUL is broadcast in system information and the UE supports SUL, shall be performed based on Srxlev criteria taking the parameter into account.

# Appendix C: Draft Reply LS for Option 2

3GPP TSG-RAN2 Meeting #113bis-e	R2-21xxxxx

Online, April 12 – 20, 2021

Title:	[draft] Reply LS related to RSS based RSRQ for LTE-MTC

Response to:	R4-2103728

Release:	Release 16

Work Item:	LTE_eMTC5-Core

Source:	[Huawei – to be] RAN2

To:	RAN4, RAN1

Cc:

Contact Person:

#### Name:	Brian Martin

#### E-mail Address:	brian DOT alexander DOT martin AT huawei DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments: -

1. Overall Description:

RAN2 thanks RAN4 for the LS in R4-2103728.

RAN2 have discussed the options listed in the LS and have decided that from a RAN2 perspective option 2 is preferred.

2. Actions:

To RAN1, RAN4:

RAN2 kindly asks RAN1 and RAN4 to take the above into account, and update the specifications accordingly.

3. Date of Next TSG-RAN2 Meetings:

TSG-RAN2 Meeting #114-e		  	19 – 27 May, 2021

TSG-RAN2 Meeting #115		23 – 27 Aug, 2021