3GPP TSG-RAN WG2 #113bis-e	R2-2103504

Electronic meeting, 12 – 20 April, 2021

Agenda item:		8.2.2.3	Activation of deactivated SCG

Title:		Issues on SCG activation procedure

Source:		NEC

Document for:		Discussion and Decision

# 1. Introduction

In RAN2#113e, a lot of agreements were made for SCG (de)activation [1].

- Agreements

- 1	NW-triggered SCG activation is indicated to the UE via the MCG.

Agreements

2	The UE behaviour when the SCG activation is indicated to the UE via the MCG is one or more of the following options:

option 1) similar to reconfiguration with sync, i.e. the UE always initiates random access to the PSCell.

option 2) in certain cases:

-	the UE does not initiate random access and monitors PDCCH on the PSCell (at the latest after the specified processing time).

-	the SCG can schedule data transmission on the PDCCH

The UE decides not to perform random access (one option to be selected):

option 2a) if the TA timer is still running and possibly other conditions (FFS how TAT starts)

option 2b) based on the contents of the SCG activation indication

- Agreements

- 1a 	SCG activation can be requested by MN/SN/UE. FFS on how to accept/reject the procedure. FFS which signalling is used.

- 3 	RRC signalling is defined for the interaction between UE/MN and MN/SN in SCG activation/deactivation. FFS if lower-layer signalling is needed.

In this contribution, we discuss the remaining issues for activation of deactivated SCG and provide our views.

# 2. Discussion

2.1	SCG activation signalling

In this discussion, we focus on the SCG activation request by the network node (i.e. MN or SN). In the last meeting, it was agreed that SCG activation can be requested by both MN and SN. Then, it was also discussed whether a receiver node can reject or not. In normal case, the receiver node does not reject, because the SCG activation is requested when it is necessary and there is no valid reason to reject it basically.

However, there could be one case where the MN may want to reject the request from the SN, where the MN has already triggered the SN change procedure by contacting a target SN right before the SCG activation request from the (source) SN. This is a kind of race condition. Although it will be a rare case, it may happen. If the specification does not allow the MN to reject the SCG activation request in this case, another complicated network behaviour needs to be specified, which may end up with sub-optimum results, e.g. throughput degradation or connection failure in current SCG.

Observation 1: A receiver node should accept the SCG activation request basically, while there may be a case where a receiver node wants to reject with valid reasons.

In the following, we assume a receiver node can reject the SCG activation request. As it was agreed that “NW-triggered SCG activation is indicated to the UE via the MCG”, it should be clarified who and how decides to trigger the SCG activation towards the UE via MCG.

When the MN requests and the SN accepts, the MN simply triggers the SCG activation to the UE. If the SN rejects, the MN may consider an alternative action, e.g. bearer type change from SN terminated to MN terminated. On the other hand, when the SN requests and the MN accepts, the MN can trigger the SCG activation to the UE. If the MN wants to reject due to some reasons, it is up to the MN (i.e. network implementation) to trigger the following actions to ensure a data transmission in SCG would not be impacted much. With these observations, it can be the MN to take a final decision to trigger the SCG activation to the UE via MCG.

Proposal 1: A receiver node can reject the request for SCG activation between MN and SN.

Proposal 2: MN is responsible for a final decision to trigger the SCG activation and send the SCG activation indication to the UE via MCG.

2.2	UE behaviour upon receiving SCG activation indication

In the last meeting, there were also some agreements on the possible options for the UE behaviour when the SCG activation is indicated.

2	The UE behaviour when the SCG activation is indicated to the UE via the MCG is one or more of the following options:

option 1) similar to reconfiguration with sync, i.e. the UE always initiates random access to the PSCell.

option 2) in certain cases:

-	the UE does not initiate random access and monitors PDCCH on the PSCell (at the latest after the specified processing time).

-	the SCG can schedule data transmission on the PDCCH

The UE decides not to perform random access (one option to be selected):

option 2a) if the TA timer is still running and possibly other conditions (FFS how TAT starts)

option 2b) based on the contents of the SCG activation indication

Firstly, it would be good to confirm some basic situations to progress this issue further. We believe when the PSCell is added but deactivated directly, the UE needs to perform random access when the SCG is activated for the first time. This is the same for PSCell change, RRC resume or HO.

Proposal 3: Random access to the PSCell is always needed at the first SCG activation after direct SCG deactivation at PSCell addition/change, RRC resume or HO.

Currently, it is being discussed whether and how to maintain the TA timer in deactivated SCG. In some cases, the TA timer may be still running at the time of SCG activation and the UE may skip random access, which is the option 2a). This may be useful if this works well.

However, even if the option 2a) could work, it may not be much reliable depending on the situation (e.g. cell deployment, UE mobility) and the network may want to ensure the access toward the PSCell at SCG activation. Thus, we would like to confirm that the network should be able to control whether random access is necessary or not at the time of activating the SCG, regardless whether the option 2a is supported or not.

Proposal 4: The network should be able to indicate whether the UE needs to perform random access to the PSCell at SCG activation. FFS how and when this is indicated.

On top of proposals above, RAN2 can progress further details of the UE behaviour upon receiving the SCG activation.

# 3. Conclusion

In this contribution we discussed the remaining issues for activation of deactivated SCG and made the following proposals.

Proposal 1: A receiver node can reject the request for SCG activation between MN and SN.

Proposal 2: MN is responsible for a final decision to trigger the SCG activation and send the SCG activation indication to the UE via MCG.

Proposal 3: Random access to the PSCell is always needed at the first SCG activation after direct SCG deactivation at PSCell addition/change, RRC resume or HO.

Proposal 4: The network should be able to indicate whether the UE needs to perform random access to the PSCell at SCG activation. FFS how and when this is indicated.

# References

[1] R2-2101951, Report on LTE legacy, Mobility, DCCA, Multi-SIM and RAN slicing

[2] R2-2100641, “SCG (de)activation initiation”, NEC