3GPP TSG-RAN WG2 Meeting #113bis Electronic	R2-2103529

Online, April 12 – April 20, 2021	Revision of R2-2101204

Agenda item:	8.6.4

Source:	Nokia, Nokia Shanghai Bell

Title:	Details of context fetch and data forwarding

WID/SID:	NR_SmallData_INACTIVE - Release 17

Document for:	Discussion and Decision

# 1	Introduction

In the previous RAN2 meetings, the following agreements were reached:

Agreements (RAN2#111-e):

1 	Small data transmission with RRC message is supported as baseline for RA-based and CG based schemes

2	RRC-less can be studied for limited use cases (e.g. same serving cell and/or for CG) with lower priority

3	Context fetch and data forwarding with anchor re-location and without anchor re-location will be considered.   FFS if there are problems with the scenario “without anchor relocation”.

4	From RAN2 perspective, stored “configuration” in the UE Context is used for the RLC bearer configuration for any SDT mechanism (RACH and CG).

5	The 2-step RACH or 4-step RACH should be applied to RACH based uplink small data transmission in RRC_INACTIVE

6	The uplink small data can be sent in MSGA of 2-step RACH or msg3 of 4-step RACH.

7	Small data transmission is configured by the network on a per DRB basis

8	Data volume threshold is used for the UE to decide whether to do SDT or not.   FFS how we calculate data volume.

FFS if an “additional SDT specific” RSRP threshold is further used to determine whether the UE should do SDT

9	UL/DL transmission following UL SDT without transitioning to RRC_CONNECTED is supported

10	When UE is in RRC_INACTIVE, it should be possible to send multiple UL and DL packets as part of the same SDT mechanism and without transitioning to RRC_CONNECTED on dedicated grant.  FFS on details and whether any indication to network is needed.

In this contribution, we discuss the further details for the SDT data forwarding between anchor and target gNBs.

# 2	Details of context fetch and data forwarding with and without anchor relocation

RAN2 has agreed to consider both context fetch and data forwarding (with and without anchor relocation) for SDT procedures in INACTIVE state that are initiated to a target gNB different than the last serving gNB (that sent the UE in INACTIVE and stores the UE AS Context). RAN2 has also agreed that the RLC bearer configuration to use for SDT is based on the UE-specific RLC configuration stored in the UE AS context.

Data forwarding for improved data latency entails that SDT data is forwarded from the target gNB to the last serving gNB, which deciphers and sends it to the 5GC. DL data may be also forwarded by the last serving gNB to the target gNB. As SDT is meant for small data only, the associated capacity demands on the Xn interface from data forwarding can be insignificant. Furthermore, the forwarding need not increase the number of signaling messages over the Xn interface since it can be performed along with the Retrieve UE Context Request message or alike message.

For the case of data forwarding without anchor relocation, the UE AS context is retained at the last serving gNB and no path switch to the target gNB is performed, reducing the associated network signaling. This strategy can be appealing whenever the data forwarding delay (one-way Xn latency) can be tolerated, which could be expected in the majority of small data use cases.

For the case of data forwarding with anchor relocation, the UE AS context is relocated and path switch to the target gNB is performed along with the data forwarding. This allows the target gNB to be ready for subsequent SDT transmissions or subsequent SDT procedures that the UE may initiate to the same target gNB.

Observation 1: Data forwarding with and without anchor relocation may provide data latency and network signalling benefits and their adoption should be up to the network.

In general, in case of SDT with anchor relocation, it is not clear which node should decide whether the full UE AS context should be fetched or not, as discussed in R3-210056. There may be situations where the target gNB could have compelling reasons to become the new anchor gNB, for example, in the case the target gNB has decided that it is beneficial to move the UE to the RRC_CONNECTED state and therefore become the new anchor gNB. This may happen if the UE has indicated that a significant amount of data is to be expected or has indicated the presence of non-SDT data in its buffer. There may be opposite situations where the target gNB could instead have a strong preference to not become the anchor gNB. This could be the case for example due to its load situation, e.g. the target gNB is in high load and prefers that the SDT transaction continues keeping the old anchor gNB. Moreover, there may be situations where the target gNB would have no compelling reason, or no preference in either direction, then the anchor gNB could decide whether to relocate the anchor function or not based on its own information: for example, based on past UE traffic activity/mobility estimation it could judge whether it is worth relocating the UE AS context.

Observation 2: To cope with the various use cases and needs, the target gNB should be able to request the UE AS context relocation when needed or indicate that it leaves the decision to the anchor gNB.

Proposal 1: The target gNB should be able to request the UE AS context retrieval when needed or indicate that it leaves the decision to the anchor gNB.

Furthermore, the UE applies ciphering to the UL SDT data based on the security information stored in the UE AS Context, where it derives the ciphering key using the Next-Hop Chaining Count (NCC) parameter provided to the UE by the last serving gNB upon RRC state transition to RRC_INACTIVE. This is according to the existing security framework as per UP-EDT. Thus, when using data forwarding (e.g. without anchor relocation), the last serving gNB should terminate the PDCP protocol layer for the SDT DRB and perform PDCP processing including deciphering.

Proposal 2: The PDCP protocol instance for an SDT DRB can be terminated at the last serving gNB when data forwarding is used.

Since the PDCP instance of the SDT DRB is to be terminated at the last serving gNB, when data forwarding is applied by a target gNB adopting a RAN split architecture, it may be considered whether the UL data could be tunnelled directly by the target gNB-DU (MAC/RLC) to the target gNB-CU-CP control-plane entity (RRC) without the need to involve the target gNB-CU-UP user-plane entity (PDCP). However, these aspects are under RAN3 responsibility and are going to be discussed in RAN3, see R3-210056, R3-210057.

In case of SDT with data forwarding without anchor relocation, as a result of the agreement of using the RLC configuration stored in UE Context for SDT, it is still needed for the target gNB to fetch the RLC context from the anchor gNB to process the received RLC PDU before data forwarding can take place. This vanishes the latency benefits of data forwarding due to the required buffering delay at the target gNB while waiting for the context and mandates a partial context retrieve also when no anchor relocation is used, which introduces additional network signalling overhead as shown in R3-210056.

Observation 3: Using the stored RLC configuration for SDT implies the need to first fetch the RLC context before data forwarding can occur, which is undesired as it vanishes the benefits of data forwarding.

Similarly, even for the case of without data forwarding, the current approach exposes latency given the RLC PDU at the target gNB can only be decoded upon receival of UE AS Context. In fact, by employing the CU-DU split architecture, this would mean the RLC context can only be provided to the DU after receiving the UE AS Context and only then the RLC PDU can be decoded and forwarded to CU for PDCP processing. Such delay in the SDT procedure seems to have also significant impact to UE power consumption given it needs to wait decoding PDCCH until the SDT procedure is completed.

Observation 4: Even without data forwarding, the agreed approach of using the stored RLC configuration for SDT implies the need to first fetch the UE AS Context before the gNB-CU can configure the RLC entity of the gNB-DU to decode the RLC PDU and forward the resulting PDCP PDU to gNB-CU – this seems to introduce significant delay for SDT.

Given the above, to enable the network to decode the RLC PDU of the SDT transmission with stored RLC configuration immediately, i.e. without requiring fetching of the RLC context, it seems possible for the UE to provide this configuration information along with the SDT transmission. This allows the target gNB to immediately decode the RLC PDU and enables immediate data forwarding to be used over Xn.

Proposal 3: Support a UE indication about the used RLC configuration in the SDT transmission to enable the target gNB to decode the RLC PDU immediately.

It is worth noting that without such UE indication about the used RLC configuration, the scenario of data forwarding with anchor relocation becomes an irrelevant scenario since the first RLC PDU cannot be forwarded from the target to the anchor gNB before fetching the RLC context. However, by the time the RLC context is fetched and the first RLC PDU is processed, the anchor relocation would be already initiated, after which data forwarding wouldn’t be possible anymore. Proposal 5 allows to support such scenario of data forwarding with anchor relocation in line with the corresponding objective in the SDT Work Item Description.

# 3	Conclusion

Observation 1: Data forwarding with and without anchor relocation may provide data latency and network signalling benefits and their adoption should be up to the network.

Observation 2: To cope with the various use cases and needs, the target gNB should be able to request the UE AS context relocation when needed or indicate that it leaves the decision to the anchor gNB.

Observation 3: Using the stored RLC configuration for SDT implies the need to first fetch the RLC context before data forwarding can occur, which is undesired as it vanishes the benefits of data forwarding.

Observation 4: Even without data forwarding, the agreed approach of using the stored RLC configuration for SDT implies the need to first fetch the UE AS Context before the gNB-CU can configure the RLC entity of the gNB-DU to decode the RLC PDU and forward the resulting PDCP PDU to gNB-CU – this seems to introduce significant delay for SDT.

Proposal 1: The target gNB should be able to request the UE AS context retrieval when needed or indicate that it leaves the decision to the anchor gNB.

Proposal 2: The PDCP protocol instance for an SDT DRB can be terminated at the last serving gNB when data forwarding is used.

Proposal 3: Support a UE indication about the used RLC configuration in the SDT transmission to enable the target gNB to decode the RLC PDU immediately.