3GPP TSG-RAN WG2 Meeting #113bis electronic          	          R2-2103586

Online, April 12 – April 20, 2021

Agenda Item:	8.9.2

Source: 	Huawei, HiSilicon

Title: 	Discussion on potential TRS/CSI-RS

Document for:	Discussion and decision

Introduction

During the last meeting, RAN2 discussed the signalling for providing and updating the TRS/CSI-RS configuration [1]. Meanwhile, RAN1 discussed the content of TRS/CSI-RS configuration and the availability issue.

In this contribution, we further discuss the signalling design for providing TRS/CSI-RS configuration based on the lasted progresses in RAN2 and RAN1.

Discussion

Providing TRS/CSI-RS configuration

According to the discussion during the last RAN2 meeting, it is agreed that candidate solutions for providing TRS/CSI-RS configuration include:

- SIB (existing or new SIB), and

- Other dedicated higher-layer signalling,

where the SIB signalling is the baseline and other higher-layer signalling can be additionally considered if justified.

RAN2#113-e [041] On signalling providing the configuration of TRS/CSI-RS occasion(s) for idle/inactive UE(s): - SIB signalling is the baseline; - Other dedicated high-layer signalling methods (e.g., dedicated RRC, RRC release message, etc.) can be additionally considered with justification. It is assumed they do not work alone. [041] RAN2 will down select from the following options on SIB signalling providing the configuration of TRS/CSI-RS occasion(s) for idle/inactive UE(s): - Option 2: Existing SIB, other than SIB1; - Option 3: New SIB type, e.g. SIB-x;

- SIB signalling

For the SIB signalling, it is agreed that RAN2 will make down selection from the existing SIB other than SIB1 and the new SIB.

Based on RAN1’s progress presented below, we think using a new SIB is preferred since RAN1 agreed that multiple RS resources can be provided to idle/inactive UEs. Considering that the number of TRS/CSI-RS may be large to ensure the performance of this mechanism, the size of the configuration may also be relatively large. Hence, introducing a new SIB can accommodate more contents. In addition, the TRS/CSI-RS configuration may only be relevant to certain UEs, putting the information in a new SIB avoids impacting UEs not supporting this, including legacy UEs, as they would do not need to receive unnecessary information as would be the case if the information would be introduced in an existing SIB.

RAN1#104-e Agreements: Multiple RS resources can be configured for TRS/CSI-RS occasion(s) for idle/inactive UEs. - FFS details (including whether or not to restrict the RS to be TRS only)

Proposal 1: A new SIB is used to provide the TRS/CSI-RS configuration to idle/inactive-mode UEs.

As for the detailed configuration, to ensure that idle/inactive UEs can use the assistance RS, RAN1 has agreed that several parameters are necessary to be included in the configuration, such as time/frequency resource, sequence generating parameter of the RS and so on. Other parameters (e.g., QCL parameters) may also be needed. These parameters for a RS resource may require about 70~80 bits after ASN.1 coding. Besides, the above agreements show that multiple RS resources may be configured for e.g., UEs in different POs/MOs. However, according to TS 38.331, the maximum TBS of a SIB is 2976 bits, which means only about 40 RS resources can be configured. It is questionable whether 40 RS resources are sufficient considering there can be at most 64 beams in FR2.

RAN1#104-e Agreements: Configuration of TRS/CSI-RS occasion(s) for idle/inactive Ues include at least: - powerControlOffsetSS, - scramblingID - firstOFDMSymbolInTimeDomain, - startingRB. - nrofRBs, - FFS other parameters - FFS applicable values Agreements: The SCS configuration of TRS/CSI-RS occasion(s) for idle/inactive UEs can be discussed and down-selected from following alternatives at RAN1#104b-eRAN1#105-e: - Alt1: same as initial BWP - Alt2: configurable parameter Agreements: The configuration of the frequency location of TRS/CSI-RS occasion(s) for idle/inactive UEs are discussed and down-selected from following alternatives at RAN1#104bis-e: - Alt-1: within initial DL BWP - Alt-2: is not restricted by initial BWP - IDLE/INACTIVE mode UE is not expected to receive TRS/CSI-RS outside the initial DL BWP. Agreements: To study QCL information of TRS/CSI-RS occasion(s) for idle/inactive UEs from following alternatives: - Alt-1: TCI state from higher layer configuration, e.g. qcl-InfoPeriodicCSI-RS - Alt-2: QCL assumptions associated with transmitted SSBs implicitly, e.g. similar to PDCCH monitoring in PO - FFS details - FFS details - Other alternatives are not precluded

Observation 1: The maximum number of RS resources that can be configured in a SIB may be around 40 whereas there can be at most 64 beams in FR2, thus it is not clear whether 40 RS resources are sufficient.

Therefore, how to design the TRS/CSI-RS configuration, including e.g., the IE structure and contents with considerations on reducing the signalling overhead is a key issue. RAN1 discussed to pre-define some parameters in specification but the parameters should be very limited, as it is not preferred to reduce the gNB flexibility. In addition, since the RS occasions are usually for connected UEs, restricting too many configuration may even have impact on Rel-15/16 network.

As one potential solution, the configuration of RS resources can be divided into two parts, one part is common information, and the other part is RS-specific information. Fig.1 illustrates how the overhead is reduced. There are N RS resources and each resource have M parameters. If the parameter 1 and parameter 2 are the same for N resources, then parameter 1 and parameter 2 can be configured only one time instead of N times. Which parameters belong to common information or RS-specific information will be discussed in RAN1.

Fig.1 Example of signalling structure with reduced overhead

Proposal 2: For the case that assistance RS is configured by SIB, the signalling overhead should be minimized, e.g. the configuration of the assistance RS comprises a common part and a RS-specific part. Details depend on RAN1 progress.

- Other higher-layer signalling

For other dedicated higher-layer signalling, a possible solution is to use the dedicated RRC messages when UEs are still in connected mode, e.g., via the RRCRelease message. This way, the TRS/CSI-RS configuration can be directly transmitted to each relevant UE and it is possible to provide different configurations on a per UE basis if needed. However, we have a consensus that this approach cannot work alone.

Considering that the RRC message does not have overhead restriction (discussed above for SIB), and may cost less in terms of signalling overhead compared with periodic broadcast signalling since the dedicated signalling is one-shot even for several specific UEs, we think we can further consider this approach when there are more conclusions from RAN1 .

Proposal 3: Wait for more progress from RAN1 before determining whether dedicated RRC signalling, e.g. RRCRelease message can be used to provide the TRS/CSI-RS configuration to idle/inactive-mode UEs.

Updating TRS/CSI-RS configuration

Since the SIB signalling has been agreed as a way to provide the TRS/CSI-RS configuration, in this section we investigate potential impacts on existing procedures when updating the TRS/CSI-RS configuration based on the premise that the TRS/CSI-RS configuration is broadcast in SIB.

First, we think the TRS/CSI-RS configuration update issue and the TRS/CSI-RS availability issue should be differentiated and clarified. In our view, the TRS/CSI-RS configuration update refers to the configuration content modification which involves the SI change procedure while the TRS/CSI-RS availability refers to whether some of the TRS/CSI-RS configurations are available, e.g. multiple TRS/CSI-RS configurations can be broadcast by the network.

For the TRS/CSI-RS availability issue, according to RAN1’s agreements, the TRS/CSI-RS availability is to be explicitly informed to idle/inactive UEs for avoiding UE’s blind detection and RAN1 would further discuss e.g. the signalling and detailed information. We can wait for more progresses from RAN1.

RAN1#104-e Update on 1/31: Agreements: For a cell with TRS/CSI-RS occasions configured for IDLE/Inactive UEs, IDLE/Inactive UE’s assumption on the availability of TRS/CSI-RS at the configured occasion(s) is informed to the idle/inactive UE based on explicit indication. - FFS details (e.g., the signalling, detailed information for the TRS/CSI-RS, etc.) - There is no intended blind detection of the presence/absence of TRS/CSI-RS at the UE side in this feature. That is, the UE assumes TRS/CSI-RS is not present if the network does not indicate it is available (or indicates it is unavailable).

For the TRS/CSI-RS configuration update issue, if the existing SI modification procedure is reused for the TRS/CSI-RS mechanism, the impact on legacy operation can be further considered in RAN2.

For example, in case the change of TRS/CSI-RS information is frequent, the SI modification will also be frequent. If the current SI modification procedure is directly reused without any enhancements, all UEs will be informed of SI change and subsequently acquire system information (at least MIB, SIB1), resulting in that irrelevant UEs (e.g., legacy UEs or UEs not supporting/using the TRS/CSI-RS mechanism) perform unnecessary SI update and waste power.

Besides, in the current SI modification procedure, the network should send SI change indication within a preceding modification period and updated SI message is broadcast in the next modification period, i.e., the system information can only be modified at certain time boundaries. Hence, if the SI modification procedure is reused for the TRS/CSI-RS mechanism, the network will not be able to modify the TRS/CSI-RS configuration at any time. These reference signals, however, are actually intended to be used by connected-mode UEs. The constraint caused by the TRS/CSI-RS SI modification mechanism may affect the use of the TRS/CSI-RS in connected-mode, which impacts the performance of connected-mode UEs.

During the last RAN2 meeting, many companies showed interest on the optimization on the SI change procedure. Hence, we propose to further consider this aspect. Possible enhancements for further study may be e.g.,

- How to ensure that the TRS/CSI-RS configuration modification only involves UEs that are using the TRS/CSI-RS mechanism or even more specifically using the RS resources which are changed.

- How to ensure that the network completes the TRS/CSI-RS configuration update procedure with less delay.

Proposal 4: Further optimization on the SI change procedure is considered to minimize the impact of the TRS/CSI-RS mechanism on legacy operations.

Conclusion

In this paper we discuss potential RAN2 impacts and work in terms of providing connected-mode TRS/CSI-RS to idle/inactive-mode UEs. Observations and proposals are summarized as below.

Providing TRS/CSI-RS configuration

Proposal 1: A new SIB is used to provide the TRS/CSI-RS configuration to idle/inactive-mode UEs.

Observation 1:	The maximum number of RS resources that can be configured in a SIB may be around 40 whereas there can be at most 64 beams in FR2, thus it is not clear whether 40 RS resources are sufficient.

Proposal 2: For the case that assistance RS is configured by SIB, the signalling overhead should be minimized, e.g. the configuration of the assistance RS comprises a common part and a RS-specific part. Details depend on RAN1 progress.

Proposal 3: Wait for more progress from RAN1 before determining whether dedicated RRC signalling, e.g. RRCRelease message can be used to provide the TRS/CSI-RS configuration to idle/inactive-mode UEs.

Updating TRS/CSI-RS configuration

Proposal 4: Further optimization on the SI change procedure is considered to minimize the impact of the TRS/CSI-RS mechanism on legacy operations.

References

- R2-2102387, Summary of [041][ePowSav] TRS/CSI-RS for IDLE INACTIVE.