3GPP TSG RAN WG2 Meeting #113bis-e                	R2-2103614

# Online meeting, 12 -20 April 2021

Agenda Item: 	8.11.2

Source:		Sony

Title:			Considerations on positioning latency

Document for:	Discussion & Decision

# Introduction

At RAN plenary #91, the following revised WID was agreed  [2], with new objectives based on mainly the conclusion of the RAN2 part of the positioning study item [1].

- Specify methods, measurements, signalling, and procedures for improving positioning accuracy of the Rel-16 NR positioning methods by mitigating UE Rx/Tx and/or gNB Rx/Tx timing delays, including [RAN1, RAN2, RAN3, RAN4] - DL, UL and DL+UL positioning methods - UE-based and UE-assisted positioning solutions - Specify the procedure, measurements, reporting, and signalling for improving the accuracy of [RAN1, RAN2, RAN3] - UL AoA for network-based positioning solutions. - DL-AoD for UE-based and network-based (including UE-assisted) positioning solutions. - Specify the enhancements of signalling, and procedures for improving positioning latency of the Rel-16 NR positioning methods, for DL and DL+UL positioning methods, including: - Latency reduction related to the request and response of location measurements or location estimate and positioning assistance data; [RAN2, RAN3, RAN1] - Latency reduction related to the time needed to perform UE measurements; [RAN1, RAN4] - Latency reduction related to the measurement gap; [RAN1, RAN4, RAN2] - Specify methods, measurements, signalling and procedures to support positioning for UEs in RRC_ INACTIVE state, for UE-based and UE-assisted positioning solutions, including [RAN2, RAN1, RAN3,RAN4]: - DL NR positioning methods and RAT-independent positioning methods - Support of UE positioning measurements for UEs in RRC_INACTIVE state - Reporting of positioning measurement or location estimate performed in RRC_INACTIVE when the UE is in RRC_INACTIVE state Note: this work will be coordinated with the SDT WI. - As 2nd priority: - UL and DL+UL NR positioning methods - Support of gNB positioning measurements for UEs in RRC_INACTIVE state - Specify on-demand transmission and reception of DL PRS for DL and DL+UL positioning for UE-based and UE-assisted positioning solutions, including: [RAN2, RAN1, RAN3] - UE-initiated request of on-demand DL PRS transmission; - LMF (network)-initiated request of on-demand DL PRS transmission; - Specify the signalling, and procedures to support GNSS positioning integrity determination, including [RAN2, RAN3]: - The assistance information that will be used to support integrity determination - The information that will be used to provide the positioning integrity KPIs and integrity results - Support of integrity for UE-based and UE-assisted A-GNSS positioning. Note: This objective is applicable to NR and E-UTRA. - Support the following enhancements of A-GNSS positioning [RAN2, RAN3, RAN4] - Specify support for BDS B2a signal - Specify support for BDS B3I signal - Specify support for NavIC to NR Note: This objective is applicable to NR and E-UTRA. - Study and specify, if agreed, the enhancements of information reporting from UE and gNB for multipath/NLOS mitigation [RAN1, RAN2, RAN3] - Discuss and specify new as well as the impact on the existing RAN4 requirements for positioning and other RRM measurements and corresponding procedures [RAN4] - Define extensions of signalling, protocol and procedure for NR positioning enhancement, as needed [RAN3]

In this contribution, we discuss our views on the introduction of the support for enhanced latency aspects

# Discussions on latency reduction

During study item phase, Reduced latency solutions was discussed, and in the subsequent work item, the following objectives were agreed [2].

- Specify the enhancements of signalling, and procedures for improving positioning latency of the Rel-16 NR positioning methods, for DL and DL+UL positioning methods, including:

  - Latency reduction related to the request and response of location measurements or location estimate and positioning assistance data; [RAN2, RAN3, RAN1]

  - Latency reduction related to the time needed to perform UE measurements; [RAN1, RAN4]

  - Latency reduction related to the measurement gap; [RAN1, RAN4, RAN2]

## Positioning Latency Reduction

- Low latency positioning is one of the KPI in NR positioning enhancement study in Rel-17. Based on the discussions in the previous meeting, the latency of the legacy NR positioning is still relatively high (e.g. more than 1000 ms) in comparison to the expected positioning latency target in Rel-17 (10s of ms). In RAN1#102e meeting, a number of agreements were made [3]:

- For reducing NR positioning latency, more efficient signaling & procedures will be investigated to enable a device to request and report positioning information, which may include, but not limited to, the following aspects:

  - DL PRS/UL SRS configuration, activation or triggering.

  - The request for positioning information (the assistance data, etc.).

  - The report of positioning information (the measurement report, etc.).

- Note: It is not within RAN1 scope to analyze positioning architecture enhancements to enable such more efficient signaling & procedures.

- Note: RAN1 does not make any assumptions on whether the LCS architecture specified in TS 23.273 is enhanced or not.

#### End to end latency:

In our previous RAN1 contribution [5], we discussed the best achievable physical layer latency of DL-TDoA in Rel-16. We observed, it is still far higher than 10 ms as the expected physical layer latency in Rel-17. In our view, RAN1 shall focus on reduction of physical layer latency. This covers at least related to the triggering aspect, reference signal transmission, processing (e.g. measurement), and reporting of the positioning measurements. In case of downlink-based positioning, physical layer latency Start and End times are defined as follows [3]:

| Method | Start | End |
| --- | --- | --- |
| UE assisted DL-only & DL-ECID & Multi-RTT | Transmission of the PDSCH from the gNB carrying the LPP Request Location Information message | Successful decoding of the PUSCH carrying the LPP Provide Location Information message |

- In case of DL-PRS transmission with low latency support, we can consider it can be triggered by L1-procedure. Hence, a DCI carries information of DL-PRS allocation is needed. This can significantly shorten the measurement gap, especially if the gNB has been configured with long PRS periodicity.

Positioning latency requirements includes both end-to-end positioning latency and physical layer positioning latency. On end-to-end latency, RAN1 has sent an LS to RAN2 for analysis of latency of NR positioning protocols defined in Rel.16 including any potential enhancements, since end-to-end latency may involve higher layer protocol and signaling to core-network. Furthermore end to end latency also has core network aspects.

One simplified example for legacy signalling sequence affecting the end to end latency for UE assisted DL-only (DL-TDoA) is elaborated here, and shown in  Figure 1. LS sends a positioning measurement request to the UE via LPP protocol (and transparent to gNB). The time when the UE receives this request is considered as the start of physical layer latency. The UE performs measurements of the PRS from multiple gNB´s within a measurement gap as the PRS may have different configuration with the existing UE active bandwidth part (BWP). Once the UE has obtained the positioning measurement data, the UE sends an uplink scheduling request (SR) to the serving gNB. The serving gNB sends the corresponding UL grant in downlink control channel (PDCCH). The UE then finally transmits the positioning measurement reports in uplink data channel (PUSCH) to the LS. The time when the gNB manages to decode PUSCH is known as end of physical layer positioning latency. There are multiple steps involved within the start and the end as described above. In the last meeting, it was proposed to define 10ms as physical layer latency for IIoT use-cases.

Figure 1: Signalling mechanism in DL-TDOA positioning technique

PRS configuration

PRS configuration

SR request

Meas GAP

PRS transmission

Positioning meas request

UE

gNB

LS

POS meas

UL grant

POS meas_rep

Observation  1: End-to-end positioning latency analysis requires involvement from the following WGs: RAN1, RAN2, RAN3 and SA2.

#### Measurement report:

Another component that can contribute to positioning latency reduction is the transmission of positioning measurement report. In the legacy NR positioning, once the UE obtains positioning measurement report the UE transmits scheduling request, requesting uplink allocation (PUSCH) with positioning measurement report as the payload. Then, the UE needs to wait for the reception of uplink grant in PDCCH prior to the PUSCH transmission. When the LS transmits positioning request to the UE, the LS can also make the serving gNB aware of the transmission of positioning request to one of the UE in that cell. To shorten this step, we consider to introduce a positioning measurement report, with configured grant (CG) mechanism. The gNB can allocate resources for the UE that has been requested to perform positioning procedure. The configured grant resources are used by the UE to transmit the positioning measurement results. Note: configured grant mechanism has been part of the NR URLLC. Hence, the specification impact can be kept minimum.

In case configured grant is not deployed, we can also consider the UE to skip scheduling request (SR) transmission. In addition, as the process in the legacy approach, it is not guaranteed that the gNB receives or immediately allocate the uplink resources (PUSCH). This will further increase the latency. A method to reduce the latency is to configure the UE to monitor downlink control channel (PDCCH) containing uplink grant after the measurement gap.

- Observation 2: Physical layer enhancements, such as scheduling information for PRS resource allocations and UE positioning measurement report is handled by RAN1.

In RAN2, responsible to support the signalling between the UE and gNB or basically related to RAN procedures covering LPP signalling, RRC signalling, and MAC layer. Optimization in those aspect can facilitate L1 improvement, including measurement reporting.

Proposal 1: Positioning latency reduction is facilitated by RAN2 in the context of RAN procedures/protocol on LPP signalling, NRPPa, RRC signalling, and MAC layer.

According to the recently closed NR positioning enhancement study item technical report [3], Aperiodic reception of DL PRS from the TRPs of the serving gNB and aperiodic reception of DL PRS from the TRPs of the neighbouring gNBs can be studied further. These type of DL PRS transmission requires triggering mechanism at the gNB/TRP so that gNB/TRP knows the time and resources to transmit DL PRS. We consider the LMF to provide positioning configuration, including triggering signal, to gNB/TRP by using NRPPa protocol.

Proposal 2: Enhance NRPPa protocol to enable LMF to provide positioning configuration to gNB/TRP, including triggering signal (e.g., trigger to transmit aperiodic DL PRS).

# Summary

In this contribution, we have discussed our view on potential positioning enhancements in NR. Our observation and proposals are listed below:

Observation  1: End-to-end positioning latency analysis requires involvement from the following WGs: RAN1, RAN2, RAN3 and SA2.

Observation 2: Physical layer enhancements, such as scheduling information for PRS resource allocations and UE positioning measurement report is handled by RAN1.Physical layer enhancements such as XXX is handled by RAN1.

Proposal 1: Positioning latency reduction is facilitated by RAN2 in the context of RAN procedures/protocol on LPP signalling, NRPPa, RRC signalling, and MAC layer.

Proposal 2: Enhance NRPPa protocol to enable LMF to provide positioning configuration to gNB/TRP, including triggering signal (e.g., trigger to transmit aperiodic DL PRS).

# References

- RP-202094 Revised SID: Study on NR Positioning Enhancements

- RP-210903, Revised WID on NR Positioning Enhancements

- TR 38.57 version 17.0 Technical Report: Study on NR Positioning Enhancements, March 2021