3GPP TSG RAN WG2 Meeting# 113bis electronic   	                            R2-2103618

Online, April 12 – April 20, 2021

Agenda Item:	8.16.2

Source: 	Sony

Title:	Cell reselection using credentials from a separate entity

Document for:	Discussion

# Introduction

In this contribution we look into SA2 response and analyse its impacts on cell reselection.

# Discussion

2.1 Credentials from a separate entity

RAN2 asked SA2 the question about Group ID and SA2 response is:

Question 1: Can RAN2 assume uniform support of external authentication related parameters (i.e., indicator for "access using credentials from a separate entity is supported", GID(s) ) , and indicator for "whether the SNPN allows registration attempts from UEs that are not explicitly configured to select the SNPN") across a network or a registration area? [SA2 answer] Yes, These parameters should be set uniformly per SNPN. Question 2: Shall Group IDs be broadcasted per SNPN or per cell? [SA2 answer] Yes, It is assumed that that the Group IDs will be broadcast per SNPN.

Based on SA2 response, our understanding is that Group ID support is uniform throughout the SNPN and a Group ID may be supported over multiple SNPNs or a single SNPN may support multiple Group IDs. RAN2 has already agreed that Group ID will be broadcast and UE AS layer will report Supported Group IDs to NAS. RAN2#113e agreements are:

In the UE, AS reports to NAS about the following broadcasted new parameters: - Indicator that "access using credentials from a separate entity is supported" in the cell per SNPN - Supported Group IDs - Indicator that "whether the SNPN allows registration attempts from UEs that are not explicitly configured to select the SNPN" per SNPN.

- The related question for RAN2 discussed during RAN2#113e was:

- Is the GID selected by NAS given to AS after registration to assist UE subsequence cell selection and reselection?

Based on SA2 response, Group ID is ubiquitously supported in an SNPN and does not require special treatment to cell selection/reselection, so we propose:

Proposal: RAN2 to agree that NAS does not provide selected Group IDs to AS layer to assist subsequent cell selection and reselection.

The other question discussed in RAN2#113e was:

- Should AS support the (IDLE/INACTIVE/CONNECTED mode) mobility scenarios between different SNPNs or SNPN and PLMN when the same credentials can be used on the source and the target networks? E.g. Can a UE move from SNPN#1 to SNPN#2 when the GID used to access SNPN#1 is supported by SNPN#2? Can a UE move between SNPN#1 to PLMN#a when the credential of PLMN#a is used to access SNPN#1? - Shall Group IDs be broadcasted per SNPN? (or per cell?)

Questions related to connected mode are in RAN3 scope and other aspects are already covered in P1.

# Conclusion

We propose RAN2 to discuss and agree on following proposals:

Proposal: RAN2 to agree that NAS does not provide selected Group IDs to AS layer to assist subsequent cell selection and reselection.

# References

[1]	RP-202363: New WID: NR Non-Public Network enhancements

[2]	R2-210xxxx: Reply LS on clarification request for eNPN features, SA2