3GPP TSG-RAN WG2 Meeting #113bis-e     			          	                         R2-2103695

Electronic Meeting, April 12 – 20, 2021

Agenda item:	8.8.2

Source:	CMCC

Title:	Discussion on slice based cell reselection

WID/SID:	NR_slice

Document for:	Discussion and Decision

1	Introduction

This contribution discusses on the slice deployment scenarios and slice based cell reselection for the Release 17 work item “Enhancement of RAN Slicing for NR”.

2	WI objectives

A Release 17 study item “Study on enhancement of RAN Slicing” was concluded in RAN2#113-e, and as the output of the study item, RAN2 part of TR 38.832[1] was approved in RAN#91-e. Furthermore, the work item “Enhancement of RAN Slicing for NR” was approved in RAN#91-e. The followings are the objectives of this work item:

The work item aims to standardize the enhancement on RAN support of network slicing. Detailed objectives of the work item are: - Support slice based cell reselection, specify mechanisms and signalling including [RAN2] - To assist cell reselection, broadcast the supported slice info of the current cell and neighbour cells, and cell reselection priority per slice in system information message. - To assist cell reselection, include slice info (with similar information as in SI message) in RRCRelease message. - Support slice based RACH configuration, specify mechanisms and signalling including, for Mobile Originating cases [RAN2] - Configure separated PRACH configuration (e.g., transmission occasions of time-frequency domain and preambles) for slice or slice group - Configure RACH parameters prioritization (e.g., scalingFactorBI and powerRampingStepHighPriority) for slice or slice group - Determine how this works with existing functionality, which may include how to perform RACH type selection (e.g., 2-step and 4-step), support of RACH fall-back cases, handling of simultaneous configuration with similar functions such as legacy RA prioritization (e.g., MPS and MCS UEs). Note: The use of Rel-17 RAN slicing enhancements in given cells shall not prevent from accessibility for Rel-15 and Rel-16 UEs. Note: RAN3 objectives to be added after RAN3 study phase is finished. Note: This work item should take SA2 output on slicing enhancement into consideration if RAN impacts are identified, e.g. the relation between Tracking Area and S-NSSAI is expected to impact the solution for slice based cell reselection.

3	Discussion

3.1	Discussion on slice deployment scenarios

SA2 studied on key issue #7 “Support of 5GC assisted cell selection to access network slice” and the latest conclusions are captured in TR 23.700-40 v2.0.0[2].

Here are TA related conclusions in SA2:

- The following is assumed e.g. to support legacy UEs: - all cells of a tracking area support the same S-NSSAI(s); and - the S-NSSAI(s) of the Allowed NSSAI are supported by all tracking areas in a registration area.

The above statement only covers the legacy case for R15 and R16. SA2 haven’t reached agreement on whether the above statement is applicable for R17 or not.

Observation 1: SA2 agreed on the homogeneous S-NSSAI within TA for R15/R16, but no agreement for R17.

As triggered by SA2’s LS, RAN2 also discussed on the TA issue, but no agreement was reached for R17. It is observed that, for R17, some companies prefer to follow the legacy principle, while other companies would like to left some flexibility for operators’ deployment. If a solution is applicable for inhomogeneous deployment, it is also applicable for homogeneous deployment. So, we propose to specify a cell reselection solution that is appliable for inhomogeneous S-NSSAI deployment within TA, which will by default applicable for homogeneous case.

Observation 2: if a solution is applicable for inhomogeneous deployment, it is also applicable for homogeneous deployment.

Proposal 1: RAN2 aims to specify a cell reselection solution that is appliable for inhomogeneous S-NSSAI deployment within TA, which will by default applicable for homogeneous case as well.

Assuming proposal 1 is agreeable. For example, as shown in Figure 1, cell 1 and cell 2 are covering the same location 1 but supporting different slices. Both slice 1 and slice 2 are available within location 1, by means like HO, re-direction or DC/CA. Similarly, both slice 1 and slice 2 are available within location 3 and location 4. Therefore, location 1, 3 and 4 may be deployed in the same RA and TA.

Figure 1: Examples for slice deployment scenarios [1]

As captured in TR 38.832[1], RAN2 common understanding is that intended slice is based on the information AS receives from NAS for the particular use case. For idle-mode mobility, intended slices = allowed S-NSSAI(s). And according to TS 38.413[3], the maximum number of allowed S-NSSAI is 8.

Table 1: Allowed S-NSSAI [3]

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Allowed S-NSSAI List |  | 1 |  |  |
| >Allowed S-NSSAI Item |  | 1..<maxnoofAllowedS-NSSAIs> |  |  |
| >>S-NSSAI | M |  | 9.3.1.24 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofAllowedS-NSSAIs | Maximum no. of allowed S-NSSAI. Value is 8. |

In Release 15 and Release 16, the UE needs to (re)select a cell which supports all slices in Allowed S-NSSAI List because of the homogenous support in a TA. However, if different cells in the same TA may support different slices, for example, when the UE with Allowed S-NSSAI List = {slice 1, slice 2} stays in Location 3, the UE cannot find a cell which can support both slice 1 and slice 2.

Observation 3: In the inhomogeneous deployment scenario, there is the case that no cell can support all the slices in Allowed S-NSSAI List in current area, i.e. each cell can only support part of the Allowed S-NSSAI List.

Therefore, we need to discuss the scenario that if deployments without homogenous support in a TA may be possible, how the UE performs cell reselection procedure when the Allowed S-NSSAI List includes more than one slices.

Proposal 2: RAN2 further discuss how the UE performs cell reselection procedure when the Allowed S-NSSAI List contains multiple slices, while there is no cell can support all the slices in the allowed list.

3.2	Intended slices for UE in RRC_INACTIVE and slice based cell (re)selection

In TR 38.832[1], there is only agreement for the intended slices for UE in RRC_IDLE as mentioned above, i.e. intended slices = allowed S-NSSAI(s). According to TS 38.331[4], upon the UE receives the RRCRelease message including suspendConfig, the UE shall suspend all DRB(s) and enter RRC_INACTIVE and perform cell selection. For UE in RRC_INACTIVE, it may have suspended DRB(s) for some slices. When the UE receives the RRCResume message, the UE shall resume all DRB(s) including DRB(s) for some slices.

5.3.8.3 Reception of the RRCRelease by the UE The UE shall: <omitted> 1> if the RRCRelease includes suspendConfig: <omitted> 2> suspend all SRB(s) and DRB(s), except SRB0; <omitted> 2> indicate the suspension of the RRC connection to upper layers; 2> enter RRC_INACTIVE and perform cell selection as specified in TS 38.304 [20]; 1> else 2> perform the actions upon going to RRC_IDLE as specified in 5.3.11, with the release cause 'other'.

Based on the above analysis, it may be more suitable to specify the intended slices = suspended slices for inactive-mode mobility. And it’s beneficial that the inactive UE selects or reselects to a cell which supports suspended slices for fast resume the corresponding slice services.

Proposal 3: For inactive-mode mobility, intended slices = suspended slices.

Proposal 4: The UE in RRC_INACTIVE should select or reselect to a cell which supports suspended slices.

3.3	Slice based cell reselection

In SA2#143E meeting, the following conclusions for network based solution without UE impact were agreed:

- To enable the possibility where certain frequencies, supported by the network i.e. PLMN or SNPN, cannot be used to access a network slice, a solution without UE impacts is required as to ensure any UE can be directed towards a frequency/cell from which the UE can access the network slice. - The principles of solution #17/46 are used as basis for network based solution with no UE impacts. The 5GC may provide to the RAN, in addition to the Allowed NSSAI as in Rel-16, additional assistance information to steer the UE to another cell. The assistance information can be at least one of the rejected S-NSSAIs, the Configured NSSAI, or Target NSSAI and RFSP. This assistance information allows the RAN to redirect the UE to a cell supporting network slices not available in a current cell and RA. NOTE: The details about the assistance information will be decided during the normative phase.

The baseline solution #17 and #46 in TR 23.700-40[2] both have no impact on UE. With the help of assistance information from core network, when RAN received RRCSetupComplete from UE containing a requested NSSAI that the current cell doesn’t support, RAN can handover or redirect the UE to another frequency or cell which supports the requested NSSAI.

The SA2’s solution #17 and #46 addresses key issue #7 “Support of 5GC assisted cell selection to access network slice” which means the solution only to address slice aware cell selection, but not cell reselection.

Observation 4: The SA2’s solution #17 and #46 addresses key issue #7 “Support of 5GC assisted cell selection to access network slice” which means the solution only to address slice aware cell selection, but not cell reselection.

To assist cell reselection, RAN can broadcast the supported slice info of the current cell and neighbour cells, and cell reselection priority per slice in system information messages. During the SI phase, some companies had concerns on security and SIB payload size. Several potential solutions were also provided, such as by providing only SST, on-demand SIB, SIB segmentation, slice grouping or slice associated UAC information. Table 1 shows the pros and cons of these solutions:

Table 1: Pros and cons of potential solutions for cell selection

| The potential solutions | Pros. | Cons. |
| --- | --- | --- |
| Option 1: Providing only SST | It can reduce the SIB payload size, may address the security concern. | Providing only SST would be too broad for UE to decide whether to access this cell as there would be at most 224=16,777,216 slices with different Slice Differentiator for each SST. The UE may not reselect a correct cell which supports the intended slices of UE. |
| Option 2: On-demand SIB or SIB segmentation | It may reduce the SIB2/3/4/5 payload size because they contain cell reselection information. | It will introduce additional delay in acquisition of on-demand SIB or SIB segmentation, while the total size of the slice information to be broadcast is not reduced. It cannot address the security concern. |
| Option 3: slice grouping | If the slice grouping or slice associated UAC information is negotiated between UE and the CN via NAS signalling, it can address the security and SIB payload size concerns. | For the initial access, the UE cannot acquire the slice grouping information from CN. This option can be only used to the cell reselection procedures after initial registration procedure. This option will impact on SA2/CT1 specs. |
| Option 4: slice associated UAC information | According to TS 24.501 [5], “Operator-defined access category definitions can be signalled to the UE using NAS signalling”, it can address the security and SIB payload size concerns. | For the initial access, the UE cannot acquire the slice associated UAC information from CN. This option can be only used to the cell reselection procedures after initial registration procedure. According to TS 38.331 [4], if the access category chosen by NAS is not included in the system information, the AS layer will consider the related access attempt allowed. If a cell is to forbid some slices, it has to put the relevant access categories in its SI messages. This may increase SIB payload size and cause more SI message updates. |

In cell reselection, the latency introduced in acquisition of on-demand SIB or SIB segmentation is usually tolerable because the reselection parameter TreselectionRAT is in seconds according to TS 38.331[4] and TS 38.304[6]. In addition, we expect that the UE can reselect to a cell that supports intended slices during the cell reselection procedure. Therefore option 1 is not suitable for cell reselection, option 2 alone cannot solve the security and payload size concerns, option 3 and 4 is more suitable for cell reselection and can be discussed further.

Proposal 5: Slice grouping or UAC is supported as baseline to address SIB concerns.

4	Conclusion

Here are the proposals for RAN slicing enhancement.

Observation 1: SA2 agreed on the homogeneous S-NSSAI within TA for R15/R16, but no agreement for R17.

Observation 2: if a solution is applicable for inhomogeneous deployment, it is also applicable for homogeneous deployment.

Proposal 1: RAN2 aims to specify a cell reselection solution that is appliable for inhomogeneous S-NSSAI deployment within TA, which will by default applicable for homogeneous case as well.

Observation 3: In the inhomogeneous deployment scenario, there is the case that no cell can support all the slices in Allowed S-NSSAI List in current area, i.e. each cell can only support part of the Allowed S-NSSAI List.

Proposal 2: RAN2 further discuss how the UE performs cell reselection procedure when the Allowed S-NSSAI List contains multiple slices, while there is no cell can support all the slices in the allowed list.

Proposal 3: For inactive-mode mobility, intended slices = suspended slices.

Proposal 4: The UE in RRC_INACTIVE should select or reselect to a cell which supports suspended slices.

Observation 4: The SA2’s solution #17 and #46 addresses key issue #7 “Support of 5GC assisted cell selection to access network slice” which means the solution only to address slice aware cell selection, but not cell reselection.

Proposal 5: Slice grouping or UAC is supported as baseline to address SIB concerns.

References

- 3GPP TR 38.832, NR; Study on enhancement of Radio Access Network (RAN) slicing (Release 17)

- 3GPP TR 23.700-40, Study on enhancement of network slicing; Phase 2 (Release 17)

- 3GPP TS 38.413, NG-RAN; NG Application Protocol (NGAP) (Release 16)

[4]  3GPP TS 38.331, NR; Radio Resource Control (RRC) protocol specification (Release 16)

[5]  3GPP TS 24.501, Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3; (Release 16)

[6] 3GPP TS 38.304, NR; User Equipment (UE) procedures in Idle mode and RRC Inactive state (Release 16)