3GPP TSG-WG2 Meeting #113bis electronic    	     R2-2103720

Online, April 12 – April 20，2021

Source: 			CMCC

Title:  Consideration on Uu adaption layer

Agenda item:	8.7.4.2

Document for: 	Discussion

Introduction

In the past RAN2 e-meeting and RAN plenary, Uu adaption layer had been discussed and some basic agreements had been achieved as following:

- For L2 UE-to-NW relay, the Uu adaptation layer at Relay UE supports UL bearer mapping between ingress PC5 RLC channels for relaying and egress Uu RLC channels over the Relay UE Uu path.

- For L2 UE-to-NW relay, the different RBs of the same Remote UE and/or different Remote UEs can be subject to N:1 mapping and data multiplexing over Uu RLC channel

- For L2 UE-to-NW relay, Uu adaptation layer is used to support Remote UE identification for the UL traffic (multiplexing the data coming from multiple Remote UEs).

- The Uu adaptation layer can be used to support DL bearer mapping at gNB to map end-to-end Radio Bearer (SRB, DRB) of Remote UE into Uu RLC channel over Relay UE Uu path

- The identity information of Remote UE Uu Radio Bearer and Remote UE is included in the Uu adaptation layer at UL in order for gNB to correlate the received data packets for the specific PDCP entity associated with the right Remote UE Uu Radio Bearer of a Remote UE.

- The Uu adaptation layer can be used to support DL N:1 bearer mapping and data multiplexing between multiple end-to-end Radio Bearers (SRBs, DRBs) of a Remote UE and/or different Remote UEs and one Uu RLC channel over the Relay UE Uu path

- The Uu adaptation layer needs to support Remote UE identification for Downlink traffic

- The identity information of Remote UE Uu Radio Bearer and the identity information of Remote UE needs be put into the Uu adaptation layer by gNB at DL in order for Relay UE to map the received data packets from Remote UE Uu Radio Bearer to its associated PC5 RLC channel

In this contribution, we will further discuss functionality design and header format for adaption layer.

Discussion

From our perspective, the uu adaption layer functionalities of L2 U2N relay described in TR38.836 are sufficient and clear [2]. For uplink relaying traffic, the different end-to-end RBs (SRB, DRB) of the same Remote UE and/or different Remote UEs can be subject to N:1 mapping and data multiplexing to one Uu RLC channel. Relay UE may need to inform gNB the source of the traffic (i.e. comes from which Remote UE). Remote UE identification should be supported at Uu adaptation layer for UL packets. Moreover, the Relay UE needs to indicate the exact Remote UE Uu Radio Bearer to gNB. The identity information of Remote UE Uu Radio Bearer needs be include at Uu adaptation layer. In case of multiple Remote UEs based relaying, the identity information of Remote UE needs also be put by Relay UE at Uu adaptation layer. However the details on the identity information of Remote UE Uu Radio Bearer and Remote UE identification has not been conclude until now.

Figure 1: Mapping between multiple remote UEs and relay UE
(adaptation layer is not supported at the PC5 interface)

Identity information of Remote UE Uu Radio Bearer

The identification of Remote UE uu Radio Bearer is used for the gNB to deliver the received data packets to the specific PDCP entity associated with the right Remote UE Uu Radio Bearer. The identification information should be uniquely identifiable for it remote UE and can be matched to specific PDCP. With the above motivations, drb-Identity and srb-Identity or logicalChannelIdentity can be considered. drb-Identity and srb-Identity are both configured per RadioBearerConfig by network during the Remote UE RRC connection setup or RRC reconfiguration. logicalChannelIdentity is configured within RLC-BearerConfig, which is more directly associated with the right Remote UE Uu Radio Bearer. Since the size of drb-Identity and srb-Identity or logicalChannelIdentity is 32, so at least 5bits indicator is enough.

Proposal 1: Considering drb-Identity and srb-Identity or logicalChannelIdentity as Identity information of Remote UE Uu Radio Bearer, and the length can be at least 5bits.

Remote UE identification

As captured in the TR38.836, Remote UE identification is supported at Uu adaptation layer to inform gNB the source of the traffic. According to the agreements on RRC connection establishment procedure, Remote UE setups RRC connection with gNB before relay UE relays data and signaling to gNB and the ue-Identity is include in RRCSetupRequest. From network point view, remote UE can be recognized uniquely after the first RRC message is successfully received by network. In additional, the PC5 connection of remote UE and relay UE is known for network. So, for the UE identification included in adaption layer, the absolute UE identification, e.g., UE-Identity (39bits) is not needed any more. On the contrary, an index indicates the ID of the remote UE that the relay UE relaying is sufficient for identify the remote UE for a specific relay UE. The size of index is up to the maximum number of remote UEs can be allowed to be relayed by one relay UE.

Proposal 2: An index indicates the ID of the remote UE that the relay UE relaying is sufficient; The size of index is up to the maximum number of remote UEs can be allowed to be relayed by the relay UE.

Conclusions

According the above discussion we have following proposals:

Proposal 1: Considering drb-Identity and srb-Identity or logicalChannelIdentity as Identity information of Remote UE Uu Radio Bearer, and the length can be at least 5bits.

Proposal 2: An index indicates the ID of the remote UE that the relay UE relaying is sufficient; The size of index is up to the maximum number of remote UEs can be allowed to be relayed by the relay UE.

References

- R2-2009122, post 111-e 627 Relay remaining issues on L2 architecture.

- 3GPP TR 38.836, Study on NR sidelink relay;

- 3GPP TS 38.331, Radio Resource Control (RRC) protocol specification