3GPP TSG-RAN WG2 Meeting # 113 bis Electronic	R2-2103831

Online, April 12 – April 20, 2021

Agenda Item:	[8.3.3] [UE notification on network switching for multi-SIM]

Source: 	Apple Inc

Title:  	Signaling aspects of MUSIM Network Switching

Document for:	Discussion and Decision

# Introduction

From the last RAN2#113 Electronic meeting, the following agreements were made with respect to the MUSIM Network Switching work item [1].

- Agreements

- 1	Switching procedure can be used to notify network A that the UE has a preference to leave RRC_CONNECTED state in network A.

- 2	The switching procedure can be used to notify network A that the UE has a preference to be kept in RRC_CONNECTED state in network A while temporarily switching to network B.

In this contribution, we present our views on the signalling aspects of MUSIM network switching, including UE – NW capability exchange to align on the attributes that UE can send while requesting for a switch.

# Discussion

## Network Switching Use cases and issues

Current MUSIM implementations have proprietary mechanisms to switch back and forth across the two networks in which the MUSIM UE SIM instances are currently camped. Typically this network switching is done without any explicit signalling to their respective networks. For example for the cases when both SIM instances are camped in IDLE/INACTIVE state in their respective networks, the MUSIM UE can periodically switch networks  to decode the respective paging occasion on each SIM instance. Since each SIM instance is in IDLE/INACTIVE mode, no explicit indication is required to the respective networks.

In other cases, when one of the SIM instance (1) is in CONNECTED state with its network (A), while the other SIM instance (2) is in IDLE/INACTIVE state with its network (B). In cases, when a higher priority activity is triggered on SIM instance (2),  current MUSIM implementations would locally suspend the CONNECTED mode activity on SIM instance (1) and do a network switching without indicating any signalling with network (A), thereby leading to potential RRC state mismatch between the SIM instance (1) and its network (A).  A sample scenario is illustrated below in Figure 1.

Figure 1 : CONNECTED / IDLE RRC State Mismatch Issue

Similarly, when one of the SIM instance (1) is in INACTIVE state with network (A), while the other SIM instance (2) is in IDLE state with its network (B), any high priority activity on SIM instance (2) would force it to enter CONNECTED state with its network (B). In such cases, the SIM instance (1) which is in INACTIVE state, will be local suspended, pending completion of the higher priority activity on SIM instance (1). At this point, SIM instance (2) would enter CONNECTED state, and any INACTIVE state paging to SIM instance (1) would not be received by the MUSIM UE. As a result, network (A) might potentially transition the SIM instance (1) to IDLE state. In the meantime, after completion of high priority activity on SIM instance (2) when the SIM instance (1) is locally resumed, it will go back to INACTIVE state, and there is a potential RRC state mismatch between SIM instance (1) and network (A). A sample scenario is illustrated below in Figure 2.

Figure 2 : INACTIVE / CONNECTED RRC State Mismatch Issue

Note that the high priority activity on SIM instance (2) can be a periodic page monitoring (for voice and/or data) or a user triggered activity that requires SIM instance (2) to enter into CONNECTED state or autonomous UE activity like OOS search, background PLMN search, high priority PLMN search etc.

This kind of abrupt network switching leads to

- Radio resource wastage as network (A) would still assume that SIM instance (1) is connected to it, whereas the MUSIM UE is actually switched to network (B).

- Poor network KPIs due to the MUSIM UE SIM instance (1) suddenly “disappearing” from network (A).

- Suboptimal UE performance in SIM instance (1), due to frequent interruptions from the other SIM instance (2) (e.g.) Degraded throughput performance in SIM instance (1)

- RRC state mismatch between UE SIM instance (1) and network (A)

Observation 1 : There are use cases when one SIM instance is in CONNECTED state, and other SIM instance is in IDLE/INACTIVE state, uninformed network switching between SIM instances can lead to poor UE and NW performance and degraded KPIs.

Observation 2 : There are use cases when one SIM instance is in INACTIVE state, and other SIM instance is in IDLE/INACTIVE state, uninformed network switching between SIM instances can lead to poor UE and NW performance and degraded KPIs.

Proposal 1 : A graceful switching mechanism has to be defined and standardized to ensure that both the MUSIM UE and the network are in sync about their respective RRC states, during the course of MUSIM suspension and resumption.

Prior to any UE – NW signalling for MUSIM network switching purpose, it is essential that both UE and NW are aligned on the UE’s MUSIM capability, and the set of attributes based on which UE can send a NW switching request. The set of attributes requested as part of the switching request can include,

- Whether the NW allows the MUSIM UE SIM instance in CONNECTED mode and intending to switch to the other NW, to request for a specific RRC State (IDLE/INACTIVE/CONNECTED)

- Whether the NW allows the MUSIM UE SIM instance in CONNECTED mode and intending to switch to the other NW, to request for a duration of switch

- Whether the NW allows the MUSIM UE SIM instance in CONNECTED mode and intending to switch to the other NW, to indicate a cause for the switch (e.g. Signalling, Voice Call etc)

- Whether the NW allows the MUSIM UE SIM instance in CONNECTED mode intending to switch to the other NW, to request for a one time switch (e.g. due to a service recovery on the other SIM instance)

- Whether the NW allows the MSUIM UE SIM instance in CONNECTED mode intending to switch to the other NW, to request for a periodic switch (similar to gap patterns) (e.g. for periodic IDLE/INACTIVE page monitoring on the other SIM instance)

Observation 3: To address MUSIM coordination issues like graceful network switching, it is beneficial for the UE to indicate its MUSIM capability to network, to help in proper coordination between UE and network for MUSIM use cases.

Observation 4 : UE and NW can apriori align on the set of attributes based on which the UE is allowed to request for graceful Network switching.

Proposal 2: UE and NW can exchange MUSIM and Network switching attribute as part of initial capability handshake

Alternatively, instead of the NW having to signal such attribute(s) individually to each MUSIM UE, the Network can choose to broadcast this and any  other MUSIM specific information common to all MUSIM UEs by means of a new System Information message.

Proposal 3 : Introduce a new MUSIM specific SIB type to carry all MUSIM related information applicable to all MUSIM UEs

It is evident that only for a UE in CONNECTED mode, it is required to indicate a Network Switching request to its NW. Based on the pre-agreed set of attributes (either via capability exchange or via broadcast), a UE in CONNECTED mode can use the UE ASSISTANCE INFORMATION signalling framework to request for a network switch.

Observation 5 : MUSIM UE in CONNECTED state need to signal some key MUSIM suspension attributes to help in graceful network switching.

Proposal 4 : MUSIM UE in CONNECTED state, can reuse the UE ASSISTANCE INFORMATION signalling approach to signal the MUSIM Suspension attributes to the network to help in graceful network switching.

In the last RAN2#113-e meeting, it is already agreed that by means of the switching request, the UE can indicate its preference to either stay or leave the RRC CONNECTED state. It is logical for the NW to maintain the RRC CONNECTED state of the UE only if the intended switching is for a short duration (e.g. to complete a short signalling activity or do IDLE/INACTIVE page decode on the other SIM instance).  In certain other cases, it is better for both UE and NW to leave the RRC CONNECTED state,  given the nature of the activity on the other SIM instance which triggered the network switching request in the first place. Based on the particular use case, the suspension duration (if allowed to be requested by UE) can help to determine the impending RRC state.

Observation 6 : The suspension duration requested by MUSIM UE can be used to determine the current RRC state of the SIM that is requesting for the suspension.

Proposal 5 : A short suspension duration for a given SIM instance implies, that the SIM instance prefers to hold the CONNECTED mode context.

Proposal 6 : A long suspension duration for a given SIM instance implies, that the SIM instance prefers to exit the current CONNECTED mode context.

It is expected that the scenario of long suspension duration would be used typically when the suspension is triggered due to a voice call scenario on the other SIM instance. In such cases, it is critical that the voice call setup on the other SIM instance is not delayed due to the RRC connection release activity on the current CONNECTED mode MUSIM UE SIM instance. In order to ensure a good user experience, the MUSIM UE should be expected to wait only for a reasonable time to receive the RRC Release message before triggering the network switch.

Observation 7 : The long suspension and follow up RRC Connection release signaling should not negatively impact the call setup duration on the other SIM instance.

Proposal 7 : MUSIM UE triggering long suspension and having an intent to go out of RRC CONNECTED mode should be expected to wait for only for a reasonable time for receiving the RRC RELEASE message, after which once the timer expires, it can autonomously switch to the other SIM instance.

To handle the case of resumption, post suspension due to MUSIM, the resumption strategy would depend on the current RRC state of the suspended SIM instance.

- In case, the suspended SIM instance is in RRC INACTIVE state, the MUSIM UE can use the RRC RESUME REQUEST with the cause rnaUpdate as a means to ensure RRC state synchronization with its respective network.

- In case, the suspended SIM instance is in RRC IDLE state, the MUSIM UE can use the legacy RRC CONNECTION SETUP procedures, to trigger any future state transitions out of RRC IDLE.

Observation 8 : Post suspension, the resumption of the suspended MUSIM instance is dependent on the current suspended UE SIM instance state (INACTIVE or IDLE).

Proposal 8 : A MUSIM UE SIM instance which is suspended and is in RRC INACTIVE state can use the RRC RESUME REQUEST with cause rnaUpdate as a means to ensure RRC state synchronization with its network.

Proposal 9 : A MUSIM UE SIM instance which is suspended and is in RRC IDLE state can use the legacy RRC CONNECTION SETUP procedures to trigger any future state transitions out of RRC IDLE.

## Minimizing interruptions due to suspended SIM instance in RRC INACTIVE state

A MUSIM UE SIM instance which is in RRC INACTIVE state during the course of it suspension (which has been triggered due to the higher priority activity on the other SIM instance), would be running an instance of timer T380. On the expiry of the timer T380, it is expected that this SIM instance for which the timer T380 has expired, shall trigger the periodic RNA update. It is to be noted that the basic premise of this SIM instance moving into RRC INACTIVE state is to be in suspended mode, so as not to interfere with the ongoing higher priority activity on the other SIM instance. So, any periodic RNA update activity triggered would result in counter suspension and can lead to ping-pong suspension/resumption scenarios, which can be avoided if possible. In such cases, network can configure a higher value of T380 for those devices which are suspended due to MUSIM cause and are in RRC INACTIVE state.

Observation 9 : A MUSIM UE SIM instance suspended due to MUSIM cause, and in RRC INACTIVE state, when its timer T380 expires can result in ping-pong suspension/resumption scenarios, which should be avoided.

Proposal 10 : Networks can configure a higher value for T380 as part of the suspend configuration, while configuring the MUSIM UE SIM instance to RRC INACTIVE state.

# Conclusion

Observation 1 : There are use cases when one SIM instance is in CONNECTED state, and other SIM instance is in IDLE/INACTIVE state, uninformed network switching between SIM instances can lead to poor UE and NW performance and degraded KPIs.

Observation 2 : There are use cases when one SIM instance is in INACTIVE state, and other SIM instance is in IDLE/INACTIVE state, uninformed network switching between SIM instances can lead to poor UE and NW performance and degraded KPIs.

Proposal 1 : A graceful switching mechanism has to be defined and standardized to ensure that both the MUSIM UE and the network are in sync about their respective RRC states, during the course of MUSIM suspension and resumption.

Observation 3: To address MUSIM coordination issues like graceful network switching, it is beneficial for the UE to indicate its MUSIM capability to network, to help in proper coordination between UE and network for MUSIM use cases.

Observation 4 : UE and NW can apriori align on the set of attributes based on which the UE is allowed to request for graceful Network switching.

Proposal 2: UE and NW can exchange MUSIM and Network switching attribute as part of initial capability handshake

Proposal 3 : Introduce a new MUSIM specific SIB type to carry all MUSIM related information applicable to all MUSIM UEs

Observation 5 : MUSIM UE in CONNECTED state need to signal some key MUSIM suspension attributes to help in graceful network switching.

Proposal 4 : MUSIM UE in CONNECTED state, can reuse the UE ASSISTANCE INFORMATION signalling approach to signal the MUSIM Suspension attributes to the network to help in graceful network switching.

Observation 6 : The suspension duration requested by MUSIM UE can be used to determine the current RRC state of the SIM that is requesting for the suspension.

Proposal 5 : A short suspension duration for a given SIM instance implies, that the SIM instance prefers to hold the CONNECTED mode context.

Proposal 6 : A long suspension duration for a given SIM instance implies, that the SIM instance prefers to exit the current CONNECTED mode context.

Observation 7 : The long suspension and follow up RRC Connection release signaling should not negatively impact the call setup duration on the other SIM instance.

Proposal 7 : MUSIM UE triggering long suspension and having an intent to go out of RRC CONNECTED mode should be expected to wait for only for a reasonable time for receiving the RRC RELEASE message, after which once the timer expires, it can autonomously switch to the other SIM instance.

Observation 8 : Post suspension, the resumption of the suspended MUSIM instance is dependent on the current suspended UE SIM instance state (INACTIVE or IDLE).

Proposal 8 : A MUSIM UE SIM instance which is suspended and is in RRC INACTIVE state can use the RRC RESUME REQUEST with cause rnaUpdate as a means to ensure RRC state synchronization with its network.

Proposal 9 : A MUSIM UE SIM instance which is suspended and is in RRC IDLE state can use the legacy RRC CONNECTION SETUP procedures to trigger any future state transitions out of RRC IDLE.

Observation 9 : A MUSIM UE SIM instance suspended due to MUSIM cause, and in RRC INACTIVE state, when its timer T380 expires can result in ping-pong suspension/resumption scenarios, which should be avoided.

Proposal 10 : Networks can configure a higher value for T380 as part of the suspend configuration, while configuring the MUSIM UE SIM instance to RRC INACTIVE state.

# Reference

[1]  RAN2#113-e Session Chair Notes – R2-2101951 – Report on LTE legacy, Mobility, DCCA, Multi-SIM and RAN Slicing