3GPP TSG-RAN WG2 e-Meeting #113bis-e	R2-2103912

April 12th- April 20th, 2021

Title:	NR-NTN: Multi-TAI Broadcast

Source: 	Fraunhofer IIS, Fraunhofer HHI

Agenda Item:	8.10.3.1 - Earth fixed/moving beams related issues

TDoc Type:	Discussion

Document for:	Decision

# Introduction

In the last meeting, the following agreements regarding tracking areas have been made in order to support a soft tracking area update (TAU) approach for moving beams.

Agreements:

- In NTN, the UE determines the TA based on the broadcast information (the use of other information is not excluded). In any case RAN2 will not go in a different direction than other groups

- In NTN, the network may broadcast more than one TACs per PLMN in a cell, which is to up to network implementation.

This document shall motivate for a change of the current specification regarding the SIB1.

# Discussion

In 5G NR, the system information block #1 (SIB1) is specified in TS 38.331 V16.4.1. It carries relevant cell-specific system information that is required by the UE to access the cell. The so-called cell access related information (CellAccessRelatedInfo) contains the public land mobile network (PLMN) identity info list (plmn-IdentityInfoList), which contains a set of PLMN-IdentityInfo elements. The PLMN identity info further contains a PLMN-IdentityList and an optional tracking area code (TAC). The tracking area identifier (TAI) is made up by the PLMN and the TAC.

The detailed SIB1 message and the relevant information elements are captured in the annex (chapter 5) of this document. A coarse structure is shown in the following:

SIB1

CellAccessRelatedInfo

plmn-IdentityInfoList

PLMNIdentityInfo 1

plmn-IdentityList

plmn identity 1

plmn identity 2

...

plmn identity maxPLMN

trackingAreaCode

PLMNIdentityInfo 2

plmn-IdentityList

plmn identity 1

plmn identity 2

...

plmn identity maxPLMN

trackingAreaCode

...

PLMNIdentityInfo maxPLMN

plmn-IdentityList

plmn identity 1

plmn identity 2

...

plmn identity maxPLMN

trackingAreaCode

As it can be seen above, it is possible to define up to maxPLMN (12) PLMN identities, but there is only one trackingAreaCode per PLMNIdentityInfo element.

Observation: SIB1 currently only supports multiple PLMNs, but not multiple TACs per PLMN.

Since it currently does not seem to be possible to broadcast multiple TACs per PLMN, we propose to add this feature to the SIB1.

Proposal: In NTN, the network shall be able to broadcast more than one TAC per PLMN in the SIB1.

# Summary

The following observations and proposals are described in this document:

Observation: SIB1 currently only supports multiple PLMNs, but not multiple TACs per PLMN.

Proposal: In NTN, the network shall be able to broadcast more than one TAC per PLMN in the SIB1.

# References

- 3PGG TS 38.331 V16.4.1 (2021-03): 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; Radio Resource Control (RRC) protocol specification (Release 16)

# Annex: Relevant message and information elements

Extracted from [1]:

SIB1 message -- ASN1START -- TAG-SIB1-START SIB1 ::= SEQUENCE { cellSelectionInfo SEQUENCE { q-RxLevMin Q-RxLevMin, q-RxLevMinOffset INTEGER (1..8) OPTIONAL, -- Need S q-RxLevMinSUL Q-RxLevMin OPTIONAL, -- Need R q-QualMin Q-QualMin OPTIONAL, -- Need S q-QualMinOffset INTEGER (1..8) OPTIONAL -- Need S } OPTIONAL, -- Cond Standalone cellAccessRelatedInfo CellAccessRelatedInfo, connEstFailureControl ConnEstFailureControl OPTIONAL, -- Need R si-SchedulingInfo SI-SchedulingInfo OPTIONAL, -- Need R servingCellConfigCommon ServingCellConfigCommonSIB OPTIONAL, -- Need R ims-EmergencySupport ENUMERATED {true} OPTIONAL, -- Need R eCallOverIMS-Support ENUMERATED {true} OPTIONAL, -- Need R ue-TimersAndConstants UE-TimersAndConstants OPTIONAL, -- Need R uac-BarringInfo SEQUENCE { uac-BarringForCommon UAC-BarringPerCatList OPTIONAL, -- Need S uac-BarringPerPLMN-List UAC-BarringPerPLMN-List OPTIONAL, -- Need S uac-BarringInfoSetList UAC-BarringInfoSetList, uac-AccessCategory1-SelectionAssistanceInfo CHOICE { plmnCommon UAC-AccessCategory1-SelectionAssistanceInfo, individualPLMNList SEQUENCE (SIZE (2..maxPLMN)) OF UAC-AccessCategory1-SelectionAssistanceInfo } OPTIONAL -- Need S } OPTIONAL, -- Need R useFullResumeID ENUMERATED {true} OPTIONAL, -- Need R lateNonCriticalExtension OCTET STRING OPTIONAL, nonCriticalExtension SIB1-v1610-IEs OPTIONAL } SIB1-v1610-IEs ::= SEQUENCE { idleModeMeasurementsEUTRA-r16 ENUMERATED{true} OPTIONAL, -- Need R idleModeMeasurementsNR-r16 ENUMERATED{true} OPTIONAL, -- Need R posSI-SchedulingInfo-r16 PosSI-SchedulingInfo-r16 OPTIONAL, -- Need R nonCriticalExtension SIB1-v1630-IEs OPTIONAL } SIB1-v1630-IEs ::= SEQUENCE { uac-BarringInfo-v1630 SEQUENCE { uac-AC1-SelectAssistInfo-r16 SEQUENCE (SIZE (2..maxPLMN)) OF UAC-AC1-SelectAssistInfo-r16 } OPTIONAL, -- Need R nonCriticalExtension SEQUENCE {} OPTIONAL } UAC-AccessCategory1-SelectionAssistanceInfo ::= ENUMERATED {a, b, c} UAC-AC1-SelectAssistInfo-r16 ::= ENUMERATED {a, b, c, notConfigured} -- TAG-SIB1-STOP -- ASN1STOP

CellAccessRelatedInfo information element -- ASN1START -- TAG-CELLACCESSRELATEDINFO-START CellAccessRelatedInfo ::= SEQUENCE { plmn-IdentityList PLMN-IdentityInfoList, cellReservedForOtherUse ENUMERATED {true} OPTIONAL, -- Need R ..., [[ cellReservedForFutureUse-r16 ENUMERATED {true} OPTIONAL, -- Need R npn-IdentityInfoList-r16 NPN-IdentityInfoList-r16 OPTIONAL -- Need R ]] } -- TAG-CELLACCESSRELATEDINFO-STOP -- ASN1STOP

PLMN-IdentityInfoList information element -- ASN1START -- TAG-PLMN-IDENTITYINFOLIST-START PLMN-IdentityInfoList ::= SEQUENCE (SIZE (1..maxPLMN)) OF PLMN-IdentityInfo PLMN-IdentityInfo ::= SEQUENCE { plmn-IdentityList SEQUENCE (SIZE (1..maxPLMN)) OF PLMN-Identity, trackingAreaCode TrackingAreaCode OPTIONAL, -- Need R ranac RAN-AreaCode OPTIONAL, -- Need R cellIdentity CellIdentity, cellReservedForOperatorUse ENUMERATED {reserved, notReserved}, ..., [[ iab-Support-r16 ENUMERATED {true} OPTIONAL -- Need S ]] } -- TAG-PLMN-IDENTITYINFOLIST-STOP -- ASN1STOP

maxPLMN INTEGER ::= 12 -- Maximum number of PLMNs broadcast and reported by UE at establisghment

TrackingAreaCode information element -- ASN1START -- TAG-TRACKINGAREACODE-START TrackingAreaCode ::= BIT STRING (SIZE (24)) -- TAG-TRACKINGAREACODE-STOP -- ASN1STOP

PLMN-Identity information element -- ASN1START -- TAG-PLMN-IDENTITY-START PLMN-Identity ::= SEQUENCE { mcc MCC OPTIONAL, -- Cond MCC mnc MNC } MCC ::= SEQUENCE (SIZE (3)) OF MCC-MNC-Digit MNC ::= SEQUENCE (SIZE (2..3)) OF MCC-MNC-Digit MCC-MNC-Digit ::= INTEGER (0..9) -- TAG-PLMN-IDENTITY-STOP -- ASN1STOP