3GPP TSG-RAN WG2 #113bis-e	R2-2103914

Electronic meeting, 2021-04-12 – 2021-04-20

Agenda Item:	8.11.2

Source:	Ericsson

Title:	Reducing Latency for Positioning procedures

Document for:	Discussion, Decision

# 1	Introduction

One of the agenda Items for release 17 is to reduce latency for positioning procedures. In this paper, we discuss how to progress on this Agenda as SA2 has also been working in this area. Hence, we need to see what SA2has been discussing and how we can collaborate with them in order to avoid any duplicate work.

    - Latency reduction related to the reporting and request of positioning assistance data (e.g., via location scheduling in advance of the time of when the location is needed) [RAN2]

SA2 based upon this work has sent an LS to RAN2(S2-2102048) which will be treated in this meeting. RAN2 needs to discuss that and discuss on the way forward on this agenda.

# 2	Discussion

Currently in 23.273 deferred MT-LR has been specified. For MO-LR, it can be assumed that deferred procedure is not needed as UE can request any time when it wants the positioning assistance data or wants the location to be estimated. However, for MO-LR procedure, in order to ensure that resources are allocated in advance or that assistance data is provided in advance, it is essential that UE also provides at what time “T” it wants positioning to be estimated. If such info is available to the UE, then UE should basically provide the information to the Network in order to execute deferred MO-LR procedure. SA2 has currently been discussing this mechanism as below. Another aspect that SA2 is discussing is to allow multiple LPP messages encapsulated in one LCS message.

Based upon the LS, SA2 has endorsed a CR [2] which allows the UE to report to LMF the wanted time “T” for when positioning estimation as part of e.g. MO-LR procedure should be performed. The information is provided to LMF via LCS message is shown below. Further, it is possible to have multiple LPP message embedded in one LCS message. The MO-LR part of CR is shown below.

## 6.2	5GC-MO-LR Procedure

Figure 6.2-1 illustrates the general network positioning requested by the UE to the serving PLMN for obtaining the location related information of itself.

Figure 6.2-1: 5GC-MO-LR Procedure

1)	If the UE is in CM-IDLE state, UE instigates the UE triggered Service Request as defined in clause 4.2.3.2 of TS 23.502 [19] in order to establish a signalling connection with the AMF.

2)	The UE sends an MO-LR Request message included in a UL NAS TRANSPORT message. The MO-LR Request may optionally include up to three LPP positioning message. Different types of location services can be requested: location estimate of the UE, location estimate of the UE to be sent to an LCS client or AF, or location assistance data. If the UE is requesting its own location or that its own location be sent to an LCS client or AF, this message carries LCS requested QoS information (e.g. accuracy, response time, LCS QoS Class), the requested maximum age of location, the requested type of location (e.g. "current location", "current or last known location") and, optionally for a current location, a scheduled location time. If the UE is requesting that its location be sent to an LCS client, the message shall include the identity of the LCS client or the AF, and may include the address of the GMLC through which the LCS client or AF (via NEF) should be accessed. In addition, a Service Identity indicates which MO-LR service of the LCS Client is requested by the UE may be included. The message also may include a pseudonym indicator to indicate a pseudonym should be assigned by the network and transferred to the LCS Client as the UE's identity. If the UE is instead requesting location assistance data, an embedded LPP message specifies the type of assistance data and the positioning method for which the assistance data applies.

For an LCS 5GC-MO-LR requesting location transfer to an LCS Client or AF, the AMF shall assign a GMLC address, i.e. VGMLC address, which is stored in the AMF. If a VGMLC address is not available, the AMF may reject the location request. The AMF verifies the subscription profile of the UE and decides if the requested service is allowed or not.

3)	The AMF selects an LMF as described in clause 5.1.

4)	The AMF invokes the Nlmf_Location_DetermineLocation service operation towards the LMF. The service operation includes an LCS Correlation identifier, the serving cell identity, the client type, an indication whether a location estimate, or location assistance data is requested and any embedded LPP message(s) in the MO-LR Request. If the UE's location is requested, the service request may include an indication if UE supports LPP, the requested QoS, Supported GAD shapes and any scheduled location time. If location assistance data is requested, an embedded LPP message will convey the requested types of location assistance data. If any of the procedures in clause 6.11.1 or 6.11.2 are used the service operation includes the AMF identity. Once an AMF has selected an LMF it must continue to use that LMF for the duration of the session.

5)	If the UE is requesting its own location, the actions described in clause 6.11 are performed together with the actions described in step 12 in clause 6.1.2 if a scheduled location time is present. If the UE is instead requesting location assistance data, the LMF transfers this data to the UE as described in clause 6.11.1. The LMF determines the exact location assistance data to transfer according to the type of data specified by the UE, the UE location capabilities and the current cell.

6)	When a location estimate best satisfying the requested QoS has been obtained or when the requested location assistance data has been transferred to the UE, the LMF returns the Nlmf_Location_DetermineLocation Response towards the AMF. The service operation includes the LCS Correlation identifier, the location estimate, if this was obtained, its age and accuracy and may include information about the positioning method.

If a location estimate was not successfully obtained, or if the requested location assistance data could not be transferred successfully to the UE, a failure cause is included in the service operation.

7)	If the location estimate was successfully obtained, the AMF invokes the Ngmlc_Location_LocationUpdate service operation towards to the VGMLC assigned in the step 2. The service operation carries the identity of the UE, the event causing the location estimate (5GC-MO-LR) and the location estimate, its age, obtained accuracy indication and the LCS QoS Class requested by the target UE. In addition, the service operation may include the pseudonym indicator, the identity of the LCS Client, AF ID, the GMLC address and the Service Identity specified by the UE, if available.

8)	If the UE did not request transfer of its location to an LCS Client or AF in step 2, steps 8 to 11 are skipped. If the VGMLC is same NF instance as HGMLC this step is skipped. Otherwise VGMLC invokes the Ngmlc_Location_LocationUpdate service operation towards to the HGMLC (the VGMLC may query the UDM of the UE to obtain the address of the HGMLC) including the information received from the VGMLC.

9a)	If the pseudonym indicator is included in the MO-LR Location Information, the HGMLC assigns a pseudonym to the UE. If the identified LCS Client is not accessible by the HGMLC, step 9a and step 10a are skipped. Otherwise the GMLC transfers the location information to the LCS client, carrying the identity or the pseudonym of the UE, the event causing the location estimate (5GC-MO-LR), the Service Identity, if available, and the location estimate and its age, in accordance with the LCS QoS Class requested by the target UE. If the UE requested LCS QoS class was Assured, GMLC sends the result to the LCS client only if the result has been indicated to fulfil the requested accuracy. If the UE requested LCS QoS class was Best Effort, GMLC sends whatever result it received to the LCS client with an appropriate indication if the requested accuracy was not met.

9b-1)	If the AF ID is included in step 1, the HGMLC assigns the NEF address based on local configuration or via NRF and invokes Ngmlc_Location_LocationUpdateNotify service request towards the NEF, carrying the AF ID. The location information parameters sent within this service operation are same as the step 9a except that no pseudonym is included.

9b-2)	If the identified AF is not accessible by the NEF, step 9b-2 and step 10b-1 are skipped. Otherwise, the NEF transfer the location information to the identified AF by invoking the Nnef_Location_LocationUpdateNotify service.

10a)	If the LCS Client does not support MO-LR (for temporary or permanent reasons) or cannot handle the location estimate of the UE, e.g. the LCS Client does not know the Service Identity, or the UE does not register to the LCS Client, the LCS Client has no corresponding data of the UE, the LCS Client shall return the Location Information ack message to the HGMLC with a suitable error cause. Otherwise, the LCS Client handles the location estimate according to the Service Identity, sends the GMLC or the HGMLC the Location Information ack message signalling that the location estimate of the UE has been handled successfully.

10b-1)	If the AF cannot handle the location estimate of the UE, e.g. the UE does not register to the AF, the AF has no corresponding data of the UE, the AF shall respond to the Nnef_Location_LocationUpdateNotify service request with a suitable error cause. Otherwise, the AF handles the location estimate according to the Service Identity and respond to the Nnef_Location_LocationUpdateNotify service request indicating that the location estimate of the UE has been handled successfully.

10b-2)	The NEF sends a Ngmlc_Location_LocationUpdateNotify service response towards the HGMLC with the outcome of the operation.

11)	If the VGMLC is same NF instance as HGMLC this step is skipped. If the identified LCS Client or AF is not accessible, the HGMLC sends a Ngmlc_Location_LocationUpdate service response to VGMLC with an appropriate error cause. Otherwise, the response shall include an acknowledgement. The message shall specify whether the location estimate of the UE has been handled successfully by the identified LCS Client or AF, and if not, the corresponding error cause obtained in step 10. In addition, the HGMLC may record charging information both for the UE and inter-working revenues charges.

12)	If the VGMLC receives the MO-LR Location Information Acknowledgement from the HGMLC, if the identified LCS Client or AF is not accessible, the VGMLC sends a Ngmlc_Location_LocationUpdate service response to AMF with an appropriate error cause. Otherwise, the response shall include an acknowledgement. The message shall specify whether the location estimate of the UE has been handled successfully by the identified LCS Client or AF, and if not, the corresponding error cause obtained in step 9 or 10. In addition, the VGMLC may record charging information both for the UE and inter-working revenue charges.

If the VGMLC receives Ngmlc_Location_LocationUpdate Request from the AMF and it is not required to send to any LCS Client or AF, the VGMLC may record charging information for the UE and response the Ngmlc_Location_LocationUpdate Request to the AMF.

13)	The AMF sends an MO-LR Response message included in a DL NAS TRANSPORT message. The response carries any location estimate requested by the UE including the indication received from E-SMLC whether the obtained location estimate satisfies the requested accuracy or not, or an indicator whether a location estimate was successfully transferred to the identified LCS client or AF. If the location estimate was successfully transferred to the identified LCS Client or AF, the MO-LR Response message shall specify whether the location estimate of the UE has been handled successfully by the identified LCS Client or AF, and if not, the corresponding error cause obtained in step 13. In addition, AMF may record charging information.

It is hence proposed that RAN2 reviews the SA2 CRs and discuss what aspect needs to be further done in RAN2 in order to avoid any duplicate work.

SA2 as part of the 5G_eLCS_ph2 WID (SP-200082) includes support of “very low latency” and is already providing the needed support.

RAN2 needs to review the SA2 work to avoid any duplication work for latency reduction.

Further, the other aspect that RAN2 may need to discuss is how to utilize this information that UE or LCS Client wants positioning to take place at scheduled time(s) T for MT-LR, deferred MT-LR periodic event and MO-LR procedures. LMF based on information in location request obtained from UE or external client decides the timing for positioning measurements and determination. One way to utilize this information would be to feed time information to RAN node which then can take the inputs for scheduling, providing UL grants. However, just the timing input may not be adequate for gNB as the important aspects of the scheduler and grant allocator is to allocate the resources in order to meet QoS. Hence, LMF should also provide QoS of positioning in terms of latency /accuracy or generic QoS for Positioning which can be useful for gNB to prioritize the resources among several users. Considering an example where multiple UEs are supposed to provide their measurement result at the same time; then the QoS need can enable the gNB to prioritize the resource in case of resource scarcity.

RAN2 to discuss how the time T can be utilized.

Positioning QoS information is provided to gNB for resource allocation prioritizations

# Conclusion

In the previous sections we made the following observations:

Observation 1	SA2 as part of the 5G_eLCS_ph2 WID (SP-200082) includes support of “very low latency” and is already providing the needed support.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 needs to review the SA2 work to avoid any duplication work for latency reduction.

Proposal 2	RAN2 to discuss how the time T can be utilized.

Proposal 3	Positioning QoS information is provided to gNB for resource allocation prioritizations

# References

RP-210903 , “Revised WID on NR Positioning Enhancements“, March 16 - 26, 2021

S2-2102047, “Addition of a Scheduled Location Time“