3GPP TSG-RAN WG2 #113bise	R2-2103927

Electronic meeting, April 12th – 20th 2021

Agenda Item:	9.1.3

Source: 	Ericsson

Title:	Comparing solution for NB-IoT paging carrier selection

Document for: 	Discussion, Decision

# 1	Introduction

In Rel-17 one of the objectives in the WID is [WID’s T-doc]:

- Introduce support for NB-IoT carrier selection based on the coverage level, and associated carrier specific configuration (e.g. maximum repetitions UL/DL, DRX configurations, etc.). [NB-IoT] [RAN2, RAN3]

In RAN2-111e the below agreements were made:

Agreements

- Paging carrier selection Improvements based on CE level is considered

- Paging carrier selection Improvements based on DRX cycle may be considered

  - whether DRX cycle is considered as part of CE level (Rmax) or can be also considered separately

- Enhancements for NPRACH Carrier selection carrier may be considered

- Paging carrier selection Improvements solely based on WUS or GWUS is not considered

- FFS service based

And in RAN 113-e

- Paging carrier selection Improvements based on CE level is considered - Paging carrier selection Improvements based on DRX cycle may be considered - whether DRX cycle is considered as part of CE level (Rmax) or can be also considered separately - Paging carrier selection Improvements solely based on WUS or GWUS is not considered - FFS service based - Select between one of the options: - Option 1: UE selects a paging carrier based on a rule configured by the network - Option 2: NW configures a specific paging carrier Working assumption: For both options, when coverage changes, mechanism that requires UE to report the update of coverage is not introduced.

In this contribution we outline a solution on how Paging carrier selection improvements based on coverage level could be achieved.

# 2	Discussion

## 2.1 Selection of Solution

Based upon the email discussion; there are 2 Options on how paging carrier can be allocated/selected.

- Select between one of the options:

    - Option 1: UE selects a paging carrier based on a rule configured by the network

    - Option 2: NW configures a specific paging carrier

Below two drawbacks of Option 2 have been mentioned:

###### 1         How the eNB (efficiently) performs load balancing/UE distribution/UE redistribution.

Currently the UE distribution is based upon below formula where weights are preconfigured by the NW and UE selects the carrier based upon UE_ID.

floor(UE_ID/(N*Ns)) mod W < W(0) + W(1) + … + W(n)

In the eNB based solution, basically the same formula can be used but now by eNB instead of by each UE. However, this is a NW based implementation and no standard impact is required.

Considering there are 6 paging carriers; eNB may categorize these carriers into 2 groups (group 1 and group 2): eNB would classify whether the UE belongs to carrier from group 1 or group 2 based upon UE_ID.

a) UE ID matching to a carrier in group 1: eNB will further assess and configure whether this UE should belong to carrier 1, carrier 2 or carrier 3.

b) UE ID matching to a carrier in group 2: eNB will further assess and configure whether this UE should belong to carrier 4, carrier 5 or carrier 6.

In the first step the number of groups can help to perform load balancing. Then further classification (mapping to each carrier) can be done by individual UE needs which will also enable load balancing. In each group, there can be carriers which are based upon below characteristics for example;

a) Power Boosting carrier

b) Non-Power boosting carrier

c) Carrier reserved for UEs supporting short DRX Cycle & Coverage level with Rmax < X

eNB based carrier assignment would be simple NW implementation and no standard impacts. Besides the flexibility that it gives would outweigh any other solution.

Further eNB (NW) has other observability such as UE paging performance, paging success rate KPIs per carrier, UL RSSI, quality report of different carriers etc; eNB can utilize this information also for load balancing.

NW based tools for paging carrier performance monitoring can be also used for load balancing. These aspects are also NW implementation and has no standard impacts.

###### 2            More complicated changes for S1 interface

This drawback for Option 2 does not hold as per the following explanation. There is already a paging container that is sent from eNB to MME. The only change that would be needed is to update this with the carrier information. Further, for Option 1 also, the update would be needed; here the update would at least be to store the UE coverage level parameter.

Both Options would need changes in S1 and these changes are normal (work as usual changes).

NW configures a specific paging carrier.

## 2.2 Paging Carrier

Multicarrier operation in NB-IoT has been supported since Rel-14. Anchor carrier provides the NPSS and NSSS whereas both anchor and non-anchor can be used for paging and random access. Anchor carrier may be power boosted compared to non-anchor carrier and it also carries synchronization signals which are to be used by UEs also camped on a non-anchor carrier. Because of power boosting the coverage of such power boosted carrier as such can be larger than the non-power boosted carrier.

From 36.304:

If P-RNTI is monitored on NPDCCH and the UE supports paging on a non-anchor carrier, and if paging configuration for non-anchor carrier is provided in system information, then the paging carrier is determined by the paging carrier with smallest index n (0 ≤ n ≤ Nn-1) fulfilling the following equation:

floor(UE_ID/(N*Ns)) mod W < W(0) + W(1) + … + W(n)

Thus, paging carrier as such is deterministic from NW and UE perspective as it involves static attributes such as UE_ID and paging carrier weights.

For coverage-based paging; it is possible to use the above formula where weight is based upon Rmax (coverage). The UE may select the paging carrier which is optimal based upon measurement. However, we potentially see a problem where UE selects the carrier.

Considering a scenario as above; where we have 4 carriers (One anchor carrier and 3 non-anchor carriers); Anchor carrier and one of the non-anchor carrier #2 are power boosted and two of the other non-anchor carriers are non-power boosted. However, assuming the Rmax value is same. Because of the power boost the number of repetitions needed in deep coverage is considered similar to Rmax to serve a UE in cell border of non-boosted carrier.

In such scenario where 4 carriers have same Rmax; a UE in one of the non-power boosted non-anchor carriers may select also the power boosted carrier; which is not the main intention. From NW perspective, the resources in anchor carrier or power boosted carrier are to be used by UE in deep coverage and UE in normal coverage may use resources from non-anchor carrier. This segregation would enable UEs in deep coverage to monitor the carrier which would basically provide similar performance in terms of latency as compared to UEs in normal coverage.

For an NB-IoT cell with a carrier being power-boosted (by for example 6dB) it is preferred to let UEs in good coverage monitor paging on the non-power boosted carriers and the poor coverage UEs on a power boosted carrier. This coverage level-based paging solution would save both UE power and spectrum resources (as less radio resources are needed to page the poor coverage UEs and one could for example multiplex paging to multiple UEs on the non-anchor carrier which could be more efficient).

For Paging Carrier selection, NW can configure UEs in good coverage in non-power boosted carrier(s) whereas UEs in poor coverage in a power boosted carrier.

In order to ensure, UE does not select wrong carrier; i.e normal UE coverage does not select power boosted carrier ; eNB should assign the paging carrier to the UE.

eNB based carrier assignment would be needed, else UE may select wrong carrier just based upon UE_ID and Rmax attributes.

One approach is to follow the legacy principle of paging, i.e MME pages the UE in the last known location first, and introduce enhancements for what paging carrier the UE uses. The solution is outlined as below in a sequential order:

- The UE monitors for paging in accordance with the pre-Rel-17 mechanism (based on UE_ID) as default behavior.

- The eNB assigns a new paging carrier different from the one based on UE_ID to the UE in connected mode or during connection establishment, whose attributes (e.g. Rmax) can be provided in a dedicated message and/or broadcasted signaling.

- When the UE is released to idle mode the eNB via paging container informs the MME about the assigned paging carrier and the UE monitors paging using the assigned carrier when it camps on the same cell where it was released and experiences similar coverage conditions.

- If the UE moves away from the cell or the coverage situation/condition changes in the same cell, the UE monitors paging based on what is signaled by the network to avoid any mismatch.

The eNB may for example signal either in dedicated signalling (as part of assigning the dedicated paging carrier) or using broadcast (SIB22-NB) what the UE should do if it moves away from the cell where it was assigned the new paging carrier or if its coverage condition changed significantly with respect to the assigned paging carrier so it would no longer be possible to receive paging on that carrier. Thus, one item to standardize is where the UE should monitor paging in such cases. The UE could be informed to monitor using e.g., one of the alternatives below (also seen in the example ASN.1 IE fallBackPagingCarrier-r17 further below):

- the anchor carrier or

- the legacy procedure based on UE_ID or

- the assigned paging carrier by dedicated signalling

Another item to standardize is how the UE should detect the intra-cell coverage condition change and thus if the assigned paging carrier is no longer good enough and needs to be changed (for example according to one of the above three alternatives). How this should be solved needs to be evaluated further but it could for example be based on a change in NRSRP or an estimated NPDCCH BLER using paging Rmax going above a certain percentage threshold, e.g. X%. The latter could be based on same/similar criteria as already used for the Msg3 CQI reporting.

As long as the UE is camping on the cell where it was assigned a paging carrier and its coverage level has not changed or is better, it monitors paging using the assigned paging carrier provided before it was released to idle mode.

SystemInformationBlockType22-NB information element

-- ASN1START

SystemInformationBlockType22-NB-r14 ::=	SEQUENCE {

dl-ConfigList-r14					DL-ConfigCommonList-NB-r14	OPTIONAL,	-- Need OR

ul-ConfigList-r14					UL-ConfigCommonList-NB-r14	OPTIONAL,	-- Need OR

pagingWeightAnchor-r14				PagingWeight-NB-r14			OPTIONAL,	-- Cond pcch-config

nprach-ProbabilityAnchorList-r14	NPRACH-ProbabilityAnchorList-NB-r14	OPTIONAL,	-- Cond nprach-config

lateNonCriticalExtension			OCTET STRING					OPTIONAL,

...,

[[	mixedOperationModeConfig-r15	SEQUENCE {

dl-ConfigListMixed-r15			DL-ConfigCommonList-NB-r14	OPTIONAL,	-- Cond dl-ConfigList

ul-ConfigListMixed-r15			UL-ConfigCommonList-NB-r14	OPTIONAL,	-- Cond ul-ConfigList

pagingDistribution-r15			ENUMERATED {true}			OPTIONAL,	-- Need OR

nprach-Distribution-r15			ENUMERATED {true}			OPTIONAL	-- Need OR

}																OPTIONAL,	-- Need OR

ul-ConfigList-r15				UL-ConfigCommonListTDD-NB-r15	OPTIONAL	-- Cond TDD

]],

[[

fallBackPagingCarrier-r17		ENUMERATED {anchor, ue_id, dedicated} OPTIONAL	-- Need OR

]]

}

DL-ConfigCommonList-NB-r14 ::=		SEQUENCE (SIZE (1.. maxNonAnchorCarriers-NB-r14)) OF

DL-ConfigCommon-NB-r14

…

Dedicated RRC configuration is introduced to allow the eNB to assign a paging carrier to a UE other than that selected based on UE_ID.

Dedicated and/or broadcast based RRC configuration is introduced for a UE to use a paging carrier other than the dedicated paging carrier in case its coverage level/condition deteriorates within the cell.

Dedicated and/or broadcast based RRC configuration is introduced for a UE to use a paging carrier other than the dedicated paging carrier in case a new cell is selected.

## 2.3 NPRACH Carrier

From the feedback provided by the deployment/field with regards to random access carrier assignment, it is considered inflexible mainly because of the two reasons below:

- It is not possible to configure an NPRACH resource for an enhanced coverage level without including the CE levels below, e.g., CE 0 must be included if it were to be configured only for CE 1 and/or CE 2.

- From current MAC specification, one can configure selection probability for anchor carrier but not for non-anchor carrier

when multiple carriers provide PRACH resources for the same enhanced coverage level, the UE will randomly select one of them using the following selection probabilities: - the selection probability of the anchor carrier PRACH resource for the given enhanced coverage level, nprach-ProbabilityAnchor, is given by the corresponding entry in nprach-ProbabilityAnchorList - the selection probability is equal for all non-anchor carrier PRACH resources and the probability of selecting one PRACH resource on a given non-anchor carrier is (1- nprach-ProbabilityAnchor)/(number of non-anchor NPRACH resources)

We see multiple advantages of allowing flexibility in carrier assignment for random access.

- NPRACH resource partitioning is simplified or even not needed for specific carriers.

- As NPRACH resource is not partitioned for one non-anchor carrier, it decreases the possibility of collision between NPRACH and NPUSCH, and also decreases the possibility of NPUSCH tone reduction due to collision between NPRACH and NPUSCH.

- If a certain CE level has larger proportion of RACH request than other; it gives flexibility to assign more resource/carriers dedicated to that CE level.

- The peak NPRACH processing load can be reduced if different carriers are assigned to different CE levels, this is helpful for capacity.

- Potential NPRACH resource overlap from neighbor cells’ non-anchor carrier can be reduced, false preambles, i.e., selected by UEs that estimate wrong CE level, caused by NPRACH interference can be reduced especially for CE0 (the symbol group hopping pattern difference with PCI is only among repetitions).

Multiple benefits are seen if flexibility is allowed in assigning random access carrier.

Provide selection probability among the non-anchor NPRACH carriers

Provide flexibility to assign NPRACH carrier to any CE level.

# Conclusion

In the previous sections we made the following observations:

Observation 1	eNB based carrier assignment would be simple NW implementation and no standard impacts. Besides the flexibility that it gives would outweigh any other solution.

Observation 2	NW based tools for paging carrier performance monitoring can be also used for load balancing. These aspects are also NW implementation and has no standard impacts.

Observation 3	Both Options would need changes in S1 and these changes are normal (work as usual changes).

Observation 4	For Paging Carrier selection, NW can configure UEs in good coverage in non-power boosted carrier(s) whereas UEs in poor coverage in a power boosted carrier.

Observation 5	eNB based carrier assignment would be needed, else UE may select wrong carrier just based upon UE_ID and Rmax attributes.

Observation 6	Multiple benefits are seen if flexibility is allowed in assigning random access carrier.

Based on the discussion in the previous sections we propose the following:

Proposal 1	NW configures a specific paging carrier.

Proposal 2	Dedicated RRC configuration is introduced to allow the eNB to assign a paging carrier to a UE other than that selected based on UE_ID.

Proposal 3	Dedicated and/or broadcast based RRC configuration is introduced for a UE to use a paging carrier other than the dedicated paging carrier in case its coverage level/condition deteriorates within the cell.

Proposal 4	Dedicated and/or broadcast based RRC configuration is introduced for a UE to use a paging carrier other than the dedicated paging carrier in case a new cell is selected.

Proposal 5	Provide selection probability among the non-anchor NPRACH carriers

Proposal 6	Provide flexibility to assign NPRACH carrier to any CE level.

# References

RP-201306, “WID revision: Additional enhancements for NB-IoT and LTE-MTC”, RAN #88e, Electronic Meeting, June 29th – July 3rd, 2020.